Shareware

By

Harmer Inc.

98 Taylor St.

Fredericktown, Ohio

43019

3-7-90

LIFTING.EXE

It is amazing how repetition reduces our lifting

capacity as is shown in this adaptation of formulas by NOISH.

TMU.EXE

This software has, in the past proven very useful to me.

By selecting a series of movements from the menu, you build a

file of movements for a given task. This file contains values

for each movement. These values are converted into time to

produce a standard. This standard is not meant to be exact

however, in comparison, if all moves are entered, the

difference should be useable. Since, if all moves are

entered, there is consistency you can develop your own fudge

factor to get closer to your own company actual times.

INDTEMP.EXE

In heat treatment of steel the tempering of a

martensitic structure is given in books as the temperature

required based on one hour at temperature. This adaptation of

formula is to convert this book tempering temperature to the

temperature required for a time you supply given in seconds.

FLANGVOL.EXE

This program is used in the design of metal stampings

and or metal stamping dies. This applies to producing flanged

holes in stampings. This applies only to flanged holes where

it is desirous to pierce a hole and then flange it without

splitting. This use of formulas is for use where the hole is

produced with ironing ( The matrix hole diameter to punch

diameter clearance is less than metal thickness. ) This

software calculates the volume of material required and from

this volume estimates the diameter of the prepierced hole.

Thank You For Your Interest

In Software

By

Harmer Inc.

98 Taylor St.

Fredericktown, Ohio

43019