LAMCON - The Laminate Construction Editor

LAMCON uses the material files created by PCAT to build layered

composite sections.   These sections are stored on disk and later

submitted to other COPAN modules for various calculations.

This program was written using many of the same procedures that

were found in PCAT.   This is to provide you with a familiar

surrounding but, more importantly, to keep from having to rewrite

all that code.  You will notice that the screen formatting is the

same in that four primary windows are used to display data  and

prompts.  These four are arranged as shown below:

------------------------------

|                 |    4     |

|                 |----------|

|        3        |          |

|                 |          |

|                 |     2    |

|                 |          |

|-----------------|          |

|        1        |          |

------------------------------

The LAMCON windows shown in Hi Res

These windows generally provide different functions and are

divided up as follows:

Window 1

It is here that the ever familiar Main> prompt will appear when

LAMCON is first booted.  The cursor will appear as a solid, non

blinking diamond.

Window 1 is where all of the keyword commands will be entered.

Most of the commands will be available via the function keys F1-

F10.   For convenience, the 25th row will show which commands are

assigned to which function keys.   By pressing the appropriate

function key, the  command will be automatically entered. A

carriage return <CR> is added to each so that immediately after

the command is displayed, it is entered.  This way, all it takes

is a single key to kick-in most routines.

Window 2

The second window in LAMCON is used to display the laminate as it

is being created.   This way, the exact composition can be

monitored while in the create or edit mode.   The window  will

display four columns of data during each session.  These columns

consist of:   Command--Reference--Quantity---Angle

More about these later.

#

Window 3

This area is where most of the larger prompts will be displayed

as well as the material catalogs which are read in from disk.

The larger prompts are those which use a few lines of explanation

such as with the definition of a valid filename.  Another example

is the configuration file routine which collect names of PCAT

MATerial files for assignment to the function keys.  This sounds

a bit confusing now but stick with it and it will soon be clear

as resin.

During create and edit sessions, it will be necessary to

reference what each ply consists of.   The best way to do this is

to select materials from the files that you have spent precious

time creating.   What LAMCON will do is display the material file

of your choice and from this you will be able to select the

desired ply or core.

Window 4

Number four is a status window which was not used in PCAT.   The

status window indicates two different things during the program

operation.  One is the CNF status, the other is the Laminate

Status.

Operational Procedures

Operating LAMCON is something that anyone can do.    On this we

will bet almost anything.   Because we realized that LAMCON  and

PCAT could be considered extraneous due to all the fine editors

on the market,  we had to put in some overtime to make sure that

these programs made file creation second to sex.   Well, maybe

not that good but we tried.

#

Screen Saver

Should you be called away from the monitor just after you

complete the LAMCON boot-up, you may return and find that LAMCON

has vanished.  However, don't panic--everything is fine.

If no activity occurs for a duration of about a  minute,  LAMCON

will assume that you got bored and went home.  At that time, the

screen will be cleared to keep your monitor in showroom

condition.

LAMCON will then "roll some dots" to let everyone know that

nothing has frozen-up.

To signal that work is ready to commence, stab any key and the

Main> will be returned.   The timer will then be reset in the

event you're called away again.  This screen saver is active only

in the Main> level.

What to do first

When the opening screen appears for the first time,  a short beep

will be heard.   The reason for this is because LAMCON attempted

to open a configuration file but was unable to because it does

not yet exist.  Your first task should be to take care of this so

that your next session will be a more silent one.

Establishing a CNF file           [SET-F]

A CNF file is a configuration file which is really an overkill on

the terminology.   All this file does is load in a list of

filenames from disk which you define.   LAMCON must get this

information from you because these is no way to know what the

filenames will be.  Now you're asking, "What files?"

The filenames which go into the CNF file are names of material

files created with  PCAT.   As you will recall from the PCAT

section of this manual, one of the concepts involved creating

multiple files for each manufacturer.  This way, the files remain

a more manageable length which keeps disk access time much  more

favorable.

#

By using the SET-F command, the files which you will be using on

a regular basis are then read each time you start LAMCON.  At the

appropriate  time,  LAMCON then assigns these filenames  to  the

function keys.  The purpose of this is to then allow  you  to

switch back and forth between material  catalogs  without  the

necessity of having to remember and stroke in the entire file name.

To activate the SET-F command, type in SET-F or press the

appropriate function key.   This will bring up a small narration

in window 3 along with the listing of the current settings.   If

this is the first time that the command is being used, then all F

keys will be set to nothing.   To enter the filenames,  press the

key (1-8)  which  corresponds to the location that you  wish  to

assign that file.  A suggestion  would be to assign all  cores to

F1, F3, F5 and F7  and all plies to F2, F4, F6 and F8.  This way,

one  side  of  the  keys is  for  cores,  the  other  for  plies.

Remember,  when assigning, you will need to press 1 for F1, 2 for

F2 and so on.  Do not use the function keys yet!

When all of the selections have been made (by the way,  it is not

necessary to fill all the keys) press the [Esc] key to record the

filenames  into  the  CNF file.   The save will be  made  to  the

default drive and path.   Remember to save the CNF along with the

LAMCON system file as the search for the CNF file will be made in

the default drive-path.

With this taken care of, the laminate stacking may now begin.

CREATING A LAMINATE            [CREATE]

Now the fun begins.  With all the CNF details out of the way,

LAMCON can now start to do what it was designed for:   stacking

laminates.

To begin the create sequence,  enter CREATE from the Main> or

press the function key which indicates this keyword.

Upon doing so,  window 3 will produce a narration asking for  the

name of the job.

#

Assigning a Job Name

A job name is an 8 character string which references the work

that you will be doing.  We understand that 8 characters does not

give you much room for creativity but do the best you can.  These

8  characters  must conform with the regulations  established  by

DOS.

The exception to this is that the job name may not contain a dot

(.) because a file extension will be added by LAMCON.   If a . is

included, it will be coldly ignored.

After the job name has been entered into window 1, press the <CR>

to signal your completion.

LAMCON  will  reproduce  the  job name and  then  wait  for  your

approval.   A  single keystroke of [Y] will confirm the job  name

selection.     If  you have a change of heart,  the [N] key  will

restart the routine and a new name can be entered.

File/Laminate Descriptions

Because numerous files will undoubtedly be created for a single

project, it is necessary to describe what each laminate

represents.   This line of text is written into the .PRP file so

that at any point in the future, the exact purpose of the

laminate will always be known.

Selecting a Material File

After the job name and description has been entered,  LAMCON will

ask for another name, this is the PCAT file that you wish to

select material from.   At this time, LAMCON will assign F1-F8

to the filenames that you designated with the SET-F command.

Now, all that is required is a single push of the appropriate

function key.   If there is a small specialty file out on disk,

this may be entered by typing in its full name.   You must

include the .MAT as LAMCON will only read MATerial files.

After the filename is entered, and if it exists on the default

drive, it will be read into memory.  On the other hand, if the

filename is mispelled or if it does not exist on the default

drive, LAMCON will tell you so.   Another filename will then be

required to continue the create sequence.

#

Groups, Plies and Angles

Now that the material filename has been entered, the file will be

read and immediately displayed in window  3.   Each material

will be assigned a reference number based on its location

within the file.

Window 1, the command window,  will then produce a small data

entry area which will be used to communicate all create commands.

---------------------------------------

I            Group No. n              I

I   Ply.No.---Qty.---Angle---Thick.   I

I                                     I

---------------------------------------

Create/Edit Command Window

The solid diamond cursor will indicate the position within the

window.   The header under which the cursor rests is the

information for which LAMCON is waiting.    There are four

headers, the first is the Ply/Command Header.

Command Header

The Ply/Command Header accepts two distinctly separate items,

commands and material references.  Material references are

discussed here.   To use the commands,  page ahead to the section

on #Create and Edit Commands#.

Material References

When a ply or core is desired, all that is necessary is to type

in the number associated with it.  This is found in window 3 in

the <Material Inventory>.   By keying in a single integer, LAMCON

will assume that you wish to use that material number as a group.

The material reference number cannot exceed the total number of

records which have been entered into the PCAT material file.

This you should not have to be told.   An entry of zero will be

construed by LAMCON as a core ply.  More about this later.

After the material reference has been entered and accepted by

LAMCON, the cursor will move to the next entry position.

#

Entering Cores from a Ply Inventory

When a laminate is being created,  it is common to use the ply

catalog first.   If the laminate is to be a sandwich, it is also

common that the core ply will consist only of a single entry,

unless you are a European user where multi-cored constructions

are more prominent.   Rather than have to load a new material

catalog, just to make the core ply specification,  LAMCON will

accept a core filler.

This is more of an aid for the user than for LAMCON as it

highlights the core ply.   As most refer to sandwich faces as

being above or below the core,  this filler keeps the sandwich in

its true perspective.

To enter a core filler,  a zero may be entered at the command

position.   A shortcut is to press the function key which

indicates  CORE.   This will save you one whole  keystroke.   The

cursor will move directly to the thickness position as quantity

and angle are not accepted by LAMCON.   Any thickness may be used

as this ply group will have to be replaced at sometime  in  the

future.

After the thickness is entered, the core will be added to the

laminate in a highlighted print so that you are reminded that

this is a core ply.   The cursor will then return to the  command

position and wait for the next material or command.

Ply Quantity

The second entry position is the ply quantity.   This allows

groups of the same material and orientation to be grouped

together.   There is no physical limit on this but rather a

logical one.  The entry must be an integer.

For a group containing a single ply, the <CR> may be pressed and

LAMCON will default to one.

Ply Angle

The third position accepts the angle of the ply with respect to

the longitudinal axis of the ply.   All that LAMCON will do is

write the .PRP file with the angle that you indicated.  LAMCON

does not calculate off-axis properties.

The <CR> default is 0 degrees.

#

Ply Thickness

Immediately after the material reference is entered at the

command position,  the default thickness will appear under this

header.   This is to show you what the current ply thickness is

set at according to your PCAT record.   If you do not like the

thickness that was entered in PCAT,  it can be changed when the

cursor is resting at the fourth position.

The default thickness WILL NOT be visible when the cursor is at

the thickness position.   So make sure you look at it before you

enter the angle.

After the thickness has been entered, the ply group information

will be posted to the screen in window number 2.

Getting the Full View   [FULL]

After you have entered a few plies, window 2 may not provide all

of the information you desire.  This is because, the normal

screen view provides only material numbers.   If you desire to

get a more complete view of the data that you have created, the

FULL command can be entered from  the first entry position.

Keying in the keyword FULL will produce a more complete view on a

more complete screen.   The full view will show each group in its

stacked location along with the group material name and the file

from which it was selected.

When working with laminates that contain a high number of groups,

the output speed to the display may be controlled.   There were

two reasons for this function:  One,  this was found to be

necessary when working with the 10+Mhz machines as the groups

would fly by before a normal human could get their fingers on the

[Ctrl] [S].   And two, because we didn't want to bother you

with the pause at the end of the screen.

So, to modify the speed,  use keys 1,2 and 3.  Just press one of

these while the laminate is being listed.  LAMCON will beep once

when the information is received.  [1] is the fastest, and [3] is

the slowest.  [3] is five times slower than [1] and really creeps

on the XT.  The initial setting is 1 but don't worry if you miss

the first listing.

#

After the listing is complete, press the [A] key and LAMCON will

scroll the list again.  The speed setting that was last pressed

will be used for all subsequent full screen listings unless

changed with another 1, 2 or 3.

To return to the edit or create session after all necessary

inquiries have been completed, two keys will be active to

initiate the transport.  These are the [Esc] and the <CR>.

When you return,  the four windows will be restored and the  data

will be reinstalled.  Everything will be just as you left it.

The LIST command     [LIST]

When in the process of stacking a laminate, the group total may

exceed the screen length.   This is about a 16 group laminate

which is getting to be on the beefy side.   But should this

happen, the 1st group will be shoved off the screen to make room

for the new one.   This is only fair as the first one has been

there the longest.

To reproduce the list, beginning with 1, the LIST command can be

entered.  This can also be invoked using the appropriate function

key.

As with the FULL command, the LIST function output speed may be

controlled with keys 1-3.   [1] is the fastest,   [3] is the

slowest.

The speed command is set to one to begin.   After the first set,

the speed remains at the last setting for all subsequent lists.

Inserting a Group    [INSE]

The INSERT command is your basic split and push.  It adds another

ply group to the laminate but at a specific location.

To insert a group, enter the command INSERT from the first

position.  LAMCON will then prompt for the location of the insert

to occur.   This is because before the new data can be accepted,

room must be made in the array.   The location will be specified

as  "above" which group.   As all things are  relative, this is

relative to the top of the screen.   Which means 1 is above 2 and

2 is  above 3 and so on.   This seems backwards when you think

about it but it works just fine when you look at it.

#

After you enter the location for the split to occur, LAMCON will

make room for the new group.  The four entry items will then be

requested.  All data requirements follow the instructions given

previously.

When the thickness is entered, the new listing is produced in

window 2 and the command window is readied for the next group.

If the INSERT command needs to be cancelled after it has been

entered, press <CR> when the location is being requested.

However, once a valid location is given, the new group must be

entered.  To remove it, use the DELETE function.

Changing/Replacing a Group     [REPL]

The REPLACE command can be used to overwrite an already existing

ply group.  By keying in and entering REPLACE, or using the

function key,  LAMCON will move its pointer to that ply while

holding on to the current stack position.   The command window

will then change to reflect the new pointer.   Any changes, or a

new ply, should then be entered.  When complete, the pointer will

move back to the previous stack position.

How to get rid of one      [DEL]

Occasionally, it may be necessary to undue a ply group.  This is

a snap using the DELETE function.   Just enter this keyword by

typing  it in or via the function keys, and LAMCON will ask you

which one you wish to remove.  By entering a valid group number,

the laminate will be restacked with the new being one less.

Once it's gone, it's gone.  There are no undelete keys.  We give

you that much credit.

Inverting the Laminate   [INV]

Because the laminate is considered as having a top and bottom

surface, these may be quickly changed by using the INVERT

function.   This command, when entered from the command position,

will invert the laminate causing the 1st ply to become the last

as well as switching everything in the middle.

This allows you to keep the laminate in whatever reference you

feel comfortable with.   There is no up or down for other COPAN

modules so do not worry about which group is the first.

#

Rotating the Laminate   [ROT]

If the need to rotate the laminate arises,  all ply groups may be

indexed together using the ROTATE  command.   This keyword is

entered from the first entry position and may be invoked from the

function keys.

When the prompt is displayed, enter the number of degrees  of

rotation that you require.   45 degrees will add 45,  -10  will

subtract 10.    After the number of degrees is entered, the

laminate will be relisted to display the new orientations.

Creating A Symmetrical Laminate   [SYMM]

As many laminates are symmetrical about their own mid-plane,

LAMCON allows you to quickly "duplicate" the  second  half.   To

invoke this command,  type in SYMM from the first entry position.

A prompt will ask you to enter the ply at  which the laminate

symmetry is to begin.  If you want to duplicate a mirror image of

all plies entered, the ply value at which the symmetry begins

will be the number of plies entered.

This command is not listed on the 25th row of the edit screen.

Saving a Completed Laminate   [SAVE]

When the stacking process is complete, or if you wish to save an

incomplete laminate, the keyword SAVE or EXIT should be used.

This of course must be entered from the Command position.   The

save process will begin immediately on the current job via the

current drive and path.

NOTE:

Try not to save incomplete jobs as this will cause a .INC file to

be created.  An incomplete job is a laminate which contains a

core filler.   The INC is a extension which is used to  indicate

INComplete.   This file will be treated differently and will

generally cause you to do some extra key pushing so try to avoid

it.

Abandoning a Job   [QUIT]

QUIT will exit the edit screen without saving.  Be careful!

#

Getting a New Job   [NEWJ]

Call a headhunter.  But seriously folks.  If there is a need to

redefine the job name before the save, this can be done with the

NEWJOB command.   Use  the function key or type it in yourself.

The routine should look familiar as you used it before entering

the create/edit session.

Getting a New Catalog   [NEWC]

To switch back and forth between the PCAT catalogs, enter the

keyword NEWCAT.   This can be done by typing it in or using the

function key.   This  must be done at the first entry position.

The third window will then clear, the original file setting

process will be presented.  A real piece of cake.

Editing an Existing Laminate File

Actually, editing and creating are the same thing.   The only

difference is where you begin.   With a create, the beginning is

always with the first group.   With an edit, the beginning is

with the last group plus one of the laminate being edited.

Everything else is the same.

To edit a laminate, EDIT must be entered from the Main> prompt.

- - - - - - - - - - - - - - - - - - -

Main>EDIT

- - - - - - - - - - - - - - - - - - -

With this,  LAMCON will produce a short narration asking for the

job name which you wish to edit.  For all completed laminates,

only 8 characters are accepted.  Any extensions will be ignored

unless it is a .INC extension.  This of course would mean that

you wish to complete a laminate which contains a core filler.

After the job has been entered, the file is read.  If you try to

fool LAMCON, you will not succeed.  A nasty error message will

let you know that LAMCON went looking for a file that did not

exist.

Immediately after the read is complete,  LAMCON will go to the

full screen view to present what information was obtained from

disk.  If the file is correct, then the [Esc] or <CR> will

continue the process.   If the wrong job has been specified, an

[N] will route you back to the "Job to edit>" prompt.

#

If the job is correct, then a material catalog is read and the

create process picks up at total groups plus one.   You should

feel right at home.

Printing Laminate Schedules    [LPT]

To get a formatted copy of a laminate,  the LPT command may be

used from the Main>.   You will have the option of using the

current job, if one exists, or you may request a new one from

within LPT.   Primarily, LPT is used for completed laminates,

however the .INC variety may also be printed.

When requesting a completed job, enter the job name only.

(8 characters maximum, no extensions please)

When requesting an incomplete file, enter the job plus .INC

LAMCON will return to ask for the next file to print when

printing is complete.   To return to the Main> prompt, enter a

<CR>.

Changing the Default Drive   [DRIVER]

Each time LAMCON reads or writes disk information, the default

drive and path is used to direct the access.   These defaults are

automatically established when LAMCON is first initialized.  This

is a result of where the user was in DOS just prior to loading

LAMCON.  For example, if LAMCON was activated by A> then the

default drive will be A.  If LAMCON was loaded with a C> then the

default will be the C drive.

Sub-directories are also supported by LAMCON.   This means that

while operating within the program,  it is possible to change not

only the drive but the path as well.   If you are unfamiliar with

the sub-directory concept, it is strongly suggested that you

spend some time with the DOS manual.  Of course it will not be as

pleasurable to read as this manual,  but it will enhance your PC

skills countless measures.

Should you find it necessary to change the drive-path while in

LAMCON, enter DRIVER from the Main>.   This keyword may be

shortened to DR or it may be invoked by pressing the appropriate

function key.

#

Upon doing so, and if done correctly, a small message will appear

in the command window:

- - - - - - - - - - - - - - - - - - - -

Procedure DRIVER  <CR> to abort

- - - - - - - - - - - - - - - - - - - -

This message will be displayed for only a few seconds when using

a  steroid free XT type machine.   If you have installed a Steve

McQueen 27 Mhz Turbo Board into your PS/2,  it may not even stay

long enough for you to realize that it was a message.

All it's supposed to do is let you know that to terminate the

DRIVER command, a null string should be entered.  That's a <CR>.

After this message has left the screen,  a prompt will appear

asking for the new drive and path.   At the same time, the

existing  drive and path will be displayed so that you know what

the current setting is.   If you are using the DRIVER command to

check the setting, and it checks out just fine, a press of the

<CR> will leave the defaults unchanged.

In the event that you decide to change the defaults, the new

drive-path can be entered in a single string.  The format is a

follows:

First two characters must be:  A: or B: or C:

(Upper or lower case ok)

Third character must be:   \   (a backslash)

Fourth :  1st level sub-directory name

Fifth  :  \   (another backslash)

Sixth  :  2nd level sub-directory name

Seventh:  \

And the pattern continues depending upon how many levels you wish

to address.

The result should look something like this:

A:\ or B:\ or c:\

All of these will cause the defaults to be set to the drive A,  B

or C respectively with the primary (ROOT) directory being used.

If a hard disk system is being used,  it makes good sense to

create a sub-directory for all COPAN/II system and data files.

If a COPAN sub-directory has been created at the first level, the

proper entry to access it will be:  C:\COPAN

#

If it was created at the second level to a first level DESIGN

directory, the entry would be:

C:\DESIGN\COPAN

If the entry is not correct, which would be an invalid format,

LAMCON will make a low pitched sound indicating its disgust.  A

message will also be diplayed to make clear the fact that the

format used was unacceptable.   The drive-path will remain

unchanged until the proper format is used.   The Main> can always

be reaccessed with the punch of the <CR>.

Reading The Disk Directory      [DIR]

If during the course of a LAMCON session it becomes necessary to

read the disk directory,  it is not required to exit to DOS and

execute the DIR command.  This command can be entered as a LAMCON

keyword and will produce all files for a given drive.   Selective

file searches can also be achieved using wild card characters.

To activate this function, enter DIR from the Main> prompt or it

may be invoked using the appropriate function keys.

A new window will appear which will accomodate four columns of

file names.   At this point, a new set of functions will be

assigned to the [F1]-[F10] keys.  These new key definitions will

be displayed on the bottom line of the screen.

To display all the files in the default drive, enter the

character [*] or press [F1].   All file names will be listed on

the screen with the total file quantity.   If the current drive

contains more that one screen of data, it will be necessary to

execute a keyboard interrupt using the combination  [Ctrl] [S].

By pressing these two keys together (actually [Ctrl] should be

pressed first and held while then pressing [S]) you will

cause the output to stop.  To resume the action, press the [S]

again.   If you need to stop it again, do another [Ctrl] [S].

The listing is not a true DIR, its more a DIR/W.   This was

because we felt the filename was more important than the size and

date.

If only certain types of files are being sought, for example, all

of the .EXE files,  it is possible to search for a specific file

extension.   After  the DIR> appears,  enter  *.EXE.   This  with

produce a listing of all .EXE files on the default drive.

#

To check on all PCAT material files the entry would be *.MAT.

The function keys include a few other entries which might be used

frequently.   If the one you require is not listed, you will be

stuck having to enter it yourself.

Where all of the information regarding disk contents has been

gathered, it is possible to return to the MAIN> by entering the

now familiar null string <CR>.

What if you want a hard copy of the directory, but don't want to

print the whole screen?  Simple.  After you have the DIR> prompt,

carefully execute this two-key sequence.  [Cntl] [PrtScn].  This

will cause all subsequent text which is printed on the screen to

be routed to the printer as well.  This means that your commands

will be printed too but that shouldn't be too serious a matter.

Don't forget to do another [Cntl] [PrtScn] after you have printed

all that you require.   This acts as a toggle to reverse the

condition.

Checking Bytes Remaining on Disk     [ROOM]

When working on hard disks, this function is not of much

interest, but floppies are a different story.  These comes a time

in every floppy's life when additional saves produce the dreaded,

insufficient disk space.  this can happen when byte management is

not practiced.   Keeping track of available disk space can save

some needless headaches when working with LAMCON.

To perform a quick check on the available disk space, enter the

keyword ROOM from the Main>.

A prompt will then ask for the drive that you are concerned with,

namely A, B or C.

LAMCON will be waiting for a single key response of A, B or C.

(If you have a D drive, you can enter D.)

After pressing this single key, LAMCON will print the number of

bytes available for use on that disk.

Control is automatically returned to the Main>.

#

Removing Disk Files   [ERASE] or [DIR]

Files on disk may be removed while operating LAMCON.  Two

different keywords are available for this function.

[DIR]

The keyword DIR produces a screen listing of the disk files.   It

is sometimes  convenient to have this list in front of you when

deleting obsolete files.   When the DIR> prompt is showing, enter

the subword KILL.   This will tell LAMCON that you wish to delete

a file while keeping the data screen unchanged.

After KILL is entered, a new prompt will ask for the name of the

file that should be deleted .   The full file name, including any

extensions, must be entered.

If the file that was entered actually exists, it will be deleted.

A  message will confirm this when the file no longer exists.  If

LAMCON does not find a file with the same name as entered, the

message will indicate that the file was not deleted.

To return to the DIR> prompt,  enter a <CR>.  Another <CR> will

produce the Main> prompt.

[ERASE]

The second way to remove files is to enter ERASE from the Main>

prompt.  This too may be invoked from the function keys.  The

only difference between this routine and the DIR method is that

with ERASE, the files will not be visible on the screen.  The

ERASE command might be used when all files from a specific job

are being removed due to obsolesence or to make room on the disk.

As with the DIR method,  if the file that was entered actually

exists, it will be deleted.  A message will confirm this when the

file no longer exists.   If LAMCON does not find a file with the

same name as entered, the message will indicate that the file was

not deleted.

To return to the Main> prompt, enter a <CR>.

#

Renaming Disk Files    [RENAME]

Should it be necessary to change the name of an existing file, it

is not required to exit to DOS.   While working within LAMCON it

is possible to rename disk files by using the command RENAME.

This keyword, or its first two letters, should be entered from

the Main> prompt.   This will cause an input statement to be

displayed in the command window which will ask for the old

filename.   The full name must be used.  This means that any file

extensions such as .MAT or .PRP must be included.

After the old name has been entered with a <CR>, a new prompt

will be displayed asking for the new name.  This is the name that

the old file will be given.   This too must be a valid DOS

filename.

When the <CR> is pressed, the operation will commence.   The

renaming is complete when a message indicates that the old file

has been renamed to the new.   Return to the Main> is automatic

when the rename operation is successful.

If the old file is not found on the default drive or if the new

name already exists,  an error message will indicate:

Terminated due to error

The routine will not exit if the rename is unsuccessful.

Instead, the procedure will restart with the Old filename>

prompt.

To terminate the RENAME procedure, enter a <CR>.

Clearing All Variables   [NEW]

During the development of LAMCON, it was necessary to repeatedly

execute certain sections in order to hunt the great bug.  Because

each run needed to begin with all variables cleared, a small

procedure was built in to set everything back to its

initialization status.

When LAMCON is first started, this procedure is activated to

ensure a correct initialization.   However, after you have worked

with LAMCON for just a single edit, the registers become filled

with all that is necessary to produce what you see (and alot that

you don't)

#

If there is a need to reset LAMCON to its powered-up status, a

NEW command can be entered to achieve this  "warm  boot".   All

variables will be immediately set to the initialization  status

and defaults for will be assigned.    (COREDATA.MAT and

PLYDATA.MAT)

After the command has been executed, LAMCON will respond with

"All Clear"

Control is immediately returned to the Main> prompt upon

completion.

NOTE:   LAMCON does not confirm a NEW request.   Once

entered, the command is executed.

Exiting LAMCON       [EXIT]

When it is time to power down LAMCON you will probably want to

exit the program.   You could just cut the power but we figured

that you may wish to use your machine for other things than

designing composite panels.

To EXIT, enter the keyword EXIT from the Main>.  This entry will

produce the Exit> prompt.  What else?  If you wish to exit to

DOS, type in DOS and hit the <CR>.

LAMCON does not confirm a DOS request.  So make sure that you

want to leave when you transmit this message.  However, if you

should commit this tragic error,  you can always do another

LAMCON from ACCESS and start the whole thing over again.