-

tm

PC-STRAN    v4.00  Users Guide

tm

PC-STRAN

A 1-,2- or 3-Dimensional

STRuctural ANalysis Program

For the IBM Personal Computer

(C)Copyright 1984 - 1989

by Joe Murphy,Ph.D.,P.E.

Version 4.00

Joe Murphy

P.O. Box 56164

Madison,  WI  53705

tm

PC-STRAN   is a registered trademark of Joe Murphy.

IBM is a registered trademark of

International Business Machines Corp.

Microsoft is a registered trademark of

Microsoft Corporation.

Hercules is a registered trademark of

Hercules Computer Technology, Inc.

Page-1

-

tm

PC-STRAN    v4.00  Users Guide

Table of Contents

General Information....................................  3

System Requirements....................................  4

Some Definitions.......................................  4

Getting Started........................................  5

Example Problem........................................  5

Tips & Techniques......................................  6

\Paths\................................................  7

Hercules MonoGraphics..................................  7

Maximums and Mass Storage..............................  7

NON-normal Program Termination.........................  7

Information about the files on the PC-STRAN Disks......  8

Previous Versions Compatability........................  9

Program details........................................ 10

PC-PROP............................................ 10

PC-MESH............................................ 11

PC-LOAD............................................ 13

PC-STRAN (General) PC-SOLVE (Particular)........... 14

Limitations...................................... 15

PC-VIEW............................................ 15

PC-RPT............................................. 15

Load Factor Analysis............................. 15

Graphics Hardcopy...................................... 16

Error Codes ........................................... 16

References............................................. 17

Acknowledgements....................................... 17

PC-STRAN Version / Enhancements........................ 18

Some Questions answered................................ 19

Permission to COPY..................................... 19

User-Supported Software................................ 21

8087 Version........................................... 22

Command Line Switches/n................................ 22

Invoice................................................ 22

Disclaimer

----------

Use of this program acknowledges this disclaimer of warranty:

This program is supplied as-is.  The Author disclaims all  warranties,

expressed or implied, including, without limitation, the warranties of

merchantability and of fitness of this program for any purpose.

Although every  effort  has  been  made  to  ensure  both  theory  and

programming  logic  in  this program are correct,  no warranty, either

expressed or implied is made by the Author that the analyses  obtained

from  its  use will be correct.  In no event will the Author be liable

to you for any damages,  including any lost profits, lost  savings  or

other incidental or consequential damages arising out of the use of or

inability  to use this program, even if the Author has been advised of

the possibility of such damages, or for any claim by any other party.

Page-2

-

tm

PC-STRAN    v4.00  Users Guide

General Information

-------------------

PC-STRAN analyzes 2- or 3-Dimensional truss/frame  structures  or  1-D

continuous beams. The general procedure is to recall three disk files:

1) a member PROPerty file containing:

a) member type

b) material properties

i) density

ii) Young's modulus

iii) (torsional) shear modulus

c) geometric properties

i) cross sectional area

ii) 2 (bending) moments of inertia

iii) (rotational) torsion constant

2) a geometric 3 dimensional MESH file containing:

a) structure's node information

i) node number

ii) 3 coordinates

iii) 3 lateral restraints

iv) 3 rotational restraints

b) structure's member information

i) beginning & end node numbers (2ai)

ii) 2 member-joint fixity factors

iii) roll angle

iv) member type (1a)

3) a LOAD file with up to 8 load cases containing:

a) loaded node number (2ai)

b) loads at 3a) with:

i) 3 applied forces

ii) 3 applied moments .

c) loaded member node numbers (2bi)

d) linearly distributed loads at member nodes

PC-STRAN employs a matrix method of structural analysis,  specifically

the direct stiffness method of solution, which is sometimes called the

displacement method. See section on References.

A truss/frame is defined as a structure whose members can be  attached

to  joints (or nodes) where only axial forces are transmitted (a truss

with frictionless swivel connections) OR where  bending  and  twisting

moments  are  also  transmitted  (a  frame with rigid connections).  A

truss/frame member then can have  one  end  of  the  member  a  swivel

connection and the other end of the SAME member a rigid connection!

Concentrated loads may only be specified at  nodes.   If  concentrated

loads  are to be applied at the interior of a member, insert a node at

that point and divide the member into  two.   For  member  loads,  the

values of the linear distribution are specified at the members' nodes.

Displacements   are  calculated  only  at  the  nodes.   If  you  want

displacements at the interior of a member, again insert a node at that

point.

For more information see section on Program details.

Page-3

-

tm

PC-STRAN    v4.00  Users Guide

System Requirements

-------------------

PC-STRAN requires:

1) structural engineering knowledge

(most IMPORTANT)

2) IBM PC or IBM XT or IBM AT with:

a) DOS 2.x or later

b) Max memory 448K (actually dependent on problem size)

c) 2 360K disk drives (preferably 2 drives & RAM disk)

d) CGA, EGA, VGA, HGC, or HGC+, 80 column monitor

Some Definitions

----------------

PC-MESH and PC-VIEW can view and rotate the 3  dimensional  structure.

Also,  defining  the coordinates of the nodes require defining a three

dimensional, right handed, Cartesian coordinate system.  This will  be

referred to as the GLOBAL system.  Below is a representation:

y

|

|

|      L

|     /|

|    / |

|   /  |

|  /   |

| /    |

|/     |

O------|------------x

/ \     |

/   \    |

/     \   |

/       \  |

/         \ |

/           \|

/             P

z

The x,y,z coordinate system has the +z axis pointing at  you,  the  +y

axis  pointing  vertically up, and the +x axis pointing to your right.

PC-MESH will initially (by default) view this way - you looking in the

-z direction.

LONG & LAT angles

If you are looking from point L towards the origin  O,  the  angle  LO

makes with its projection PO on the xz plane is defined the "LAT"itude

angle,  measured  positive  from the xz plane toward the +y axis.  The

angle PO makes with the  z  axis  is  defined  the  "LONG"itude  angle

measured  positive  from  the  +z  axis toward the +x axis.  These two

angles are similar to LONGitude and LATitude on a globe of the world.

Page-4

-

tm

PC-STRAN    v4.00  Users Guide

Getting Started

---------------

First copy Disk2 onto a PROGRAM Disk and Disk1 onto a DATA Disk.   The

steps specified will be for a two disk system.  Two disk drives with a

RAM disk is IDEAL.

1. Boot up the computer.  Put the DATA Disk1 copy  with  PC-STRAN  and

DATA on it in the default drive and PROGRAM Disk2 in the other drive.

2. Type PC-STRAN at the prompt (e.g. A>pc-stran)

3. You will be queried for the Path and PROBLEM filename for the  DATA

files, the Path for the PROGRAM files, the Path for the STORAGE files,

and the Path for the WORK files.  Give appropriate responses.

4. When in the PC-STRAN Main Menu, you can choose to transfer  to  PC-

PROP, PC-MESH, PC-LOAD, PC-SOLVE, or PC-VIEW.

5. Once you are done with PC-PROP, PC-MESH, PC-LOAD  or  PC-VIEW,  you

can transfer to the PC-STRAN Main Menu.

6. When you choose PC-SOLVE, option  4,  the  RESult  and  DEFormation

files  will be written using the Path and PROBLEM filename of the DATA

files while the STORAGE and WORK files will be written and deleted  on

their specified Paths.

7. To start out, work through the example problem of a truss.

Example Problem  -- a 3-D truss with 6 nodes & 12 members

---------------

1. After booting up, put Disk1 in drive A, Disk2 in drive B, and  type

PC-STRAN at the prompt (e.g. A>pc-stran)

2. Enter A:\EXAMPLE1. ,B:\ ,A:\ and  A:\  for  the  Path  and  PROBLEM

filename, PROGRAM Path, STORAGE Path and WORK Path.

3. At PC-STRAN Main Menu select option 1 -- transferring to PC-PROP

4. At PC-PROP Main Menu select option "4) Print all ...records".  When

queried where to print, select option "s) Screen".

5. After viewing the records and returning to PC-PROP  Main  Menu  try

editing a record.

6. To file the updated records select option "9) FILE all records...".

7. When you want to return to the main menu, select option "0) END ...

transfer...".

8. PC-MESH and PC-LOAD are  similar  to  PC-PROP.  Read  the  internal

instructions for viewing the mesh in PC-MESH.

Page-5

-

tm

PC-STRAN    v4.00  Users Guide

9.  PC-SOLVE  is fairly straightforward.  After the analysis ends, you

will return to the main menu.  You can return to DOS  or  transfer  to

PC-VIEW  to  view  the  deflected structure.  If you return to DOS the

DATA Directory should have the following files  on  it.  EXAMPLE1.PRP,

.MSH, .LDS, .RES, .DEF.

10. To get a REPORT, load BASICA (e.g. A>BASICA ).  Then  load  PC-RPT

(e.g.  LOAD"pc-rpt"  ).   Answer  A:\EXAMPLE1.  to  "Path  and PROBLEM

filename of DATA files", and A:\ to "Path of WORK files".  PC-RPT adds

the appropriate filename extensions.  PC-RPT will prompt if  you  want

to do a load factor analysis.   EXAMPLE1.RPT is an ASCII file that you

can   send   to  a  printer  with  the  print  command  (e.g.  A>print

example1.rpt)  or  view  on  screen  with  the  more   command   (e.g.

A>more<example1.rpt).

11. That's all there is to it.

Tips & Techniques

-----------------

Keep programs separate from data. If you use a  hard  disk,  copy  all

.EXE,  .COM and .BAS files into a separate PROGRAM subdirectory.  Copy

all other files into a DATA subdirectory.  For each problem you  could

create a separate subdirectory.

Keep units CONSISTENT!  All forces MUST  have  the  same  units!   All

length units MUST be the same!

If you have a RAM drive, designate it as the drive for the WORK files!

File records often (select option 9) when using  PC-PROP,  PC-MESH  or

PC-LOAD - you will not end the program but will save data if something

should happen.  Option 0 will NOT file data.

Rotating the structure  in  PC-MESH  should  flag  obvious  errors  in

geometry and in member end connectivity.

The default structure size in  ViewMesh of PC-MESH and PC-VIEW  allows

the  ENTIRE  structure  to be viewed from any angle.  If the structure

appears small use the zoom feature.

The default node restraints in PC-MESH is 0 (0 movement).  The default

member end connections in PC-MESH is 0 (0 fixity factor).

When minimizing the node bandwidth (option m in PC-MESH) use a  target

value  of  2.   After repeated passes, note the minimum bandwidth that

was attained during a pass.  Do  NOT  resequence  the  nodes.   Choose

option  m  again  but  now with the minimum bandwidth (you know it was

attainable) as the target bandwidth.  Resequence the nodes.

The experienced user does not have to use PC-PROP, PC-MESH or  PC-LOAD

to  create  or update the *.PRP, *.MSH or *.LDS files.  Any ASCII word

processor can be used.  If in doubt about how the files should be  set

up check the data files on the data disk!

Page-6

-

tm

PC-STRAN    v4.00  Users Guide

\Paths\

-------

Beginning with Version 4.00, PC-STRAN accepts Paths to DATA, PROGRAM,

STORAGE and WORK files.

Hercules MonoGraphics

---------------------

Beginning with Version 4.00, PC-STRAN graphics will work with the

Hercules Graphic Card and Hercules Graphic Card  Plus.    One  has  to

install  a  Hercules  DRIVER by typing MSHERC at the DOS prompt before

starting PC-STRAN.  See the Information about the files on the

PC-STRAN Disks.

Maximums and Mass Storage

-------------------------

PC-STRAN has the following maximums:

Number of member types             =    64

Number of nodes (NN)               =   128

Number of members (NM)             =   256

Semibandwidth (IH)                 =   255

Number of degrees of freedom (DOF) =   768

Number of load cases (LC)          =     8

Number of members with

linearly distributed loads        =   128 per load case

Number of records, K matrix (NR)   = 32727

NR = IH * [(IH + 1)/2 + (DOF - IH)]

PC-STRAN needs the following mass storage:

On the WORK files (RAM) drive

ERASE-ME.TMP

On the STORAGE files drive, a maximum of 456K

ERASE-ME.ROT    1152 bytes * NM

ERASE-ME.MB#      72 bytes * NM * LC

ERASE-ME.ND#      24 bytes * NN * LC

NON-normal Program Termination

------------------------------

If you have a non-normal program termination (normal is  option  0  in

PC-STRAN  to  DOS),  please  let  me  know  if  you think it is in the

program. Before using PC-STRAN again  check  for  and  ERASE  all  the

ERASE-ME.* files.

Page-7

-

tm

PC-STRAN    v4.00  Users Guide

Information about the files on the PC-STRAN Disks

-------------------------------------------------

This information is provided so that you'll know the function of  each

file  distributed  to  you,  and whether or not each file is optional.

The following files (unless noted by an * ) have been provided on  the

distribution diskettes:

1.      PC-STRAN.DOC - This is the main documentation file.  It is not

needed after the documentation has been printed.

2.      DOC.BAT - The file is used to assist you in printing  out  the

documentation.  It is also not needed after the documentation has been

printed.

3.      ADDED.DOC - If this file is present,  it  contains  additional

documentation.

4.      filename.LDS - file containing LoaDS for  INput  in  PC-STRAN.

EXAMPLE_.LDS are provided as optional examples.

5.      filename.MSH - file containing MeSH geometry - INput.

EXAMPLE_.MSH are provided as optional examples.

6.      filename.PRP - file containing member PRoPerty data - INput.

EXAMPLE_.PRP are provided as optional examples.

7.*     filename.RES - file containing PC-STRAN RESults - OUTput.

This file is INput to PC-RPT for writing "custom" reports.

8.*     filename.RPT - file containing RePorT of analysis.

PC-RPT generates this as OUTput.

9.*     filename.DEF - file  containing  DEFlected  node  geometry  of

loaded  structure.   Together  with filename.MSH,  PC-VIEW can overlay

loaded structure on top of non-loaded structure.

10.      PC-LOAD.EXE - A child program to  create  or  update  a  file

containing  applied  LOADs at the nodes of the structure. This program

is called from PC-STRAN.

11.      PC-MESH.EXE - A child program to  create  or  update  a  file

containing  the unloaded structure geometry or MESH.  Also the program

is for VIEWING the structure (rotating &  zooming).  This  program  is

also called from PC-STRAN.

12.      PC-PROP.EXE - A child program to  create  or  update  a  file

containing  the  PROPerties  of  the  members  of  the structure. This

program is also called from PC-STRAN.

13.      PC-SOLVE.EXE - A child program that SOLVES the  1-,2-  or  3-

Dimensional  STRuctural  ANalysis  of  the  structure using, as input,

files created by PC-PROP, PC-MESH, and PC-LOAD.

Page-8

-

tm

PC-STRAN    v4.00  Users Guide

14.      PC-STRAN.EXE - A parent program that controls the 1-,2- or 3-

Dimensional STRuctural ANalysis of the structure. After  entering  DOS

to start, type in PC-STRAN (e.g. A>PC-STRAN)

15.       PC-VIEW.EXE  -  A  child  program  for  VIEWing  the  loaded

structure (dashed lines) over the original structure.  Similiar to

PC-MESH.   Also  for  viewing  graphs  of memeber load, shear, moment,

slope and displacement diagrams.  This program is also called from

PC-STRAN.

16.      PC-RPT.BAS - A stand alone source program that  can  be  user

modified  for custom report writing.  INput is filename.RES and OUTput

is filename.RPT.

17.      README.1ST - A file on steps to use PC-STRAN.  This  file  is

not needed after the documentation has been printed.

18.*     ERASE-ME.ROT -  A  temporary  STORAGE  file  for  the  member

stiffness  matrices  and local-to-global ROTation matrices.  It should

be killed or erased by PC-STRAN upon successful termination.

19.*     ERASE-ME.ND#, .MB# - Temporary STORAGE  files  for  NoaD  and

MemBer loads of load case #.  It is killed by PC-STRAN.

20.*     ERASE-ME.TMP - A TeMPorary WORK file  for  PC-STRAN  results.

The  results  are  reformatted  for filename.RES.  It is killed by PC-

STRAN.  Also TeMPorary WORK for PC-VIEW.  It is erased by PC-VIEW.

21.      HIRESPIX.COM -  A  stand  alone,  terminate-and-stay-resident

(TSR),  program similar to IBM's GRAPHICS.COM for Epson and compatible

dot matrix printers.  Works with CGA, EGA, VGA, HGC and HGC+.

22.       MSHERC.COM  -  A  stand  alone,  terminate-and-stay-resident

(TSR), driver for monographics with Hercules HGC and HGC+ cards.

Previous Versions Compatability

-------------------------------

For Versions before 2.0

If you have made DATA files with a previous version  of  PC-STRAN  and

you  want  to  use those files with this version,  you must RENAME the

files to have a common filename and extensions PRP, MSH and LDS.  Also

insert a line (use EDLIN) at the very beginning of the LDS file with a

1 in the second column.  (It signifies 1 load case.)

For Versions before 2.5

If you have made a .LDS file compatible with Version 2.0 (see  above),

run the UPDATE25.BAS program to rewrite the .LDS file!

Page-9

-

tm

PC-STRAN    v4.00  Users Guide

Program details

---------------

PC-PROP

-------

PC-PROP creates a catalog of member properties.  The created file  is

stored on disk and used as an input file for PC-STRAN. This eliminates

specifying   member   properties   every  time  an  analysis  is  run.

Substituting a member in an analysis requires changing only the member

type field in PC-MESH.

PC-PROP can recall a member PROPerty file from disk, and modify member

records, add records, delete records, print all the records  to  disk,

printer or screen, and/or file the updated records.

PC-PROP is menu driven with scroll record  data  entry.  There  are  8

fields in each member type record.  They are:

1) Member Type name - alphanumeric characters

2) Density - force/length cubed

3) Young's Modulus - force/length squared

4) (torsional) Shear Modulus - force/length squared

5) Cross Sectional Area - length squared

6) (bending) Moment of Inertia about local y axis -

length fourthed

7) (bending) Moment of Inertia about local z axis -

length fourthed

8) (rotation) Torsion Constant about local x axis -

length fourthed

Fields 1,3,5 MUST be defined.  Fields 6,7 are necessary  when  bending

moments are to be transmitted.  Fields 4,8 are necessary when twisting

moments  are  to  be  transmitted.   If  8  is  left  blank it will be

approximated by PC-STRAN using information in fields 5,6,7 by:

4

A

TC = _________________

2

(2*PI) (Iyy+Izz)          PI=3.141593

This approximation is not good for  cross  sections  with  holes.  See

References section for computing Torsion Constants.

Page-10

-

tm

PC-STRAN    v4.00  Users Guide

PC-MESH

-------

PC-MESH creates a file  of  the  geometry  of  the  structure  to  be

analyzed.   The  created  file  is stored on disk and used as an input

file to PC-STRAN.  The program allows the user to view  the  structure

from any angle (see Some Definitions section).  The program is divided

into two general parts; one for specifying node (or joint) information

and the other for specifying member information.

PC-MESH can recall a geometric MESH file from disk, and modify node or

member records, add nodes or members, delete nodes or  members,  print

all node or member records to disk, printer or screen, and/or file the

updated node and member records.

PC-MESH is menu driven with scroll record data  entry.  There  are  10

fields in each node record.  They are:

1) Node Number - 1 to 3 digits

2) Global x coordinate - length

3) Global y coordinate - length

4) Global z coordinate - length

5) Node restraint in global x direction   | 0=restrained

6) Node restraint in global y direction   |  0=default

7) Node restraint in global z direction   | 1=NOT

8) Rotation restraint about global x axis |   restrained

9) Rotation restraint about global y axis |

10) Rotation restraint about global z axis |

There are 6 fields in each member record.  They are:

1) Beginning Node number - 1 to 3 digits

2) Ending Node number - 1 to 3 digits

3) Member connection to beginning node | see

4) Member connection to ending node    | below

5) Member roll angle - degrees

6) Member type name - alphanumeric characters

Member-to-node connection values can have  one  of  ten  single  digit

values.

0 is for a swivel connection and is the default value.

1 is for a fixed or rigid connection.

2-9 are for semirigid connections (see References) with fixity factors

of .2 to .9 (i.e. the fixity factor is the specified single  digit

divided  by  ten).   The semirigid connection can be thought of as

two rotational and one torsional springs at the member ends, with:

flexural (or torsional) rigidity       3 FF

spring constant = ________________________________   X  ______

element length               1 - FF

Note: The fixity factor (FF) is the same for the two bending axes  and

the torsional axis.

Page-11

-

tm

PC-STRAN    v4.00  Users Guide

To  understand  the  definition of a roll angle consider that a member

has a LOCAL set of coordinate axes.  The X (caps for LOCAL) axis  runs

+  from  the  beginning  of  the member to its end passing through the

member's centroid.  The member's Y and  Z  axes  define  the  member's

Moments  of  Inertia  Iyy  and Izz. Remember that the LOCAL coordinate

system has to be right-handed too.  To obtain the roll angle translate

the member's beginning node to the Global origin and move the member's

end node to the +x global axis WITHOUT twisting the member about  its'

X  LOCAL  axis.   The angle measured from the +z global axis to the +Z

LOCAL axis is the member's roll angle. Special rules are required  for

vertical  members.   Foremost is that end nodes have to be higher that

beginning nodes. Note the roll angle is measured + from +z  toward  +x

to +Z.

Roll angle (RA) is the angle the local +Z axis makes with a line (call

it P) which lies in the the cross-section (i.e., local Y-Z plane)  and

is  parallel to the global x-z (i.e., horizontal) plane.  The angle is

measured  positive  from  line   P   toward   the   local   +Z   axis.

Mathematically:

^     ^   ^       ^

[ y x ( Y x Z ) ] . Z = cos(RA)

where x is cross product

. is dot product

^ is vector

y

In the example on the right,              |

the vertical member runs                  |       X

o to X. It would have a -10               |       |

degree roll angle or +350                 |       |

degrees.  In the picture                  |       |

z'o is parallel to zO.                    O---Y---|----x

Also Zo is perpendicular                 /     \  |

to Yo.                                  /       \ |

/         \|

With PC-MESH you can view             /          .o

the structure interactively,         z         . /

specifying your line of sight                .  /

(see Some Definitions).  You               .   /

can rotate and also shift your           .    /

line of sight, up, down, left,         Z     z'

right, and zoom in and out of the

structure.  Fixed member connections

are drawn to its nodes while swivel connections stop short of its

nodes.  Instructions are given with the ViewMesh option in PC-MESH.

PC-MESH has an option to reduce or minimize the node  bandwidth.   The

node   bandwidth   is   linearly   related  to  the  stiffness  matrix

semibandwidth.  The time required to solve a linear  system  increases

as  the  stiffness  matrix  bandwidth  squared (for the same number of

degrees of freedom). Also because the number of records is limited  to

32727,  a  minimum  bandwidth  means  the fastest solution and maximum

problem size.

See also Command Line Switches/n.

Page-12

-

tm

PC-STRAN    v4.00  Users Guide

PC-LOAD

-------

PC-LOAD creates a file of applied loads to a specific structure.  The

created  file is stored on disk and used as an input file to PC-STRAN.

This eliminates specifying loads every time an analysis is run.  Eight

different loading cases for the same structure can be specified in one

file.  For each loading case, the members' weight can be  INCLUDED  or

IGNORED.  The members' weight load is calculated by:

member weight = density * cross sectional area * length

acting in the -y GLOBAL direction.

PC-LOAD can recall a LOAD file from disk, and  for  each  case  modify

load  records,  add loads, delete loads, print all the load records to

disk, printer or screen, and/or file the updated load records.

PC-LOAD is menu driven with scroll record  data  entry.  There  are  7

fields in each applied node load record.  They are:

1) Node number - 1 to 3 digits

2) Force in global +x direction - force

3) Force in global +y direction - force

4) Force in global +z direction - force

5) Moment about global x axis, + going from y to z -

force*length

6) Moment about global y axis, + going from z to x -

force*length

7) Moment about global z axis, + going from x to y -

force*length

There are 5 fields in each applied member load record.  They are:

1) Beginning Node number - 1 to 3 digits

2) Ending Node number - 1 to 3 digits

3) Value of linearly distributed load at the

beginning node - force/length

4) Value of linearly distributed load at the

ending node - force/length

5) Distributed Load Type - alphabetic characters

Nine different choices of distributed load type are permitted:

XA refers to a linearly distributed load in the GLOBAL x direction

per unit of member Actual length.  YA and ZA are similar.

XP refers to a linearly distributed load in the GLOBAL x direction

per unit of member Projected length on the yz plane.  YP  and  ZP  are

similar.

XL refers to a linearly distributed load in the LOCAL X  direction

per unit of member actual length.  YL and ZL are similar.

Gravity loads would be YA with negative values.  Snow loads  would  be

YP  with negative values.  Uplift wind pressure loads might be YL with

positive values.

Page-13

-

tm

PC-STRAN    v4.00  Users Guide

PC-STRAN (General) PC-SOLVE (Particular)

----------------------------------------

PC-STRAN  analyzes  3  dimensional  structures  whose   members   are

essentially one dimensional connected together at joints or nodes. The

connectivity  of  members  to  the  nodes  can either be friction-less

swivels or fixed or semirigid.  One end of a member can be  fixed  and

the other swivel.  PC-STRAN requires as input the following 3 files:

1) a PROPerty file on member types

2) a geometric MESH file with node

and member information

3) a LOAD file specifying loads at the nodes and members

Because each  node  can  have  6  possible  degrees  of  freedom  -  3

translation  and 3 rotation - care must be taken in PC-MESH to release

the nodes.  Consider the following types of structures:

1) Rigid beam   - dx=dz=ry=rx=0  2 DOF's per node left

2) Plane truss  - dz=ry=rx=rz=0  2 DOF's per node left

3) Plane frame  - dz=ry=rx=0     3 DOF's per node left

4) Space truss  - ry=rx=rz=0     3 DOF's per node left

5) Space frame  -                6 DOF's per node left

Since PC-STRAN can handle part frame and  part  truss  structures  the

maximum  records  of the K matrix is not a multiple of number of nodes

but is a separate maximum (see Maximums).  After PC-STRAN reads in the

input files and checks that the information is consistent (e.g. member

types in MESH file exist in PROPerty file, LOADed nodes exist in  MESH

file,  etc.)  the  total  number of records is calculated and printed.

Output is directed to a user specified file and includes the following

for all load cases:

1) all the input

2) degrees of freedom of the structure

semibandwidth of K matrix

number of records in K matrix

3) equivalent node loads for member loads

4) member end forces

in local coordinate system

5) node movement and forces (reaction & applied)

in global coordinate system

As PC-STRAN is analyzing, messages are printed to the monitor (e.g. as

member stiffnesses are added to the global matrix).  Member  stiffness

matrices,   in   local  coordinates,  and  local-to-global  coordinate

transformation matrices, are stored in a temporary  STORAGE  file,  so

they  only  have to be calculated once thereby reducing computer time.

The global matrix file is stored in available DOS memory.

Page-14

-

tm

PC-STRAN    v4.00  Users Guide

Limitations

-----------

PC-STRAN assumes small deformations and member  direction  cosines  to

remain  the  same  throughout the loading and deformation history. The

global matrix (17 digits precision) is solved  using  Crout  reduction

and Gauss-Jordan elimination, with matrix skyline profiling.

PC-VIEW

-------

PC-VIEW views  the  deflected  loaded  structure  over  the  original

unloaded  structure.  PC-VIEW needs as input a file with extension DEF

created by  PC-STRAN  during  the  structural  analysis.   The  loaded

structure  is  drawn  with  dashed lines.  Two additional features are

incorporated in PC-VIEW not  found  in  PC-MESH.   They  are  a  SCALE

adjustment  to  make  the node displacements bigger or smaller and the

LOAD CASE specification (1 to 8 max).   Only  node  displacements  are

graphed  not  member bending. Instructions are given with the ViewMesh

option in PC-VIEW.

PC-VIEW also has a graphics option with  instructions  to  view  load,

shear,  moment,  slope  and  displacement  diagrams of the structure's

members.

See also Command Line Switches/n.

PC-RPT

--------

PC-RPT writes the structural analysis report.  PC-RPT needs as  input

a  file  with  extension RES created by PC-STRAN during the structural

analysis.  PC-RPT creates a file with extension RPT which is an  ASCII

file.   This  file can be sent to a printer with the DOS print command

(e.g. A>print filename.RPT) or to screen with  the  DOS  more  command

(e.g. A>more<filename.RPT). PC-RPT is provided in BASIC source code so

you  can  "customize"  your reports.  To run  PC-RPT you have to be in

BASIC (e.g A>basica ).  When in BASIC, LOAD"pc-rpt" and press the  RUN

key.   Before  you  customize PC-RPT it is suggested you make  a  copy

of  the  one provided and customize the copy.  REMEMBER  you  have  to

read everything from filename.RES in the order given in PC-RPT but you

can write to filename.RPT in any fashion you wish.

Load Factor Analysis

--------------------

PC-RPT can also provide limitless linear  combinations  of  the  eight

possible  load  cases.   This  is quite useful for Load and Resistance

Factor Design (LRFD).  After the individual load cases are printed  to

file,  PC-RPT  will  prompt for factors to multiply each load case and

print the linear additions of the results into the  *.RPT  file.   The

user might have designate load case 1 for permanent loads, load case 2

for  floor  live  loads,  load  case 3 for snow loads, etc.  This then

allows the user  to  combine  the  load  cases  in  any  way  desired.

Entering ALL zeroes for the load factors terminates PC-RPT.

[Note: To get just the weight of the structure as a load case, put  on

Page-15

-

tm

PC-STRAN    v4.00  Users Guide

a zero force concentrated load and specify weight included option!]

Graphics Hardcopy

-----------------

Using DOS 2.x or later and an IBM dot matrix  printer  (or  one  that

will  understand  the  graphics commands) you can do a graphics screen

print. Before running PC-STRAN type  GRAPHICS  while  in  DOS  command

(e.g.  A>graphics  ). Then while in the graphics of PC-MESH or PC-VIEW

you can print the screen by pressing the Shift-PrtSc keys.   Refer  to

your DOS manual GRAPHICS Command.

HIRESPIX is provided and is similar to IBM's  GRAPHICS  but  prints  a

smaller graph.  Install HIRESPIX instead of, or AFTER, GRAPHICS.

Error Codes

-----------

Run-time error codes of QuickBASIC 4.5 (not all-inclusive)

6   Overflow

7   Out of memory

11   Division by zero

24   Device timeout

25   Device fault

27   Out of paper

51   Internal error

53   File not found

57   Device I/O error

61   Disk full

68   Device unavailable

70   Permission denied

71   Disk not ready

72   Disk-media error

75   Path/File access error

76   Path not found

If you get the error - Bad file number at 416 - when using

PC-RPT.BAS, load BASICA with the switch /f:4, i.e., A>BASICA/f:4 .

Page-16

-

tm

PC-STRAN    v4.00  Users Guide

References

----------

Przemieniecki, J.S.

1968.  Theory of Matrix Structural Analysis.

McGraw-Hill Book Co., Inc.,

New York. (see pp.61-82)

Timoshenko, S. and J.N. Goodier

1951.  Theory of Elasticity.

Second Edition, McGraw-Hill Book Co., Inc.,

New York. (see p.267)

Seely, F.B. and J.O. Smith

1955. Advanced Mechanics of Materials.

Second Edition, John Wiley & Sons, Inc.,

New York. (see pp.276-279)

Popov, E.P.

1968.  Introduction to Mechanics of Solids.

Prentice-Hall, Inc.,

Englewood Cliffs.

Isaacson, E. and H.B. Keller

1966.  Analysis of Numerical Methods.

John Wiley & Sons, Inc.,

New York.  (see pp.50-52)

Willems, N. and W.M. Lucas, Jr.

1978.  Structural Analysis for Engineers.

McGraw-Hill Book Co., Inc.,

New York. (see Appendix D - torsional constants)

Grooms, H.R.

Algorithm for Matrix Bandwidth Reduction.

1972.  Journal of the Structural Division,

Proceedings of the American Society of Civil Engineers,

Vol. 98, No. ST1, pp.203-214.

Wang, C.K.

1983.  Intermediate Structural Analysis.

McGraw-Hill Book Co., Inc.,

New York. (see Chapter 20, Rigid Frames with Semirigid Connections)

Acknowledgements

----------------

I gratefully acknowledge the following engineers

Catherine M. Marx, P.E.

Ron Wolfe, P.E.

Dave Gromala, P.E.

They and users are entitled to credit for things you like in PC-STRAN.

Page-17

-

tm

PC-STRAN    v4.00  Users Guide

PC-STRAN Versions / Enhancements

--------------------------------

1.0  -  First documented stand alone programs

-  PC-PROP, PC-MESH, PC-LOAD, PC-3DSA

-  64 member types,64 nodes,64 members,80 dof's

1.2  -  PC-3DSA renamed to PC-STRAN

-  programs chained

-  128 nodes,256 members,254 dof's

-  disk storage of WORK files

1.4  -  upgrade of 1.2

-  192K of memory

2.0  -  add PC-VIEW, PC-WRITE

-  255 dof's,8 load cases

-  transfer information between programs

2.5  -  upgrade of graphics

-  linearly distributed member loads (128)

-  members' weight load option (for all members)

-  moment/shear diagrams for members (256)

-  scroll method of selection

-  greater than 255 dof's

2.57 -  8087 version of 2.5

3.00 -  add PC-SOLVE as subprogram to PC-STRAN

-  direct memory allocation algorithm for K matrix &

load vectors (resulting in significant speed improvement)

-  use of 16 bytes 0:04F0-0:04FF for data passing

between program modules

-  autocheck and autoutilization of 87 math coprocessing chip

3.10 -  show nodes option in PC-MESH and PC-VIEW

-  finer Zoom increments in PC-MESH and PC-VIEW

-  course and fine Shift adjustments in PC-MESH and PC-VIEW

-  assembly language profile routine in PC-SOLVE

3.20 -  PC-WRITE renamed to PC-RPT

-  node bandwidth minimization (reduction) option in PC-MESH

-  capabilities for semirigid joints

3.22 -  command line switches for non-8087 and non-EGA compatibles

3.50 -  expanded drive letters A-Z

-  compiled with QuickBASIC 4.0

(runs faster though larger programs)

-  infinite number of linear combinations of load cases

in PC-RPT (useful in load factor analysis)

-  PC-STRAN Main Menu in each program module

4.00 -  path capabilities for DATA, PROGRAM, STORAGE and WORK files

-  PC-STRAN is a parent process with children, PC-PROP,

PC-MESH, PC-LOAD, PC-SOLVE and PC-VIEW

-  PC-RPT incorporates user-defined header and page numbering

-  data entry uses scrolling of records in editing window

data is not reformatted

-  graphics and HiResPix (screen dump driver) works with

CGA, EGA, VGA, HGC and HGC+ and 100% compatibles

Page-18

-

tm

PC-STRAN    v4.00  Users Guide

Some Questions answered

-----------------------

1.  "Will it help if I buy more RAM memory for my PC?".  ANSWER:  Yes,

if with enough memory and purchase of the appropriate software you can

create  a  "RAM" (electronic) disk drive to hold the TEMPorary storage

WORK files needed by PC-STRAN.

2.  "In what language was PC-STRAN written?".  ANSWER: It was  written

in Microsoft QuickBASIC, then compiled with Microsoft's QuickBASIC 4.5

Compiler.

3.  "Can I obtain the source code for the program?".  ANSWER: No.

4.  "What is your update policy?".  ANSWER: Presently  I  will  notify

registered  users  of  updates  by  postcard.   Or you can mail me two

diskettes with postage and mailer, or a check for $10,  every  six  to

nine months or so, and I will send you the then-current version.

5.  "Do you have any  other  programs  that  you  are  distributing?".

ANSWER:  Yes,  PCBRIDGE(tm)  is  a  structural  analysis program for a

continuous beam with moving vehicle load envelopes.

6.  "Is there a version of PC-STRAN for other computers?" ANSWER: No.

7.  "What about program support?". ANSWER: I  will  give  support  and

reply  to written questions from registered users.  I will also notify

registered users of any program problems.

Permission to COPY

------------------

Clubs and other non-profit organizations are granted permission by the

author to freely copy these programs and documentation  and  share  it

with their members, so long as:

1.  No price is charged for the software or documentation. However,  a

distribution  cost  may  be  charged for the cost of the diskettes, so

long as it is not more than $5 per diskette.

2.  Club members  are  informed  of  the  user-supported  concept  and

encouraged to support it with their registrations.

3.  The program or documentation are not modified in any way  and  are

distributed together.

See User-Supported Software.

Page-19

-

tm

PC-STRAN    v4.00  Users Guide

User-Supported Software

-----------------------

User-Supported Software

If you are using this program

and find it to be of value

your registration, $45, is required.

Joe Murphy

Post Office Box 56164

Madison,  WI  53705

Regardless of whether you register,

you are encouraged to copy and share these programs.

tm

PC-STRAN   is NOT public  domain  or  free  software,   but  is  being

distributed as "user-supported" software.

Non-registered users of this software are granted a limited license to

make an evaluation copy for trial use  on  a  private,  non-commercial

basis,   for  the  express purpose of determining whether the programs

are suitable for their needs.  At the end of this  trial  period,  you

should either register your copy or discontinue using the programs.

This means that if you use  this  program  for  any  real  benefit  to

yourself or others, you should pay for your copy.  That way the author

will be able to provide you with support and updates.

A registration entitles  you  to  use  the  program  on  any  and  all

computers  available  to  you,   with  the  following restriction:  If

others have access to this program or may use  it,   then  you  should

purchase a site license.

All users are granted a limited license to copy the programs  for  the

trial  use  of  others and subject to the following limitations.  This

license does NOT include distribution  or  copying  of  this  software

package:

1. In connection with any other product or service.

2. For general use within a company or institution.

3. For any consideration fee.

4. Distribution in modified form,

i.e., the documentation must be included.

User-supported software is a way in  distributing  computer  programs,

based on these beliefs:

1.  That the value and utility of software is  best  assessed  by  the

user on his/her own system.

2.  That the creation of personal computer software can and should  be

supported by the computing community.

3.  That distribution of programs should be  encouraged,  rather  than

restricted.

Page-20

-

tm

PC-STRAN    v4.00  Users Guide

Anyone may request a copy  of  a  user-supported  program  by  sending

blank,  formatted disk(s) to the author of the program.  An addressed,

postage-paid return mailer must accompany  the  disk  (no  exceptions,

please).  A  copy of the program, with documentation on the disk, will

be sent by return mail.  The program will  carry  a  notice  with  the

registration  fee  and  the  program's  author.  A person who uses the

program is expected to become a registered user.

Free distribution of software and registration for its use  eliminates

costs for advertising and copy protection schemes.

Users obtain quality software at a reasonable price. They can  try  it

out  before  buying, and do so at their own pace and in the comfort of

their own home or office.

User-supported software only works if the software  is  actually  user

supported.

If you use the software, you should show  your  support  and  register

your  copy.   It's  only through your registration that the author can

afford to improve the software and bring you  improved  versions.   If

you  don't  like  the  software,  or feel that it is sorely lacking in

some area(s),  then please don't hesitate to let the author know.

Page-21

-

tm

PC-STRAN    v4.00  Users Guide

8087 Version

------------

PC-STRAN v3.00 and later checks if a  87  math  coprocessing  chip  is

available.   If PC-STRAN detects it is, PC-STRAN branches to code that

utilize assembly language subroutines written for the 87  chip  during

the number crunching Crout reduction & Gauss-Jordan elimination in

PC-SOLVE. See below.

Command Line Switches/n

-----------------------

tm

PC-STRAN   has an override switch at the command  line.   To  use  the

override option, type at the command line

pc-stran/n

#5##+

n     [100% IBM CGA] [87 in PC clone] [transfer prompts]

1          no

2                         no

3          no             no

4                                          no

5          no                              no

6                         no               no

7          no             no               no

#5##+

[100% IBM CGA]

On some non-IBM CGA and EGA cards the node number option  in  PC-MESH

and  PC-VIEW  produces numbers with lines through the numbers. If this

is the case use option 1,3,5 or 7.

[87 in PC clone]

On some PC clones the program senses an 87 chip present when  one  is

not installed.  If this is the case use option 2,3,6 or 7.

[transfer prompts]

When returning to PC-STRAN main menu the program default prompts  "Do

you  want  to return ...? Y/N".  To turn these default prompts off use

option 4,5,6 or 7.

Invoice

-------

If you have need for an invoice  so  that  you  can  initiate  payment

through  your  company  accounting  system,  the  following invoice is

provided.  For those of you in this category, or for those who wish an

invoice for your tax records, it should serve the purpose.

Page-22

-

tm

PC-STRAN    v4.00  Users Guide

--------------------

INVOICE

--------------------

Purchased from:

Joe Murphy

P.O. Box 56164

Madison,  WI  53705

DATE:     /     /                           Invoice No. 890101

--------------------------------------------------------------

PRICE     PRICE

PRODUCT                    QTY       EACH      EXTENDED

-------------------------  ---       -----     --------

tm

PC-STRAN  , support

programs, and

documentation            ___        $45      _____.__

Version 4.00

---------------------------

SUB

---------------------------

5% State sales tax                  SALES TAX   ___.__

(Wisconsin state residents)

TOTAL

--------------------------------------------------------------

Please make check payable to : Joe Murphy

You may retain this Invoice

for your tax records.

Page-23