NAME

rc85prg - program an RC-85 repeater controller.

SYNOPSIS

rc85prg  [options]  prgfile

DESCRIPTION

RC85PRG will program an A.C.C. RC-85 controller, using a telephone

modem with a PC running MS-DOS.  Both dialup and control receiver access

methods are supported.  The modem must be capable of sending digits 0

through 9, * and #.  The Hayes (TM) modem is supported by default.  RC85PRG

has been compiled with default sequences for the owner unlock code, the control

operator code, the user autodialer programming codes, and the controller

phone number.  Each of these sequences can be redefined by command-line

options.  <prgfile> is an input file containing the programming commands.

If not specified on the command line, stdin is used.  Hence pipes (|) or

redirection (<) may be used to provide programming commands.  The

MBBIOS interrupt-driven I/O package by AA4RE must be loaded prior

to invoking RC85PRG.  Contact your local packet BBS sysop if you need

help in obtaining AA4RE's excellent package.

OPTIONS

If RC85PRG is invoked improperly, it will display a usage

summary, and indicate the current value of all settable options.

Options include:

-v 			enable verbose mode.

-t			enable test mode (simulate operation, no delays).

-u			indicate autodialer is kept unlocked.

-w N			wait N secs after dialing the RC85, to permit

the phone answer message to finish.

-p N			pause N secs after each command, to permit the

response message to finish.

-m COMn:baud,8N1	select a modem port and parameters.  Any COM

port supported by MBBIOS is permitted.

-d phonenumber		override the default phone number of the con-

troller.  Include any exchange access and pause

for dialtone characters in this string.

-o owner_code		override the default owner unlock code.

-c ctrlop_code		override the default control op code.

-2 code			override the default 2-digit slot programming

code.

-3 code			override the default 3-digit slot programming

code.

-r			indicate the modem audio drives a radio transmitter.

-i STRING		specifies a modem initialization string.

-i0 STRING		overrides the default modem command-prefix.

-i1 STRING		overrides the default modem command-suffix.

-s			enter an interective phrase-to-code translation

mode (see SPEECH, below).

-sd			display all the speech vocabulary words.

-s FILE			load a file of words and their translations.

MODEM REQUIREMENTS

RC85PRG will default to the appropriate sequences used by Hayes(TM)-

compatible modems.  This sequence differs, depending on whether a phone

or radio connection is used (see the -r option).  For telephone access to

the RC85 controller, commands are prefixed by "ATDT", and suffixed by ";".

For radio access, commands are prefixed by "ATH1DT" and suffixed by ";H0".

This permits the off-hook LED to be wired to PTT, and modem audio output

wired into the mike audio input with appropriate padding resistors.

These sequences can be changed with the -i options.  An initialization

string is specified by the -i option, while the prefix and suffix strings

are set by the -i0 and -i1 options.

PRGFILE FORMAT

This file is an ASCII file containing one command per line.

Comments may be placed anywhere, and are indicated by a '#' and

continue to the end of the line.  Although a '#' can be used in

control sequences sent via radio, the '#' must be used to terminate a

programming sequence sent via the telephone.  Hence its use as a

comment delimiter is not ambiguous.  Commands consist of two characters

beginning in column 2.  Case is not significant.  Commands are:

CU			unlock the controller lock, to permit

message programming (for example).

CL			lock the controller (after a CU command).

CS string		send <string> to the controller, permitting an

arbitrary RC85 command to be issued.  <string>

is composed of words separated by spaces or tabs.

Each word is looked up in the vocabulary tables,

and its value, if found, replaces the word in <string>.

CO string		send the control-operator prefix followed by

<string> to the controller.  <string> is expanded

in the same fashion as for the CS command.

S n s p			program the autodialer slot number <n>, identified

by string <s>, to dial the phone number <p>.

string <s> may be used to record the callsign

or any other text; no spaces are permitted, and

this information is otherwise ignored.  The S

command is equivalent to a sequence of CS commands

to enter unlocked mode (if slots 0 through 9 are

selected), to unlock the autodialer (if needed, see

-u), to clear the current slot contents, and

to program a new phone number.  If the slot number

<n> is negative, the slot is programmed to speak

the number being dialed.  If phone number <p> is

omitted, then the slot <n> is cleared, thereby erasing

any previous phone number.

MP			program the previously-sent message.  Equivalent

to CS *0.

MR			readback the message being programmed.  Equivalent

to CS *2.

MA			abort the message being programmed.  Equivalent

to CS *4.

ID dots/dashes		causes a Hayes-compatible modem to send a Morse

code identification string.  This command is func-

tional only when radio access (-r option) is set.

The ID command should be used as often as needed

in the PRGFILE so as to identify your station every

10 minutes.

EXAMPLE PRGFILE

# Program phone numbers into three slots

S3	Fire	268-5501

S4	LPSO	232-9211

S11	N5KNX	555-1234

# Program emergency msgs

cu

cs *1113	#slot 3 = Fire

cs fire		#equivalent to code *634

mp

cs *1114	#slot 4 = LPSO

cs L P S O	#equivalent to codes 53 71 73 63

mp

cl

co 75 #Select stardust courtesy tone

id -./...../-.-/-./-..-	# send N5KNX in morse (ignored unless -r option set)

THEORY OF OPERATION

Rc85prg first validates its command-line arguments. It then dials the

RC85 controller, if telephone access was selected, and waits (see -w) a while

for the dialing and rc85 greeting message to complete.  Rc85prg will then read

the input file looking for programming commands.  Each line of the input file

constitutes one command.  If the command is CS, CO or ID, each following word

(words are separated by blanks or tabs) is replaced by its translation, if

any, from the vocabulary tables.  This is most useful for speech-related

commands.  Each command is then sent to the RC85 (suffixed automatically

by the required # tone) and then the program pauses (see -p) for the spoken

acknowlegement.

Commands should be given in the proper sequence, that is,

unlocked-mode commands should not be mixed with control-operator commands.

Autodialer slot programming does not require explicit unlocking;

the RC85 is unlocked automatically if slots 0 through 9 are selected.

After end-of-file is encountered, the appropriate lock and hangup commands

are sent to the RC85, and then the modem is told to hang up the phone line.

If the program is interrupted by ctrl-C, it will attempt to terminate the

session gracefully.

SPEECH

Since the speech vocabulary is so limited, an option exists (-s) to

permit various word combinations to be tried until a sentence is perfected.

The -s option enters an interective mode, in which each input line is

translated and displayed as a sequence of speech codes, where possible.  The

byte count is also displayed, to help the user keep messages within their

length limits.  A list of words beginning with a desired letter(s) is obtained

by beginning a line with '?' followed by the letter(s) desired.  A '?' by

itself will display all words in the vocabulary.

The entire vocabulary can also be displayed by using the -sd option.  RC85PRG

will halt after the display is completed.

The vocabulary can be augmented by using the -s option followed by a file name.

This file consists of one pair of words per line.  The first word of the

pair will be translated into the second word when encountered in a CS, CO or

ID command.  Comments may be inserted in the file by placing a "#" in column

one.

EXAMPLE

rc85prg -o 1989082344 -c 654 -d 121,9,5551212  setimers.r85

dials 555-1212 and programs the controller from the file

named setimers.r85.

grep ^S11 autodial.r85 | rc85prg -o 1989082344 -c 654 -d 121,9,5551212

uses the grep program to find the line beginning with S11

in the file named autodial.r85, and provides this line to

rc85prg for programming.

rc85prg -sd > vocab.85

dumps the speech vocabulary into the file named vocab.85.

LIMITATIONS

RC85PRG makes a one-pass translation of CO, CS and ID commands.

Also, the wait and pause timers may be inaccurate if a multitasker is

used.  The usual times required by the RC85 to execute commands can be

lengthened if it needs to ID the repeater.  I tried disabling the repeater

(CO 01) before I started a programming session, but this seemed to

break some programming commands, despite documentation to the contrary.

AUTHOR

James Dugal, N5KNX @ W5DDL (packet) jpd@usl.edu (Internet)

P.O. Box 44844,  Lafayette, LA  70504

Latest version available by anonymous FTP from PC.USL.EDU; look in

the pub/ham directory.