WELCOME TO THE SSTV AND FAX PROGRAM SSTVFAX VERSION ON5KN_16

AUTHOR:ON5KN BBS @ON7RC-5

BELGIUM

THIS IS AN EXPERIMENTAL RECEIVE AND TRANSMIT PROGRAM WRITTEN IN

ASSEMBLER AND TO USE ON ANY IBM COMPATIBLE UNDER MSDOS

THE UPSET WAS MADE TO BE ABLE TO RECEIVE AND TRANSMIT SSTV AND

FAX WITH A MINIMUM OF ADDITIONAL EXTERNAL HARDWARE

FOR RECEPTION OF BOTH MODES YOU ONLY NEED A SIMPLE 741-IC

TRANSMITTING TAKES PLACE VIA A LM324 AND A XR2206

BOTH TRANSMIT AND RECEIVE HARDWARE ARE POWERSUPPLIED BY

THE RS232 FROM THE PC

THE SCHEMATICS ARE ADDED IN THE FILE SSTVFAX2.SCH WICH YOU

SIMPLY CAN PRINT UNDER MSDOS ON	A GRAPHICS PRINTER

USAGE: COPY SSTVFAX2.SCH PRN /B

BE SURE TO MAKE THE CORRECT ADJUSTMENTS FOR THE TX-MODULE

SYNCPULSES MUST BE 1200HZ

FULL WHITE MUST BE BELOW OR EQUAL TO 2300HZ

TO OBTAIN FULL WHITE SCREEN USE:

IN RX-MODE CLEAR SCREEN TO BLACK WITH C,E OR V

BRING TO KEY-MODE WITH K

INVERT FULL BLACK SCREEN TO FULL WHITE WITH I

SOME DYNAMIC ADJUSTMENT MAY BE NECESSARY TO FINE-ADJUST THE

1200HZ PULSES (TRY ON A BLANK SCREEN)

A 4-LEVEL GREYSCALE IS AUTOMATED ON BOTTOM OF EACH PICTURE

SSTV CAN BE RECEIVED FROM ALL ACTUAL PRESENT MODES HOWEVER

THE IMAGES WILL BE DISPLAYED IN GREY-LEVELS EVEN FOR COLOUR

SENDED PICTURES

TRANSMISSION OF SSTV IS POSSIBLE IN 16 GREY-LEVELS AND IN

FOUR STANDARD MODES 8SEC, 16SEC, 32SEC,(NORMAL) AND 64SEC

(IN DOUBLE MODE)

FAX WILL BE RECEIVED IN 120 OR 90 LPM

TRANSMISSION OF FAX WILL ALSO BE IN 120 OR 90 LPM AND AN

OPTION PERMITS TO SEND EACH LINE TWICE (DOUBLE MODE)

THE RECEPTION SCREEN CAN BE ONE OF THE CURRENT MODES EXCEPT

S-VGA WICH IS NOT YET SUPPORTED

SOME VIDEO CARDS MAY NOT BE COMPATIBLE

IN SSTV BEST SYNC WILL BE OBTAINED WHEN RIGHT BOTTOM OF IMAGE

IS POINTED WITH RIGHT MOUSE BUTTON

START POINT OF IMAGE ON SCREEN CAN BE REDEFINED WITH MOUSE-

POINTING AND LEFT MOUSE BUTTON

ON SOME MODES OF COLOUR SSTV IT MAY BE INTERESTING TO USE

DOUBLE COMPACTATION AND OR POINT RIGHT BOTTOM SYNCRO POINT

AT END OF SCREEN

ON RECEPTION OF 96 SEC OR MORE IMAGES USE OF "U" TO DISABLE

VERTICAL SYNC UNTIL NEXT "T" AVOID SPURIOUS SYNC BY QRM

AS TIMING IS CRITICAL ON SUCH AN APPLICATION THE POSSIBILITIES

ARE FUNCTION OF THE SPEED OF YOUR PC AND TYPE OF SCREEN

THE PROGRAM WAS WRITTEN ON  AT 286/12MC 640K VGA

MAINLY CGA AND HERCULES MODES ARE NOT FULLY QUALIFIED BUT

CAN GIVE ACCEPTABLE RESULTS IN RECEPTION

A XT PC/4.7MC WILL NOT RESPECT THE TIMING STANDARDS IN TRANSMISSION

ANOTHER FEATURE WICH EXPANDS THE POSSIBILITIES IS A STANDARD

MOUSE WITH MOUSE-DRIVER

THE PICTURES CAN BE STORED ON DISK AND VIDPIC.COM ALLOWS YOU

TO PICK SCREENS FROM OTHER APPLICATIONS

IF YOUR SYSTEM IS EQUIPED WITH A HAND-SCANNER YOU ARE ABLE

TO CREATE YOUR OWN PICTURES AND PROCESS THEM WITHIN SSTVFAX

MAKING SLOWSCAN AND FAX ON A REAL LOW-BUDGET WICH WAS THE

MAIN TARGET OF THIS PACKAGE AND TO BRING THESE MODES OF

RECEPTION AND TRANSMISSION AS A SIMPLE ADD TO THE BECOMING

COMMON-USE-PC IN THE RADIO-AMATEUR WORLD

TRY FIRST THE RECEPTION MODULE !

IT IS EASY AND YOU WILL SEE THE ACTIVITIES ON HF AND VHF

NEAR 14.230KHZ YOU ARE SURE TO SEE THEM FIRST

ENJOY THIS FREE SOFTWARE PACKAGE AND LET ME KNOW WHAT YOU

ARE THINKING ABOUT AND GIVE SOME SUGGESTIONS

MAKE REFERENCE TO SSTVFAX2

IF YOU MAKE A COPY FOR FURTHER DISTRIBUTION PLEASE LEAVE IT

IN THE SELF-EXPLODING FORM SSTVFAX2.COM OR AT LEAST BE SURE

THAT IT INCLUDES SSTVFAX2.TXT/SSTVFAX2.SCH/SSTVFAX.EXE/VIDPIC.COM

AND THIS SSTVFAX2.DOC FILES

LAST REMARK ... YOUR TIMER MAY BE DISTURBED AFTER RUNNING

SSTVFAX JUST TILL ANOTHER START-UP OF YOUR PC

HAVE A LOT OF FUN AND SEE YOU SOON ON THE BANDS

73 ON5KN BBS @ON7RC-5 BELGIUM