SUBPART D - TECHNICAL STANDARDS

97.301  Authorized frequency bands.

The following transmitting frequency bands are available to an

amateur station located within 50 km of the Earth's surface, within the

specified ITU Region, and outside any area where the amateur service is

regulated by another country or United States government agency:

(a) For a station having a control operator holding a Technician,

General, Advanced, or Amateur Extra Class operator license:

WAVELENGTH     ITU           ITU           ITU      SHARING REQUIREMENTS

BAND      REGION 1      REGION 2      REGION 3   (See 97.303, Para.)

VHF         MHz           MHz           MHz

6 m         ---        50-54         50-54         (a).

2 m      144-146       144-148       144-148       (a).

1.25 m      ---        220-225          ---        (a),(b),(e).

UHF         MHz           MHz           MHz

70 cm    430-440       420-450       430-440       (a),(b),(f).

33 cm       ---        902-928          ---        (a),(b),(g).

23 cm    1240-1300     1240-1300     1240-1300     (j).

13 cm    2300-2310     2300-2310     2300-2310     (a),(b),(j).

do.   2390-2450     2390-2450     2390-2450     (a),(b),(j).

SHF         GHz           GHz           GHz

9 cm        ---        3.3-3.5       3.3-3.5       (a),(b),(k),(l).

5 cm     5.650-5.850   5.650-5.925   5.650-5.850   (a),(b),(m).

3 cm     10.00-10.50   10.00-10.50   10.00-10.50   (a),(c),(i),(n).

1.2 cm   24.00-24.25   24.00-24.25   24.00-24.25   (a),(b),(i),(o).

EHF         GHz           GHz           GHz

6 mm     47.0-47.2     47.0-47.2     47.0-47.2

4 mm     75.5-81.0     75.5-81.0     75.5-81.0     (b),(c),(h).

2.5 mm   119.98-120.02 119.98-120.02 119.98-120.02 (k),(p).

2 mm     142-149       142-149       142-149       (b),(c),(h),(k).

1 mm     241-250       241-250       241-250       (b),(c),(h),(q).

---      above 300     above 300     above 300     (k).

(b) For a station having a control operator holding an Amateur

Extra Class control operator license:

WAVELENGTH     ITU           ITU           ITU      SHARING REQUIREMENTS

BAND      REGION 1      REGION 2      REGION 3   (See 97.303, Para.)

MF          kHz           kHz           kHz

160 m    1810-1850     1800-2000     1800-2000     (a),(b),(c).

HF          MHz           MHz           MHz

80 m     3.50-3.75     3.50-3.75     3.50-3.75     (a).

75 m     3.75-3.80     3.75-4.00     3.75-3.90     (a).

40 m     7.0-7.1       7.0-7.3       7.0-7.1       (a).

30 m     10.10-10.15   10.10-10.15   10.10-10.15   (d).

20 m     14.00-14.35   14.00-14.35   14.00-14.35

17 m     18.068-18.168 18.068-18.168 18.068-18.168

15 m     21.00-21.45   21.00-21.45   21.00-21.45

12 m     24.89-24.99   24.89-24.99   24.89-24.99

10 m     28.0-29.7     28.0-29.7     28.0-29.7

(c) For a station having a control operator holding an Advanced

Class operator license:

WAVELENGTH     ITU           ITU           ITU      SHARING REQUIREMENTS

BAND      REGION 1      REGION 2      REGION 3   (See 97.303, Para.)

MF          kHz           kHz           kHz

160 m    1810-1850     1800-2000     1800-2000     (a),(b),(c).

HF          MHz           MHz           MHz

80 m     3.525-3.750   3.525-3.750   3.525-3.750   (a).

75 m     3.775-3.800   3.775-4.000   3.775-3.900   (a).

40 m     7.025-7.100   7.025-7.300   7.025-7.100   (a).

30 m     10.10-10.15   10.10-10.15   10.10-10.15   (d).

20 m     14.025-14.150 14.025-14.150 14.025-14.150

do.   14.175-14.350 14.175-14.350 14.175-14.350

17 m     18.068-18.168 18.068-18.168 18.068-18.168

15 m     21.025-21.200 21.025-21.200 21.025-21.200

do.   21.30-21.45   21.30-21.45   21.30-21.45

12 m     24.89-24.99   24.89-24.99   24.89-24.99

10 m     28.0-29.7     28.0-29.7     28.0-29.7

(d) For a station having a control operator holding a General Class

operator license:

WAVELENGTH     ITU           ITU           ITU      SHARING REQUIREMENTS

BAND      REGION 1      REGION 2      REGION 3   (See 97.303, Para.)

MF          kHz           kHz           kHz

160 m    1810-1850     1800-2000     1800-2000     (a),(b),(c).

HF          MHz           MHz           MHz

80 m     3.525-3.750   3.525-3.750   3.525-3.750   (a).

75 m        ---        3.85-4.00        ---        (a).

40 m     7.025-7.100   7.025-7.100   7.025-7.100   (a).

do.      ---        7.225-7.300      ---        (a).

30 m     10.10-10.15   10.10-10.15   10.10-10.15   (d).

20 m     14.025-14.150 14.025-14.150 14.025-14.150

do.   14.225-14.350 14.225-14.350 14.225-14.350

17 m     18.068-18.168 18.068-18.168 18.068-18.168

15 m     21.025-21.200 21.025-21.200 21.025-21.200

do.   21.30-21.45   21.30-21.45   21.30-21.45

12 m     24.89-24.99   24.89-24.99   24.89-24.99

10 m     28.0-29.7     28.0-29.7     28.0-29.7

(e) For a station having a control operator holding a Technician

or Novice Class operator license:

WAVELENGTH     ITU           ITU           ITU      SHARING REQUIREMENTS

BAND      REGION 1      REGION 2      REGION 3   (See 97.303, Para.)

HF          MHz           MHz           MHz

80 m     3.70-3.75     3.70-3.75     3.70-3.75     (a).

40 m     7.050-7.075   7.10-7.15     7.050-7.075   (a).

15 m     21.10-21.20   21.10-21.20   21.10-21.20

10 m     28.1-28.5     28.1-28.5     28.1-28.5

(f) For a station having a control operator holding a Novice Class

operator license:

WAVELENGTH     ITU           ITU           ITU      SHARING REQUIREMENTS

BAND      REGION 1      REGION 2      REGION 3   (See 97.303, Para.)

VHF         MHz           MHz           MHz

1.25 m      ---        222.10-223.91    ---        (a),(b),(e).

UHF         MHz           MHz           MHz

23 cm    1270-1295     1270-1295     1270-1295     (i).

97.303  Frequency sharing requirements.

The following is a summary of the frequency sharing requirements

that apply to amateur station transmissions on the frequency bands

specified in Section 97.301 of this Part.  (For each ITU Region, each

frequency band allocated to the amateur service is designated as either

a secondary service or a primary service.  A station in a secondary

service must not cause harmful interference to, and must accept

interference from, stations in a primary service.  See Sections 2.105

and 2.106 of the FCC Rules, "United States Table of Frequency

Allocations" for complete requirements.) (* Because of their great

length, 2.105 & 2.106 are not included in the references here.  The

following sharing requirements are sufficient for amateur reference.)

(a) Where, in adjacent ITU Regions or Subregions, a band of

frequencies is allocated to different services of the same category, the

basic principle is the equality of right to operate.  The stations of

each service in one region must operate so as not to cause harmful

interference to services in the other Regions or Subregions.  (See ITU

"Radio Regulations," No. 346 (Geneva, 1979).)

(b) No amateur station transmitting in the 1900-2000 kHz segment,

the 1.25 m band, the 70 cm band, the 33 cm band, the 13 cm band, the 9

cm band, the 5 cm band, the 3 cm band, the 24.05-24.24 GHz segment, the

76-81 GHz segment, the 144-149 GHz segment and the 241-248 GHz segment

shall cause harmful interference to, nor is protected from interference

due to the operation of, the Government radiolocation service.

(c) No amateur station transmitting in the 1900-2000 kHz segment,

the 3 cm band, the 76-81 GHz segment, the 144-149 GHz segment and the

241-248 GHz segment shall cause harmful interference to, nor is

protected from interference due to the operation of, stations in the

non-Government radiolocation service.

(d) No amateur station transmitting in the 30 meter band shall

cause harmful interference to stations authorized by other nations in

the fixed service.  The licensee of the amateur station must make all

necessary adjustments, including termination of transmissions, if

harmful interference is caused.

(e) The 1.25 m band is allocated to the amateur, fixed and mobile

services in the United States on a co-primary basis.  The basic

principle that applies is the equality of right to operate.  Amateur,

fixed and mobile stations must operate so as not to cause harmful

interference to each other.

(f) In the 70 cm band:

(1) No amateur station shall transmit north of Line A in the

420-430 MHz segment.

(2) The 420-430 MHz segment is allocated to the amateur

service in the United States on a secondary basis, and is allocated in

the fixed and mobile (except aeronautical mobile) services in the

International Table of allocations on a primary basis.  No amateur

station transmitting in this band shall cause harmful interference to,

nor is protected from interference due to the operation of, stations

authorized by other nations in the fixed and mobile (except aeronautical

mobile) services.

(3) The 430-440 MHz segment is allocated to the amateur

service on a secondary basis in ITU Regions 2 and 3.  No amateur station

in this band in ITU Regions 2 and 3 shall cause harmful interference to,

nor is protected from interference due to the operation of, stations

authorized by other nations in the radiolocation service.  In ITU Region

1, the 430-440 MHz segment is allocated to the amateur service on a

co-primary basis with the radiolocation service.  As between these two

services in this band in ITU Region 1, the basic principle that applies

is the equality of right to operate.  Amateur stations authorized by the

United States and radiolocation stations authorized by other nations in

ITU Region 1 shall operate so as not to cause harmful interference to

each other.

(4) No amateur station transmitting in the 449.5-450 MHz

segment shall cause interference to, nor is protected from interference

due to the operation of stations in, the space operation service and the

space research service or Government or non-Government stations for

space telecommand.

(g) In the 33 cm band:

(1) No amateur station shall transmit from within the States

of Colorado and Wyoming, bounded on the south by latitude 39 degrees N.,

on the north by latitude 42 degrees N., on the east by longitude 105

degrees W., and on the west by longitude 108 degrees W.  This band is

allocated on a secondary basis to the amateur service subject to not

causing harmful interference to, and not receiving protection from any

interference due to the operation of, industrial, scientific and medical

devices, automatic vehicle monitoring systems or Government stations

authorized in this band.

(2) No amateur station shall transmit from those portions of

the States of Texas and New Mexico bounded on the south by latitude 31

degrees 41 minutes N., on the north by latitude 34 degrees 30 minutes

N., on the east by longitude 104 degrees 11 minutes W., and on the west

by longitude 107 degrees 30 minutes W.

(h) No amateur station transmitting in the 23 cm band, the 3 cm

band, the 24.05-24.25 GHz segment, the 76-81 GHz segment, the 144-149

GHz segment and the 241-248 GHz segment shall cause harmful interference

to, nor is protected from interference due to the operation of, stations

authorized by other nations in the radiolocation service.

(i) In the 1240-1260 MHz segment, no amateur station shall cause

harmful interference to, nor is protected from interference due to the

operation of, stations in the radionavigation-satellite service.

(j) In the 13 cm band:

(1) The amateur service is allocated on a secondary basis in

all ITU Regions.  In ITU Region 1, no amateur station shall cause

harmful interference to, and is not protected from interference due to

the operation of, stations authorized by other nations in the fixed

service.  In ITU Regions 2 and 3, no station shall cause harmful

interference to, and is not protected from interference due to the

operation of, stations authorized by other nations in the fixed, mobile

and radiolocation services.

(2) In the United States, 2300-2310 MHz segment is allocated

to the amateur service on a co-secondary basis with the Government fixed

and mobile services.  In this segment, the fixed and mobile services

must not cause harmful interference to the amateur service.  No amateur

station transmitting in the 2400-2450 MHz segment is protected from

interference due to the operation of industrial, scientific and medical

devices on 2450 MHz.

(k) No amateur station transmitting in the 3.332-3.339 GHz and

3.3458-3525 GHz segments, the 2.5 mm band, the 144.68-144.98 GHz,

145.45-145.75 GHz and 146.82-147.12 GHz segments and the 343-348 GHz

segment shall cause harmful interference to stations in the radio

astronomy service.  No amateur station transmitting in the 300-302 GHz,

324-326 GHz, 345-347 GHz, 363-365 GHz and 379-381 GHz segments shall

cause harmful interference to stations in the space research service

(passive) or Earth exploration-satellite service (passive).

(l) In the 9 cm band:

(1) In ITU Regions 2 and 3, the band is allocated to the

amateur service on a secondary basis.

(2) In the United States, the band is allocated to the amateur

service on a co-secondary basis with the non-Government radiolocation

service.

(3) In the 3.3-3.4 GHz segment, no amateur station shall cause

harmful interference to, nor is protected from interference due to the

operation of, stations authorized by other nations in the radiolocation

service.

(4) In the 3.4-3.5 GHz segment, no amateur station shall cause

harmful interference to, nor is protected from interference due to the

operation of, stations authorized by other nations in the fixed and

fixed-satellite service.

(m) In the 5 cm band:

(1) In the 5.650-5.725 GHz segment, the amateur service is

allocated in all ITU Regions on a co-secondary basis with the space

research (deep space) service.

(2) In the 5.725-5.850 GHz segment, the amateur service is

allocated in all ITU Regions on a secondary basis.  No amateur station

shall cause harmful interference to, nor is protected from interference

due to the operation of, stations authorized by other nations in the

fixed-satellite service in ITU Region 1.

(3) No amateur station transmitting in the 5.725-5.875 GHz

segment is protected from interference due to the operation of

industrial, scientific and medical devices operating on 5.8 GHz.

(4) In the 5.650-5.850 GHz segment, no amateur station shall

cause harmful interference to, nor is protected from interference due

to the operation of, stations authorized by other nations in the

radiolocation service.

(5) In the 5.850-5.925 GHz segment, the amateur service is

allocated in ITU Region 2 on a co-secondary basis with the radiolocation

service.  In the United States, the segment is allocated to the amateur

service on a secondary basis to the non-Government fixed-satellite

service.  No amateur station shall cause harmful interference to, nor

is protected from interference due to the operation of, stations

authorized by other nations in the fixed, fixed-satellite and mobile

services.  No amateur station shall cause harmful interference to, nor

is protected from interference due to the operation of, stations in the

non-Government fixed-satellite service.

(n) In the 3 cm band:

(1) In the United States, the 3 cm band is allocated to the

amateur service on a co-secondary basis with the non-government

radiolocation service.

(2) In the 10.00-10.45 * ("GHz" missing) segment in ITU

Regions 1 and 3, no amateur station shall cause interference to, nor is

protected from interference due to the operation of, stations authorized

by other nations in the fixed and mobile services.

(o) No amateur station transmitting in the 1.2 cm band is protected

from interference due to the operation of industrial, scientific and

medical devices on 24.125 GHz.  In the United States, the 24.05-24.25

GHz segment is allocated to the amateur service on a co-secondary basis

with the non-government radiolocation and the Government and

non-government Earth exploration-satellite (active) services.

(p) The 2.5 mm band is allocated to the amateur service on a

secondary basis.  No amateur station shall cause harmful interference

to, nor is protected from interference due to the operation of, stations

in the fixed, inter-satellite and mobile services.

(q) No amateur station transmitting in the 244-246 GHz segment of

the 1 mm band is protected from interference due to the operation of

industrial, scientific and medical devices on 245 GHz.

97.305  Authorized emission types.

(a) An amateur station may transmit a CW emission on any frequency

authorized to the control operator.

(b) A station may transmit a test emission on any frequency

authorized to the control operator for brief periods for experimental

purposes, except that no pulse modulation emission may be transmitted

on any frequency where pulse is not specifically authorized.

(c) An amateur station may transmit the following emission types

on the frequencies indicated, subject to the standards specified in

section 97.307(f) of this Part.

WAVELENGTH FREQUENCIES      EMISSION TYPES             STANDARDS (see

BAND                       AUTHORIZED          97.307(f), paragraph:)

MF:

160 m   Entire band       Phone, image, RTTY,

data                 (1), (2), and (3).

HF:

80 m    Entire band       RTTY, data           (3), (9).

75 m    Entire band       Phone, image         (1), (2).

40 m    7.000-7.075 MHz   RTTY, data           (c)*, (9).

(*-"c" should be "3"?)

40 m    7.075-7.100 MHz   Phone, image         (1), (2), (9) and (11).

40 m    7.10-7.15 MHz     RTTY, data           (1), (9).

40 m    7.15-7.30 MHz     Phone, image         (1), (2).

30 m    Entire band       RTTY, data           (3).

20 m    14.00-14.15 MHz   RTTY, data           (3).

20 m    14.15-14.35 MHz   Phone, image         (1), (2).

17 m    18.068-18.110 MHz RTTY, data           (3).

17 m    18.110-18.168 MHz RTTY, data           (3).

15 m    21.0-21.2 MHz     RTTY, data           (3), (9).

15 m    21.20-21.45 MHz   Phone, image         (1), (2)

12 m    24.89-24.93 MHz   RTTY, data           (3).

12 m    24.93-24.99 MHz   Phone, image         (1), (2).

10 m    28.0-28.3 MHz     RTTY, data           (4).

10 m    28.3-29.5 MHz     Phone, image         (1), (2), and (10).

10 m    28.5-29.0 MHz     Phone, image         (1), (2).

10 m    29.0-29.7 MHz     Phone, image         (1).

VHF:

6 m     50.1-51.0 MHz     MCW, phone, image,

RTTY, data           (2), (5).

6 m     51.0-54.0 MHz     MCW, phone, image,

RTTY, data, test     (2), (5), and (8).

2 m     144.1-148.0 MHz   MCW, phone, image,

RTTY, data, test     (2), (5), and (8).

1.25 m  Entire band       MCW, phone, image,

RTTY, data, test     (2), (5), and (8).

(*-missing here was "UHF:")

70 m*   Entire band       MCW, phone, image,

RTTY, data, test     (6), (8).

(*-"m" should read "cm" here and through the SHF category.)

33 m*   Entire band       MCW, phone, image,

RTTY, data, test,

pulse                (7), (8), and (12).

23 m*   Entire band       MCW, phone, image,

RTTY, data, SS, test (7), (8), and (12).

13 m*   Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

SHF:

9 m*    Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

5 m*    Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

3 m*    Entire band       MCW, phone, image,

RTTY, data, SS, test (7), (8), and (12).

1.2 m*  Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

EHF:

6 m*    Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

(*-"m" should read "mm" here and through rest of paragraph.)

4 m*    Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

2.5 m*  Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

2 m*    Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

1 m*    Entire band       MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

---     Above 300 GHz     MCW, phone, image,

RTTY, data, SS,

test, pulse          (7), (8), and (12).

97.307  Emission standards.

(a) No amateur station transmission shall occupy more bandwidth

than necessary for the information rate and emission type being

transmitted, in accordance with good amateur practice.

(b) Emissions resulting from modulation must be confined to the

band or segment available to the control operator.  Emissions outside

the necessary bandwidth must not cause splatter or keyclick interference

to operations on adjacent frequencies.

(c) All spurious emissions from a station transmitter must be

reduced to the greatest extent practicable.  If any spurious emission,

including chassis or power line radiation, causes harmful interference

to the reception of another radio station, the licensee of the

interfering station is required to take steps to eliminate the

interference, in accordance with good engineering practice.

(d) The mean power of any spurious emission from a station

transmitter or external RF power amplifier transmitting on a frequency

below 30 MHz must not exceed 50 mW and must be at least 40 dB below the

mean power of the fundamental emission. For a transmitter of mean power

less than 5 W, the attenuation must be at least 30 dB.  A transmitter

built before April 15, 1977 or first marketed before January 1, 1978 is

exempt from this requirement.

(e) The mean power of any spurious emission from a station

transmitter or external RF power amplifier transmitting on a frequency

between 30-225 MHz must be at least 60 dB below the mean power of the

fundamental.  For a transmitter having a mean power of 25 W or less, the

mean power of any spurious emission supplied to the antenna transmission

line must not exceed 25 uW and must be at least 40 dB below the mean

power of the fundamental emission, but need not be reduced below the

power of 10 uW.  A transmitter built before April 15, 1977 or first

marketed before January 1, 1978, is exempt from this requirement.

(f) The following standards and limitations apply to transmissions

on the frequencies specified in section 97.305(c) of this Part.

(1) No angle-modulated emission may have a modulation index

greater than 1 at the highest modulation frequency.

(2) No non-phone emission may exceed the bandwidth of a

communications quality phone emission of the same modulation type.  The

total bandwidth of an independent sideband emission (having B as the

first symbol), or a multiplexed image and phone emission, shall not

exceed that of a communications quality A3E emission.

(3) Only a RTTY and data emission using a specified digital

code listed in section 97.309(a) of this Part may be transmitted.  The

symbol rate must not exceed 300 bauds, or for frequency-shift keying,

the frequency shift between mark and space must not exceed 1 kHz.

(4) Only a RTTY and data emission using a specified digital

code listed in section 97.309(a) of this Part may be transmitted.  The

symbol rate must not exceed 1200 bauds, or for frequency-shift keying,

the frequency shift between mark and space must not exceed 1 kHz.

(5) A RTTY, data or multiplexed emission using a specified

digital code listed in section 97.309(a) of this Part may be

transmitted.  The symbol rate must not exceed 19.6 kilobauds, or for

frequency-shift keying, the frequency shift between mark and space must

not exceed 1 kHz.  A RTTY, data or multiplexed emission using an

unspecified digital code under the limitations listed in section

97.309(b) of this Part also may be transmitted.  The authorized

bandwidth is 20 kHz.

(6) A RTTY, data or multiplexed emission using a specified

digital code listed in section 97.309(a) of this Part may be

transmitted.  The symbol rate must not exceed 56 kilobauds, or for

frequency-shift keying, the frequency shift between mark and space must

not exceed 1 kHz.  A RTTY, data or multiplexed emission using an

unspecified digital code under the limitations listed in section

97.309(b) of this Part also may be transmitted.  The authorized

bandwidth is 100 kHz.

(7) A RTTY, data or multiplexed emission using a specified

digital code listed in section 97.309(a) of this Part or an unspecified

digital code under the limitations listed in section 97.309(b) of this

Part may be transmitted.

(8) A RTTY or data emission having designators with A, B, C,

D, E, F, G, H, J or R as the first symbol; 1, 2, 7 or 9 as the second

symbol; and D or W as the third symbol is also authorized.

(9) A station having a control operator holding a Novice or

Technician operator license may only transmit a CW emission using the

international Morse code.

(10) A station having a control operator holding a Novice or

Technician operator license may only transmit a CW emission using the

international Morse code or phone emissions J3E and R3E.

(11) Phone and image emissions may be transmitted only by

stations located in ITU Regions 1 and 3, and by stations located within

ITU Region 2 that are west of 130 degrees West longitude or south of 20

degrees North latitude.

(12) Emission F8E may be transmitted.

97.309  RTTY and digital emission codes.

(a) Where authorized by Sections 97.305(c) and 97.307(f) of this

Part, an amateur may transmit a RTTY or data emission using the

following specified digital codes:

(1) The 5-unit, start-stop, International Telegraph Alphabet

No. 2, code defined in International Telegraph and Telephone

Consultative Committee Recommendation F.1, Division C, and extensions

as provided for in CCITT Recommendation T.61 (Malaga-Torremolinos,

1984).

(2) The 7-unit code specified in International Radio

Consultative Committee Recommendation CCIR 476-1 (1978) 476-3 (1982),

476-4 (1986) or 625 (1986).

(3) The 7-unit code defined in American National Standards

Institute X3.4-1977 or International Alphabet No. 5 as defined in

International Telegraph and Telephone Consultative Committee

Recommendation T.50 or in International Organization for

Standardization, International Standard ISO 646 (1983).

(b) Where authorized by Sections 97.305(c) and 97.307(f) of this

Part, a station may transmit a RTTY or data emission using an

unspecified digital code, except to a station in a country with which

the United States does not have an agreement permitting the code to be

used.  RTTY and data emissions using unspecified digital codes must not

be transmitted for the purpose of obscuring the meaning of any

communication.  When deemed necessary by an EIC to assure compliance

with the FCC Rules, a station must:

(1) Cease the transmission using the unspecified digital code;

(2) Restrict the transmission of any digital code to the

extent instructed;

(3) Maintain a record, convertible to the original

information, of all coded communications transmitted.

97.311  SS emission types.

(a) SS emission transmissions by an amateur station are authorized

only for communications between points within areas where the amateur

service is regulated by the FCC.  SS emission transmissions must not be

used for the purpose of obscuring the meaning of any communication.

(b) Stations transmitting SS emission must not cause harmful

interference to stations employing other authorized emissions, and must

accept all interference caused by stations employing other authorized

emissions.  (For the purposes of this paragraph, unintended triggering

of carrier operated repeaters is not considered to be harmful

interference.)

(c) Only the following types of SS emission transmissions are

authorized (hybrid SS emission transmissions involving both spreading

techniques are prohibited):

(1) Frequency hopping where the carrier of the transmitted

signal is modulated with unciphered information and changes frequency

at fixed intervals under the direction of a high speed code sequence.

(2) Direct sequence where the information is modulo-2 added

to a high speed code sequence.  The combined information and code are

then used to modulate the RF carrier.  The high speed code sequence

dominates the modulation function, and is the direct cause of the wide

spreading of the transmitted signal.

(d) The only spreading sequences that are authorized must be from

the output of one binary linear feedback shift register (which may be

implemented in hardware or software).

(1) Only the following sets of connections may be used:

Number of stages                    Taps used

in shift register                  in feedback

7                              7, 1.

13                          13, 4, 3, and 1.

19                          19, 5, 2, and 1.

(2) The shift register must not be reset other than by its

feedback during an individual transmission.  The shift register output

sequence must be used without alteration.

(3) The output of the last stage of the binary linear feedback

shift register must be used as follows:

(i) For frequency hopping transmissions using x

frequencies, n consecutive bits from the shift register must be used to

select the next frequency from a list of frequencies sorted in ascending

order.  Each consecutive frequency must be selected by a consecutive

block of n bits.  (Where n is the smallest integer greater than log2(x).

(ii) For direct sequence transmissions using m-ary

modulation, consecutive blocks of log2 m bits from the shift register

must be used to select the transmitted signal during each interval.

(e) The station records must document all SS emission transmissions

and must be retained for a period of 1 year following the last entry.

The station records must include sufficient information to enable the

FCC, using the information contained therein, to demodulate all

transmissions.  The station records must contain at least the following:

(1) A technical description of the transmitted signal;

(2) Pertinent parameters describing the transmitted signal

including the frequency or frequencies of operation and, where

applicable, the chip rate, the code rate, the spreading function, the

transmission protocol(s) including the method of achieving

synchronization, and the modulation type;

(3) A general description of the type of information being

conveyed (voice, text, memory dump, facsimile, television, etc.);

(4) The method and, if applicable, the frequency or

frequencies used for station identification;

(5) The date of beginning and the date of ending use of each

type of transmitted signal.

(f) When deemed necessary by an EIC to assure compliance with the

rules of this part, a station licensee must:

(1) Cease SS emission transmissions;

(2) Restrict SS emission transmissions to the extent

instructed;

(3) Maintain a record, convertible to the original information

(voice, text, image, etc.) of all spread spectrum communications

transmitted.

(g) The transmitter power must not exceed 100 W.

97.313  Transmitter power standards.

(a) An amateur station must use the minimum transmitter power

necessary to carry out the desired communications.

(b) No station may transmit with a transmitter power exceeding 1.5

kW PEP.  Until June 2, 1990, a station transmitting emission A3E is

exempt from this requirement provided the power input (both RF and

direct current) to the final amplifying stage supplying RF power to the

antenna feed line does not exceed 1 kW, exclusive of power for heating

the cathodes of vacuum tubes.

(c) No station may transmit with a transmitter power exceeding 200

W PEP on:

(1) The 3.70-3.75 MHz, 7.10-7.15 MHz, 10.10-10.15 MHz and

21.1-21.2 MHz segments;

(2) The 28.1-28.5 MHz segment when the control operator is a

Novice or Technician operator; or

(3) The 7.050-7.075 MHz segment when the station is within ITU

Regions 1 or 3.

(d) No station may transmit with a transmitter power exceeding 25

W PEP on the VHF 1.25 m band when the control operator is a Novice

operator.

(e) No station may transmit with a transmitter power exceeding 5

W PEP on the UHF 23 cm band when the control operator is a Novice

operator.

(f) No station may transmit with a transmitter power exceeding 50

watts PEP on the UHF 70 cm band from an area specified in footnote US7

to Section 2.106 of the FCC Rules, unless expressly authorized by the

FCC after mutual agreement, on a case-by-case basis, between the EIC of

the applicable field facility and the military area frequency

coordinator at the applicable military base.  An Earth station or

telecommand station, however, may transmit on the 435-438 MHz segment

with a maximum of 611 W effective radiated power (1 kW equivalent

isotropically radiated power) without the authorization otherwise

required.  The transmitting antenna elevation angle between the lower

half-power (-3 dB relative to the peak or antenna bore sight) point and

the horizon must always be greater than 10 degrees.

(g) No station may transmit with a transmitter power exceeding 50

watts PEP on the 33 cm band from within 241 km of the boundaries of the

White Sands Missile Range.  Its boundaries are those portions of Texas

and New Mexico bounded on the south by latitude 31 degrees 41 minutes

North, on the east by longitude 104 degrees 11 minutes West, on the

north by latitude 34 degrees 30 minutes North, and on the west by

longitude 107 degrees 30 minutes West.

97.315  Type acceptance of external RF power amplifiers.

(a) No more than 1 unit of 1 model of an external RF power

amplifier capable of operation below 144 MHz may be constructed or

modified during any calendar year by an amateur operator for use at a

station without a grant of type acceptance.  No amplifier capable of

operation below 144 MHz may be constructed or modified by a non-amateur

operator without a grant of type acceptance from the FCC.

(b) Any external RF power amplifier or external RF power amplifier

kit (see section 2.815 of the FCC Rules), manufactured, imported or

modified for use in a station or attached at any station must be type

accepted for use in the amateur service in accordance with Subpart J of

Part 2 of the FCC rules.  This requirement does not apply if one or more

of the following conditions are met:

(1) The amplifier is not capable of operation on frequencies

below 144 MHz.  For the purpose of this part, an amplifier will be

deemed to be incapable of operation below 144 MHz if it is not capable

of being easily modified to increase its amplification characteristics

below 120 MHz and either:

(i) The mean output power of the amplifier decreases, as

frequency decreases from 144 MHz, to a point where 0 dB or less gain is

exhibited at 120 MHz; or

(ii) The amplifier is not capable of amplifying signals

below 120 MHz even for brief periods without sustaining permanent damage

to its amplification circuitry.

(2) The amplifier was originally purchased before April 28,

1978, and has been issued a marketing waiver by the FCC, or the

amplifier was purchased before April 28, 1978, by an amateur operator

for use at that amateur operator's station.

(3) The amplifier was:

(i) Constructed by the licensee, not from an external RF

power amplifier kit, for use at the licensee's amateur station; or

(ii) Modified by the licensee for use at the licensee's

amateur station.

(4) The amplifier is sold by an amateur radio operator to

another amateur radio operator or to a dealer.

(5) The amplifier is purchased in used condition by an

equipment dealer from an amateur radio operator and the amplifier is

further sold to another amateur radio operator for use at that

operator's amateur station.

(c) A list of type accepted equipment may be inspected at FCC

headquarters in Washington, DC or at any FCC field location.  Any

external RF power amplifier appearing on this list as type accepted for

use in the amateur service may be marketed for use in the amateur

service.

97.317  Standards for type acceptance of external RF power amplifiers.

(a) To receive a grant of type acceptance, the amplifier must

satisfy the spurious emission limitations of section 97.401(d) or (e)

of this Part, as applicable, when the amplifier is:

(1) Operated at its full output power;

(2) Placed in the "standby" or "off" positions, but still

connected to the transmitter; and

(3) Driven with at least 50 W mean RF input power (unless

higher drive level is specified).

(b) To receive a grant of type acceptance, the amplifier must not

be capable of operation on any frequency or frequencies between 24 MHz

and 35 MHz.  The amplifier will be deemed incapable of such operation

if it:

(1) Exhibits no more than 6 dB gain between 24 MHz and 26 MHz

and between 28 MHz and 35 MHz.  (This gain will be determined by the

ratio of the input RF driving signal (mean power measurement) to the

mean RF output power of the amplifier); and

(2) Exhibits no amplification (0 dB gain) between 26 MHz and

28 MHz.

(c) Type acceptance may be denied when denial would prevent the

use of these amplifiers in services other than the amateur service.  The

following features will result in dismissal or denial of an application

for the type acceptance:

(1) Any accessible wiring which, when altered, would permit

operation of the amplifier in a manner contrary to the FCC Rules;

(2) Circuit boards or similar circuitry to facilitate the

addition of components to change the amplifier's operating

characteristics in a manner contrary to the FCC Rules;

(3) Instructions for operation or modification of the

amplifier in a manner contrary to the FCC Rules;

(4) Any internal or external controls or adjustments to

facilitate operation of the amplifier in a manner contrary to the FCC

Rules;

(5) Any internal RF sensing circuitry or any external switch,

the purpose of which is to place the amplifier in the transmit mode;

(6) The incorporation of more gain in the amplifier than is

necessary to operate in the amateur service; for purposes of this

paragraph, the amplifier must:

(i) Not be capable of achieving designed output power

when driven with less than 40 W mean RF input power;

(ii) Not be capable of amplifying the input RF driving

signal by more than 15 dB, unless the amplifier has a designed

transmitter power of less than 1.5 kW (in such a case, gain must be

reduced by the same number of dB as the transmitter power relationship

to 1.5 kW; this gain limitation is determined by the ratio of the input

RF driving signal to the RF output power of the amplifier where both

signals are expressed in peak envelope power or mean power);

(iii) Not exhibit more gain than permitted by paragraph

(c)(6)(ii) of this Section when driven by an RF input signal of less

than 50 W mean power; and

(iv) Be capable of sustained operation at its designed

power level.

(7) Any attenuation in the input of the amplifier which, when

removed or modified, would permit the amplifier to function at its

designed transmitter power when driven by an RF frequency input signal

of less than 50 W mean power; or

(8) Any other features designed to facilitate operation in a

telecommunication service other than the Amateur Radio Services, such

as the Citizens Band (CB) Radio Service.