WELCOME TO THE SSTV AND FAX PROGRAM SSTVFAX4 VERSION 4.45

AUTHOR:ON5KN BBS @ON7RC-5

BELGIUM

***THIS IS NOT A PERFECT PACKAGE AND WILL NEVER BE***

THIS IS AN EXPERIMENTAL RECEIVE AND TRANSMIT PROGRAM WRITTEN IN

ASSEMBLER AND TO USE ON ANY IBM COMPATIBLE UNDER MSDOS

THE UPSET WAS MADE TO BE ABLE TO RECEIVE AND TRANSMIT SSTV AND

FAX WITH A MINIMUM OF ADDITIONAL EXTERNAL HARDWARE

FOR RECEPTION OF BOTH MODES YOU ONLY NEED A SIMPLE 741-IC

TRANSMITTING ONLY 4 RESISTORS AND 2 CAPACITORS ARE NEEDED

RECEIVE HARDWARE IS POWERSUPPLIED BY THE RS232 FROM THE PC

THE SCHEMATICS ARE ADDED IN THE FILE SSTVFAX4.SCH WICH YOU

SIMPLY CAN PRINT UNDER MSDOS ON	A GRAPHICS PRINTER

USAGE: COPY SSTVFAX4.SCH PRN /B

THE INTERFACE IS BUILD WITHOUT ANY ADJUSMENT COMPONENT AND

DOES NOT REQUIRE ANY ADJUSTMENT

ALTHOUGH THE OUTPUT LEVEL TO YOUR TX-INPUT CAN BE ADAPTED

TO YOUR NEED BY CHANGING THE VALUE OF R4 (22K)

A 16-LEVEL GREYSCALE IS AUTOMATED ON TOP OF EACH SSTV PICTURE

SSTV CAN BE RECEIVED FROM ALL ACTUAL PRESENT MODES HOWEVER

THE IMAGES WILL BE DISPLAYED IN GREY-LEVELS EVEN FOR COLOUR

SENDED PICTURES

TRANSMISSION OF SSTV IS POSSIBLE IN 16 GREY-LEVELS AND IN

FOUR STANDARD MODES 8SEC, 16SEC, 32SEC,(NORMAL) AND 64SEC

(IN DOUBLE MODE)

INITIAL FORMAT IS 8SEC

WHEN POINTING THE UPPER-LEFT CORNER YOU ARE FREE TO CHOICE

THIS POINT ANYWHERE ON THE SCREEN

WHEN POINTING THE BOTTEM-RIGHT CORNER ACCEPTING THE MOUSE

POSITION WILL KEEP A 1 TO 1 RATIO

MOVING THE MOUSE ON BOTTEM-RIGHT WILL REDUCE OR ENLARGE THE

TRANSMITTED PORTION OF THE SCREEN ALTHOUGH MAINTAINING IT

IN THE SELECTED MODE BUT TRY TO DISPLACE DIAGONALLY

FAX WILL BE RECEIVED AND TRNSMITTED IN 60,90,120,180,240,

AND 480 LPM IN IOC 288 AT NORMAL AND IN IOC 576 IN DOUBLE

480 LPM WILL GIVE YOU DRAFT QUALITY

TRANSMISSION OF FAX WILL SEND EACH LINE TWICE IN NORMAL AND

FOUR TIMES IN DOUBLE

IN FAX THE FORMAT FUNCTION WILL SELECT THE PORTION OF SCREEN

WICH WILL BE TRANSMITTED IN A 1 TO 1 RATIO

ON STARTING UP THE PROGRAM SSTVFAX4.COM YOU MAY ADD A TIMING

CORRECTION PARAMETER I.E.   SSTVFAX4.COM 2\E\-30

THE 2 = COM PORT IN USE (MAY BE 1 OR 2)

THE E = EGA VIDEO MODE (MAY BE C, H, E, OR V)

THE -30 = TIMING CORRECTION FOR FAX (MAY BE FROM +99 TO -99)

TO DETERMINE THIS FACTOR TWO WAYS OF DOING

THE DIFFICULT ONE IS RECEIVING A FAX OF WICH YOU ARE SURE OF

ACCURACY AND TRY AND CORRECT IN STEPS

THE EASY WAY IS TO LOCATE YOUR PC-OSCILLATOR ON 14MHZ BAND

THE STANDARD IS 1.193.180 HZ * 12 = 14.318.160 HZ

SO YOU MAY EXPECT TO FIND IT BETWEEN 14.300 KHZ AND 14.340 KHZ

RECEIVE IT USING A SHORT WIRE FROM THE ANTENNA TO THE PC

AND READ THIS FREQUENCY AS F0 IN HZ

DIVIDE FO BY 240 GIVING F1

SUBSTRACT FROM 59.682 THIS F1 GIVING THE TIMING CORRECTOR AND SIGN

AS AN EXAMPLE YOUR ZEROBEAT IS 14.330.880 HZ AS F0

THIS GIVES US 14.330.880 / 240 = 59712 AS F1

AND 59.682 - 59.712 = -30 AS CORRECTION FACTOR

EVEN A READING ERROR OF 1.000 HZ WILL GIVE A GOOD APPROACH

THE RECEPTION SCREEN CAN BE ONE OF THE CURRENT MODES EXCEPT

S-VGA WICH IS NOT YET SUPPORTED

SOME VIDEO CARDS MAY NOT BE COMPATIBLE

IN SSTV BEST SYNC WILL BE OBTAINED WHEN RIGHT BOTTOM OF IMAGE

IS POINTED WITH RIGHT MOUSE BUTTON

START POINT OF IMAGE ON SCREEN CAN BE REDEFINED WITH MOUSE-

POINTING AND LEFT MOUSE BUTTON

ON SOME MODES OF COLOUR SSTV IT MAY BE INTERESTING TO USE

DOUBLE COMPACTATION

ON RECEPTION OF 96 SEC OR MORE IMAGES USE OF "U" TO DISABLE

VERTICAL SYNC UNTIL NEXT "T" AVOID SPURIOUS SYNC BY QRM

AS TIMING IS CRITICAL ON SUCH AN APPLICATION THE POSSIBILITIES

ARE FUNCTION OF THE SPEED OF YOUR PC AND TYPE OF SCREEN

THE PROGRAM WAS WRITTEN ON  AT 286/12MC 640K  VGA 256K

MAINLY CGA AND HERCULES MODES ARE NOT QUALIFIED BUT CAN BE

BE USED WITH LIMITED RX-ONLY FUNCTIONS

A LOWER SPEED LIMIT IS BUILT-IN ON THE START-UP AND IF BELOW

PROGRAM WILL ABORT

THE MINIMUM MEMORY SPACE REQUIRED IS 400KB IF BELOW PROGRAM

WILL ABORT ON START

A FEATURE WICH EXPANDS THE POSSIBILITIES IS A STANDARD

MOUSE WITH MOUSE-DRIVER

THE BUILT-IN SCAN FUNCTION IS BASED ON THE GENIUS GS-4500

THE SSTVFAX4 PROGRAM DRIVES DIRECTLY ITS INTERFACE CARD

AND CONVERTS ITS BLACK AND WHITE DOTS TO A 16 GREY LEVEL

THE AMOUNT OF FREE MEMORY DETERMINES THE MAXIMUM SCAN-LENGHT

BEST RESULTS ARE OBTAINED IN HORIZONTAL SCANNING AND EGA

ALL PICTURES CAN BE STORED ON AND RETRIEVED FROM DISK IN

GIF87a FORMAT. THE 4-GREY-PLANES FORMAT OF SSTVFAX3 AND

SSTVFAX2 IS STILL SUPPORTED FOR LOADING PICTURES.

IF YOUR SYSTEM IS EQUIPED WITH ANOTHER SCANNER YOU ARE ABLE

TO CREATE YOUR OWN PICTURES AND IMPORT THEM AS GIF-FILES

MAKING SLOWSCAN AND FAX ON A REAL LOW-BUDGET WAS THE

MAIN TARGET OF THIS PACKAGE BY INTRODUCING THESE MODES OF

RECEPTION AND TRANSMISSION AS A SIMPLE ADD TO THE BECOMING

COMMON-USE-PC IN THE RADIO-AMATEUR WORLD

TRY FIRST THE RECEPTION MODULE !

IT IS EASY AND YOU WILL SEE THE ACTIVITIES ON HF AND VHF

NEAR 14.230KHZ YOU ARE SURE TO SEE THEM FIRST

ENJOY THIS FREE SOFTWARE PACKAGE AND LET ME KNOW WHAT YOU

ARE THINKING ABOUT AND GIVE SOME SUGGESTIONS

MAKE REFERENCE TO SSTVFAX4 VERSION 4.45

IF YOU MAKE A COPY FOR FURTHER DISTRIBUTION PLEASE LEAVE IT

IN THE SELF-EXPLODING FORM SSTVFAX4.COM OR AT LEAST BE SURE

THAT IT INCLUDES SSTVFAX4.TXT/SSTVFAX4.SCH/SSTVFAX4.EXE

AND THIS SSTVFAX4.DOC FILES

LAST REMARK ... YOUR TIMER MAY BE DISTURBED AFTER RUNNING

SSTVFAX4 JUST TILL ANOTHER START-UP OF YOUR PC

HAVE A LOT OF FUN AND SEE YOU SOON ON THE BANDS

73 ON5KN BBS @ON7RC-5 BELGIUM