HamComm

Version 2.3

February 5th 1994

W. F. Schroeder

DL5YEC

HamComm 2.3                                 Table of Contents

Table of Contents

1 Introduction  ........................................... 1

2 Trial Use License ....................................... 1

3 Exclusion of warranty ................................... 2

4 System requirements ..................................... 2

5 Installation  ........................................... 3

6 Startup ................................................. 3

7 Config file HC.CFG  ..................................... 4

8 Screen layout ........................................... 4

9 Help system ............................................. 5

10 Interface schematics ................................... 5

11 Receive circuit  ....................................... 5

12 PTT circuit  ........................................... 6

13 Transmit circuit ....................................... 6

14 External converter ..................................... 7

15 External AFSK  ......................................... 7

16 Signal decoding  ....................................... 8

17 Macro keys ............................................. 8

18 Macro files  ........................................... 9

19 SHIP/SYNOP decoder ..................................... 9

20 Scope function ........................................ 12

21 Spectrum function  .................................... 13

22 Bitlength statistics .................................. 15

23 Converter checkout .................................... 16

24 Port address .......................................... 16

25 Op-Amp power supply  .................................. 18

26 IRQ number ............................................ 20

27 Appendix A - Interface schematics  .................... 21

28 Appendix B - Key assignment summary  .................. 22

i

HamComm 2.3                Manual              February 5th 1994

1.  Introduction

HamComm  is  a program for ham radio communications.  Version

2.2  supports  reception  and  transmission  of amateur radio

teletype  and Morse code signals.  Weather station reports in

the SHIP and SYNOP format can also be decoded.

A  converter or modem chip is not required.  The audio output

of  the  receiver  is  connected  to  the  serial port of any

PC/XT/AT  compatible computer thru a very simple and low-cost

circuit.  Only one IC is needed (Op-Amp LM741 or similar) and

a  few  diodes, capacitors and resistors.  The supply current

is  drawn  from  the serial port.  For transmission the audio

signal  from  the  speaker or serial port is connected to the

microphone  input  of  the  transmitter  thru  a  passive R/C

filter.     Audio    frequency   generation   and   decoding,

serial/parallel conversion and all other signal processing is

done by the program.

2.  Trial Use License

HamComm 2.3 is NOT a public domain program.

Copyright (c) 1990,1991,1992,1993 and 1994 by W. F. Schroeder.

All rights reserved.

You are hereby granted a limited license to use this software

for  evaluation  purposes  for  a  period of 30 days.  If you

intend  to  continue  using  this  software  after the 30 day

evaluation  period,  you  must make a registration payment of

$20 or DM30 to:

W. F. Schroeder

Augsburger Weg 63

D-33102 Paderborn

Germany

Please  note:  the bank will charge up to $5 to cash a cheque

from   a  foreign  country.   For  this  reason  please  send

banknotes or add another $5 to your cheque.

- 1 -

HamComm 2.3                Manual              February 5th 1994

3.  Exclusion of warranty

The  author  provides  absolutely  no warranty, to the extent

permitted  by  applicable  state  law.  Except when otherwise

stated  in  writing,  the author and/or other parties provide

this  program  "AS  IS"  without warranty of any kind, either

expressed  or  implied,  including,  but  not limited to, the

implied  warranties  of  merchantability  and  fitness  for a

particular  purpose.   The  entire risk as to the quality and

performance  of  the program is with you.  Should the program

prove  defective,  you  assume  the  cost  of  all  necessary

servicing, repair or correction.

In no event unless required by applicable law will the author

and/or  any  other  party  who may modify and/or redistribute

this program be liable to you for damages, including any lost

profits,   lost  monies,  or  other  special,  incidental  or

consequential  damages arising out of the use or inability to

use  (including but not limited to loss of data or data being

rendered inaccurate or losses sustained by third parties or a

failure  of  the  program to operate with any other programs)

this   program,   even  if  you  have  been  advised  of  the

possibility  of  such  damages, or for any claim by any other

party.

4.  System requirements

HamComm 2.3  will  run  under  MS-DOS  3.x  or higher on any

PC/XT/AT-compatible  computer  with  at  least  320KB of free

memory.   A  harddisk is recommended.  Because of the size of

the  program  file  there may be not enough space to copy all

required  files onto a 720KB diskette.  On slow systems, e.g.

8088  CPU, some functions may not work as expected or may not

work at all.

HamComm  will  automatically detect the type of video adapter

in  use.  MDA, CGA, EGA, VGA and Hercules are supported.  The

graphics  display  functions  are not available on MDAs since

the  MDA  has  no graphics mode.  No attempt has been made to

avoid screen flicker (snow) on cheap CGAs.

HamComm  will probably not run under any kind of multitasking

software  like Desqview, Windows, Windows NT or OS/2 since it

needs  direct control of the interrupt controller, timer chip

and serial I/O hardware.

- 2 -

HamComm 2.3                Manual              February 5th 1994

5.  Installation

For   installation   on   a  harddisk  you  should  create  a

subdirectory  HAMCOMM  and  CD to that directory.  HamComm is

distributed  as  a  .ZIP or .EXE file.  To unpack a .ZIP file

you  need  a program like UNZIP or PKUNZIP.  The .EXE version

is  a  'self-extracting  archive' that contains all the files

and  a build-in unpack program.  In both cases the files will

be created in the current directory.

Please read the files README, HC.CFG and CHANGES !

6.  Startup

To  start  HamComm  type  HC at the DOS prompt and hit ENTER.

The  interface circuit is not needed if you just want to play

with the program.

HamComm  normally uses the currently selected video mode.  If

you  have  an EGA or VGA card with an EGA/VGA monitor you can

also  switch to 43 or 50 lines.  Try option -L43 to switch to

43  lines on EGA/VGA cards or -L50 for 50 lines on VGA cards.

If  the  card  already  is in one of these modes, option -L25

selects 25 lines.

Many  SuperVGA  cards  have special text modes e.g.  80x60 or

132x44.   Activate  the desired mode before starting HamComm.

Most  cards come with a utility program to do this.  Only the

modes  where  the video display buffer starts at segment B800

are supported.

HamComm  assumes  that  the original PC display character set

known as "codepage 437" is active.

On  startup the program identifies the video card and monitor

in  use  and  automatically  selects  the  graphics  mode for

highest   resolution.    Some  program  functions  require  a

graphics   mode   for  display  of  the  input  signal.   The

identification  may  fail  because of compatibility problems.

By selecting one of the following commandline options HamComm

can be forced to use the specified mode:

Option  Mode      Resolution

-cga    6        640x200  2-color

-ega   10h       640x350 16-color

-vga   12h       640x480 16-color

-herc  Hercules  720x348  2-color

- 3 -

HamComm 2.3                Manual              February 5th 1994

If  you  have  a computer with LCD display and readability is

poor try the following DOS command before starting HamComm:

mode bw80

The  'mode'  program is supplied with MS-DOS and is used here

to switch to black and white mode.

7.  Config file HC.CFG

When  HamComm  is  started the program automatically searches

for  a  configuration  file  called  "HC.CFG".   The  current

directory is searched first, then all other directories along

the  current  "PATH".   The  supplied HC.CFG file is an ASCII

text file and can be sent to a printer or changed with a text

editor.   Please  take the time to read it carefully and make

the  required  adjustments  for  your  system.   At least you

should substitute DL5YEC with your own callsign.

HC.CFG  includes  explanations and examples for all available

configuration  commands.   It  is  only special in that it is

executed  automatically  on  program  startup.   You can also

write your own .CFG files and execute them from the FILE menu

while  running  HamComm.   This  could be used to set several

parameters like baud and shift to special values in one step.

You could also build a set of .CFG files for loading standard

texts  in different languages.  In combination with the macro

keys they could then be loaded with a single key.

8.  Screen layout

HamComm  has an SAA-like user interface with pull-down menus,

dialog  boxes  and online helptexts.  The menu bar at the top

of  the screen is always visible while the display is in text

mode.   The  bottom  line has information about the currently

selected menu item.

Every  name  on  the  menu  bar  has a highlighted character.

Press  and hold the ALT key and type that character to select

the  corresponding  menu.   The  cursor-left and cursor-right

keys  can  now  be  used to switch to the previous/next menu.

Use the cursor-up and cursor-down keys to move to the desired

function and hit the ENTER key.

Every  menu  line  also  has a highlighted character.  Typing

that character (without the ALT key) will get you directly to

the corresponding function.

- 4 -

HamComm 2.3                Manual              February 5th 1994

Some  frequently  used  menu  entries  have  been assigned to

function  keys.   The  F8 key for example activates the SCOPE

function.

The  ESC  key  can  be  used  at  any  time to abort the menu

selection.

9.  Help system

The  ALT-H  and  F1  keys both start the help system but show

different  kinds  of  information.   The  ALT-H  key displays

general  information about the program, the F1 key displays a

help text specific to the current situation.

10.  Interface schematics

The  interface  schematics  are  included  in the online help

texts.

1. Start the program,

2. Hit ALT-H to start the help subsystem,

3. Hit the letter 'O' to select topic 'Overview',

4. Hit the TAB key to select topic 'Converter' and hit return,

5. Scroll the help text using the cursor-up/cursor-down keys

and read the text.

Hit ESC to leave the help system.

See also appendix A for a plain ASCII version.

11.  Receive circuit

The  operational amplifier (OpAmp) is used to bring the audio

signal  from  the  receiver  up  to  RS232 level.  The supply

current  is  drawn  from  the  DTR and RTS pins of the serial

port.   The  four  diodes  (1N4148 or similar) of the receive

circuit form a standard bridge rectifier.  The 1uF capacitors

are used for buffering.

The  input  signal amplitude should be at least 100mVpp.  The

100nF  capacitor  removes  any DC bias.  Since the OpAmp runs

with  maximum gain there will be a (more or less) rectangular

waveform  at  its  output.  It should have an amplitude of at

least +/-5V to reliably drive the RS232 input.

- 5 -

HamComm 2.3                Manual              February 5th 1994

The  operational  amplifier  LM741  was choosen because it is

inexpensive   and  widely  available.   However  with  todays

technology its electrical characteristics are not impressive.

An LF356 or TL071 for example is much faster and has a higher

input  resistance.   If  you  want  to substitute a different

opamp  type,  keep  in  mind  that  the  serial port can only

deliver a supply current of a few milliamperes.

12.  PTT circuit

The  RTS  output  of the COM port is not only used to provide

the  supply  current  for  the  opamp,  but  also  to key the

transmitter.   A Diode is used to protect the base of the PTT

transistor  against the negative voltage of the RTS output in

receive  mode.   The  resistor  is required to limit the base

current.   In  transmit  mode  the  RTS  and  DTR pins change

polarity.   RTS  is now positive and the transistor pulls the

PTT  line  to  ground.   This circuit has been tested with an

FT747, FT757GXII and a TS440S.  On some old rigs there may be

a  high voltage on the PTT line.  The required current to key

the  transmitter may also be too high for a small transistor,

so better check BEFORE connecting it to the computer.

13.  Transmit circuit

AFSK  tone  signals  for  transmission  are  available at two

places:

1. at the speaker connector

2. at the serial port.

With  many  motherboards one side of the speaker is connected

to  the  +5Volt  supply  thru  a  resistor, the other side is

pulled  to ground by a transistor.  In contrast to most other

areas  of  the  PC's hardware there is no standard design for

the  speaker  output, so there may be variations.  In general

one  side of the speaker will be near ground or +5Volt, while

the  other side has a square-wave signal.  This signal is fed

to two R/C filters to smooth it out.  The microphone input is

very sensitive so a variable resistor is used for attenuation

and a capacitor to remove any DC.

- 6 -

HamComm 2.3                Manual              February 5th 1994

If  you  don't want to make any modifications to the PC or if

the  speaker  output  is  not accessible, you can also get an

AFSK   signal  from  the  TxD  pin  of  the  COM  port.   The

frequencies  are  not  quite  as  accurate  as at the speaker

output  and will often deviate by 5 to 10Hz, but this usually

gives  no  problems.  The signal level at the TxD pin is much

higher  than  at  the speaker output, so you probably have to

change  the  component  values  for  the  R/C  filter and the

attenuator a bit.

14.  External converter

An  option  "External Converter" is available on the "Keying"

menu.   It can also be controlled by the ALT-F9/ALT-F10 keys.

If  this  option  is  enabled,  a  mark/space  signal from an

external  converter  or modem chip is expected at the CTS pin

of  the  COM  port.  Please note that the normal interface is

still  required.   There  will  be  no reception if it is not

connected  or doesn't get the audio signal from the receiver.

Connect  the  input  of  the  converter  and the input of the

HamComm interface in parallel to the receiver output.

Filter   converter   for   RTTY   are  normally  designed  to

differentiate  between two tones.  For CW reception a kind of

tone decoder is required that can tell the tone from noise.

15.  External AFSK

AFSK  tones  for  transmission  are normally available at the

speaker  output or the TxD pin of the COM port.  If you don't

want  to  use those signals for whatever reason, you can find

an  FSK  signal  at  the  DTR  pin of the COM port.  For RTTY

transmission  DTR  is  negative for 'mark' state and positive

for  'space' state.  In CW mode DTR is negative for 'no-tone'

and  positive  for  'tone'.   During  reception DTR is always

positive.

Note  that in receive mode the RTS pin is negative and always

positive  during  transmissions.  It can thus be used to gate

the DTR signal for keying a transmitter in CW mode.

If  you attach any additional components to these pins please

keep  in  mind  that the DTR/RTS signals supply the power for

the OpAmp.

- 7 -

HamComm 2.3                Manual              February 5th 1994

16.  Signal decoding

The  amplified  audio  signal  is  connected to the DSR modem

status  input  of  the  serial port where every zero crossing

generates  an interrupt.  HamComm determines the time between

successive interrupts using the PCs timer chip and calculates

the   corresponding   tone   frequency.   Due  to  the  timer

resolution  of  about  one  microsecond  the  result is quite

accurate  and  constitutes  the  base  for all further signal

processing.

For  RTTY  decoding  the  tone  is  compared to the currently

selected  center  frequency to decide wether this is a 'mark'

or  'space'  signal.  The mark/space signal is sampled at the

proper time according to the current baud rate collecting all

bits of a character.

For  CW  decoding  the  program has to differentiate the tone

from noise.  To be regarded a valid signal the tone has to be

between  the  currently selected mark/space frequencies for a

certain  amount  of  time.   The program maintains a floating

average  of  the  length  of the dots and dashes to adjust to

varying speeds.

When  the character is complete it is converted to ASCII code

and displayed in the RX window.

17.  Macro keys

A  macro  key  can  be  used  to replay a previously recorded

sequence  of  keystrokes.   The  digits on the top row of the

keyboard  are  used  as  macro  keys  while  the  ALT  key is

depressed.    They  are  normally  located  right  above  the

QWERTY...   row.   Do not confuse these keys with the numeric

keypad!

In  the  beginning  there  are  no keystrokes assigned to the

macro  keys, so pressing ALT-1, ALT-2...ALT-9 or ALT-0 has no

effect.   To  start  recording,  select item 'Macro recorder'

from  the  FILE  menu.   A message box will appear confirming

that  recording has been started.  When the ENTER key is hit,

the  message  box  will  go  away  and  from now on up to 250

keystrokes  will be recorded along with normal processing.  A

blinking  "! RECORDING !" message is shown at the lower right

corner of the screen to remind you that the recorder is still

active.

- 8 -

HamComm 2.3                Manual              February 5th 1994

Now hit one of the macro keys to stop recording and to assign

the  keystroke  sequence to that key.  This also replaces any

previous  assignment.   A  message box will appear to confirm

that  macro  recording  has terminated.  Every time the macro

key  is  hit the recorded keystroke sequence will be replayed

just like it had been typed again.

To  terminate  recording  without changing any macro key just

select  'Macro recorder' again from the FILE menu.  A message

box  will  appear  to  confirm  that macro recording has been

canceled.

To  clear  a  macro  key start recording in the usual way and

directly  after  that  hit the key to clear.  This will erase

any  previous  recording assigned to that macro key.  Again a

message box will confirm the result.

18.  Macro files

Macro  definitions  can be saved to a file by selecting "Save

macros"  from  the FILE menu.  The usual file select box pops

up  with  a suggested .MAC file type.  All recorded keystroke

sequences will be saved to the selected file.

To  load a .MAC file select "Load macros" from the FILE menu.

All  macro  definitions  found  in  the selected file will be

added to or replace the current assignments.  Macro keys that

didn't  hold any keystrokes at the time the file was written,

will not be changed when the file is loaded.

There  is  also  a  way  to automatically load your favourite

macros  definitions.  When HamComm is started it searches for

the   default   macro  file  HC.MAC,  first  in  the  current

directory, then in every directory along the PATH.

19.  SHIP/SYNOP decoder

Weather  reports  are  transmitted by many stations troughout

the world, 24 hours a day.  If you live in europe try to copy

Quickborn  Meteo  (near  Hamburg,  Germany) on 4583, 7646 and

11638kHz, or Bracknell (UK) around 4488kHz.  In north america

try  CFH (Halifax, Nova Scotia) on 4271, 6496.5 and 10536kHz.

There   are   many   others   between   4000   and   5000kHz.

Transmissions  are  normally in Baudot, 425Hz shift, 50 or 75

baud.

- 9 -

HamComm 2.3                Manual              February 5th 1994

The SYNOP format is used for reports from land stations, SHIP

format  is  used  for  reports  from ships and other maritime

vessels.   The  messages include data about temperature, wind

speed/direction,   cloud   cover,  precipitation,  dew-point,

pressure  and other meteorological information.  SHIP reports

also include the current position of the vessel.

Land  stations are identified by a five-digit station number.

These  are  fixed stations, so their geographical position is

not  transmitted  with  a SYNOP report.  To give you a better

idea  where  the  reporting station is located, HamComm comes

with  a  list  of  about  10000  station  numbers,  names and

geographical  position.   The program will automatically pick

the  correct  entry  from  this  list  while decoding a SYNOP

report.

A typical message looks like this:

zczc 548

sien42 edzw 141500

aaxx 14154

01465 42889 42715 10084 20022 40159 52033 81048=

02060 41480 40000 11088 21113 40060 52035 72272

83530 333 83694=

nnnn

This  is  what you get if the SHIP/SYNOP decoder ist switched

off.

The   first  line  of  a  message  starts  with  zczc  and  a

three-digit transmission sequence number.  The second line is

a header describing the following information.  Here 'sien42'

indicates  a  "synoptic  report  at  intermediate  hours" for

northern  europe,  'edzw'  is  the  international four-letter

location  indicator  of  the station originating or compiling

the  bulletin  and  '141500'  means  3pm UTC on day 14 of the

current  month.   'aaxx'  is the indicator for SYNOP reports,

'nnnn' marks the end of a message.

There  are  other  formats  in  use  for  different  kinds of

information.   Currently  HamComm  only  supports decoding of

SHIP  and  SYNOP  reports.  A message may contain one or more

reports separated by = characters.  Explaining the reports in

detail is beyond the scope of this manual.

- 10 -

HamComm 2.3                Manual              February 5th 1994

With the WX decoder switched on (from the TEXT menu) the same

message looks like this:

zczc [start] 548 [message 548]

sien42 [Synoptic reports at intermediate hours (SYNOP, SHIP)]

[Northern Europe]

edzw [Offenbach (MET/COM Centre)]

141500 [day:14 UTC:1500]

aaxx [SYNOP]

14154 [day:14 UTC:1500]

[Wind speed obtained from anemometer (knots)]

01465 [Norway, 5848'E TORUNGEN (LGT-H)]

42889 [manned] [cloud height:2000-2500m] [visibility:75km]

42715 [cloud cover:4/8] [wind dir:270 deg, speed:15]

10084 [air temp:+8.4]

20022 [dew-point temp:+2.2]

40159 [pressure at sea level:1015.9hPa]

52033 [pressure:increasing] [change in 3h:3.3hPa]

81048 [cloud info]

=

02060 [Sweden, 6832'E NAIMAKKA]

41480 [manned] [cloud height:300-600m] [visibility:30km]

40000 [cloud cover:4/8] [wind dir:calm, speed:0]

11088 [air temp:-8.8]

21113 [dew-point temp:-11.3]

40060 [pressure at sea level:1006.0hPa]

52035 [pressure:increasing] [change in 3h:3.5hPa]

72272 [past wx: snow, or rain & snow mixed,

cloud cover > 1/2 of sky]

[wx now: Snow]

83530 [cloud info]

333 [section 3]

83694 [clouds:3/8, stratocumulus, 1000-1500m]

nnnn [EOM]

HamComm  displays  the  incoming text just as before.  The WX

decoder  watches  the  characters  go by, waiting for certain

keywords  like zczc (start of msg), aaxx (SYNOP report), bbxx

(SHIP  report)  and  nnnn (end of msg) to synchronize with an

incoming  message.   If  it  thinks it knows what the text is

about,  it  starts  to  insert  comments.   The  comments are

surrounded   by   square   brackets   and   therefore  easily

distinguished  from  the  normal  text  (there  are no square

brackets in the Baudot character set).

- 11 -

HamComm 2.3                Manual              February 5th 1994

There are other ways to build a SHIP/SYNOP decoder.  It could

collect  all data for a report and then display the result in

a  nicely  formatted  way.  However, there is a problem.  The

kind of RTTY transmission used for these reports has no error

correction,   not   even   error  detection.   It's  hard  to

automatically  classify  a  message  as  ok  or garbled.  For

example: if the program says there are +30 degrees celsius in

Greenland and it's december, you will hopefully not buy that.

HamComm  has  no idea where Greenland is and not the foggiest

about  the  climate  there,  so  it  can't catch that type of

error.

The  program  displays  the incoming text as received, so you

can  see  if there is garbage coming in.  Usually it is quite

obvious  if the input is bad.  With a WX decoder working like

a 'black box' it is sometimes very hard to see how it came to

its conclusions and what to believe or not.

20.  Scope function

In  RX  mode  the tone decoder routine calculates the current

signal  frequency for use by other parts of the program.  The

SCOPE  function  uses  this  value  to display a graph of the

input  frequency  vs.   time.   This  is the blue line on the

SCOPE screen.

The tone decoder also maintains a floating average frequency.

The  effect  is very similar to a low-pass filter and is used

for  noise  reduction.  The result is displayed at the top of

the screen as a red line.

The  solid  green  line  marks  the currently selected center

frequency  and the dotted lines above and below show the mark

and  space  tones.  For RTTY reception the center line should

be  in  the  middle between the mark and space tones.  For CW

reception  the signal should match the center frequency.  All

signals  outside  the  area  marked  by  the dotted lines are

ignored.

- 12 -

HamComm 2.3                Manual              February 5th 1994

The display can be controlled with the following keys:

F1  display help screen

F10  activate menu bar

ESC  same as F10

'+'  toggle display of decoded signal

'*'  toggle grid display

'B'  toggle position of red line

HOME  reset sample rate to 1ms

PG-UP  sample rate faster

PG-DOWN  sample rate slower

ENTER  hold display immediately

The  right  mouse  button holds the display at the end of the

current  sweep.  The left mouse button activates the menu bar

just like F10.

The  red  line  (input  signal  after  low-pass)  is normally

displayed  at the top of the screen to not interfere with the

blue  line.   In its second position it will overlay the blue

line to compare the two signals.

Display  of  the decoded signal is normally disabled.  It can

be found at the bottom of the screen and shows the mark/space

states for RTTY and tone/no-tone states for CW reception.

With  some  practice it is quite easy to determine the signal

you  are  listening  to  by  watching the SCOPE display.  CW,

AMTOR   ARQ  and  FEC,  Packet  Radio  and  RTTY  show  their

characteristic  patterns and you can also watch unstable VFOs

sweeping slowly across the band.

21.  Spectrum function

On  entry  to the Spectrum function a graphics mode according

to the video card and monitor in use is activated.

Every  millisecond  the  current  input frequency is checked.

For  every  frequency  there  is  a  counter and the one that

corresponds  to  the  current  frequency is incremented.  All

counter  values  are  displayed  as  vertical lines with each

sweep  from  left to right.  The higher the count, the longer

the  line.   What  you see on the screen is therefore not the

true  audio spectrum but the result at the output of the tone

decoder  routine.   An  opamp  and  a few lines of code can't

substitute a real spectrum analyzer.

- 13 -

HamComm 2.3                Manual              February 5th 1994

When set to 1Hz per pixel SPECTRUM has the best resolution of

all display functions.  For example, the actual shift used by

an RTTY station can be determined quite accurately.

The display can be controlled with the following keys:

F1  display help screen

F10  activate menu bar

ESC  same as F10

'-'  toggle raw/filtered data

'+'  toggle 'shaddow' (color displays only)

'*'  toggle dot/line mode

HOME  reset, start at 200Hz, 3Hz per pixel

PG-UP  zoom in

PG-DOWN  zoom out

LEFT  lower frequencies

RIGHT  higher frequencies

SPACE  clear display

ENTER  hold display

The  right  mouse  button holds the display at the end of the

current  sweep.  The left mouse button activates the menu bar

just like F10.

If  you  have a slow PC then don't use the SPECTRUM function,

you  might  get  frustrated.   There  simply isn't enough CPU

power  to keep the display in motion.  AT class maschines and

real fast XTs should be ok.

The   SPECTRUM   function   has   been   found  to  be  quite

entertaining, especially with a color display.  Next time you

got  visitors  in  your  shack who don't know what's going on

just  start  this  function,  turn  the  lights low and sweep

across a crowded band.

- 14 -

HamComm 2.3                Manual              February 5th 1994

22.  Bitlength statistics

The  BITLENGTH function is a graphics display of the MARK and

SPACE pulse length as detected by HamComm.  It's main purpose

is to quickly get an idea about the speed of the signal.

With every transition of the decoded MARK/SPACE signal a high

or  low  pulse  ends.   The  upper  histogram shows the pulse

duration  statistics  of the high pulses, the lower histogram

shows the statistics of the low pulses.

Before  using  this function the center frequency must be set

to the center between the MARK and SPACE tones, otherwise the

pulse width display will suffer from distortion.

The  shortest  pulses of an RTTY signal are normally equal to

the length of one bit.  Therefore the leftmost peak indicates

the  baud  rate  of  the  incoming  signal.   A  noisy signal

produces  lots  of  short spikes.  They tend to accumulate at

the  left end of the histogram and should be ignored for baud

analysis.

The  BITLENGTH  function tries to identify the leftmost valid

peek  automatically,  but  this  may fail with noisy signals.

Below  each  histogram  the  baud  rate  corresponding to the

highest  peak  is displayed.  There is also a MARKER that can

be moved freely using the LEFT and RIGHT keys.

The display can be controlled with the following keys:

F1  display help screen

F10  activate menu bar

ESC  same as F10

PG-UP  zoom in

PG-DOWN  zoom out

LEFT,RIGHT  move marker

CONTROL-LEFT  move marker, fast mode

CONTROL-RIGHT   "     "      "    "

ENTER  hold display

The  marker  can also be controlled by moving the mouse while

holding  down  the  right  mouse  button.   The  baud  number

corresponding  to  the  marker  position  is displayed at the

upper left corner of the screen.

The left mouse button activates the menu bar just like F10.

- 15 -

HamComm 2.3                Manual              February 5th 1994

23.  Converter checkout

So  you  have  build  the  interface  and made the neccessary

connections  to  the  radio.  You fire up the program and you

see - nothing.

To  find  out  what  is going wrong let's start at the serial

port.  HamComm needs to know two parameters about a COM port:

1. the port address

2. the IRQ number

The  port  address  and  IRQ  number  for  COM1  and COM2 are

standardized  and  therefore  predefined  in  HC.CFG  to  the

following values:

addr   IRQ

COM1   3F8h    4

COM2   2F8h    3

For other serial ports there is no real standard, so COM3 and

COM4 are not defined in the supplied HC.CFG.  Undefined ports

can not be selected from the PORT menu.

24.  Port address

Start  the  program  and select entry 'HamComm' from the INFO

menu.  A small window appears displaying, among other things,

the  port  address  and IRQ of the serial ports as defined in

HC.CFG.   If  the  values  for the port you want to use don't

match the hardware, correct HC.CFG with a text editor.

Remove  the  interface for now.  Select the correct port from

the  port  menu,  then  hit  F3 to activate the RX/TX screen.

Using a voltmeter check the RTS and DTR outputs of the serial

port with respect to the GND pin.  RTS should have a negative

voltage and DTR should be positive.

Now  switch to transmit (TX) mode.  This can be done from the

MODE  menu  but it's easier to hit control-T.  When switching

to  TX  mode  both RTS and DTR reverse their polarity, so RTS

should now be positive and DTR should be negative.

- 16 -

HamComm 2.3                Manual              February 5th 1994

If they don't, here is a list of what might be wrong:

- the voltmeter is broken.

- you are measuring at the wrong socket.

The serial ports of a PC are normally

25-pin or 9-pin male SUB-D sockets.

- you are measuring at the wrong pins.

4     7                              4   5

RTS   GND                            DTR GND

----1-----|-----|-----------13---     ----1-----|-|----

\         |     |               /     \         | |   /

\  o o o o o o o o o o o o o  /       \  o o o o o  /

\  o o o o o o o o o o o o  /         \  o o o o  /

\             |           /           \   |     /

14-----------|---------25             \6-|---9/

DTR                         RTS

20                          7

- you have selected the wrong com port. Check the PORT menu.

- the port address as defined in HC.CFG for this port is not

correct.  Check the INFO menu and HC.CFG.

- the com port has a different address from what you think.

The address can often be selected with jumpers on the

serial card.  Check the documentation for your computer.

On some newer PCs and notebooks the com port address can

be changed or the port can be disabled with a setup

program.

- if only one of the signals (RTS or DTR) changes polarity,

the line driver for the other signal may be broken.

- 17 -

HamComm 2.3                Manual              February 5th 1994

25.  Op-Amp power supply

Now  let's  take  a  look  at the interface.  By far the most

common problems are:

- wrong or missing connections

- solder bridges and cold solder joints

If  you think the problem is with the interface take the time

and carefully inspect it under a magnifying glass.

If that doesn't work, proceed as follows.

Switch  off  the  computer  and  connect the interface to the

serial  port.  Don't connect it to the radio yet, connect the

audio  input  to  GND.   If  possible the interface should be

directly  plugged  to the com port.  Any cable between the PC

and  the  interface  should  be  avoided,  it may degrade the

signal quality.

Switch  the  PC on, start HamComm, select the right port from

the  PORT  menu  and hit F3 for the RX/TX screen.  Check that

RTS  and DTR still change polarity when you toggle between RX

and TX mode.

Check  the supply voltages at the op-amp with respect to GND.

The  positive  voltage  at pin 7 should be +5V or higher, the

voltage at pin 4 should be -5V or lower (more negative).

+V out

8  7  6  5

|  |  |  |    Operational Amplifier

|------------|

|            |  LM741 or TL071

|            |

|-|          |  DIL-8 package, top view

|          |

|-|          |   +V = positive supply

|            |   -V = negative supply

|            |   -I = inverting input

|------------|   +I = non-inverting input

|  |  |  |    out = output

Pin 1  2  3  4

-I +I -V

Check  the  voltages  both  in  RX and TX mode at the op-amp.

Here they should NOT change polarity, otherwise the IC may be

permanently  damaged.   If  they do or if the readings change

significantly  in  RX  and  TX  mode,  check  the  wiring and

- 18 -

HamComm 2.3                Manual              February 5th 1994

polarity  of  the  four diodes between the PC and the op-amp.

Also check the two capacitors used for buffering the supply.

Although  only  a current of a few milliamperes is needed for

the  op-amp,  some  com  ports  can  not deliver the required

supply  voltages.   You  may try to find an op-amp that draws

less  current  but  the easiest solution is to use two 9-Volt

batteries:

bat 1      bat 2

||         ||

op-amp pin 7 <---||---GND---||---> op-amp pin 4

+||-       +||-

If  the power supply is ok we will now try to find out if the

op-amp  works  correctly.   First  check  the  voltage at the

inverting  and  non-inverting  inputs.  Both should read 0 or

VERY  close  to  0 volts with respect to GND.  If they don't,

there is probably a wiring error or the chip is broken.

Temporarily  connect a resistor of about 1M (megaohm) between

the  non-inverting input (pin 3) and the positive supply (pin

7).   Be  very  careful  not to short any pins.  The op-amp's

output  should  go  as  high  as it can go, which is normally

about 1 volt below the positive supply.

Remove  the resistor and connect it between the input (pin 3)

and the negative supply (pin 4).  The output should now be as

low as it can go, about 1 volt above (more positive) than the

negative supply.

Check  the  output  signal directly at the DSR pin of the com

port to make sure that the wiring is ok.  This is only a very

simple  test  wich  the  op-amp will only fail if it is badly

damaged.

The  output  of  the  op-amp  will also be positve or negativ

without  the  resistor.  This is normal and doesn't mean that

the chip is broken.

- 19 -

HamComm 2.3                Manual              February 5th 1994

26.  IRQ number

Now connect the input of the interface to the audio output of

the  receiver.  The op-amp is quite sensitive and should work

even  with very low signals.  Some radios have a socket for a

tape  recorder  where  the  amplitude  is  independent of the

volume setting.  Try that first.

Tune in to some strong signal, preferably to a constant tone.

At  the  DSR pin of the com port there should now be a square

wave  signal.   The amplitude at DSR should have at least the

+/-5V swing required to reliably drive an RS-232C input.

Start  HamComm  to  check  if  the  signal  at  DSR generates

interrupts.  Select the right port from the PORT menu and try

the TUNE, SCOPE or SPECTRUM function from the MODE menu.

If  all  of  these functions seem to be dead, HamComm doesn't

get  any  hardware  interrupts  from the serial port.  Select

entry  'HamComm'  from  the INFO menu and check the displayed

IRQ  number  for  the  port  you want to use.  If it's wrong,

specify the right one in HC.CFG.

Make  sure  that  the  port  really uses the IRQ you think it

does.   Many  serial cards have jumpers or small switches for

IRQ line selection.  Some newer PCs and notebooks can do this

from a setup program.

Please  note, that an IRQ line can only be used by one device

at  a  time.   For  example, if you have a mouse on IRQ 4 you

can't use a COM port on the same IRQ at the same time.

- 20 -

HamComm 2.3                Manual              February 5th 1994

27.  Appendix A - Interface schematics

RS-232C

Receive Circuit                         25  9

pin pin

D1

LM741-------*--------*-I<--*--> DTR 20   4

|      |        | D2  |

3 |\ |      |      ---->I---

>------||---*-----|+\ 7     |      | |

C1   |     |  \______|______|_|________> DSR  6   6

0.1uF  |   2 |  /6     |      | | D3

|  ---|-/       |      | --I<--

Signal        |  |  |/ |4     |      |   D4  |

from          /  /     ---*---|------*--->I--*--> RTS  4   7

Receiver    R1\  \R2      |   |              |

100K/  /100K    |   | +            |

\  \       === ===  C2,C3      |

|  |       === ===  1 uF       |

|  |     +  |   |   16V        |

>-----------*--*--------*---*--------------|--> GND  7   5

|                  |

PTT Circuit               |E                 |

(omit for receive-only)   ^\      R3         |  D1-D5

Q1            \I    1K     D5  |  1N914 or

2N2222         I---/\/\----I<--|  1N4148

/I B

PTT <----------------------/

C

Transmit Audio Circuit    R2         R3

(omit for receive-only)   15K        15K

--------*---/\/\---*---/\/\----*----<  Out

C1      |       |          |        R4 |

.1 uF   / R1    |          |       220K\

<----| |---->\ 10K  === C2     === C3       /     PC Speaker

MIC         / var. === .022   === .022     \

Out         |       |  uf      |  uF       |

<------------*-------*----------*           *----<  +5V

|

GND

- 21 -

HamComm 2.3                Manual              February 5th 1994

28.  Appendix B - Key assignment summary

--- All Windows ---

F1              display help text

F2              RX/TX window, CW mode

F3              RX/TX window, Baudot mode

F4              RX/TX window, ASCII mode, 7 bit

F5              RX/TX window, ASCII mode, 8 bit

F6              Bitlength display

F7              Spectrum display

F8              Scope display

F9              Tune window

F10             Menu bar

ALT-1           execute macro 1

ALT-2           execute macro 2

ALT-3           execute macro 3

ALT-4           execute macro 4

ALT-5           execute macro 5

ALT-6           execute macro 6

ALT-7           execute macro 7

ALT-8           execute macro 8

ALT-9           execute macro 9

ALT-0           execute macro 0

ALT-F4          Terminate programm

ALT-F5          WX decoder off

ALT-F6          WX decoder on

ALT-F7          AFC off

ALT-F8          AFC on

ALT-F9          external converter off

ALT-F10         external converter on

ALT-C           Callsign decoder

ALT-X           Terminate programm

--- TUNE Window ---

HOME            reset center frequency

LEFT            decrease center frequency

RIGHT           increase center frequency

- 22 -

HamComm 2.3                Manual              February 5th 1994

--- RX/TX Window ---

SHIFT-F1        Standardtext 1

SHIFT-F2        Standardtext 2

SHIFT-F3        Standardtext 3

SHIFT-F4        Standardtext 4

SHIFT-F5        Standardtext 5

SHIFT-F6        Standardtext 6

SHIFT-F7        Standardtext 7

SHIFT-F8        Standardtext 8

SHIFT-F9        Standardtext 9

SHIFT-F10       Standardtext 10

LEFT            move TX cursor left, 1 character

RIGHT           move TX cursor right, 1 character

UP              move TX cursor up, 1 line

PAGE-UP         move TX cursor up, 1 page

DOWN            move TX cursor down, 1 line

PAGE-DOWN       move TX cursor down, 1 page

HOME            move TX cursor to first line

END             move TX cursor to last line

INSERT          toggle insert mode on/off

DELETE          delete character under cursor

BACKSPACE       delete character left to cursor

TAB             toggle NORMAL/REVERSE keying direction

ENTER           transfer text line to transmit buffer

CONTROL-A       move TX cursor to beginning of line

CONTROL-D       insert time and date

CONTROL-E       move TX cursor to end of line

CONTROL-F       open/close logfile

CONTROL-H       same as BACKSPACE

CONTROL-I       same as TAB

CONTROL-L       toggle AutoUnshift on/off

CONTROL-M       same as ENTER

CONTROL-P       toggle transmit monitoring on/off

CONTROL-S       send textfile

CONTROL-T       toggle transmit/receive mode

CONTROL-W       toggle Line/Word mode

CONTROL-X       clear TX line

CONTROL-Z       insert time

CONTROL-LEFT    roll RX window backwards, 1 line

CONTROL-HOME    roll RX window backwards, 1 page

CONTROL-RIGHT   roll RX window forewards, 1 line

CONTROL-END     roll RX window forewards, 1 page

CONTROL-PAGE-UP resize RX/TX window

CONTROL-PAGE-DOWN "      "     "

- 23 -

HamComm 2.3                Manual              February 5th 1994

--- SCOPE Display ---

ESCAPE          same as F10

'+'             toggle display of decoded signal

'*'             toggle grid display

'B'             toggle position of red line

HOME            reset sample rate to 1ms

PAGE-UP         sample rate faster

PAGE-DOWN       sample rate slower

ENTER           hold display immediately

--- SPECTRUM Display ---

ESC             same as F10

'-'             toggle raw/filtered data

'+'             toggle 'shaddow' (color displays only)

'*'             toggle dot/line mode

HOME            reset, start at 200Hz, 3Hz per pixel

PAGE-UP         zoom in

PAGE-DOWN       zoom out

LEFT            lower frequencies

RIGHT           higher frequencies

SPACE           clear display

ENTER           hold display

--- BITLENGTH Display ---

ESC             same as F10

PAGE-UP         zoom in

PAGE-DOWN       zoom out

LEFT            move marker left

RIGHT           move marker right

CONTROL-LEFT    move marker left, fast mode

CONTROL-RIGHT   move marker right, fast mode

SPACE           clear display

ENTER           hold display

- 24 -

HamComm 2.3                                             Index

AFSK output................6

Bitlength.................15

CGA........................2

Converter, external........7

Converter..................5

Converter.................16

CW.........................8

Desqview...................2

EGA........................3

ESC........................5

Exclusion of warranty......2

External AFSK..............7

external converter.........7

F1.........................5

Floppy.....................2

Function keys..............5

Harddisk...................2

HC.CFG.....................4

Help system................5

Installation...............3

Interface schematics.......5

Interface.................16

Introduction...............1

Laptop.....................4

LCD........................4

License....................1

Macro files................9

Macro keys.................8

Menu bar...................4

Multitasking...............2

Operational Amplifier......5

PTT circuit................6

Receive circuit............5

RTTY.......................8

Scope function............12

Screen layout..............4

Selection..................4

SHIP/SYNOP.................9

Signal decoding............8

Speaker....................6

Spectrum function.........13

Startup....................3

SuperVGA...................3

System requirements........2

Transmit circuit...........6

VGA........................3

Video adapter..............2

Windows....................2

XT.........................2

i