STAC ELECTRONICS

TECHNICAL SUPPORT BULLETIN

NUMBER:     018

SUBJECT:    WINDOW3.0

DATE:       September 18, 1991

TIME:

SOURCE:         Tech Support, Engineering, User's Manual

SUPERCEDES/     TECH003 Dated 4/14/1991

SUPERCEDED BY:

RELEASE STATUS: Public

TITLE:

WINDOWS 3.0 in enhanced mode and STACKER

OVERVIEW:

WINDOWS is a memory-intensive program.  It can be run in enhanced mode to

take advantage of extended memory, and such operation is compatible with

STACKER.

ISSUES/COMMENTS:

1) When the XT/8 coprocessor board is present, its address range must be

excluded in the WINDOWS SYSTEM.INI file.  Overlooking this step can

lead to corruption of the Windows directory.  Omitting the EMMExclude

can also lead to "Insufficient Memory" errors when launching Windows

programs.

Use of the AT/16 coprocessor presents no such problems.

2) Permanent swap files cannot reside on the STACKER volume; (Windows

does not present it as an option).

3) Temporary swap files should also reside on the uncompressed drive.

4) With third party (non-Microsoft) memory managers, certain files

must exist in the identical directory paths on both the compressed

Stacker drive and in the uncompressed boot drive.

386MAX           BLUEMAX             QEMM386

-----------      ------------        ------------

386MAX.SYS       BLUEMAX.SYS         QEMM386.SYS

386LOAD.SYS      385LOAD.SYS         LOADHI.SYS

386MAX.VXD       BLUEMAX.VXD         WINHIRAM.VXD

RECOMMENDED ACTIONS:

1) If WINDOWS is run in enhanced mode AND the XT/8 coprocessor board is

installed, add an EMMExclude statement to WINDOWS\SYSTEM.INI under the

heading [386ENH] (see User's Manual, "Microsoft Windows 3.0 in 386

Enhanced Mode Users").  This should be done BEFORE WINDOWS is loaded,

and can be accomplished using any text editor.  (If the coprocessor is

disabled, SYSTEM.INI may be modified using NOTEPAD.)

2) If a permanent swap file will be part of the configuration, leave

sufficient uncompressed space on the host volume.  If there is not

enough uncompressed space and you have Stacker version 2.0 or greater,

use SDEFRAG to physically shrink the STACVOL file.

If you are using version 1.12 or lower,the STACKER volume must be

SREMOVEd, and a smaller one created.

3) Temporary swap files left on the uncompressed drive will not cause

fragmentation in the STACKER volume.

4) If the STACKER volume becomes corrupted or write-protected due to an

interrupted 386-enhanced session:

a) verify that STACKER coprocessor card address has been excluded in

SYSTEM.INI;

b) run SCHECK/F on the STACKER volume;

c) delete the WIN386.SWP (temporary swap) file;

d) run SCHECK/F again;

e) reboot before writing anything to disk.