MAILBOX.DOC  for WB6CGW  MAILBOX6.BAS

WRITTEN FOR:  Microsoft Quick Basic Version 4   / CLONE / TNC

PURPOSE:

Mailbox  is  a communications program for Radio  Amateurs   using

packet  radio.  It can be used to communicate with  other  packet

stations  or  it  can be used in an unattended mode to  wait  for

a connecting station to leave mail. Transfer of binary files is

possible between two stations using the program.

*****************************************************************

Mailbox6 is an enhanced version of Mailbox5, which was written

for GWBASIC. MAILBOX6.ARC contains the Quick Basic source code

and a stand alone EXE file.

Versions five and six are similar in operation with the following

major differences:

1. Faster operation - This program keeps up with the serial port

and it is unlikely that the receive buffer will overflow.

2. MAIL6.DAT

To allow for ease of setting program parameters, such as call

signs and TNC setup strings, I have included a data file which

can be changed with a word processor to suit your needs.

(see MAIL6DAT.DOC)

MAILBOX6 looks for the data file, MAIL6.DAT, and uses it to preset

its' initial parameters. If the data file is not found it defaults

to its' internal parameters. If you are running basic you can, of

course, change the lines at the front of the program.

3. Function Keys

Function keys can be preset in MAIL6.DAT and can be set on the fly.

4. DOS SHELL

This is the Basic SHELL command which allows you to suspend the

program and return to the DOS command line. Type EXIT to return

to the mail box program. The communications program stops so it

is not possible to transfer files while at the DOS command line.

5. When typing on line 25, received data will be displayed on the

monitor.  In version five, if data was received while  typing,

the received data was saved in a buffer until carriage return

was struck.

REGARDING HAND SHAKING BETWEEN THE CLONE AND THE PK-232

The following pins are necessary:

CLONE END:                         PK-232  J2

DB9 CONNECTOR   DB25 CONNECTOR

3          2          (transmit data)       -> 2

2          3          (receive data )       <- 3

7          4          (request to send)     -> 4

8          5          (clear to send)       <- 5 (toggles)

6          6          (data set ready)      <- 6 (always high)

5          7          (signal ground)           7

1          8          (data carrier detect) <- 8

4          20          (data terminal ready) -> 20

When  the program is run the following options are available from

the main menu.  Choose the mode you wish to operate in,  type the

number and hit return. (enter)

Note: TNC must be connected or there will be a device time-out.

1. INITIALIZE TNC

The  program  attempts to initialize the TNC by  sending  TNC$(1)

through TNC$(9) to the TNC.  TNC$(1) is the TNC auto-baud  signal

and is sent three times, while TNC$(2) through nine are sent only

once. If you add more strings, be sure to change TNCN to indicate

the  total number. If your TNC is  like mine you won't have

to change anything.

2.   WAIT FOR MAIL

The  program closes any open files,  lists the BBS stations  that

are  monitored  and  waits for mail or a connection.  Hit  ^Q  to

return to the menu.  If you have mail on a monitored BBS,  a file

containing  the  date and time is opened under the name  of  that

BBS. A connecting station can up-load a file or leave a  message.

After disconnect, the program returns to the check for mail mode.

If  there  is  no activity for several minutes  the  screen  will

blank to protect the terminal.

NOTE:

The program waits to receive a match with the string contained

in the array BBS$(x). When a match is detected it looks at the

rest of the line to see if there is a match with MAILCALL$.

Use a word processor to change these two strings in the data

file, to match the strings of the Bulletin Board System that

you are monitoring.

3.   COMMUNICATIONS

This mode operates as a terminal program,  allowing control

of the TNC and contact with other stations.  Hit

ALT  Q  to return the the main menu.  All  of  the  functions

available   from  the  main  menu  are  available  while  in  the

communications  mode by hitting the ALT key along with one  other

key. (hit ALT H while in the com mode to see a list of choices.)

If there  is  no activity for several minutes the screen will

blank leaving only a blinking cursor.

4.   LOG FILE ON

Opens  log  file into which text sent and text received is saved

to disk  under the  file name of your choice. The extension is

provided  by  the program. When asked for a filename enter a name

or hit return and the  program will provide the name PACKETLG.

The first extension will  be .100 and will increment with each

additional  PACKETLG. Files  can  be opened quickly with out giving

any thought to  the actual name.  (a download file works the same

way except that only received text is saved to disk )

5.   CLOSE LOG FILE

Closes any open log or download file.

6.   TYPE FILE TO DISK

Allows creating a text file without leaving the program.  Editing

is only possible on the line being typed.  End with /ex on a line

by itself.

7.   VIEW FILE

Allows viewing any ASCII file on disk.

8.   UPLOAD FILE

Up-load a binary or ASCII file while in this mode.

(binary files require that both stations use this program)

NOTE:

I have noticed that there are times when I receive ASCII packets

that  have  obvious errors. It follows that if errors  can  creep

into the ASCII packets they can also disrupt packets while in the

transparent mode. If an error were to creep into a binary file it

would most likely render the file useless. I was hoping to rely on

the TNC to  pass only valid packets but it looks like that isn't

always the case. It might be necessary to incorporate some form of

software error checking to make a reliable binary transfer.

9.   DISPLAY FILES

Display a directory of a disk drive.

0.  EXIT

Final Note:

While I use this program regularly, I have not been able to test

it 100%. I have used a second computer to simulate transfer of

binary files, but until I actually do it on the air I can't be sure

how well it works. If you get the program running I'd like to set

up a QSO and try to transfer binary files.

I am working on a "C" version of this program. If you have any

comments or ideas for improvement, please leave a message @WB6ASR

WB6CGW

#######################################################