Hierarchical Routing Designators.

A system that supports hierarchical routing will identifies

itself with the "H" feature letter in it's SID.

A hierarchical routing designator is composed of any number

of fields delimited by dot ("."). The fields are in the order

more to less specific location, from left to right (see examples below).

A hierarchical routing designator may contain up to 31 charactors

beyond the initial (six character maximum) traditional designator,

including the dot delimiters. There is no explicit limit on the number

of fields. Each field may contain at most six characters, not including

the dot delimiters.

Examples:

w0rli.norcal.usa

ja2xxx.32.j2net.jpn.asia

amsat

md.usa

95060.ca.usa

How does compatiblity with existing systems work?

When a system that handles hierarchical designators receives

a message from a system that does not handle them there is no

problem. Things work as they always have.

When a system that handles hierarchical designators sends

a message to a system that does not handle them, it sends

the leftmost field of the designator as the "@ BBS" field.

How does forwarding work?

For each message, each field of the hierarchical designator becomes

a candidate key for routing the message. They leftmost field of

the hierarchical designator that matches an entry in any routing

list is used to forward the message. For example:

ja2xxx.32.j2net.jpn.asia

If ja2xxx is in my route list, I route to him directly.

If 32 is in my route list, I use that route.

If j2net is in my route list, then I use it.

If jpn is in my route list, then I use it.

If asia is in my route list, then I use it.

Stations outside asia would all have asia in their route list,

routing their traffic to some nearby HF gateway which can route

to asia. The HF station that routes to Japan would route using

the jpn part of the designator. Inside Japan, the gateway station

would route toward the JA2 districts using the j2net part of

the designator. Inside JA2, stations would route toward the

correct region using the 32 part of the designator. Inside

the 32 region, stations would route directly to ja2xxx using

the first part of the designator.

One useful way to think about hierarchical designators is to

think of the "." as meaning "is within" . Thus the designator

w0rli.norcal.usa means "w0rli, who is in norcal, which is in usa".