DOVE3WWG DOVE OSCAR-17 Telemetry Decoder Mon Feb 26 23:46:37 1990

�����������������������������������������������������������������

New Version 1.3 Features:

�������������������������

*	Option -c on the command line, causes the display to be

shown in color.

*	Option "-s callsign" (or prefix) limits data capture to

frames starting with callsign or prefix. This is handy if

you have more that one HAPN card active in your PC. For

example:

D> DOVE3WWG -s DOVE-

will only capture those frames starting with "DOVE-".

*	Should run on just about any kind of monitor. No graphics

are required, and color is optional. Hercules is supported.

Version 1.0 Features:

���������������������

*	Raw input from a file. This makes the program useful

to non HAPN card users and permits review of old data.

*	NK6KTLM file compatible. If you've captured files or

edited them into the format required by the NK6KTLM

decoder, then DOVE3WWG will process them asis.

*	HAPN card support.  Just point your antenna, and leave

DOVE3WWG running. It obtains its data directly from

the HAPN card (requires M25 & HAPN1.SYS however). No

FIXUP is required on the captured telemetry file since

C25 is not used.

*	When capturing from the HAPN card, a telemetry file name is

generated automatically for ease of capture. File names

are based upon UTC date and time. If by chance the generated

name exists, no problem-- the file is appended to.

*	When output redirection is not used, the formatted screen

continually updates as new packets come in (from file or

HAPN card). The bottom 2 lines scroll to show other

information packets.

*	When output is directed to a file (for printing), the

format is automatically adjusted for the purpose. It

can be used in this form as input to other graphical

software packages.

*	Packets from the HAPN card are UTC date and time stamped.

This is useful when data capture of the satellite time

stamps is poor.

*	Data not understood by this program is not discarded. It

simply passes through (and captured).

Copyright Status:

�����������������

Its free. It would be nice if everyone could send a small

donation to AMSAT, or join. But it is not expected or required.

Please copy it and distribute it amongst your friends.

Although this program is free, certain authorized organizations

may charge a fee for copying and distribution (hopefully just to

recover costs).

Installing:

�����������

The DOVE3WWG is designed to be completely self contained and

"hassle free". However there are some small points to be dealt with

first.

Only the executable file DOVE3WWG.EXE is required for its operation.

There are no supporting data files to make it operate. Just copy this file

where your other command files go (according to your PATH).

For proper UTC time generation, there must exist an environment

variable TZ. This is the variable as used by the Microsoft C library. The

following shows an example of how to set it:

set TZ=EST5EDT

The example shows that:

1.	The local time zone is Eastern Standard Time (EST).

2.	UTC is 5 hours ahead (TZ=EST+5EDT is also valid).

This can be a signed number.

3.	The time zone is Eastern Daylight savings Time

when it is in effect (EDT)

4.	Microsoft default if the variable is NOT defined

is "set TZ=PST8PDT" (DOVE3WWG will complain if it

is not set).

If daylight savings is never in effect, then omit the "EDT".

You will also have to adjust this value when you switch to/from

daylight savings time. Don't forget! The telemetry capture files will

contain bad time stamps if you do!

If the H.A.P.N. telemtery capture feature is to be used, there

are some addtional prerequisits.

1.	H.A.P.N. card must be installed (of course).

2.	HAPN1.SYS driver must be installed.

3.	M25.EXE must have been installed for use.

Non H.A.P.N. users will still be able to feed file data into

the program for useful decoding.

Ok, now lets play!

General Introduction:

���������������������

The DOVE3WWG program has 2 basic "input modes" and 2 two

basic "output modes". You may mix and match them as you wish. But

first lets just list the modes available:

Input Modes:

������������

a)	Standard input (by DOS redirection)

b)	H.A.P.N. packet radio card input

Output Modes:

�������������

a)	Standard output to a file (DOS redirected)

b)	Formatted screen (no redirection)

Furthermore, there is one more "output" that is built into

this program. When the H.A.P.N. card is used for "input", a telemetry

capture file is opened for appending (if it exists). The name is generated

by taking the UTC date and time and formatting a file name in the current

directory as follows:

ydddhhmm.O17

where:	y	is the last digit of the year

y = 0 for 1990, y = 1 for 1991, etc.

ddd	is the day of the year.

ddd = 0 is jan 1, 31 is feb 1, etc.

hhmm	UTC hour and minute.

.O17	represents OSCAR-17

Command Options:

����������������

DOVE3WWG -h		Gives help (or -?)

or

DOVE3WWG [-c] [-s prefix]

where:	-c		Uses colour (or color when state side).

-s prefix	Capture only frames starting with 'prefix'.

(Normal use is -s DOVE or -sDOVE)

note:			Options are case sensitive.

Examples of Use:

����������������

D> DOVE3WWG				Captures from HAPN card

and updates screen parameters

as they come in. A capture

file is written also.

D> DOVE3WWG -sVE3			Same as above, but this tests

its ability to reject all but

VE3 frames (use -s DOVE for

satellite).

D> DOVE3WWG <00550245.O17		This command reviews an old

telemetry file that was

captured starting on Feb 25, 1990 0245 UTC. The values

are updated on the screen as if they came in from the

H.A.P.N. card, only much faster.

D> DOVE3WWG <00550245.O17 -c		This command is the same as

the above, except that the

formatted screen display is done in colour.

D> DOVE3WWG <00550245.O17 >FEB25.RPT	This command processes the

same telemetry file, except

that the information is formatted and written to

report file FEB25.RPT. The format differs from the

screen to be more suitable for printing. The output

file can also be used as input to graphing packages.

D> DOVE3WWG >FEB25.RPT			This command captures from

the H.A.P.N. card, but gen-

erates the report into file FEB25.RPT while doing

so. A telemetry capture file is written as well.

Testing it Out:

���������������

The best thing to do is to test this program out BEFORE a real

DOVE satellite pass. There is a sample TEST.O17 file to feed into the

program as input. This will show you what you can expect from a live

pass in "fast forward" (assuming you are watching the formatted screen).

Try the following:

D> DOVE3WWG < TEST.O17

Did the initial UTC date and time at the top right look correct?

If not, adjust the value of your environment variable TZ.

After having done that, now is a good time to test the data

capture facility (non H.A.P.N. card users ignore). Make sure your

HAPN1.SYS is installed, and M25.EXE has been run prior to invoking

the following command (with no arguments):

D> DOVE3WWG

In this mode, the command formats a screen and will await

telemetry packets to decode. The top right (again) should show the correct

UTC date and time. Since this test is probably being done at a time when the

satellite is unavailable, it is best to try it by tuning your packet

radio to a busy packet frequency. You should begin to see packet traffic

appear on the bottom 2 lines which will scroll away. No parameters will

update on the screen, unless by a fluke that some traffic happened to

possess the correct format.  While this is happening, data is being

captured to a *.O17 file, and the UTC date and time should be updating

on the screen when the program is not busy processing packets (time updates

on the screen cease when traffic is present, and updates resume at idle

periods).

While capturing data from the H.A.P.N. card, use ^C or the

Control-Break key to terminate the program. All files and the H.A.P.N.

card will be closed down gracefully.

Telemetry File Format:

����������������������

A couple of notes on the capture file are in order. First of

all, each packet is time stamped as it comes in. An example appears

as follows:

:: Thu Mar 01 04:19:52 1990 UTC

DOVE-1>TIME-1:

PHT: uptime is 076/13:47:53.  Time is Thu Mar 01 04:16:47 1990

The line starting with "::" identifies the line that was added

by DOVE3WWG, and is only as accurate as your PC system clock. However,

in this case, we can see that either my PC clock is out, or the DOVE

clock has drifted (probably my PC), or maybe both.

The purpose of the time stamps are to overcome the problem of

a series of packets that might be recieved without time stamps under

marginal receive conditions. These lines are easy to remove if that

becomes necessary, since each line begins with "::".

Only DOVE-1>TLM: packets are decoded. The following are also

from the satellite, but are simply recorded without interpretation

(time stamps removed here):

DOVE-1>STATUS:

80 00 00 85 B0 18 77 02 00 B0 00 00 B0 00 00 00 00 00 00 00

DOVE-1>WASH:

wash addr:1900:0000, edac=0x07

DOVE-1>TIME-1:

PHT: uptime is 076/13:48:03.  Time is Thu Mar 01 04:16:57 1990

DOVE-1>BCRXMT:

vbat= 11.082 vlo1= 10.569 vlo2= 10.069 vmax= 11.569 temp=  5.451

A couple of telemetry packet look as follows:

DOVE-1>TLM:

00:58 01:58 02:88 03:31 04:58 05:58 06:6E 07:45 08:6C 09:66 0A:A1

0B:DB 0C:E8 0D:D8 0E:00 0F:24 10:D0 11:93 12:00 13:02 14:B0 15:9B

16:92 17:8C 18:8F 19:90 1A:8D 1B:86 1C:94 1D:90 1E:26 1F:5F 20:BF

DOVE-1>TLM:

21:A6 22:77 23:11 24:15 25:2E 26:00 27:00 28:00 29:00 2A:00 2B:00

2C:00 2D:2A 2E:01 2F:9E 30:CC 31:9F 32:16 33:64 34:C6 35:9F 36:A8

37:A8 38:BA 39:99 3A:00

This program tries to be forgiving about this data. Rather than

rely on the "DOVE-1>TLM:" to follow a consistant format, the program does

some simple tests on a given line to see if it is a "TLM" line. A line that

follows the format of:

21:A6 22:77 23:11 24:15 25:2E 26:00 27:00 28:00 29:00 2A:00 2B:00

will be accepted regardless of its origin. Some may be using TNC's that

capture the block of TLM data as follows:

DOVE-1>TLM:00:58 01:58 02:88 03:31 04:58 05:58 06:6E 07:45 08:6C 09:66 0A:A1

0B:DB 0C:E8 0D:D8 0E:00 0F:24 10:D0 11:93 12:00 13:02 14:B0 15:9B

16:92 17:8C 18:8F 19:90 1A:8D 1B:86 1C:94 1D:90 1E:26 1F:5F 20:BF

DOVE-1>TLM:21:A6 22:77 23:11 24:15 25:2E 26:00 27:00 28:00 29:00 2A:00 2B:00

2C:00 2D:2A 2E:01 2F:9E 30:CC 31:9F 32:16 33:64 34:C6 35:9F 36:A8

37:A8 38:BA 39:99 3A:00

In this case, DOVE3WWG starts its simple tests beyond the first ":",

and proceeds as if there was no difference. This permits you to feed it

your existing NK6KTLM data (DOVE only) files as well.

Live Capture:

�������������

The biggest challenge here is knowing when to listen for it. Hope-

fully you already have a program to help with that. If not, there are several

available from AMSAT-NA and other organizations. Make sure you use up to date

keplerian data. It should be no older than a couple of months at most.

Finally, just tune to 145.825 Mhz. I find that the computer sometimes

copies better at 145.830 at the beginning of the pass. This is due in part to

doppler shift (and imperfections in my rig). I later switch back to 145.825.

Your mileage may vary, enjoy!

Appendix A:

�����������

(From Bob Grier N4HY)

Spacecraft:  DOVE-1:				  Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Descripion:	 C:	      B:	   A:	   Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx E/F Audio(W)+0.000	  +0.0246      0.000	   V(p-p)

1    Rx E/F Audio(N)+0.000	  +0.0246      0.000	   V(p-p)

2    Mixer Bias V:  +0.000	  +0.0102      0.000	   Volts

3    Osc. Bisd V:   +0.000	  +0.0102      0.000	   Volts

4    Rx A Audio (W):+0.000	  +0.0246      0.000	   V(p-p)

5    Rx A Audio (N):+0.000	  +0.0246      0.000	   V(p-p)

6    Rx A DISC:     +10.427	  -0.09274     0.000	   kHz

7    Rx A S meter:  +0.000	  +1.000       0.000	   Counts

8    Rx E/F DISC:   +9.6234	  -0.09911     0.000	   kHz

9    Rx E/F S meter:+0.000	  +1.000       0.000	   Counts

A    +5 Volt Bus:   +0.000	  +0.0305      0.000	   Volts

B    +5V Rx Current:+0.000	  +0.000100    0.000	   Amps

C    +2.5V  VREF:   +0.000	  +0.0108      0.000	   Volts

D    8.5V BUS:      +0.000	  +0.0391      0.000	   Volts

E    IR Detector:   +0.000	  +1.000       0.000	   Counts

F    LO Monitor I:  +0.000	  +0.000037    0.000	   Amps

10    +10V Bus:      +0.000	  +0.05075     0.000	   Volts

11    GASFET Bias I: +0.000	  +0.000026    0.000	   Amps

12    Ground REF:    +0.000	  +0.0100      0.000	   Volts

13    +Z Array V:    +0.000	  +0.1023      0.000	   Volts

14    Rx Temp:	     +101.05	  -0.6051      0.000	   Deg. C

15    +X (RX) temp:  +101.05	  -0.6051      0.000	   Deg. C

16    Bat 1 V:	     +1.7932	  -0.0034084   0.000	   Volts

17    Bat 2 V:	     +1.7978	  -0.0035316   0.000	   Volts

18    Bat 3 V:	     +1.8046	  -0.0035723   0.000	   Volts

19    Bat 4 V:	     +1.7782	  -0.0034590   0.000	   Volts

1A    Bat 5 V:	     +1.8410	  -0.0038355   0.000	   Volts

1B    Bat 6 V:	     +1.8381	  -0.0038450   0.000	   Volts

1C    Bat 7 V:	     +1.8568	  -0.0037757   0.000	   Volts

1D    Bat 8 V:	     +1.7868	  -0.0034068   0.000	   Volts

1E    Array V:	     +7.205	  +0.07200     0.000	   Volts

1F    +5V Bus:	     +1.932	  +0.0312      0.000	   Volts

20    +8.5V Bus:     +5.265	  +0.0173      0.000	   Volts

21    +10V Bus:      +7.469	  +0.021765    0.000	   Volts

22    BCR Set Point: -8.762	  +1.1590      0.000	   Counts

23    BCR Load Cur:  -0.0871	  +0.00698     0.000	   Amps

24    +8.5V Bus Cur: -0.00920	  +0.001899    0.000	   Amps

25    +5V Bus Cur:   +0.00502	  +0.00431     0.000	   Amps

26    -X Array Cur:  -0.01075	  +0.00215     0.000	   Amps

27    +X Array Cur:  -0.01349	  +0.00270     0.000	   Amps

28    -Y Array Cur:  -0.01196	  +0.00239     0.000	   Amps

29    +Y Array Cur:  -0.01141	  +0.00228     0.000	   Amps

2A    -Z Array Cur:  -0.01653	  +0.00245     0.000	   Amps

2B    +Z Array Cur:  -0.01137	  +0.00228     0.000	   Amps

2C    Ext Power Cur: -0.02000	  +0.00250     0.000	   Amps

2D    BCR Input Cur: +0.06122	  +0.00317     0.000	   Amps

2E    BCR Output Cur:-0.01724	  +0.00345     0.000	   Amps

2F    Bat 1 Temp:    +101.05	  -0.6051      0.000	   Deg. C

30    Bat 2 Temp:    +101.05	  -0.6051      0.000	   Deg. C

31    Baseplt Temp:  +101.05	  -0.6051      0.000	   Deg. C

32    FM TX#1 RF OUT:+0.0256	  -0.000884    +0.0000836  Watts

33    FM TX#2 RF OUT:-0.0027	  +0.001257    +0.0000730  Watts

34    PSK TX HPA Temp+101.05	  -0.6051      0.000	   Deg. C

35    +Y Array Temp: +101.05	  -0.6051      0.000	   Deg. C

36    RC PSK HPA Temp+101.05	  -0.6051      0.000	   Deg. C

37    RC PSK BP Temp:+101.05	  -0.6051      0.000	   Deg. C

38    +Z Array Temp: +101.05	  -0.6051      0.000	   Deg. C

39    S band TX Out: -0.0451	  +0.00403     0.000	   Watts

3A    S band HPA Temp+101.05	  -0.6051      0.000	   Deg. C

Special Thanks:

���������������

Hats off to all on the Microsat team for making the project a

reality. Many thanks also for sharing the telemetry decoding information

at an early stage. Of course, special thanks to:

idacrd!mac@princeton.edu (Robert McGwier N4HY)

AMSAT-NA:

���������

The Radio Amateur Satellite Corporation,

850 Sligo Ave.,

Silver Spring, MD USA 20910

(301) 589-6062

H.A.P.N.

��������

(for the HAPN-1 Packet Radio Adapter)

Hamilton and Area Packet Network

Box 4466, Station D,

Hamilton, Ontario, Canada

L8V 4S7

The Author:

�����������

Warren W. Gay VE3WWG

3 Goldsmith Avenue,

St. Catharines, Ontario, Canada.

L2M 2V7

Packet radio:	VE3WWG @ VE3NUU

or:		       @ VE3HPL

or:		       @ VE3RD

Unix:		!ncrcan!brambo!wwg