Documentation for SolveCHN version 1.4

--------------------------------------

SolveCHN is distributed as shareware.  You are authorized to evaluate this

program for thirty (30) days in order to determine whether it is suitable for

your needs.  After this, you are required to submit a registration fee of $20

US for a single user.  Corporations may want to consider a site license for

$250 US.  Registered users will receive the latest version and point release

upgrades free of charge.

Registration fees may be sent to:       Coments, suggestions for new features

Andrew Williams                          and bug reports (!?) may be sent by

5 Research Parkway                            mail to the address shown,

Wallingford, CT   06492                      CIS 70724, 3556 or Internet:

williamsa@delphi.com

(Previous versions were not available to the public.)

Operating Environments

----------------------

SolveCHN, in its various versions (see below), is compiled as a text-based DOS

application for the IBM PC and compatible computers in order to allow the

largest number of users to take advantage of it.  It has been tested under

OS/2 2.0 as a DOS application in both full screen and windowed DOS sessions.

It has also been tested as a DOS application running under Microsoft Windows

3.0/3.1 in standard mode as a full screen DOS application and in enhanced mode

as a full screen or windowed DOS application.  It behaves well under any of

these enviroments.  In fact it will run in as little as 128k of RAM under DOS,

OS/2 2.0 or Microsoft Windows.

General Instructions

--------------------

SolveCHN is designed to automate the tedious process of figuring out the

composition of a freebase / salt / solvate combination.  Traditionally this

has been done by manually plugging values into a spreadsheet or other number

cruncher, viewing the results and adjusting accordingly until a solution is

found.

SolveCHN removes the drudgery from this process by asking for some simple

parameters:

Base:               The empirical formula of the freebase

Salt:               The empirical formula of the salt

Solvate:            The empirical formula of the solvate

Note when entering formulas that if there is only one atom of any

particular type this must be specified.  Thus the formula C12H23N3O1

is accepted while C12H23N3O will generate an error.  Upper and/or

lower case is accepted.  **THE TOTAL NUMBER OF ANY GIVEN ATOM MUST BE

SUMMED.**  For example, C2H5O1H1 as the formula for ethanol will

generate erroneous results.  It should be entered as C2H6O1.

Experimental %C:    The %C found by combustion analysis

Experimental %H:    The %H found by combustion analysis

Experimental %N:    The %N found by combustion analysis

The upper limit for the 'salt factor':

The maximum amount of the salt (acid) component to be

considered.

The upper limit for the 'solvate factor':

The maximum amount of the solvate component to be considered.

The lower limit for the 'salt factor':

The minimum amount of the salt component to be considered.

The lower limit for the 'solvate factor':

The minimum amount of the solvate component to be considered.

The accuracy desired:

Traditionally, 0.4% has been regarded as proof of elemental

composition.  If a solution is found with this accuracy

requirement, a better solution may be found with a more

stringent requirement for accuracy.

SolveCHN was designed for organic substrates, and does not support transition

metals, lanthanides, actinides, etc. as valid atoms.  This will be changed in

a "soon to be released" version so that all atoms are recognized.  To be

explicit, the following atoms are currently supported:

Carbon      Oxygen    Fluorine    Lithium      Silicon

Hydrogen    Sulfur    Chlorine    Sodium       Phosphorous

Nitrogen              Bromine     Potassium    Selenium

Iodine      Calcium

After these parameters are entered, the program begins to test all possible

salt and solvate combinations in a methodical fashion until the accuracy

requirements are met, or until the upper limits are reached.  This can take a

minute or two depending upon several factors:

1) most importantly, the speed of your microprocessor

2) the version of the program you are running and the presence of a

math coprocessor (see below)

3) the upper limits themselves

While these somewhat time-consuming calculations are taking place, a mistake

in input might be recognized.  Accordingly, it is possible to abort the

calculations by pressing any key while they are proceeding.  A message

indicating that the calculations were interrupted is shown.  The options to

edit the existing entries, enter a new entry or quit the program are then

presented.  These options are also available when a solution is found or the

upper limits are reached.

Editing the existing (or brand new) entries is designed to be as easy as

possible.  The editing function is in 'insert' mode by default; that is, a

character is inserted into the formula at the cursor position without

disrupting the rest of the formula or number.  The standard left, right, up

and down arrow, home, end, delete, tab and backspace keys are implemented.

The F1 key brings up context-sensitive help while entering or editing an

entry.  The F9 key causes the program to jump to the calculation phase using

the existing entries.  For example, if no solution is found on the first try

and a mistake in input is recognized, only the incorrect entry need be

modified.  Pressing the F9 key then proceeds immediately to the calculation

phase.  In addition, the ESCape key will abort the program while editing any

entry.

SolveCHN was designed to be small, fast and unobtrusive.  This is one reason

that it does not change your DOS screen colors or use a graphical interface.

Presently, SolveCHN does not attempt to globally optimize the salt / solvate

combination for accuracy.  This provides the fastest possible feedback on

whether a 'hit' will be found.  Future revisions may include this feature

depending upon user demand.  Porting SolveCHN to enviroments other than DOS

is under consideration.

Main Processor and Math Coprocessor Support

-------------------------------------------

The SLVCHN.ZIP file should contain the files noted in the CONTENTS.TXT

file.  If it does not, contact the sysop of the BBS where you obtained the

file or contact the author on CompuServe: 70724,3556.

SolveCHN is designed to run on all IBM-compatible personal computers, from

those based on the 8088 to i486 microprocessors.  SolveCHN uses extensive

floating-point mathematics.  SOLVECHN.EXE can emulate an 80x87 math

coprocessor in software, but this causes slower execution and necessitates a

larger file size for the executable.  Personally, I like maximum performance

at minimal hard drive space, so there are several versions of the program

included.  To obtain optimal performance, you should use the version designed

for your machine as shown below:

For 8088 or 8086 machines with no math coprocessor, rename

SOLVECHN.086 to SOLVECHN.EXE and type SOLVECHN to run the program.

For 8088 or 8086 machines with an 8087 math coprocessor, rename

SOLVECHN.087 to SOLVECHN.EXE and type SOLVECHN to run the program.

For 80286 or higher machines with no math coprocessor, rename

SOLVECHN.286 to SOLVECHN.EXE and type SOLVECHN to run the program.

For 80286 or higher machines with an 80287 or higher math

coprocessor, rename SOLVECHN.287 to SOLVECHN.EXE and type SOLVECHN

to run the program.

Note:    i486SX based systems do not have a math coprocessor unless they have

been upgraded.  i486DX systems have an integrated math coprocessor.