QDIGIT

Copyright (C) 1992, 1993, 1994  P.C. Jahans

All rights reserved.

OVERVIEW

QDigit is an easy-to-use digitizing program which works with many brands

of digitizer tablets at speeds of up to 9600 baud in either binary or

ASCII format.  Point and linear data, including contours, outcrops, well

locations, etc., can be digitized from any map.  Output is sent to text

files in the form of 2D coordinates and other associated data supplied by

the user.  Options, calibration, and communication settings can be saved

to and retrieved from configuration files.  Especially suited to

geological applications, QDigit can also be used for any kind of 2D

digitizing: topographic maps, cross-sections, drawings, graphs, etc.  An

affine transform is used to produce accurate results even from distorted

maps (those which are no longer rectangular but more like irregular

quadrilaterals).  Other features include context-sensitive online help,

full mouse support, file viewing and printing.

DOCUMENTATION

Except for the online help, this is it.  The context-sensitive online

help is fairly extensive and covers everything you need to know to

configure and operate QDigit.  Once the program has been configured to

your digitizer, you can use it right away.  To get familiar with the

program's features, highlight the menu items in turn and press <F1> to

bring up the help window describing each item.  Most people don't want to

read a manual anyway.  If you want to skip most of this stuff, just scan

the configuration section (and of course the disclaimer) and crank up the

program.  The interface is pretty standard.

INSTALLATION

Place QDIGIT.EXE and QDIGIT.HLP in a directory specified in the PATH

command or in a working directory from which you will be digitizing your

maps.  A default configuration file will automatically be saved to and

retrieved from the same directory that the program resides in.  You may

save and retrieve configurations using any other directory and file name

specified with the Setup|Save Setup and Setup|Load Setup menu commands.

Also, typing QDIGIT /c <filename> on the DOS command line causes QDigit

to read in the specified configuration file upon startup.  Installation

is not complete until QDigit has been configured to your digitizer.  The

following section explains how to do it; a brief version is available

online.

CONFIGURING TO THE DIGITIZER

Before using QDigit to digitize a map, the program must be configured to

your digitizing tablet.  This is a two step process: 1) communications

settings, and 2) setting the digitizer data format.  You must refer to

the documentation that came with your digitizer to determine what

parameters are required for these two steps.  This is the only critical

part to the installation process.

Step 1.  Start QDigit.  Your digitizer must be connected to one of the

serial ports available on your computer.  Note that most computers do not

have ports 3 or 4 built in; these are usually added later by purchasing a

separate comm card.  Only standard IRQ configurations are currently

supported.  Select Setup|Advanced Setup|Comm Settings from QDigit's menu

and choose the communication settings appropriate for your digitizer.

Most newer tablets will operate at 9600 baud, 8 data bits, 1 stop bit, no

parity.  Another common setting is 9600 baud, 7 data bits, 2 stop bits,

even parity.

Step 2.  If your digitizer is set to binary mode output, select

Setup|Advanced Setup|Data Format from QDigit's menu and choose the

appropriate binary format.  Low resolution and high resolution binary are

standard formats which can be output by most tablets such as Calcomp

(TM).  Others, notably Summagraphics (TM), require either the MM or UIOF

binary format.  Note that low resolution binary generally goes with a

comm setting of 7 data bits, even or odd parity, and that high resolution

usually coincides with 8 data bits, no parity.

If your digitizer is set to ASCII mode output, select Setup|Advanced

Setup|Data Format|ASCII from QDigit's menu.  You must determine from your

digitizer's documentation and/or any supplied utility programs the

positions and lengths (number of characters) for the X and Y coordinates

and the button character.  An example ASCII format is shown:

Output from digitizer:     SBXXXXXXYYYYYYRL

where:

S     status flag

B     button character

X     digit for the X coordinate

Y     digit for the Y coordinate

R     carriage return

L     line feed

In this example, your ASCII settings should be:

Button Position -- 2

X Coordinate Starting Position -- 3, Length -- 6

Y Coordinate Starting Position -- 9, Length -- 6

If there is more than one button character, use the first one, last one,

or the one corresponding to the button you wish to be sensed by the

program.  ASCII formats run at slower sampling rates and are more

difficult to configure.  It is generally more efficient and easier to use

a binary format.

USING QDIGIT

Once QDigit has been configured to your digitizer it is ready to use.  It

is a good idea to save the configuration at this point with the

Setup|Save Setup menu selection, and even to copy the configuration file

to a safe place.  This avoids having to reconfigure to the digitizing

tablet in case the working configuration file is changed or deleted.

To digitize a map (or drawing, etc.) QDigit should be calibrated to the

coordinates used by the map.  This will create output data in your

working coordinates instead of digitizer coordinates and, more

importantly, compensates for the likelihood that your map is not placed

exactly square to the axes of the digitizing tablet.  Tape the map down

onto the tablet surface, then select Setup|Calibrate|Calibrate Map from

the menu.  Choose X-Y coordinates or Lon-Lat coordinates, then follow the

instructions on the screen.  QDigit will ask for four known calibration

points that you must digitize on your map and enter into a dialog box on

the screen.  Points are digitized by pressing the main button of the

digitizer's cursor (or puck or mouse) -- a beep will be heard when the

point coordinate is received.  Do this as carefully as possible -- the

accuracy of all subsequent digitizing depends on how well you do this

step.  Once the four calibration points are obtained, you are ready to

digitize your map.

The menu items under Digitize indicate the type of digitizing you can do.

Points and lines are generic digitizing functions useful for any map,

drawing or graph, while contours, wells and outcrops are especially

suited to geologic and topographic mapping.  Note that the Digitize|Well

Locations function can be used for any kind of point digitizing where

information must be saved with every 2D coordinate.  If outcrops are to

be digitized, QDigit will request that you digitize the south and north

ends of any line parallel to north.  This is required to obtain correct

azimuths for outcrops.  Alternatively, you may use the Setup|Calibrate|

Calibrate North menu selection.

When any menu item under Digitize is selected, a file selector dialog box

appears, in which you should enter an output file name (or click on an

existing one to be appended to) to which data will be sent.  The numeric

format of the 2D coordinates can be specified, if desired, using the

Setup|Set Precision menu selection.  Digitizing commences when you press

the digitizer's main cursor button or press the digitizing pen to the

tablet.  When doing lines or contours do not hold the button down while

the cursor is stationary as this will cause a stream of identical

coordinates to be saved to the output file.  Output consists of 2D

coordinates (and possibly other related data) saved one per line to the

output file.  If digitizing lines or contours, pressing the <Enter> or

<Return> key during a pause in digitizing will cause the word 'ENDLINE'

to be saved to the file.  This is used as a marker for the end of a line

so that more than one line may be digitized and saved in the same file.

Digitizing can be terminated at any time by pressing <F3> or <Esc>; data

is always saved to the file as digitizing proceeds.  Data is always

appended to output files; existing data is never erased by QDigit.

Upon exiting the program (using File|Exit or <Alt-X>) the current

configuration, calibration and other items are automatically saved.  You

may stop during a long night of digitizing, go for a coffee and come back

later to pick up exactly where you left off as long as you didn't remove

you map from the digitizing tablet.  Removing the map, of course, means

having to recalibrate it.

Operational note: If the digitizing tablet is turned off or is not

connected, user interaction with the menu system may be slowed down while

the coordinate display is activated.  Toggling the coordinate display off

with <F9> eliminates the effect.

Digitizing hint: cover the map to be digitized with wax paper, onion

skin, cellophane or similar material.  After digitizing each line or

point, mark the covering sheet with a felt pen or highlighter to keep

track of what has been just been digitized.  A future update of QDigit

will provide a graphics screen showing lines as they are digitized so

that this step will not be necessary.  But hey, this version is free!

DISTRIBUTION AND SHAREWARE FEE

This program may be freely distributed and you are encouraged to do so.

Individuals, students and educational institutions may license one copy

free of charge (ie. the shareware payment is voluntary).  If more than

one copy is required, or if the program is used in a professional or

commercial environment, a fee payment is required after a 30-day trial

period as follows:

Individuals, educational institutions:

Free     for the first copy

$25      for extra copies

Industry:         $50      for the first copy

$25      for additional copies

Payment may be made in US or Canadian dollars in the form of a cheque or

money order addressed to the author at the address below.  Please do not

send cash.

Payment of the fee entitles the user to a registered copy and one free

upgrade to the next release of QDigit which will feature HPGL graphics

file output, redundant point elimination (line simplification), geo-

physical and dipmeter log digitizing, and graphical line representation

on-screen, OR a Windows version with the same new features when it is

released later in 1994.

Anyone wishing to charge a fee for distributing copies of this program is

authorized to do so provided:

1. A nominal fee is charged to cover only the copying, postage and

other such distribution costs.

2. The offering and sale of this program will be stopped at any time

the author so requests.

3. Problems or complaints will be reported to the author for

resolution.

DISCLAIMER OF WARRANTY

BECAUSE OF THE VARIOUS HARDWARE AND SOFTWARE ENVIRONMENTS INTO WHICH THIS

PROGRAM MAY BE PUT, THIS PROGRAM IS PROVIDE "AS IS" WITHOUT WARRANTY OF

ANY KIND, EITHER EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE.  THE USER MUST ASSUME THE ENTIRE RISK OF USING THE PROGRAM.  IN

NO EVENT WILL THE COPYRIGHT HOLDER BE LIABLE TO YOU FOR DAMAGES,

INCLUDING ANY GENERAL, SPECIAL, INCIDENTAL OR CONSEQUENTIAL DAMAGES

ARISING OUT OF THE USE OR INABILITY TO USE THE PROGRAM, INCLUDING BUT NOT

LIMITED TO LOSS OF DATA OR DATA BEING RENDERED INACCURATE OR LOSSES

SUSTAINED BY YOU OR A FAILURE OF THE PROGRAM TO OPERATE WITH ANY OTHER

PROGRAMS.

BUGS

Should you find a bug in the program please report it, being as specific

as possible, with all relevant details as to its occurrence.  As well,

suggestions for improvements and new features are encouraged.  Please

contact the author by e-mail or regular mail at the address below.

CONTACTING THE AUTHOR

This program was written while the author was a graduate student at the

University of Alberta.  The university itself is in no way associated

with the program, nor should it be contacted for any information, advice,

or complaints regarding it.  All statements and opinions contained in the

program and its documentation are those of the author.  Please submit all

correspondence to:

Peter C. Jahans

Dept. of Geology

University of Alberta

Edmonton, Alberta

CANADA

T6G  2E3

(internet: pcjahans@cepheus.ucs.ualberta.ca)