Mview works backwards.  Cutting a hole in your pspace to view your model

is a real pain in the !#@%!   Too much stretching, panning, zooming and

general fiddling is required.  You should be able to set up ports while

looking at your entire model, define areas, set scales and such. The

included files alleviate much of this uneccessary and short-sighted

hassle.

The Setport and Getport lisp routines will allow you to save a series of

viewports to your current working directory.  The files ceated are very tiny.

This will allow recalling views for assembly of a sheet of multiple views at

multiple scales while in the current drawing or other drawings.

Only 3 things are required, diagonal picks to define the area, a scale

expressed in a ratio, and a file name.  It would be great if someone would

add the ability to save the layer structure with this "viewport" file.

The following is an example of applying this technique to architectural

plan drawings.

Saving views

1. Go into any of your plan drawings, load setport.lsp.

2. Make sure you are in model space and zoom e.

3. Type "setport" to start program and follow the prompts.

When asked for scale type the ratio:

"48" if you want 1/4"=1'

"96" if you want 1/8"=1'

"192" for 1/16"=1',etc.

(This has become second nature at our office.)

4. End or quit the drawing.

Sheet Assembly

1. Start new drawing in same directory.

2. Load Setport.lsp

3. Xref the drawings you were looking at before.

4. Tilemode 0

5. While in pspace xref a border sheet at a scale of one.

6. Type "getport" and follow the prompts.  ".vpt" file extension is assumed

so you don't have to type it.  The pick responds sluggishly but will work.

Note:  Of course you don't have to move between files to do this but

our sheet files xref most of their information for plotting. (standard

title block, various plans, details, etc.)  Only sheet specific info is

contained in that file.