Additions by Don Ward, Foster City, Ca on 11/09/86.

Please note that I am NOT the author of IBMAUX.DOC, IBMAUX1.SYS or IBMAUX2.SYS

and have therefore included an UNMODIFIED copy of the archive IBMAUX20.ARC

which comprises those 3 files.

You may use these programs at your own risk.  Neither I nor the authors of the

IBMAUX20.ARC shall be held responsible for any damage or loss of data that may

be incurred through the use of the programs documented herein.  You may not

sell the programs.  Any conditions stated in IBMAUX20.ARC also apply.

IBMAUX20.ARC provides device drivers AUX1 and AUX2, which give remote console

capabilities to com ports 1 and 2.  I wanted two drivers AUX3 and AUX4 which

would provide identical capabilities for my com ports 3 and 4.  The programs

IBMAUX3.SYS and IBMAUX4.SYS included here provide those capabilities.

IBMAUX3.SYS (Ports 2E8...2EF, IRQ4) and IBMAUX4.SYS (Ports 2E0...2E7, IRQ3)

were made by using the debugger on copies of IBMAUX1.SYS and IBMAUX2.SYS

respectively.

I added a modification note to the end of the files I created.  This increased

the lengths of the files, but not the memory space they take when installed as

drivers.

If you have com ports 3 and above with other addresses, the information below

should help you make suitable modifications.  Please note that I did not make

any changes with regard to the interrupt vectors - this seemed to take care of

itself.  I ensured this by using a copy of IBMAUX1 to make IBMAUX3, and a copy

of IBMAUX2 to make IBMAUX4, because it happens that my COM3 and COM4 ports use

the same IRQ's as my COM1 and COM2 ports respectively.

Offset is from the START of the file.  You need to add 100H if you are using

the debugger in the normal way.

AUX1 thru' AUX4 below refer to IBMAUX1.SYS thru' IBMAUX4.SYS respectively.

All offset and byte values are in hex.

Offset  AUX1 AUX2 AUX3 AUX4        NOTES

------  ---- ---- ---- ----        -----

D      31   32   33   34         'n' in AUXn (device name)

121      31   32   33   34         'n' in IBMAUXn.SYS (installation message)

15C      31   32   33   34         'n' in AUXn (installation message)

7D      F8   F8   E8   E0         Port Addresses from here on

7E      03   02   02   02

9B      FC   FC   EC   E4

9C      03   02   02   02

B5      FD   FD   ED   E5

B6      03   02   02   02

BF      F8   F8   E8   E0

C0      03   02   02   02

D6      FC   FC   EC   E4

D7      03   02   02   02

DE      FD   FD   ED   E5

DF      03   02   02   02

F9      FC   FC   EC   E4

FA      03   02   02   02

107      FC   FC   EC   E4

108      03   02   02   02

111      F8   F8   E8   E0

112      03   02   02   02

25D      FB   FB   EB   E3

25E      03   02   02   02

265      F8   F8   E8   E0

266      03   02   02   02

26D      F9   F9   E9   E1

26E      03   02   02   02

275      FB   FB   EB   E3

276      03   02   02   02

27D      F9   F9   E9   E1

27E      03   02   02   02

285      FC   FC   EC   E4

286      03   02   02   02

Don Ward 11/09/86.  Can be contacted via California 415 area BBS's:

(Charisma, Foster City, 349-6576 or Technet, San Carlos, 595-5079 & 574-3663)