Documentation for SPEED.COM

This program compares the microprocessor and memory operations in your

computer with the "standard" IBM/PC. Peter Norton's Norton Utilities has

a routine called SI (SYSINFO.COM) that attempts to measure the performance

of a compatible against an IBM/PC. This program expands upon that idea and

gives a more complete showing of what the actual performance of a machine

may be in "real world" applications.

For a fair check, you should boot from the DOS disk so that no extra device

drivers are installed (that use interrupts and slow things down). With the

standard IBM/PC, each of the operations indicated will show 100% (or nearly

so). Any slight difference is due to asynchronism between the programs

operation and the system clock.

The response from my "turbo-XT" looks like this:

����������������������������������������������������������������������������ͻ

� �DESIGNS �         IBM COMPATIBLE PERFORMANCE ANALYZER         � (C)  1986 �

����������������������������������������������������������������������������ͼ

Copyright (C) 1986 Richard B. Johnson

Checking memory block write   ticks: 129 compared to IBM/PC 391%

Checking register to memory   ticks: 262 compared to IBM/PC 187%

Checking memory to register   ticks: 279 compared to IBM/PC 175%

Checking register to register ticks: 326 compared to IBM/PC 169%

Checking divide by register   ticks: 102 compared to IBM/PC 501%

Checking divide by memory     ticks: 169 compared to IBM/PC 357%

Checking multiply by register ticks: 115 compared to IBM/PC 418%

Checking multiply by memory   ticks: 143 compared to IBM/PC 389%

Checking stack operations     ticks: 248 compared to IBM/PC 179%

Checking far jumps, far calls ticks: 309 compared to IBM/PC 167%

Total time is: 2082 clock ticks, (115 seconds) compared to IBM/PC 247%

This means that my system is 247 percent faster than an IBM/PC. Note that

memory operations (stack, register to memory, etc.) are not quite as fast

as register only operations. This shows that the probable bottle-neck in

my system is memory access. This "turbo" system runs a V-20 at 8MHz.

If you take this program to computer swap-meets, you can compare the speed of

various "clones" before you purchase or update your system. It can even help

you negotiate for a better price if the speed is not quite as fast as

the saleman says!

Cheers,

Dick Johnson

SYSTOP

Program Exchange

3603 Chestnut Street Apt. C

Lafayette, CA. 94549

(415) 283-3505  data

(415) 283-2809  voice