This program was developed as an alternative to IBM'S "ADVANCED DIAGNOSTICS"

for the hard disk. Although you get more than a hard disk diagnostic with

IBM'S diagnostics, you pay $295 for the package. All I ask in return for

the FREE use of this diagnostic is that you do not change the copyright

nor charge for distribution of this program.

Any problems or suggestions should be forwarded to:

JIM BRACKING

967 PINEWOOD DRIVE

SAN JOSE, CA 95129

(408) 725-0628

PROGRAM FEATURES

----------------

This program is menu driven and in most cases should be self-explanatory. It

run on a IBM AT, PC or compatible computer.

The diagnostics are divided into two categories, destructive and non destructive

tests. These tests can be useful in installing a new disk, finding and

flagging bad data areas or testing the general working condition of your hard

disk and controller.

NON-DESTRUCTIVE TESTS

---------------------

In an IBM compatible hard disk bios the last cylinder is reserved for running

diagnostic tests on. This program uses that cylinder for diagnostic writes.

F1.......WRITE/READ/VERIFY TEST     This test verify's that read and write work

by writing one sector then reading it and

comparing it to the value it wrote.

F2.......SEEK TEST                  This test first verify's every cylinder

then exercises the read/write heads by

performing a crossing seek pattern.

F3.......HEAD SELECT                This test selects one head on every

cylinder.

F4.......ECC TEST                   Most controllers have some form of onboard

error correction that writes four bytes of

ECC data with every data record. When the

data is read these four bytes of ECC are

used to determine the validity of the data

and make corrections for any incorrect

bits depending on the controller. This test

does a read long, modify's four bits and

write's the record back with a write long.

The next read should return the corrected

data.

F5.......RUN ALL TESTS              This runs tests 1 through 4 and allows the

the user to loop up the tests.

F6.......PREPARE FOR RELOCATION     Moves the read/write heads to the landing

zone. The computer should be shut off after

executing this or the heads will move back

onto the data surface.

F7.......DEFECT SCAN                Reads every sector the specified number of

times and reports the errors.

DESTRUCTIVE TESTS

-----------------

F8.......SURFACE ANALYSIS           This test writes the worst case data

pattern (6DB6) on every sector and reads

it back three times. If any errors are

found they are reported and the track if

flagged.

F9.......FORMAT DISK                This performs a physical format of the hard

disk and should not be confused with DOS

format. If you are formatting a disk for

the first time then you should specify "y"

to clear the defects, otherwise a quick

scan will be made to identify any flagged

tracks. After the format is done the tracks

that were flagged as bad will be flagged

again.

F10......FLAG DEFECTS               Its a sad but true fact that most hard

come from the factory with some known

defects and some will be developed over

time. This routine is used to flag those

defects to prevent any data from being

written there.

A-F10....FORMAT TRACK               The same as the format drive except it

only formats one track.

MISCELLANEOUS FUNCTIONS

-----------------------

A-F9.....TOGGLE PRINTER ON/OFF      The printer can be used to log any errors.

A-F10....EXIT                       Return to DOS