In the last message on memory, I talked a lot about memory arrays  and

how they are arranged. You should go scrounge that mesage up and  read

it first, before you dig into this one.

MAKE  SURE you take all precautions against static damage.  Touch  the

unpainted metal insides of the machine for a second or two before  you

touch  anything electronic. Don't hand a card to someone else- put  it

down, have them, touch top of the power supply, and pick it up.  Don't

let  anything  electronic get near regular white  styrofoam  or  white

packing  peanuts,  or  platic  bags  of  any  kind.  Use  "pink  poly"

antistatic bags, 3M dark gray antistat bags, or black d-stat bags. Use

common sense, that helps a lot.

I  assume  you have already figured what column of 9 chips (or  8,  if

parity is not used) contains the bad chip. See previous message.

The next problem is of course to figure out which chip in that  column

is  the baddie. Most computers have a bios that gags on memory  errors

but  doesn't  provide much useful information. It might give  you  the

address  of the failing chip, necessary information, but  that's  only

half  the story. You also need the failing bit number, a value from  0

to  7, or Parity Bit. You might get lucky, and have the power-on  self

test  die  on the bad chip. Then it should tell you the  failing  bit,

well most POSTs tell you that. Better to get a good memory tester  and

let it cook for a few hours. There is a real good one floating  around

as shareware called RAMTEST3 and if you can find it, use it. It  tests

EMS  ram,  conventional, expanded too. If you can't  get  it  anywhere

else, join the EXEC PC BBS at 414-964-5160. If I uploaded this message

to your BBS, chances are I uploaded RAMTEST3 as well- look around.

Anyhow,  a ramtest should give the bit number of the failure.  What  I

often  do  if  the bit number is uncertain or if it seems  to  be  the

parity  bit is to shift the entire column up one chip and see if  I've

identified  the right column. The error should move to another bit  if

so.  If the error bit is erratic or moves around between  tests,  then

it's likely the parity chip is bad. Or several chips are quite  seated

snugly  in their sockets. Try pushing them all down into  the  sockets

firmly.

If your memory test gives a bit number, that should be the chip within

the row thats bad. Often, they are numbered 0 1 2 3 4 5 6 7 P, so  the

first  one  is 0, etc. Which end is which is anybody's  guess;  memory

board makers can lay out a board any way they like. Look for hints  on

the  board itself, sometimes the bit numbers are silkscreened  on  the

board in tiny white letters, or green ones.

If  your memory test gives the bad bits as a bit pattern it will  look

something like this:

0101 1010 WRITTEN

0101 0010 READ

0000 1000 XOR      although many don't give this line. As you can see,

7654 3210 (bit 3)  the bit that's different is #3. Replace the  fourth

chip  in the column and test again. If a  different

bit is implicated each time, it might be the parity chip, or maybe the

address  decode  or refresh logic on the board has gone  south.  These

chips  are almost certainly soldered in, so buy a new board or  get  a

good scope and and Electrical Engineering degree.

Things you should try before anything else: tamp all the memory  chips

down  into the sockets. Don't flex the board to much-- it will  break.

Or a marginal trace will let go and you will have a real flakey  board

on your hands.

Take  all the boards out of the computer one at a time and  check  the

gold fingers at the bottom. Any that look real grungy should be attack

with  a clean pencil eraser. Don't touch the fingers-- skin oils  make

them  oxidize. Sometimes just pulling the card and looking hard at  it

will fix it. Really.

Good luck,

Thomas M. Peters

Stevenson's Office Equipment

4517 N. Oakland Ave

Milwaukee, WI  53211

414-964-6330

############################