_______

____|__     |               (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

This program replaces int 15h function 87h

It runs on 80286 only!  No 8086, no 8088, and (in spite of upward

compatibility) no 80386!

It serves to make moves to and from extended memory more

efficient, resulting in better performance if you do many such

moves.

Whenever the 8086 addresses memory, it performs the following

address-calculation:  10h x segment + offset.     Since both

segment and offset have 16 bits, the address is limited to 20

bits, or 1,048,576.  This is the addressing-capacity of the 8086.

The 80286 has 24 addressing-bits, but in real mode, it performs

the same address-calculation as the 8086, still limiting memory

to 1,048,576, well, actually a little bit more because of carry

into to 21st bit.

To address more memory, the 80286 must be switched to protected

mode. That's fine, but it is a terrible job to get the 80286 back

to real mode.

But there is another way.

In fact, address calculation of the 80286 in real mode is

different.

Whenever in real mode a segment register is loaded, the 80286

multiplies its new value by 10h and stores the result in the

24-bit descriptor cache.  The segment register is thenceforth

ignored, to address memory, the 80286 adds the offset to the

value in the descriptor cache.

In real mode, the contents of the descriptor cache is always 10h

times the contents of the segment register.  Its upper four bits

and its lower four bits are always zero.

There is no way to load the segment register and the descriptor

cache independently.

No, there is!  There is the LOADALL instruction, which loads

every register independent from any other register.  The exciting

thing about this instruction is that it is possible to set the

upper four bits of the descriptor cache while remaining in real

mode.

This program is a TSR.  It takes very little memory.  It uses

loadall to set a descriptor cache, and moves data to/from any

place in memory.  Thus it replaces the protected mode and

cumbersome return to real mode which is used by your ROM-BIOS.

This program is available as shareware.  You are free to use it.

You are free to give it away to any friend, provided it is not

modified in any way, the source is included, and no charge is

payed, except for a small amount for the diskette.

Whether you are satisfied or not, I would appreciate if you drop

me a line or phone me.

Feico Nater

Beukweg 24

7556 DE Hengelo

the Netherlands

phone 31 74 438373

Dit programma vervangt int 15h functie 87h

Het loopt uitsluitend op een 80286!  Niet op 8086, niet op 8088,

en (ondanks de opwaartse compatibiliteit) niet op 80386!

Het dient om efficienter gegevens te kunnen verplaatsen van en

naar extended memory, waardoor de performance van de computer

verbetert.

Steeds als de 8086 het geheugen adresseert, dan wordt de volgende

adres-berekening uitgevoerd:  10h x segment + offset.  Segment

en offset hebben elk 16 bits, en het adres is dus beperkt tot 20

bits, of 1,048,576.  Dit is de adresseercapaciteit van de 8086.

De 80286 heeft 24 adresbits, maar in real mode, doet hij dezelfde

adres-berekening als de 8086, waardoor het geheugen nog steeds

beperkt is tot 1,048,576, eigenlijk iets meer door de overdracht

in het 21ste bit.

Om meer geheugen te adresseren moet de 80286 in protected mode

worden gezet.

Dat is geen probleem, maar terugschakelen naar real mode is

bijzonder omslachtig.

Maar er is een andere manier.

Eigenlijk is de adresberekening door de 80286 in real mode iets

anders.

Als in real mode een segmentregister wordt geladen, dan

vermenigvuldigt de 80286 de nieuwe waarde met 10h.  Het resultaat

wordt opgeslagen in de descriptor cache, die 24 bits groot is.

Het segmentregister wordt verder genegeerd, om geheugen te

adresseren telt de 80286 de offset op bij de waarde in de

descriptor cache.

In real mode bevat de descriptor cache dus altijd 10h keer zoveel

als het segmentregister.  De hoogste vier bits en de laagste vier

bits zijn altijd nul.

Er is geen manier om het segmentregister en de descriptor cache

onafhankelijk van elkaar te laden.

Of toch! Het kan met de LOADALL instructie, die elk register

onafhankelijk van elk ander register laadt.  Het opwindende van

deze instructie is dat het mogelijk is de hoogste vier bits van

de descriptor cache een waarde te geven zonder naar protected

mode te gaan.

Dit programma is een TSR.  Het neemt niet veel geheugen in

beslag.  Het gebruikt LOADALL om de descriptor cache te laden en

verplaatst gegevens van en naar elke plaats in het geheugen.

Zodoende vervangt het de methode via protected mode en de

omslachtige terugkeer naar real mode die in het ROM-BIOS wordt

gebruikt.

Dit programma is beschikbaar als shareware.  U mag het vrijelijk

gebruiken. U mag een copie aan vrienden geven, mits het niet op

enigerlei wijze gewijzigd wordt, de brontekst erbij geleverd

wordt en geen betaling wordt verlangd, behalve een klein bedrag

voor het schijfje.

Tevreden of niet, ik zou het op prijs stellen als u me een

briefje schrijft of me opbelt.

Feico Nater

Beukweg 24

7556 DE Hengelo

tel 074 438373