ZXMIT V2.00

-----------

Spectrum/Amiga File Transfer Program

------------------------------------

Copyright (c) D.J.H.Hartley, 1990,1993.

ZXmit can be used and distributed freely, but it is not public domain.

The author retains copyright. It may not be sold for profit, or included

with any commercial product without prior permission from the author.

This document must be supplied with the program and must not be modified.

Format: ZXMIT TO|FROM|INFO <file> [NOHEAD]

ZXmit is a general purpose file transfer program for transferring files

between a ZX Spectrum and the Amiga, using the serial communication ports

of both computers.

It has only been tested on a 48K Spectrum and Interface 1 and these

documents are for the 48K Spectrum and Interface 1 although, without too

much difficulty, it should work for any Spectrum.

SERIAL CABLE

------------

Make up cable as follows:

Spectrum (Interface 1)             Amiga

2 (TxD)---------------------------2 (TxD)

3 (RxD)---------------------------3 (RxD)

4 (DTR)---------------------------4 (RTS)

5 (CTS)---------------------------5 (CTS)

7 (GND)---------------------------7 (GND)

ZXMIT HEADERS

-------------

ZXmit headers are optional and are attached to the beginning of a file

saved on the Amiga. This allows ZXmit to retransmit the file back to the

Spectrum as faithfully as possible or to provide information to other

programs that may use the headers. They increase the file size by 12

bytes, and should be used except when transferring files that are to be

included in an Amiga program, raw bitmap graphics for example.

These headers are compatible with the Speculator ZX Spectrum Emulator.

SPECTRUM TO AMIGA TRANSFER

--------------------------

In an Amiga CLI/Shell type:

ZXMIT TO <file> [NOHEAD]

where <file> is the file name you want it saved as. The optional NOHEAD

does not save a ZXmit header to the disc.

Then on the Speccy one of the following:

SAVE *"b"               for BASIC programs

SAVE *"b"LINE n         for BASIC programs + start line

SAVE *"b"CODE s,l       for code/data in memory

SAVE *"b"SCREEN$

etc.

AMIGA TO SPECTRUM TRANSFER

--------------------------

On the Amiga:

ZXMIT FROM <file>

On the Speccy one of:

LOAD *"b"               for BASIC programs

LOAD *"b"CODE           for code/data in memory (or screens)

LOAD *"b"SCREEN$

etc.

If the file on the Amiga was saved with a ZXmit header then ZXmit will

use this information as a header to send to the Speccy ie. BASIC/CODE,

start, length, etc. If the file doesn't have a header then a default

header of type 3 (code) and start address=25000 is sent to the Spectrum.

GENERAL

-------

The fault `wrong file type' (always on the receiving computer) indicates

a header format error. An alternative type of fault is that one side has

finished while the other is still in the process of transferring. These

errors are typically due to noise during transfer. Assuming that the

header that is to be transmitted is not at fault (corrupted), then abort

both computers and try the transfer again. ZXmit can be aborted at any

time using Ctrl-C.

INFORMATION ABOUT THE FILE ON THE AMIGA

---------------------------------------

If you have saved the file on the Amiga with a header then information

about the file can be displayed on the Amiga. This also allows you to

see if the file does have a header.

ZXMIT INFO <file>

The information is then displayed on the screen. A type 0 is a BASIC

program and type 3 is code. Types 1 and 2 are numeric and string arrays

respectively. The number after the type indicates Speculator compression,

0=not compressed. Note ZXmit does not compress transmitted files and does

not retransmit compressed files. Length is the number of bytes in the data

block. Start is either the start address of code or the run line for BASIC

programs, values >= 16384 indicate that the program does not auto-run.

BASIC is the number of bytes taken by the program part of a program.

PROTOCOL

--------

The protocol used is the default Speccy one when it boots, that is:

9600 baud, 8 bits, no parity, 1 stop bit, RTS-CTS. But as the protocol is

default on the Speccy (just make sure if you have changed the baud rate

on the Speccy that you change it back to 9600) and in the Amiga is hidden

inside ZXmit, you don't have to worry about it.

Any queries, suggestions, spare cash, beer, etc. to:

Danny Hartley

1 Alexandra Drive

Prestatyn

Clwyd

LL19 8BW

U.K.