--------------------------------------------------------------------

XDC - X-10 Command Line Control

--------------------------------------------------------------------

Version 1.10

May 1996

Copyright 1994-1996 - D.Huras - All Rights Reserved

-------------------------------------------------XDC.DOC v1.10------

CONTENTS

--------------------------------------------------------------------

1. DESCRIPTION

2. FEATURES

3. WARRANTY & LICENSE

4. COMMAND LINE ARGUMENTS

5. XDC DOS ENVIRONMENT VARIABLE

6. COMMUNICATION PORTS

7. X-10 COMMAND DIRECTIVES

8. PROGRAM OVERVIEW

9. XDC ERRORLEVELS

10. RELEASE HISTORY

1. DESCRIPTION

--------------------------------------------------------------------

XDC is a command line driven program which sends 'immediate' command

directives to the X-10 CP290 Computer Interface.  The CP290 will act

on these directives and impose the requested X-10 command signals

onto the powerline.  The registered version of XDC is supplied with

the registered versions of X10DC (X-10 Direct Control) and X10EC

(X-10 Event Control) as a registration incentive; it is not

shareware and is not to be copied or distributed.

2. FEATURES

--------------------------------------------------------------------

-XDC supports all of the X-10 Commands supported by X10DC/X10EC

including the 5 undocumented commands, specifically:

- ON/OFF/DIM

- ALL LIGHTS ON/OFF

- ALL UNITS OFF

- FADE-OFF

- X-LIGHT

-XDC supports 16 discrete brightness levels for the DIM Command.

-XDC supports the synchronization of the CP290 Clock to the PC Clock

or the converse.

-Standard Commports 1 - 4 are supported as well as a User Defined

CommPort that can be comprised of any IRQ + Base I/O Address.

-A single X-10 Command can be passed to XDC on the command line OR

multiple X-10 Commands can be passed to XDC by creating a file of

X-10 Commands and passing the filename to XDC using the /F command

line argument.

Note: You cannot target multiple units with a single X-10 command

directive using XDC except where multiple units are implied

(ie. ALL LIGHTS ON).

3. WARRANTY & LICENSE

--------------------------------------------------------------------

XDC is supplied AS IS. The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties

of merchantability and of fitness for any purpose.  The author

assumes no liability for damages, direct or consequential, which may

result from the use of XDC.

You may use the program on a single machine, and make a backup copy,

but you are prohibited from making or transferring additional

copies.

You acknowledge that you have read this license, understand it and

agree to be bound by its terms as the complete and exclusive

statement of the agreement between us, superseding any proposal or

prior agreement, oral or written, and any other communications

between us relating to the subject matter of this license.

4. COMMAND LINE ARGUMENTS

--------------------------------------------------------------------

The following command line arguments can be entered in any order:

/Cn       - COMMPORT where n = 1,2,3 & 4 [DEFAULT: 1]

/Annnn(n) - Base I/O Address applicable to User Defined CommPort

(Must be specified as a decimal number)

If /I argument is present, but /A is not; then Base

I/O Address will default to 1016 (3F8 Hex).

/In       - IRQ applicable to User Defined CommPort where n = 1 - 15

If /A argument is present, but /I is not; then IRQ

will default to 4.

/F:[d]:\filespec - File Name containing X-10 Command directives

to be pressed. (eg. /F:C:\X10\X10.CMD)

/Q       - Quiet Mode; Display no messages during program execution

/SYNC    - Synchronize CP290 using PC Clock (will wait until top of

the minute, i.e. seconds=0)

/SYNCPC  - Synchronize PC Clock to CP290 Clock (immediate)

/?       - Display Command Line Arguments & Environment Variable syntax

h,u,c,l  - X-10 Command Directive where:

h = House Code (A - P)

u = Unit Code (1 - 16)

c = X-10 Command (ON,OFF,DIM,ALLLIGHTSON,ALLLIGHTSOFF,

ALLUNITSOFF,FADEOFF,XLIGHT)

l = Level (1 - 16) when applicable

Notes:- The /C argument will be ignored if either the /I or /A

argument is passed on the command line.

- Command line options override any settings stored within

DOS environment variable, XDC.

- Multiple occurrences of the same command line argument

are accepted; the last instance will be used by XDC.

- X-10 Commands ALLLIGHTSON,ALLLIGHTSOFF,ALLUNITSOFF can

be abbreviated to ALON,ALOFF,AUOFF respectively.

- The Clock Synchronization command line arguments are mutually

exclusive, if multiple arguments are included on the command

line only the last instance will be acted upon.  However, both

"SYNC" and "SYNCPC" can be included within command files passed

with the /F: argument.

5. XDC DOS ENVIRONMENT VARIABLE

--------------------------------------------------------------------

To minimize program size and improve performance, XDC does not

support an INI file of program parameters.  However, you can avoid

using command line arguments by setting up a DOS environment

variable that contains program settings.  The DOS environment

variable 'XDC' is used to pass settings to XDC.EXE.  You use the

DOS 'SET' command to assign a value to an environment variable.

The syntax is:

SET XDC=c,i,a,q

where: c = CommPort (1 thru 4)

i = IRQ (1 - 15)

a = Base I/O address (must be > 0)

q = Quiet Mode

You do not need to specify values for parameters that you do

not wish to change.  However, XDC environment variable parameters

are positional so you must use comma's as placeholders if you

wish to skip a parameter.  Trailing comma placeholders are not

required.  Examples are shown below:

To turn on Quiet Mode:      SET XDC=,,,Q

To set CommPort to 2:       SET XDC=2

To set CommPort to 2

and turn Quiet Mode On:     SET XDC=2,,,Q

To set IRQ to 5:            SET XDC=,5

To set IRQ to 5 and

Base I/O Address* 12832:    SET XDC=,5,12832 *specify in decimal

The DOS environment variable only needs to be SET once so a logical

place for it is within your AUTOEXEC.BAT file.

Note: CommPort and IRQ+Base I/O Address are mutually exclusive

and you should set either one or the other.  If both are

set then IRQ+Base I/O Address override CommPort.

6. COMMUNICATION PORTS

--------------------------------------------------------------------

XDC supports "pseudo standard" CommPorts 1 thru 4 using the

following settings:

+----------+------+------------------+

|          |      | Base I/O Address |

| CommPort | IRQ  +---------+--------+

|          |      | Decimal |  HEX.  |

+----------+------+---------+--------+

|    1     |   4  |  1016   |  3F8   |

+----------+------+---------+--------+

|    2     |   3  |   760   |  2F8   |

+----------+------+---------+--------+

|    3     |   4  |  1000   |  3E8   |

+----------+------+---------+--------+

|    4     |   3  |   744   |  2E8   |

+----------+------�---------+--------+

If one of these CommPort settings matches your hardware

configuration simply use the /C command line argument with the

appropriate CommPort.  The XDC DOS Environment variable can also be

used.

If your hardware configuration differs from the table above, then

you can define your own CommPort by using the /I and /A command line

arguments or by setting the XDC DOS Environment variable.  IRQ can

be set to any value between 1 and 15 (inclusive).  Base I/O Address

can be set to any value greater than zero.  CAUTION: your hardware

may "lock up" if incorrect settings are used.  You should supply

both arguments (/A & /I), however, if you only supply one of the

arguments then the defaults applicable to CommPort 1 will apply to

the other argument.  (eg. if only the /I5 argument is supplied, then

the /A argument defaults to 1016).  Note that if XDC receives either

/I or /A or both, then any /C (CommPort) setting is ignored.

NOTES:

------

1) The Base I/L Address must be specified in decimal notation.

2) PS/2 Commports 3 - 8 must be specified using the /I and /A arguments.

IBM PS/2 CommPort Parameters are shown below:

+----------+------+------------------+

|   PS/2   |      | Base I/O Address |

| CommPort | IRQ  +---------+--------+

|          |      | Decimal |  HEX.  |

+----------+------+---------+--------+

|    1     |   4  |  1016   |  3F8   |

|    2     |   3  |   760   |  2F8   |

|    3     |   3  | 12832   | 3220   |

|    4     |   3  | 12840   | 3228   |

|    5     |   3  | 16928   | 4220   |

|    6     |   3  | 16936   | 4228   |

|    7     |   3  | 21024   | 5220   |

|    8     |   3  | 21032   | 5228   |

+----------+------+---------+--------+

7. X-10 COMMAND DIRECTIVES

--------------------------------------------------------------------

X-10 Command Directives are structured as follows:

h,u,c,l

where: h = House Code (A - P)

u = Unit Code (1 - 16) [Not Required for Global Commands]

c = X-10 Command:

ON

OFF

DIM

ALL LIGHTS ON [can abbreviate to: ALON]*Global Command

ALL LIGHTS OFF [can abbreviate to: ALOFF]*Global Command

ALL UNITS OFF [can abbreviate to: AUOFF]*Global Command

FADEOFF [Dim Light to 1 then OFF]

XLIGHT [Exclusive Bright Light]

l =  Level [Only REQUIRED for DIM, but also accepted for

ALL LIGHTS OFF + XLIGHT]

The h,u,c,l parameters are positional within the X-10 Command

Directive and the order cannot be changed. To skip a parameter it is

necessary to include a comma as a place holder.  Any extraneous

parameters will be accepted (and ignored). Examples are shown

below:

A,1,ON      Turn device A1 ON

A,1,DIM,16  DIM device A1 to 16 (Maximum Brightness)

A,,ALON     Turn all Lights on House Code A ON

8. PROGRAM OVERVIEW

--------------------------------------------------------------------

When the program loads, it looks for the XDC variable in the DOS

Environment and then proceeds to parse it if found.  If any errors

are detected then the program will terminate with an appropriate

message, otherwise it will parse the command line arguments (which

override the XDC environment variable).   Once again, if any errors

are found it will terminate with an appropriate message.  If the

/SYNC or /SYNCPC argument is found on the command line it will be

processed before any X-10 command directives on the command line.

If the /F: argument was included on the command line then XDC will

attempt to open the file and process the X-10 command directives.

Otherwise, XDC will look for a X-10 command directive on the command

line and execute it.  Note: If both the /F: argument AND an X-10

Command Directive name are passed on the command line, XDC will

ignore the X-10 Command directive from the Command line and process

the X-10 Command Directives within the file specified with /F:.

If XDC encounters an invalid X-10 command directive it will

terminate with a message displaying the offending directive.  When

XDC sends a command directive to the CP290, it waits for an

acknowledgement before sending another command directive.

9. XDC ERRORLEVELS

--------------------------------------------------------------------

XDC will return one of the following ERRORLEVELS to the operating

system upon program termination.

Normal Termination

------------------

0 - Program terminated normally; no errors

Execution Errors

----------------

97 - CP290 returned bad Checksum

98 - CP290 did not respond

99 - Fatal Communications Error

Syntax Errors

-------------

1 - Invalid COMMPORT on Command Line

2 - Invalid IRQ on Command Line

3 - Invalid BASE I/O Address on Command Line

5 - Invalid FileName on Command Line

6 - Invalid Command Line Argument

7 - No X-10 Command to execute

8 - Invalid XDC DOS Environment Variable

9 - Invalid COMMPORT in XDC Variable

10 - Invalid IRQ in XDC Variable

11 - Invalid BASE I/O Address in XDC Variable

14 - Invalid X-10 Command

15 - Invalid House Code in X-10 Command

16 - Invalid Unit Code in X-10 Command

17 - Invalid Dim Level in X-10 Command

10. RELEASE HISTORY

--------------------------------------------------------------------

VERSION    DATE     REVISION DESCRIPTION

=======  =========  ================================================

1.10   May   '96  �Clock Sychronization feature added

1.01   Oct.  '94  �Respect for X10DC & X10EC Registration Keys added

1.00   April '94  �First Release

1.0�   April '94  �Initial Beta Version