GRABBER/AI(tm)  Release 2.10 Shareware Issue

8514/A Adapter Interface Version

----------------------

The Screen Capture Program for MS-DOS

and 8514/A-Compatible Video Graphics

DOCUMENTATION AND TECHNICAL NOTES

August 1, 1995

Copyright 1987-1995 by

Gerald A. Monroe

All rights reserved.

+-------------------------------------------------+

|  Although they are shareware,  GR4AI and  this  |

|  documentation file are copyrighted.  They are  |

|  not in the  public  domain  and they are  not  |

|  "freeware."   Please  consult  page 6  of the  |

|  REGISTER.DOC  file  for  important  copyright  |

|  information and license provisions.  All use,  |

|  copying or distribution of  GR4AI.EXE or this  |

|  document is  governed by the terms and condi-  |

|  tions  of the  Shareware License contained in  |

|  the REGISTER.DOC file.                         |

+-------------------------------------------------+

Thank  you  for your  interest  in  the GRABBER(tm)  screen

capture system!

The GRABBER  package is being marketed in this special form

as shareware.  This  means you have the opportunity  to use

and  evaluate it for 30  days before you  decide whether to

buy.   If  you  continue to  use  GRABBER.EXE, any  of  its

"offspring" programs, or  any of the other programs  in the

GRABBER package beyond such 30-day period, then you are re-

quired  to  send your  registration  fee  to the  publisher

according  to the  instructions  in the  REGISTER.DOC file.

When  you register, you will  receive a copy  of the latest

release of  the registered GRABBER package  that you order.

You  may  not continue  using  GRABBER  beyond such  30-day

period  without registering.  For registration instructions

and a  complete  statement  of  your license  to  use  this

software, please consult the REGISTER.DOC file.

(IBM  and PC  are trademarks  of International  Business Machines

Corporation.   MS-DOS and  Windows  are trademarks  of  Microsoft

Corporation.    Other  trademarks   referenced  herein  are   the

properties of their respective owners.)

Table of Contents

Overview of GR4AI.EXE . . . . . . . . . . . . . . . . . . . .   3

Overview of the 8514/A Adapter Interface  . . . . . . . . . .   3

About This Documentation  . . . . . . . . . . . . . . . . . .   4

System Requirements for GR4AI.EXE . . . . . . . . . . . . . .   4

Types of Screens that GR4AI.EXE Can Capture . . . . . . . . .   4

The AI Driver Must Be Loaded Before GR4AI.EXE . . . . . . . .   5

How to Install GR4AI.EXE in Memory  . . . . . . . . . . . . .   5

How to Capture the Screen . . . . . . . . . . . . . . . . . .   6

Ascending  Two-Tone   Chirp  Indicates  a   Successful  Screen

Capture  . . . . . . . . . . . . . . . . . . . . . . . . .   6

One Low-Pitched Beep Indicates No AI Driver Is Present  . . .   6

Two Low-Pitched Beeps Indicate an Unrecognized Video Mode,  Or

the AI Driver Is Not In Use  . . . . . . . . . . . . . . .   6

Three Low-Pitched Beeps Indicate a File System Error  . . . .   7

Changing the Destination Drive or Directory . . . . . . . . .   7

Configuration Options for GR4AI.EXE . . . . . . . . . . . . .   7

How to use .EXE Offspring Files Created by GR4AI.EXE  . . . .   8

Options for Running GR4AI.EXE's Offspring Files . . . . . . .   8

Changing GR4AI.EXE's Hotkey Combination . . . . . . . . . . .   9

Displaying GR4AI.EXE's Offspring Files on Other Computers . .   9

Converting GR4AI.EXE Offspring to .PCX and .GIF Formats . . .  10

Programmers' Note:  Calling GR4AI.EXE From Your Own Software   11

How to Contact the Publisher  . . . . . . . . . . . . . . . .  12

GR4AI.EXE       Documentation and Technical Notes    Page 3 of 12

1.   Overview of GR4AI.EXE

GRABBER/AI, or  "GR4AI.EXE," is an alternative  version of the

GRABBER.EXE screen  capture program.    It is  a  memory-resident

screen  capture program designed  exclusively for capturing high-

resolution  graphic images  from PC  systems that  are compatible

with the IBM 8514/A  Adapter Interface, and which are  running in

"real mode" under MS-DOS.   (GR4AI does not capture  screens from

Microsoft Windows or Windows applications.)

Like GRABBER.EXE,  the GR4AI.EXE  program captures screens  to

ordinary MS-DOS executable program files with an ".EXE"  filename

extension.  Because  of this,  you can conveniently  go back  and

redisplay the picture later,  simply by entering the name  of the

file to  which the  picture was  saved.   The ".EXE"  image files

created by GR4AI.EXE will be referred to throughout this document

as "offspring" files.

2.   Overview of the 8514/A Adapter Interface

The 8514/A  Adapter  Interface,  or  "AI,"  is  a  programming

interface  that lets  application programs  work with  your high-

resolution  video adapter.  The AI is loaded as a memory-resident

program  in your  computer's  memory by  means  of an  AI  driver

supplied by the  video adapter manufacturer  or by a  third-party

source.   For most video adapters, an  AI driver is not available

because the AI is not a very widely supported standard.  However,

AI drivers are available for some adapters, and when loaded, they

allow  GR4AI.EXE  to  capture  certain types  of  high-resolution

graphics.

Video adapters  for which  an AI  driver is  available include

IBM's  8514/A and XGA cards, and cards based on ATI Technologies'

Mach-8 and Mach-32 controller chips such as the Graphics Vantage,

Graphics Ultra, and 8514/Ultra.  The  AI driver for these IBM and

ATI  cards is usually named HDILOAD.EXE.  A third-party AI driver

published by RIX Softworks  Inc. is available for cards  based on

the Tseng Laboratories ET4000 controller chip.

GR4AI.EXE       Documentation and Technical Notes    Page 4 of 12

3.   About This Documentation

This document must  be read in  conjunction with  GRABBER.DOC,

which is the documentation file for GRABBER.EXE.  This is because

GR4AI.EXE  is installed in memory  and operated in almost exactly

the  same way as the GRABBER.EXE program.  This document does not

explain in  detail  how to  install  and operate  GR4AI.EXE,  but

instead explains what  the AI  is, and what  the few  operational

differences are between GR4AI.EXE and GRABBER.EXE.

4.   System Requirements for GR4AI.EXE

To use GR4AI.EXE, you will need the following:

-  An  IBM-PC, PC-XT, PC-AT,  PS/1, PS/2,  or 100% compatible

computer.

-  At  least one  "local"  floppy  disk drive  or  hard  disk

drive.

-  MS-DOS, or IBM PC-DOS, Version 3.0 or later.

-  A  video adapter  that  is  compatible  with the  IBM  VGA

adapter.

-  An AI  driver that  designed for  compatibility with  your

VGA adapter.

5.   Types of Screens that GR4AI.EXE Can Capture

GR4AI.EXE is designed  to capture screens  from programs  that

run in "real mode" or "virtual 8086" mode under MS-DOS, and which

use  the AI to generate  screen images.   GR4AI.EXE captures only

graphics screens, not character-based text images.  GR4AI.EXE  is

designed to capture the following AI graphics modes:

- 640 x 480  (16 or 256 colors)

- 800 x 600  (16 or 256 colors)

- 1,024 x 768  (16 or 256 colors)

- 1,280 x 1,024  (16 or 256 colors)

GR4AI.EXE can  only capture a screen if the program generating

the image is actually using  the AI to do its screen  work.  Note

GR4AI.EXE       Documentation and Technical Notes    Page 5 of 12

that this is  not always the case.   For example, if you  have an

IBM 8514/A  adapter,  or  another  manufacturer's  card  that  is

compatible with the 8514/A  at the register level (such as an ATI

8514/Ultra, Graphics Ultra or Graphics Vantage card), then  it is

possible for some application programs to work with the screen by

writing directly to the adapter's hardware registers and ignoring

the AI  completely.   GR4AI.EXE cannot  capture any  screens from

such applications which write directly to the hardware registers.

6.   The AI Driver Must Be Loaded Before GR4AI.EXE

Whatever  kind  of  video  hardware  is  installed,  GR4AI.EXE

expects to find an AI driver  already resident in memory when you

initially  install GR4AI.EXE in  memory.  If an  AI driver is not

already present, GR4AI.EXE will not install.

7.   How to Install GR4AI.EXE in Memory

To use  GR4AI.EXE,  load it  into  your computer's  memory  by

entering a command that follows this example:

GR4AI x:\pathname

GR4AI.EXE will not stay  resident in memory (and you  will not

be able to  capture screens)  until you tell  GR4AI.EXE where  to

place its offspring  files, using  the syntax shown  above.   For

more information see Section 4 of GRABBER.DOC.

When you install GR4AI.EXE in memory, it will display a start-

up message in  a blue  box in  the corner  of your  screen.   The

start-up  box is followed by a several lines of information about

the  way GR4AI.EXE is currently configured to run on your system.

If you  want to change this  configuration, you can do  so at the

time  you  install  GR4AI.EXE  in  memory  by  adding  additional

parameters  on   the  command   line  after  your   "x:\pathname"

parameter, or  you  can  change  it after  GR4AI.EXE  is  already

installed.   See Section 13 of this document for more information

about changing the destination path for offspring files.

GR4AI.EXE       Documentation and Technical Notes    Page 6 of 12

8.   How to Capture the Screen

To  capture  the  screen,  press  GR4AI.EXE's  current  hotkey

combination,  as  listed  on  the  start-up  information  screen.

GR4AI.EXE's default hotkey is [Ctrl]-[Alt]-[F10] (i.e., press and

hold both the [Ctrl] key and  the [Alt] key, then press the [F10]

key, then release all three keys).

9.   Ascending  Two-Tone  Chirp  Indicates  a  Successful  Screen

Capture

If you hear  an ascending, two-tone chirp from your computer's

speaker, it  means that GR4AI.EXE has  successfully completed the

screen  capture, and  the offspring  file containing  the current

screen image is located in the "Drive/path for .EXE files."

10.     One Low-Pitched Beep Indicates No AI Driver Is Present

If  you  hear one  low-pitched  beep from  your  speaker after

pressing  the hotkey,  no offspring  file was  created.   This is

because either (a) your  AI driver was removed from  memory after

GR4AI.EXE  was installed, or (b)  the AI driver  is still present

but  GR4AI.EXE is  unable  to  locate  it.    You  should  remove

GR4AI.EXE  from  memory  immediately   by  entering  the  command

"GR4AI UNLOAD",   then  re-load  your  AI  driver,  then  re-load

GR4AI.EXE according to the instructions in Section 7.

11.     Two  Low-Pitched  Beeps  Indicate  an  Unrecognized  Video

Mode, Or the AI Driver Is Not In Use

If you  hear two  low-pitched beeps  from  your speaker  after

pressing  the hotkey,  no offspring  file was  created.   This is

because either (a) you are trying to capture a graphics mode that

is not listed above  in Section 5, or (b) the  foreground program

that is generating the image  is not using the AI to  perform its

screen work,  and  is  probably writing  directly  to  the  video

adapter's hardware registers.

GR4AI.EXE       Documentation and Technical Notes    Page 7 of 12

12.     Three Low-Pitched Beeps Indicate a File System Error

If  you  hear three  low-pitched beeps  from your  speaker, it

means that no offspring  file was created.  GR4AI.EXE  was unable

to  complete the screen capture  because of a  file system error.

For  a  list of  possible  errors and  how  to correct  them, see

Section 7 of GRABBER.DOC.

13.     Changing the Destination Drive or Directory

To change  the drive  or  directory where  GR4AI.EXE  deposits

offspring  files, enter  another GR4AI.EXE  command like  the one

described  in  Section 7 of  this  document.    From  that  point

forward,  GR4AI.EXE  will  place  offspring  files  in  the   new

destination you specify.

14.  Configuration Options for GR4AI.EXE

GR4AI.EXE responds  to many of the  same command-line switches

used  by  GRABBER.EXE,  which  are  described  in  Section 12  of

GRABBER.DOC.  These switches allow you to modify the way in which

GR4AI.EXE performs certain operations.

The  following  command-line   switches  are  available  under

GR4AI.EXE.   For an explanation  of what these  switches do under

GRPCX.EXE, how to  activate them,  and any  limitations on  their

usage,  refer to GRABBER.DOC.  The switches, and the locations in

GRABBER.DOC where they are explained, are:

- INT=nn                 (Section 12.1)

- INT=OFF                (Section 12.2)

- PROMPT and NOPROMPT    (Section 12.3)

- MOUSE and NOMOUSE      (Section 12.4)

- SOUND and NOSOUND      (Section 12.5)

- BATCHCAPTURE[:]        (Section 12.6)

- NOSWAP                 (Section 12.7)

- NOEMS                  (Section 12.8)

- NOUMB                  (Section 12.9)

- UNLOAD                 (Section 12.10)

GR4AI.EXE       Documentation and Technical Notes    Page 8 of 12

15.     How to use .EXE Offspring Files Created by GR4AI.EXE

Each .EXE offspring file created by GR4AI.EXE is  a standalone

MS-DOS  executable program containing  two parts:   (1) the image

you captured, plus (2) a proprietary program which redisplays the

captured image.

To  view the  image contained within  an .EXE  offspring file,

simply run  the offspring file like any other executable program.

You don't need any other software to redisplay the image.

To remove the  image from  the screen, simply  press any  key.

The image  will disappear,  and the .EXE  offspring program  will

exit back to the DOS prompt or to the batch file which called it.

For  on-screen  help and  other  useful  information with  any

GRABBER.EXE offspring image  file, include the  /? switch on  the

offspring's command line.  For example:  SCREEN00 /?

16.     Options for Running GR4AI.EXE's Offspring Files

GR4AI.EXE's  offspring  files  support the  same  command-line

options as  are  supported by  the offspring  files generated  by

GRABBER.EXE.   These include the "FADEIN"  and "FADEOUT" options,

the /Tn option  for timed execution,  the /F and  /N options  for

exiting to DOS  without erasing  the screen, and  the /K  keylist

option for batch file  operation and menuing.  For  a description

of what  these options can  do and how  to use them,  consult the

following sections of GRABBER.DOC:

- FADEIN                 Section 13.1

- FADEOUT                Section 13.2

- /Tn                    Section 13.3

- /F                     Section 13.4

- /N                     Section 13.4

- /K "keylist"           Section 13.6

You  should note that, for technical reasons, it is not always

safe  to use the /F  or /N command-line  options with GR4AI.EXE's

offspring files.  These two options can cause a system crash when

you  are executing  a GR4AI.EXE  offspring at  the DOS  prompt or

within a batch file, and should not be used in those  situations.

You  may  use  these  two  options  when  executing  a  GR4AI.EXE

offspring  file from inside another program by  way of a SHELL or

GR4AI.EXE       Documentation and Technical Notes    Page 9 of 12

EXEC-type function, but ONLY IF the parent program re-initializes

the AI (with the AI's "HINIT" or "HSYNC" functions) or closes the

AI (with the AI's "HCLOSE" function) immediately upon return from

the child program.  If you are not sure about the other program's

ability  to do this,  you should assume that  it cannot and avoid

the  /F and /N command-line  options with the GR4AI.EXE offspring

file.

17.     Changing GR4AI.EXE's Hotkey Combination

The default  hotkey combination  which activates  GR4AI.EXE is

[Ctrl]-[Alt]-[F10].   If you  wish, you can  change this  default

hotkey  combination  to  something  else,  and  save  the  change

permanently  to  the  GR4AI.EXE  file so  that  the  change  will

automatically take  effect each  time  you install  GR4AI.EXE  in

memory.

To accomplish  a hotkey  change, use  the "SHIFT="  and "KEY="

command-line options.  To  save a hotkey change  permanently, use

the "SAVEHOTKEY" command-line option.  All of these  options work

exactly the same way  under GR4AI.EXE as under GRABBER.EXE.   See

Section 15  of GRABBER.DOC for a  description of the SHIFT=, KEY=

and SAVEHOTKEY options and  how to use them.  Refer to Appendices

A and B of  GRABBER.DOC for a list of the codes that must be used

with the SHIFT= and KEY= options to specify a new hotkey.

18.     Displaying GR4AI.EXE's Offspring Files on Other Computers

An offspring file generated by GR4AI.EXE can be redisplayed on

a  computer other  than  the one  from  which it  was  originally

captured, but ONLY IF:  (1) the other computer is configured with

an AI driver of its own, and that AI driver is resident when  you

attempt  to run the offspring file, OR  (2) the other computer is

compatible with the VESA  Video Bios Extension (the "VESA  VBE"),

and that  VESA VBE support is present when you attempt to run the

offspring file.

In general, you should  not assume that any other  computer on

which you want to run a GR4AI.EXE offspring file has an AI driver

available,  because  the  AI  is  not  a  very  widely  supported

standard.   However,  most other  computers with  Super-VGA video

adapters (i.e., video  cards that are  capable of displaying  the

kinds  of screens  described in  Section 3.3 of  GRABBER.DOC) are

either  supplied with a VESA VBE driver from the manufacturer, or

GR4AI.EXE       Documentation and Technical Notes   Page 10 of 12

have  a  VESA VBE  driver that  is  available from  a third-party

source.

For more  information about the  VESA VBE,  see Section 16  of

GRABBER.DOC

19.     Converting GR4AI.EXE Offspring to .PCX and .GIF Formats

GR4AI.EXE  is supplied  with  two  companion  utilities  which

convert the images contained in GR4AI.EXE  offspring files to the

popular .PCX and .GIF graphics file formats.  These utilities are

called GRAI2PCX.EXE  and GRAI2GIF.EXE.   These utilities function

in exactly the same way as the GR2PCX.EXE and GR2GIF.EXE programs

which  are also  included  in  this  GRABBER  package.    To  use

GRAI2PCX.EXE and GRAI2GIF.EXE, your AI  driver must be loaded  in

memory.  For  instructions on using  these programs, consult  the

CONVERT.DOC file.

GR4AI.EXE       Documentation and Technical Notes   Page 11 of 12

20.  Programmers' Note:  Calling GR4AI.EXE From Your Own Software

If  you are a programmer developing your own software, you can

invoke  GR4AI.EXE to capture the screen from within your own pro-

gram, assuming  that GR4AI.EXE  is  already resident  in  memory.

This is  accomplished by using  GR4AI.EXE's external  programming

hook, which  is  a customized  call to  Interrupt 16h,  described

below.   You can write a routine  to implement this hook in 80x86

assembly language or in any high-level programming language which

supports BIOS-level interrupt calls.

INTERRUPT 16h, FUNCTION 4741h

On entry: AX = 4741h

DL = 0  (to check only if GR4AI.EXE is resident)

DL = 1  (to capture the current screen)

On return:   AX = 4147h  (if GR4AI.EXE is installed in memory)

For DL=0

or DL=1:  BL  contains the  logical drive to  which the

capture would be attempted (for DL=0), or was

attempted  (for DL=1).   0  =  default drive,

1 = drive A, 2 = drive B, and so forth.

For DL=1:

If CF=0:  Capture was  successful.   ES:DX points  to

an   ASCIIZ   string  which   contains  the

[drive:][\path\]filename   of   the   newly

created offspring (the drive  is also coded

in BL; see above).

If CF=1:  If  DX  = FFFFh,  then  a  capture  attempt

failed  due to  a  critical  disk error  or

disk-full   error  on   the  logical  drive

encoded in BL  (see above).   Speaker  will

beep.

If  DX does  not  equal FFFFh,  capture was

not   attempted.     GR4AI.EXE   was   busy

servicing a  prior capture request, or some

other  disk I/O  was occurring, or  DOS was

not  in  a reenterable  state.    Try again

momentarily.

GR4AI.EXE       Documentation and Technical Notes   Page 12 of 12

21.  Technical Questions?  Reporting a Problem or Bug?

>>  Do  not contact  the Public  Software  Library if  you are

experiencing  a technical problem  with GR4AI.EXE or  if you have

any  questions about how GR4AI.EXE is supposed to work.  Instead,

contact the publisher directly, as described in this section. <<

The  publisher  wants  to  know  if  GR4AI.EXE  ever  fails to

function on your system  as documented in this manual.   Also, if

you feel there is an  aspect of the program which is  not clearly

or completely described here, we appreciate your suggestions.

Please  read  all of  the  documentation  before submitting  a

problem report.  This way you can be certain whether there really

is a problem, or whether  the program is being  put to a use  for

which it is not designed or intended.  When submitting  a problem

report, be as specific as possible about what happens and in what

sequence or situation.  Be sure to list these specific things:

(1)  All  of the  information provided on the  screen after you

enter  the  GR4AI  command  at  the  DOS prompt  with  the

program already resident in memory.

(2)  Your brand and model of video adapter.

(3)  Your brand and model of PC.

(4)  Your brand  and version of DOS (for example, MS-DOS V6.20,

DR-DOS V5.0, PC DOS V4.0, etc.).

(5)  Your brand and model of video adapter.

(6)  The program from which you are  trying to capture when the

problem occurs.

(7)  The  contents  of both  your  CONFIG.SYS and  AUTOEXEC.BAT

files.

Without  all of this information,  it is difficult  to solve a

problem, and a response will be delayed.

The publisher can be reached at the following addresses:

Mail: Gerald A. Monroe              Fax:  (216) 333-6299

P.O. Box 16296                Telephone:  (216) 333-6075

Cleveland, Ohio 44116

U.S.A.

CompuServe:  72321,1257

Internet:    72321.1257@compuserve.com

* * *    End of GR4AI.DOC   * * *