November 20, 1990

This utility is intended to turn off the A20 address line

on AT class machines equipped with the Chips and Technologies

CHIPSET.

The purpose for doing so, is to allow the Borland TPCXINST

program to perform the tests required to install the extended mode

TPC compiler.

Apparently, some machines (like mine) boot up with the A20 line

already enabled.  This is to allow maximum performance with

OS/2 type operating systems.  This utility allows the user to

enable or disable the A20 line directly or with a batch file

at boot up time.

Reg 6F (RB12, A20 address line) and Reg 60 (RA0, Processor Reset)

are a feature of the NEAT ChipSet.  They provide a fast means of

switching the 80286 in and out of protected mode.

OS/2 makes frequent operating system calls that must be executed

in Real mode.  To perform these calls, the CPU typically must issue

two commands to the 8042 (or equivalent) keyboard controller

to enter protected mode and enable the A20 address line.

Reg 60 provides a processor reset, and Reg 6F controls the A20

address line control.  Because these registers are controlled by

direct I/O port writes, instead of software interrupts, they

execute very quickly.

This utility is intended to execute from the AUTOEXEC.BAT file

when the system is started.

Bruce Gavin

4730 Signal Rock Rd

Colorado Springs CO 80922-1623

(719) 380-9095 (voice)

CompuServe: 70137,3244

Online Reference BBS:

(719) 591-0302 24 hours, 2400/8N1

Programmers Online Reference Guides

Norton Guide databases

Home of FIND, ALTER, FRESHEN, and other fine utilities.