ADDRAM 1.0

Copyright (c) 1989 by Marty Del Vecchio

Adds memory to DOS using EMS 4.0 memory.

Based on the program EEMRAM

written by Chris Dunford

Table of Contents

I.   What You Should Have                         1

II.  What the Program Does                        1

III. What You Need                                2

IV.  How to Do It                                 3

V.   Message Reported by Programs                 4

A)  ADDRAM                                   4

B)  SUBRAM                                   5

C)  EMSDATA                                  6

VI.  Notices/Disclaimer                           7

VII. Questions                                    7

I.  What You Should Have

ADDRAM consists of six files:

1)  ADDRAM.COM      Program  that  uses  EMS  4.0  memory  to

extend DOS beyond 640K.

2)  ADDRAM.ASM      Assembly source for above.

3)  SUBRAM.COM      Program  that  undoes  ADDRAM--it returns

DOS to 640K

4)  SUBRAM.ASM      Assembly source for above.

5)  EMSDATA.EXE     Program that  gives information about the

EMS installed in your system.

6)  ADDRAM.DOC      This file--documentation.

II.  What the Program Does

ADDRAM  can  increase  the  size  of  your  DOS  memory.    DOS is

restricted to  an arbitrary  limit of 640K of memory.  This is split up

into 10 segments (0 to 9) of 64K each, from  address 0000  to 9FFF hex.

DOS can  only handle  one large contiguous memory block, and ordinarily

this block is 640K large.

This limit is arbitrary because of the placement  of video buffers

immediately after  the reserved DOS space.  The EGA/VGA graphics buffer

is located at A000.  The  monochrome text  buffer is  at B000,  and the

CGA/EGA/VGA text  buffer is  at B800.   Thus,  if you  are using a mono

card, then there is 64K of address space available right at the  end of

DOS that should be added to DOS.

However, the DOS limit is widely known as 640K, and there are very

few memory cards designed to put memory at  these locations  for use by

DOS.  However, expanded memory cards that conform to the EEMS (Enhanced

Expanded Memory Specification) standard  from AST  have the  ability to

map memory  at virtually any location in the PC's address space.  Chris

Dunford wrote a program called EEMRAM  (EEMRAM11.ARC) that  mapped EEMS

memory at the unused video buffers and added it to DOS.

When EMS  4.0 was introduced, it was a combination of "normal" EMS

3.2 (the type of memory provided by the Intel Above Board, for example)

and "enhanced" EMS (AST's memory cards).  Part of the EMS 4.0 spec says

that cards may be able to map memory at different  locations in  the PC

address space,  just like EEMS cards.  This program is designed for EMS

4.0 cards that can do this.

However, it must be noted that this capability is an OPTION in the

EMS  4.0  spec.    Thus,  there  are  expanded  memory  cards  that are

advertised as "EMS 4.0" cards.  However, because they cannot map memory

wherever it  is needed,  they are insufficient for the purposes of this

program.  For this program, you need a "true" EMS  4.0 card.   For more

Page 1

particulars, see below.

Page 2

III.  What You Need

ADDRAM is  designed to  extend DOS  beyond 640K whenever possible,

using EMS 4.0 memory.  It  does  this  by  addressing  EMS  memory into

unused video  buffer memory.  ADDRAM can only increase DOS' memory size

under certain conditions:

1)   You must already have exactly 640K  of DOS  memory installed.

No more, no less.

2)   The machine  MUST have  an EMS  4.0 memory  card that can map

expanded memory into the A000 and B000 segments,  where video

memory is  located.  The vast majority of EMS cards available

cannot do this,  because  they  were  designed  prior  to the

release of  the EMS 4.0 specification.  This program requires

functions found in EEMS  cards,  and  NEWLY-DESIGNED  EMS 4.0

cards.   Many EMS  cards are  sold today as "EMS 4.0" capable

that cannot perform all of the  EMS 4.0  functions needed for

this particular purpose.

The following  paragraphs represent my personal experience in

working with memory cards, and their fitness for a particular

purpose.  This should not be interpreted as a general view of

the cards  discussed--this  is  only  a  discussion  of their

suitability for use by ADDRAM.

All  of  the  AST  Rampage products (Rampage 286, Rampage 286

Plus, etc.) have the required capability, since they are true

EEMS/EMS 4.0  cards.   However, you  need to  get the EMS 4.0

driver from AST for these products--ADDRAM does not work with

a plain  EEMS driver.   If you only have the old EEMS driver,

use EEMRAM11.ARC by Chris Dunford.

The Intel Above  Board  products  (Above  Board,  Above Board

Plus) do not have the capability needed by ADDRAM.  The Above

Board is an old-style EMS card, and the new Above  Board Plus

is  prohibited  from  addressing  memory in the A000 and B000

segments to prevent conflicts with video memory.   Thus, they

cannot be used by ADDRAM to add memory to DOS.

The Charge  Card from  All Computers,  Inc. can be used.  Use

the device driver "ALLEMM4.SYS" included on the ALL disks.

QEMM from Quarterdeck and  386 to  the Max  from Qualitas can

also be used, but they can automatically add memory to DOS if

requested.  This program  should be  unnecessary if  you have

either of these products or a similar product.

The  program   EMSDATA.EXE,  included   here,  can  help  you

determine if your EMS card can be used.

3)   There must be unused memory starting at segment A000.  If you

Page 3

have  an  EGA  or  VGA  system,  you cannot add memory to DOS

unless you use the All Charge Card.  This is  because EGA/VGA

cards use the A000 segment for high resolution graphics video

buffers, and thus the EMS card cannot address memory there.

If you have the Charge Card, and you do not  use your EGA/VGA

in graphics  mode, but  only in  text mode,  you can overcome

this restriction by adding "i=A000-AFFF" to the command line.

This takes  away the graphics buffer and gives it to DOS, and

thus any program that  tries  to  use  EGA/VGA  graphics will

crash your  system.  It is up to you to decide if you want to

make this tradeoff.  However, beware that some  EGA cards use

the A000  segment even  when not  in graphics mode.  You will

have to experiment with your particular  card to  see if this

space can be used.  Such results cannot be guaranteed.

If  you  have  a  standard  monochrome or Hercules monochrome

video card, then you can add 64K to DOS, for a total of 704K.

If you have a CGA system, you can add 96K to DOS, for a total

of 736K.

IV.  How to Do It

First, you have to ensure that your card  can map  memory into the

A000 and B000 segments.  Run the program EMSDATA.EXE to see if your EMS

card can currently do this.  Some EMS software drivers  (the driver you

load in CONFIG.SYS) must be explicitly told on their command lines that

they can use this memory for mapping.

For example, if you have a  monochrome system  and an  AST Rampage

286, you  have to add the following to your 'DEVICE = REMM.SYS' command

line:  "/i=A000-AFFF".   This tells  the EMS  driver that  it is  OK to

address memory  into this  region.  If you have a CGA, you must specify

"/i=A000-B7FF".  Run the program EMSDATA.EXE  to determine  if your EMS

driver  can  map  memory  into  these  locations--the last paragraph it

prints out deals with this matter.

Once you have ensured that the  EMS card  can map  memory into the

video buffer space, you must run the program ADDRAM.COM.  ADDRAM.COM is

the program that actually maps memory into the  video buffer  space and

tells DOS that it is there.

To run  this program,  just type  'ADDRAM' at  the DOS prompt.  It

will automatically add as much memory as possible,  up to  96K, to DOS.

It will  report on  its success  or failure.   If you want to limit the

amount of memory added to  DOS  for  some  reason,  you  can  do  so by

specifying the  maximum number  of EMS  pages (16K each) on the command

line.  For example, to add only 32K  of memory  to DOS,  you would type

'ADDRAM /2' on the command line.

Page 4

V.  Messages Reported by Programs

A)  ADDRAM

Here is a list of the messages reported by ADDRAM:

"Expanded Memory Manager not found--no EMS installed."

This message  indicates that  no expanded memory was found in your

system.  If you have an EMS card in your  machine, be  sure to add

the  device  driver  statement  in  your CONFIG.SYS file.  See the

instructions that come with your EMS card to do this.

"Expanded Memory Manager reports an error."

This message indicates that the EMS  software  is  in  a  state of

error.   This may be a software or hardware error--run EMSDATA.EXE

to determine the cause.  Because of this error, ADDRAM  cannot add

memory to DOS.

"This program requires version 4.0 expanded memory."

As discussed above, the EMS driver must be version 4.0 or above.

"This program requires exactly 640K of installed DOS memory."

As discussed  above, this  program cannot  be used if your machine

has more than 640K or less than 640K of DOS memory installed.   If

you have  more, then  you don't  need this  program.   If you have

less, then you should use your EMS card  to backfill  DOS to 640K.

To do this, see your EMS card's instruction manual.

"This program's memory allocation block is not the last

in DOS' chain.  Cannot change memory of not at end."

This message  indicates that  the memory block allocated to ADDRAM

is not at the END of DOS.  You might get this  message if  you try

to  run  ADDRAM  from  inside  of another program, such as the DOS

shell of Lotus 1-2-3.  If you get  this message,  exit the program

you are currently in and try again.

"Expanded Memory Manager cannot map memory at segment A000."

As discussed  above, your  EMS card  must be able to map memory at

segment A000 in order for it to be added to DOS.   This is because

DOS can only work with one large contiguous block of memory.

"Not enough expanded memory available."

If there  is not  enough expanded  memory free,  you will get this

error.  If you are adding 64K to DOS, then  you naturally  need to

Page 5

have  64K   of  expanded   memory  available.    Run  the  program

EMSDATA.EXE to determine the amount of free expanded memory.

"Error while mapping expanded memory at A000."

This message indicates that the EMS driver reported an error while

memory was  being mapped  at A000.   This  may be  a hardware or a

software error--in any case, no memory can be added to DOS.

"Error assigning name to expanded memory handle."

Under EMS version prior  to 4.0,  a chunk  of EMS  memory was only

known by  a number  (known as  a handle).  With EMS 4.0, a program

can identify its chunk of memory with an 8-character name.  ADDRAM

assigns the name "ADDRAM00" to its memory.  This is so that SUBRAM

can later find it and deallocate it.  This message  indicates that

an error  occurred wile assigning this name to the EMS memory.  In

this case, no memory can be added to DOS.

"ADDRAM's name already in use by another handle."

This message indicates that  another  program  has  used  the name

"ADDRAM00" for  its EMS  memory.  In this unlikely case, no memory

can be added to DOS.

"XX KB memory successfully added to DOS."

This message indicates success.  "XX" will be a number, either 16,

32, 48,  64, 80, or 96.  This specifies the amount of memory added

to DOS.  To verify  that  this  is  the  case,  run  DOS' 'CHKDSK'

program.  At the end it will report the amount of DOS memory.

B)  SUBRAM

SUBRAM  can  be  used  to  un-do  the  effects of ADDRAM.  It will

restore DOS to 640K and release the expanded memory used.  Here are the

messages returned by SUBRAM:

"Only 640KB DOS memory--no memory has been added by ADDRAM."

This message  indicates that  DOS is  currently at  640K, and thus

ADDRAM has not been run successfully.

"Expanded Memory Manager not found--no EMS installed."

This message indicates that no  EMS  memory  is  installed  in the

system, and thus ADDRAM was not run successfully.

"No memory has been added by ADDRAM."

This  message  indicates  that,  although  EMS  memory is present,

Page 6

ADDRAM has not been run successfully to add memory to DOS.

"This program's memory allocation block is not the last

in DOS's chain.  Cannot change memory if not at end."

This message indicates that the memory  block allocated  to SUBRAM

is not  at the  END of DOS.  You might get this message if you try

to run SUBRAM from inside of  another  program,  such  as  the DOS

shell of  Lotus 1-2-3.   If you get this message, exit the program

you are currently in and try again.

"Error deallocating EMS memory."

This message indicates that an  error  occurred  while  SUBRAM was

freeing  the  EMS  memory  added  by  ADDRAM.    This  indicates a

malfunction in the EMS driver.

"DOS memory reset to 640KB, EMS memory freed."

This message indicates success--DOS was restored to 640KB, and the

EMS memory added by ADDRAM was freed for use by other programs.

C)  EMSDATA

EMSDATA.EXE  is  designed  to  tell  you  about  your  current EMS

configuration.  Just type  'EMSDATA' at  the DOS  command prompt.   The

program's output  is generally longer than one screen--you can pause it

by hitting Ctrl-S (the Ctrl key and the S key at the  same time).   You

can resume the program by hitting any other key.

This  program  will  tell  you  numerous  things  about  your EMS,

including the software version number, its current state,  the EMS page

frame segment  address (where application programs address EMS memory),

the total amount of EMS memory and amount of free EMS memory.

EMSDATA will also give a list of the allocated handles  (blocks of

EMS memory  in use by programs), the pages and bytes allocated to each,

and if the EMS  is version  4.0 or  higher, the  name assigned  to each

handle.   It is  rare for  a program  to assign a name to a handle, but

ADDRAM assigns the name 'ADDRAM00' to its handle.

EMSDATA  will  also  give  technical  information  about  your EMS

hardware,  include  the  raw  page  size (16KB is the standard EMS page

size, but some implementations have smaller sizes), total number of raw

pages, and number of free raw pages.

EMSDATA  will  report  the  number  of "additional register sets."

This refers to the number of mappings the  EMS hardware  can keep track

of in  addition to the current mappings.  Multitasking programs such as

Desqview use alternate register  sets  for  quick  switching  of tasks.

This is  one way  to tell  if you  have a true EMS 4.0/EEMS system.  If

Page 7

this number  is 0,  then you  most likely  do not.   If  this number is

greater than 1, then you most likely do.

The  context  save  area  size  refers  to  the amount of memory a

program must use to save the  current page  mapping context.   Resident

programs that  use EMS memory (such as Sidekick Plus, and RAM disks and

disk caches that uses EMS) must save the  current context  before using

their  EMS  memory.    The  smaller  this  area is, the faster they can

operate.

The number of DMA channels available  on an  EMS board  can affect

its performance in copying memory from expanded to conventional memory.

The  next  paragraph  printed  by EMSDATA discusses whether or not

your EMS card has been used to backfill DOS to 640K.   If it  has, then

programs like Desqview can use this page size to multitask programs in.

If not, then Desqview cannot easily  run multiple  programs in expanded

memory in this manner.

The next  paragraph (and chart) discuss the exact memory locations

at which your EMS card  can  and  cannot  address  memory.    The final

paragraph discusses  whether or  not your EMS card is currently capable

of mapping memory at segment A000, and thus whether it  can potentially

be used  by ADDRAM.   Again,  these values may be changed, depending on

your EMS card, by adding parameters to the  device driver  statement in

your CONFIG.SYS file.  See your card's manual for more details.

VI.  Notices/Disclaimer

This  program  is  released  to  the public domain for free use by

anybody.  However, it cannot be sold at any price.

The author,  Marty Del  Vecchio, hereby  disclaims any warranties,

express or  implied.   This program  is not  guaranteed to work at all.

Its author cannot be responsible for any loss or any damage of any kind

resulting in its use.  Use at your own risk.

All of  the brand  names used  are trademarks and/or copyrights of

the appropriate companies.

VII.  Questions

If you have any questions or  bug reports,  you can  contact me at

home at (508) 820-1544.  Or you can find me at the Channel 1 BBS system

in Boston.  The number there is (617) 354-8873.  Leave a message in the

main conference for 'MARTY DEL VECCHIO'--note the space!

Page 8