By Craig Allsop

I figured I might as well give everyone the macros from my source that

display the copper bars.  (They were  written for Tasm,  Ideal  mode -

but are easily converted) - I reveal all!

macro   waithorz                ; DX = 3DAh!

local   a,b

a:      in      al,dx

rcr     al,1

jc      a

b:      in      al,dx

rcr     al,1

jnc     b

endm

macro   waitvert

local   a,b

a:      in      al,dx

test    al,8

jnz     a

b:      in      al,dx

test    al,8

jz      b

endm

macro   copper                  ; SI = Colour table, CX = # of Colours

local   a               ; BL = Palette to change

mov     ah,0c9h

mov     dl,3

a:      mov     dl,0c8h

mov     al,bl

out     dx,al           ; Select palette

inc     dl

outsb

outsb                   ; Send first 2 values

lodsb                   ; Get next one ready

mov     bh,al           ; and hang onto it

mov     dl,0dah

waithorz                ; Wait for the horz. retrace

mov     dl,ah

mov     al,bh

out     dx,al           ; Send last value (Causes static!)

dec     cx

jnz     a               ; Continue...

endm

To use them, requires a 'waitvert' before using 'copper' to execute it,

and 'waitvert' requires dx = 3dah. Simple really. Eg:

mov     si, <address of palette>

mov     cx, <number of colours/lines>

mov     bl, <palette to change>

mov     dx, 3dah

waitvert

copper

You will notice that I send two values before waiting for the retrace,

because the  VGA only loads them into a buffer,  and only updates  the

palette registers when it gets the last value,  which is also when you

get the static 'dot' appearing on the screen.  Since I'm doing it this

way,  on slow machines it works perfectly,  and on faster machines you

get static on the right side  of the  screen,  but you can  remove the

overscan on the right side to fix that problem.

Regards,

Craig.

If anyone uses the above macros, or modify the above code for suitable

functions in another lanugage, then be my guest,but please give credit

where its due, and mention my name  or my alias  'Daemon' - the  above

macros must conatin this paragraph in any source and are Public Domain

in all other respects.