SUMMARY OF GRAPHICS.COM REPLACEMENT ROUTINES (Revised 01-22-90)

by Gene Quong

To print stock or commodity charts to any printer that supports

graphics, such as a dot matrix printer, you should first try pressing

<p> or <P> when a particular stock in shown on the screen.  We

currently support Epson FX-80 compatible dot matrix printers.  The

specific model that we use to print stock charts is an Epson FX-80.

In the near future we intend to add support for additional printer.

The second avenue which you may pursue is the DOS external  command

file, GRAPHICS.COM which is on the DOS system diskette that came with

your computer.  If you type GRAPHICS at the DOS prompt before running

the chart program this will make a graphics routine resident in memory that

enables the Shift-PrintScreen key command to send graphics to the printer.

Ordinarily, the Shift-PrtSc combination will only print out ASCII

characters.  The GRAPHICS.COM program currently only supports CGA,

EGA, and VGA video cards.  IT DOES NOT SUPPORT HERCULES GRAPHICS

CARDS.

In case GRAPHICS.COM does not work for your particular monitor and

printer combination, we are including on this distribution diskette

some shareware and public domain programs.

Each of the enclosed P.D. programs makes itself resident in memory

so you should reboot your system with Ctrl-Alt-Del before loading

and trying a new program.  The programs enclosed are:

CITOH.COM           HERCTOSH.COM        MLPS.COM

DMP200.COM          LASERJET.EXE

DUMPHP.COM          OKIDMP.COM

EGAEPSON.COM        OKIDUMP.COM

EP2GEM.COM          OKIPRTSC.COM

FXPRTS.COM          SCRNDUMP.COM

GEM10GRF.COM        SETUPFX.COM

GEMGRAPH.COM        THINKJET.EXE

GRAFTRAX.COM        TOSHGRPH.COM

GRAPHICR.COM        TOSHDUMP.EXE

--------------------------------

C.ITOH.COM

"This is Marty Smith's program modified for use with the

C.Itoh Prowriter printer.  It replaces IBM's GRAPHICS.COM."

--------------------------------

DMP200.COM

This program works with a Radio Shack DMP-200 printer off an IBM-PC

equipped with an Enhanced Graphics Adaptor (EGA).

Author:  Dr. R. Brooks Van Horn, Jr.

701 Fall Place

Herndon, VA. 22070

--------------------------------

DUMPHP.COM

This program permits a graphics screen to be dumped to an HP LaserJet

printer.  The program expects the output to come from COM1:.

Author:

Scott Pakin

6007 N. Sheridan Rd.

Chicago, IL    60660

--------------------------------

EGAEPSON.COM

This program works with Epson FX series printers off an IBM-PC equipped

with an Enhanced Graphics Adaptor (EGA).

Author:

Dr. R. Brooks Van Horn, Jr.

--------------------------------

EP2GEM.COM

"Modifies the vertical spacing on a Gemini printer to eliminate blank

lines when used with a graphics program designed to output to an Epson."

Makes highly elliptical chartwheels appear circular.  May also eliminate

double-spacing encountered on other program printouts sent to a Gemini

10 printer.

--------------------------------

FXPRTS.COM

"Replacement for GRAPHICS.COM for the Epson FX-80 and FX-100 printers.

The program remains resident in memory to be activated with the

Shift-PrtSc keys."

--------------------------------

GEM10GRF.COM

"The GRAPHICS.COM program has been edited to support the GEMINI-10

printers using the CGA interface.  The default GRAPHICS.COM supplied

with MS-DOS causes a venetian blind effect on copied images.  This

edit replaces the graphics header for proper printer setup on the

GEMINI-10."

Author:

Steve Hathaway

10260 SW Bryton Ct.

Wilsonville, OR 97070

--------------------------------

GEMGRAPH.COM

"Gemgraph.com allows the use of a Gemini 10X for graphic

screen dumps. It is DOS 2.0 graphics.com modified to insert the

correct number of line feeds. Results are excellent."

Author: Unknown

--------------------------------

GRAFTRAX.COM

This program works with Epson MX printers with the GRAFTRAX 80 option.

It is also supposed to work with older non-Epson-compatible Star printers.

"If you print the screen in GRAPHICS-mode, a LEFT SHIFT PrtSc will

print the screen LARGE--filling nearly an 8 1/2 x 11 page--and then

insert carriage returns to restore Top Of Form relative to where it

started.  If you do a RIGHT SHIFT PrtSc, the screen is printed about 5

1/4  x  2  3/4 inches. No TOF reset is made, and multiple screen prints

will be contiguous.

"In either graphics print mode, you can abort printing by hitting

the ESC key.  This can be used to suppress the carriage returns at the

end of the large screen print, for instance.  You cannot abort a screen

print in text mode, however, since it is your IBM ROM routine."

Author:

Marty Smith

SOURCE ST2259           (713) 661-1241  (Office)

COMPUSERVE 72155,1214   (713) 464-6737   (Home)

--------------------------------

GRAPHICR.COM

This program prints hi-res graphics in reduced scale.  Unlike the other

programs, this program does not rotate the chartwheel but prints it right

side up.  However, the chartwheel has the shape of an ellipse.

Author:

P.E. Schmid

--------------------------------

HERCTOSH.COM, Version 1.1

Copyright 1986 by Stephen G. Simpson

This is a memory-resident, Hercules-to-Toshiba, graphics screendump.

Once installed in memory, it occupies 640 bytes.

To print Hercules monochrome graphics page 0 on a Toshiba P1340

or similar printer, type Alt-0.  To print page 1, type Alt-1.

To generate a formfeed, type Alt-9.

Individual users, and distributors of user-supported software,

may freely copy and disseminate this program in unmodified form.

Other vendors should contact the author for terms.

If you find this program useful, please send $5.00 to the author:

Stephen G. Simpson, 1162 S. Garner Street, State College, PA 16801.

--------------------------------

LASERJET.EXE

This utility will enable you to print the contents of your IBM PC or

PC/XT display screen to the Hewlett-Packard LaserJet printer.

The utility is designed to work with a CGA card.  An EGA card will

produce an elliptical chartwheel.

The following commands must be in your AUTOEXEC.BAT file to help

insure proper execution of the print screen utility:

MODE COM1:9600,N,8,1,P

MODE LPT1:=COM1

LASERJET A

Reboot your computer to automatically execute the commands in the

AUTOEXEC.BAT file and load the print screen utility into memory.

Replace the 'LASERJET A' command with 'THINKJET' to use the THINKJET.EXE

driver instead.  Both drivers work on the HP LaserJet with the difference

that THINKJET.EXE produces a larger chartwheel.

Although not required, several printing options are available with the

utility.  If you wish to use any of the following options, they must be

entered within five seconds after pressing the 'Shift' and 'PrtSc'keys.

Only one option from each group (orientation, print position and paper

ejection) may be selected.  The printing process will begin five seconds

after the last key is pressed.

Group           Key            Option

-----           ---            ------

Orientation     * P       Portrait (LaserJet only)

L       Landscape (LaserJet only)

Print Position  * T       Top half of page

M       Middle of page

B       Bottom half of page

Paper Ejection  * Y       Form feed at end of print

N       No form feed at end of print

* indicates default value

Author:

Developed but not officially supported by Hewlett-Packard.

--------------------------------

OKIDMP.COM

"Run OKIDUMP once at boot time to install it and then whenever you hit

<R>ight Shift PrtSc you'll get a small graphics dump to your OKIDATA

printer. Hit <L>eft Shift PrtSc and you'll get a larger printout rotated

90 degrees. This routine is invisible to normal text screen dumps."

Known to work on an OKI 84 printer.

--------------------------------

OKIDUMP.COM

Simply run OKIDUMP as you would GRAPHICS.COM (see the DOS manual).

It acts just like GRAPHICS.COM for OKIDATA printers (without plug'n'play).

Author: unknown

--------------------------------

OKIPRTSC.COM

This program "replaces the GRAPHICS.COM program supplied as part of the

IBM PC-DOS operating system (an External DOS command).  It provides

both text and graphics capabilities for IBM PC owners who have a color

graphics adapter and an Okidata 84/92/93 printer (without the Plug-N-Play

option).

"In graphics mode, 320 x 200 medium resolution pictures are shown with

varying shades of gray to represent the colors on the screen. A 640 x

200 graphics image is shown simply in black & white, the same as it

appears on the screen.

"Additionally, in graphics mode, [OKIPRTSC] automatically places the

picture in the middle of the page (no need to move the tractors or roll

the platen to get the picture centered on the page)."

Author:

Barry R. Nance

Business Software Services

--------------------------------

SCRNDUMP.COM

To enhance graphics printing for any combination of the following:

EGA/EPSON Extended PrtSc Driver

EGA/OKIDATA 192 Extended PrtSc Driver

EGA/OKIDATA 92 Extended PrtSc Driver

EGA/CITOH Extended PrtSc Driver

CGA/EPSON Extended PrtSc Driver

CGA/OKIDATA 192 Extended PrtSc Driver

CGA/OKIDATA 92 Extended PrtSc Driver

CGA/CITOH Extended PrtSc Driver

HERCULES/EPSON Extended PrtSc Driver

HERCULES/OKIDATA 192 Extended PrtSc Driver

HERCULES/OKIDATA 92 Extended PrtSc Driver

HERCULES/CITOH Extended PrtSc Driver

SCRNDUMP Copyright 1986 by Hardwood Software Associates

Usage:  SCRNDUMP /g /p

Where:  g = E[GA] for Enhanced Graphics Adapter(640x350 Color)

C[GA] for Color Graphics Adapter(640x200 Mono only)

H[ercules] for Hercules Monochrome Graphics Adapter(720x348 Mono)

p = E[PSON] for EPSON/IBM printer

C[ITOH] for CITOH PROWRITER/PROWRITER II printer

1[92] for OKIDATA 192 printer

9[2] for OKIDATA 92 printer

Purpose:

To enhance the PrtSc key to include printing the graphics screen.

Operation:

Press 'Shift/PrtSc' to activate.  If no other key is pressed within 10

seconds the regular text print screen routine is used.  To print the

graphics screen, press the appropriate number key from the upper row.

To print the first graphics page press '1', the second (if your graphics

board has one) is printed by pressing '2' and so on.  When printing the

regular text screen, the time delay can be canceled by pressing the

SPACEBAR.  Graphics printing in progress can be canceled by pressing

'ESC'.

--------------------------------

THINKJET.EXE

See LASERJET.EXE.

--------------------------------

TOSHDUMP.EXE

Toshiba 1351 Printer Graphics Screen Dump

For the IBM-PC and compatible micro-computers.

Written by: Gary E. Walker

Vertical Market Software, Incorporated

Westfield Boulevard

Indianapolis, IN

Use just as you would use GRAPHICS.COM for the Epson

--------------------------------

TOSHGRPH.COM

This program bills itself as version 4.0 of GRAFTRAX, but for the

Toshiba P1340, P1351, or P351 printers.  It uses the same commands as

GRAFTRAX.

--------------------------------

Tandy's GRAPHICS.COM

Another set of printer options is provided by the GRAPHICS.COM that

is distributed with Tandy (Radio Shack) computers.  Options A thru E

support [A] the Tandy CGP-220 printer, [B] the Tandy DMP Standard printer,

[C] PC compatible printers, [D] PC compatible printers in Tandy mode,

and [E] the Tandy DMP-110 printer.

This screen dump utility must be obtained directly from Tandy.

--------------------------------

MLPS.COM

If Shift-PrtScr is not moving your paper to the next top of form then

load this memory-resident program beforehand.

--------------------------------

You can make loading MLPS.COM and any other form of GRAPHICS.COM automatic

by putting MLPS and the name of the GRAPHICS program in your AUTOEXEC.BAT

file.  Of course, the programs must either be in your root directory or

in a bootable path.

If you end up using any of these programs, send a $5.00 contribution

to the programmer responsible for it.