OTVIEW

Version 3.1

Merritt Island Technlogies

253 Merritt Square, Suite 616

Merritt Island, FL 32952

Voice: (407)690-3500

BBS/FAX: (407)459-0969

CONTENTS

1    Disclaimer

2    Overview

3    Hardware Requirements

4    Installation and Configuration

5    Connecting the ONE TOUCH II to Your Computer

6    Navigating the OTVIEW Menus

7    Main Menu

7.1  View/Change ONE TOUCH Settings

7.2  Download ONE TOUCH Data

7.3  Read Stored ONE TOUCH Data

7.4  Clear ONE TOUCH Memory

7.5  Update Patient Name

7.6  Exit Program

8    Display ONE TOUCH Data Menu

8.1  Display ONE TOUCH Data

8.2  Print ONE TOUCH Data

8.3  Plot ONE TOUCH Data

8.4  Save ONE TOUCH Data to Disk

8.5  Append ONE TOUCH Data

8.6  Return to Main Menu

9    Troubleshooting

10   Copyright Information

11   Sample Output

1	 DISCLAIMER

OTVIEW is a tool to allow access to the various capabilities and data

normally made available to the ONE TOUCH II Blood Glucose Meter user

by the Manufacturer. Among these is the capability to change the

meter operating parameters, retrieve stored blood glucose readings,

and to completely purge the internal memory. OTVIEW will allow you to

take control of these same functions available to all OT II users,

but through the convenience of a computer and keyboard interface. As

with any tool of this type, It is NOT to be used as a Diagnostic

tool, nor be used as an indicator of any need to modify a treatment

regimen.  ONLY you and your Doctor should ever modify your medication

dosages and schedules.

2	OVERVIEW

OTVIEW is a small and simple yet powerful tool designed to let you

control the operation of and retrieve data from the ONE TOUCH II

blood glucose meter manufactured by LifeScan, Inc.  It will allow you

to display and modify any of the meter's operating parameters, clear

the internal memory, and to download, annotate, print, plot, and

store readings from the meter.  Details on how to perform each of

these functions is described in the following text.  This program has

evolved from over two years of development and testing, and we have

included many features and improvements suggested by a variety of

people who used this program daily over a long period of time.  We

are indebted to everyone who has made inputs to help make OTVIEW a

better program, and feel we have produced an extremely useful, high

quality product that will greatly enhance both the use of the ONE

TOUCH II meter, and the recording and analysis of blood glucose

levels.

As always, we welcome any questions, comments, suggestions, and

complaints from anyone who uses this program on a regular basis and

has ideas on how to make it better.  Tom McKeever and Mike Peacock

can be reached at the SPACECON BBS (1:374/22, 94:4070/1, or

150:401/8) at (407) 459-0969 at any time.  Phone messages may be left

at our voice mailbox at (407) 690-3500, and E-Mail may be sent to

Tom.McKeever@TECHTALK.COM.

This program was created using Turbo C by Borland, and makes

extensive use of the C Extended Libraries (CXL) (c) by Mike Smedley.

Please note that due to significant changes in the file formats

beginning with version 3.0, this edition of OTVIEW cannot be used

with data files created under OTVIEW versions 2.x or earlier.

3	HARDWARE REQUIREMENTS

OTVIEW is meant to run on an IBM PC or fully compatible clone running

DOS 2.1 or later, and a having little over 200 K of available memory.

It is for reasons of memory and disk space conservation that OTVIEW

was written for DOS. A Windows version can be made available, but

uses up to 10 times the hard drive space and considerably more

memory.  Windows is therefore not required, although OTVIEW will run

under it.. OTVIEW is small enough to run from a floppy diskette, on a

laptop computer, or can be installed on your hard drive. OTVIEW fully

supports Microsoft Mouse compatible pointing devices, and if one is

detected during startup, it may be used to navigate the various menus

and displays.  If you wish to use the data graphing function, then an

EGA or VGA type graphics adapter with appropriate monitor is

required.

Access to a spare serial (COM) port is also necessary, however a

dedicated port is not necessary since it will only be needed for

brief periods when you are actually displaying or modifying operating

parameters or downloading data.  Most modern computers have at least

two serial ports, one of which is usually connected to a mouse, and

the other to a modem.  Many users we have talked to simply unplug

their modem temporarily to connect the ONE TOUCH meter.  If this

becomes inconvenient, serial A-B switch boxes are available from most

computer supply stores (or from us) so you won't have to connect and

disconnect cables.  A special cable, (Available from free of charge

from Johnson and Johnson at 1-800-227-8862) is needed to connect the

ONE TOUCH II to your Computer's Serial (COM) port. This cable has

been available at no charge to registered ONE TOUCH II users, and is

terminated with a 25 pin male connector.  If your computer requires a

female and/or a 9 pin connectors, adapters and gender changers are

also available at most computer supply stores, or may be purchased

from us at the same time you order your registered version of OTVIEW.

4	INSTALLATION AND CONFIGURATION

OTVIEW may be run from either a floppy diskette, or a hard disk drive

by simply copying the files to the appropriate subdirectory.  Note

that the only file required to use OTVIEW is the executable file

itself, no other driver or configuration files are needed.  OTVIEW

may be run from any subdirectory in your DOS 'PATH' string, however

it is necessary to set your default directory to wherever you wish

data files to be saved to or loaded from prior to starting the

program (this version does not allow directory paths to be specified

for data file names).  The simplest approach is to make a separate

subdirectory to contain all program, documentation, and data files.

The only parameter that needs to be configured for OTVIEW is the

patient name.  Start the program, and at the main menu select 'Update

Patient name'.  Enter the name of the person who the readings are

taken from, and press 'Enter' to save.  The use of this parameter is

more fully described below.

5	CONNECTION OF ONE TOUCH II TO YOUR COMPUTER

Setting up OTVIEW with your ONE TOUCH II is quite easy.

1. Connect the Serial cable to an available serial port on your

computer.  (Your Computer and the ONE TOUCH II should both

be off.)

2  Plug in the other end to the receptacle on the ONE TOUCH II Meter.

3. Turn on your Computer and log to the directory where OTVIEW

resides. (e.g. cd\OT3)

4. Type OTVIEW and hit the ENTER key.

5. Read the Opening Screen and hit ENTER to continue, or Escape

to exit.

6. If you wish to display/modify your ONE TOUCH operating parameters

or download stored readings, turn on the ONE TOUCH II and verify

that it is flashing 'INSERT STRIP'

6	NAVIGATING THE OTVIEW MENUS

Moving around within the OTVIEW menus can be done by any of three

methods.  The program fully supports most mouse type pointing

devices.  On menus and submenus where a list of options is displayed,

move the mouse to highlight the entry desired.  The left mouse button

simulates the 'Enter' key to select the highlighted option, and the

right button simulates the 'Escape' key used to abort an operation

and return to the previous menu level.  Each of the menu entries has

a letter (usually the first) capitalized and highlighted.  A specific

option can be selected and activated by simply pressing the same key

on the keyboard.  Also, entries may be selected by using the keyboard

cursor keys, and selected by pressing 'Enter' or aborting the

operation by pressing 'Escape'.  A blinking cursor is displayed when

keyboard input is expected from the user, for example in specifying

patient names or file names, or entering comments in the data display

screen.

7	THE 'MAIN' MENU

The Main Menu gives you the following choices:

"View/change ONE TOUCH settings"

"Download ONE TOUCH data"

"Read stored ONE TOUCH data"

"Clear ONE TOUCH memory"

"Update patient name"

"eXit program"

(Note that the "hot-Keys" are capitalized and highlighted)

Selecting either the 'View', 'Download', or 'Clear' options requires

communications with the ONE TOUCH and will bring up a submenu asking

for the COM port number to which the device is connected.  The menu

will display an entry for all serial ports detected by the program,

and will give an error if it sees that none are present.  Turn on the

ONE TOUCH and verify that it is flashing 'INSERT STRIP'.  Within

about twenty seconds the program should display a message showing

that communications has been established and the current baud rate.

If this does not work or an error message is displayed, then refer to

the later section on 'Troubleshooting'.

7.1	VIEW/CHANGE ONE TOUCH SETTINGS

When communications have been established, the screen showing current

values for the changeable operating parameters will be presented.

Any of these parameters can be changed by either selecting the

capitalized/highlighted letter and hitting Enter, or selecting it

with the mouse. A list of available selections will then be

displayed. Scroll to the setting you wish to change to with the arrow

keys or the mouse, or, in the case of numerical values such as time

or dates, to the number to be entered/replaced. Hit Enter when

satisfied.  When a new value has been chosen, the meter will be

updated immediately and the new selected value should then be

displayed on the screen.  A brief description of each of these

parameters and available options is described below:

WARNING: BE CAREFUL OF WHAT YOU DO HERE.. THESE SETTINGS CAN BE SET

TO GIVE  YOU TOTALLY INACCURATE READINGS AND DATA DUMPS.. USE WITH

CAUTION !

Strip Code : This parameter is a number between 1 and 16 and should

be set according to the  value shown on the box of test strips that

you are using.

Language : The ONE TOUCH II is capable of displaying prompts and

messages in seven  languages.  In order to keep the size and

complexity of OTVIEW reasonable, the program communicates with the

device only in English.  If you are using any other language, the

program will display it as the active value, but will switch the

meter to English when bringing up this screen.  If a value other than

English is selected, it will be displayed on the menu, but the ONE

TOUCH II prompts will not change to  the desired language until this

screen is exited.  Similarly, when a memory dump is requested, OTVIEW

will change the meter to English before downloading the data, then

change it back to the selected language when complete.

Units: Used to select the units of measure, either milligrams per

deciliter (MG/DL), or millimoles per liter (MMOL/L).

Beep: Select 'BEEP' if you wish the meter to beep after each function

or when a new message is displayed, or 'NOBEEP' it you want it to be

silent.

Punctuation: If you have selected MMOL/L as your units, then this

option may be used to specify whether a decimal point or a comma is

to appear in the value for each reading.

Date Format: Tells the meter if you want dates displayed as

month/day/year (M/D/Y), or as day/month/year (D/M/Y).

Time Format: Tells the meter if you want time displayed in 12 hour

AM/PM type format (AM/PM),  or as 24 hour format (24:00).

Date: The current date from the meter will be displayed according to

the currently  selected date format.  You may also select and modify

the month, day, or year.

Time: The time from the meter's clock will be continually read and

displayed on the  screen per the currently selected time format.  You

may set the time in the meter  by selecting either hours, minutes, or

seconds.

Day: The day of the week as calculated from the date in the meter is

shown.  This value  may not be modified, and hopefully matches what

the day really is !

Baud: Used to select the speed with which the meter will talk to your

computer.  The available options are 9600, 2400, 1200, and 300 bits

per second.  Most modern computers will support communications at the

highest rate, so you should normally use the 9600 value.  If you

start having time-out or checksum errors while using OTVIEW, you

might try using a lower speed and see if it helps, although this will

slow down the operation of the program.  For example, downloading 250

readings from the meter takes only about 15 seconds at 9600 baud, but

would require 1 minute at 2400, 2 minutes at 1200, and 8 minutes at

300 baud !  Note also that although the baud rate will be updated as

soon as a new value is selected, the communications rate will not

change immediately.  It will be necessary to exit the program, turn

the meter off and back on again, and then restart OTVIEW to allow it

to sync up with the meter at the new speed.

Mode: This value is used to tell the meter how to communicate with

the computer that is connected to it on the data port.  The available

options are 'RS-232', 'DTAMGR', 'DATA A', and 'DATA B'.  When using

OTVIEW on your personal computer, this value should always be set to

'RS-232', Selecting any other value will cause the ONE TOUCH II to

stop talking to your computer, and you will not be able to use OTVIEW

until the meter has been configured back to 'RS-232'.  If this should

accidentally happen, refer to the section on Troubleshooting to see

how this may be done.

7.2	DOWNLOAD ONE TOUCH DATA

Once Communications has been established with the ONE TOUCH II, you

may download the data currently stored in the meter's memory.  The

program will display the number of readings received from  the meter

as they are transmitted.  At this time the data is only stored in

your PC's system memory and  has not yet been written to disk. The

"Display ONE TOUCH data" menu will now be presented to you.  This

menu will be discussed below.

7.3	READ STORED ONE TOUCH DATA

Selecting this option will display the names of previously-stored ONE

TOUCH II data files  residing in the current directory. OTVIEW files

can be identified by the 'DAT' extension in the  DOS file name.

Highlight the one of interest with the arrow keys or mouse, and

select it with  either the left mouse button or the Enter key. If the

file selected is not a valid OTVIEW data file,  then an error message

will be displayed.  After selection, the "Display ONE TOUCH data"

screen  will then be presented.  This menu will be discussed below.

7.4	CLEAR ONE TOUCH MEMORY

As previously cautioned, this command will clear the ONE TOUCH II

on-board memory.  It is only used when data readings have been saved

to a disk file and you wish to start a "fresh" sequence of readings

in the meter.  This is done to prevent duplicate readings from being

stored in a concatenated (sequentially added) data file.  It may take

as long as 10 to 20 seconds for this operation to complete.  BE

CAREFUL WITH THIS COMMAND. DATA IS IRRETRIEVABLE !

7.5	UPDATE PATIENT NAME

The data file saved to disk can be customized with the patient's name

and/or other Identification.  Use This command and entry field to

enter the designated patient's name or ID.  This entry will be

written back into the OTVIEW program file, so it is only necessary to

enter this information once.

7.6	EXIT PROGRAM

Terminates the program's execution and returns you to DOS.

8	THE 'DISPLAY ONE TOUCH DATA' MENU

This menu gives you the following choices:

Display ONE TOUCH data

Print ONE TOUCH data

Plot ONE TOUCH data

Save ONE TOUCH data to disk

Return to main menu

8.1	DISPLAY ONE TOUCH DATA

This selection will present you with a very detailed summary of the

ONE TOUCH II data downloaded from the meter or read from the disk

file. The display is titled with the name of the currently-loaded

data file, and shows a header containing the date and time from the

meter of when the most recent download was performed, as well as the

name or ID of the patient. In addition, the ONE TOUCH II Units

setting, Minimum and Maximum Check Values, and Serial number are

displayed as read directly from the meter. Minimum, Maximum and

Average blood glucose values are calculated and displayed at the top

of the screen.

The rest of the display contains a line for each individual data

entry read from the meter.  Readings are sorted by date and time,

with the most recent entries first and the oldest last.  Each entry

includes the day, date, and time the reading was taken, the actual

blood glucose value in the appropriate units, and the event number

you may have entered for the reading (or zero for default).  If a

sample was taken that was measured as off scale high, the program

will substitute a value of 600 MG/DL (or 33.3 MMOL/L) for that entry.

Also displayed is a 'FLAGS' field for each line.  If an entry is

marked as a check or control reading by the meter, then a 'CHK' or

'CON' flag will appear in this field, respectively.  For each entry,

the ONE TOUCH II contains error checking information to ensure that

the data contained in its memory has not been corrupted in some way.

If the meter has declared a reading as invalid, or an error was

detected by OTVIEW when it was downloaded from the meter, then an

'INV' indicator will appear in the flags field.  Entries marked as

check, control, or invalid are not included in minimum, maximum, or

average value calculations.

To the right of the screen is an area where you can record notes and

comments for each entry.  Examples of how this feature may be used

can be seen in the example included in this document and in the

sample file OTSAMPLE included with the program.  As with the other

menus, either the mouse or the cursor keys may be used to scroll

through the data.  The screen may be scrolled up and down by

positioning the mouse cursor in the header or in the bottom line

respectively and pressing the left button, or you may use the up and

down cursor keys to scroll one line at a time, or page up/page down

to scroll one screen at a time.  The 'Home' key will put you on the

first reading in the file, and 'End' will bounce you to the last

entry.  A pair of blinking brackets will be displayed around the

reading on the row where the cursor is currently positioned in order

to help you line up your comments with the appropriate entry.

The cursor key can be positioned anywhere on the screen for entering

notes by pressing the left mouse button or the cursor keys.  When

entering comments from the keyboard, the left cursor, right cursor,

insert, delete, backspace, and enter keys function just like a normal

text editor.  Note that the 'Notes' field is the only portion of the

data file that the program will allow the user to modify directly.

In order to ensure accuracy and integrity of the data, OTVIEW will

not make or allow any changes to any other fields in the readings.

Please keep in mind that when viewing or making notes on data that

has just been downloaded from the meter (identified by the title

'Display Download Data' at the top of the screen), the entries are

still in the computer memory and has NOT been saved to a disk file

yet.  To do this, you must select 'Save ONE TOUCH Data to Disk' (see

below).  If you attempt to exit the program without first saving the

data to disk, the program will display a warning asking if you first

wish to do this before you exit.  If you decide to terminate OTVIEW

anyway, the data (and notes you have made) will be lost and must be

downloaded again.

Note: Some of the newer version of the ONE TOUCH II meter now

calculate and display the average value for blood glucose reading.

This number may be different than that calculated by OTVIEW in that

the meter maintains an incremental average (that is, each new reading

is averaged in with the previous value to form a new average) whereas

OTVIEW computes a new value from scratch based on each individual

reading.  The program also does not include readings that are marked

as check, control, or invalid.  So although the actual numbers may be

different, the average value calculated by OTVIEW is considered to be

more accurate than the one given by the meter.

8.2	PRINT ONE TOUCH DATA

This selection will print out all the information shown in the above

display to the default printer, including the file name, patient

name, and any annotations made in the entries. There can be up to 250

readings, so allow sufficient paper for the print-out!  (This feature

has been disabled in the demonstration version OTDEMO).

8.3	PLOT ONE TOUCH DATA

This selection will display a sub-menu that will allow you to enter

the starting and ending dates for the ONE TOUCH II data currently in

memory that you wish to plot.  These default to the dates of the

earliest and latest readings, and follow whatever date format is used

in the entries (M/D/Y or D/M/Y).  The second sub-menu will show how

many readings were found within the specified date interval (or an

error message if none were found), and will also allow you to set the

high and low limit for the vertical axis of the plot.  These will

default to the highest and lowest values found within the date

interval, and will be in whatever units were specified in the file.

As stated above, values flagged as check, control, or invalid are not

considered valid entries for plotting. After entering these limits,

the plot will be displayed on the screen until any key or mouse

button is pressed. The patient's name, data file name, and units will

be displayed at the top of the screen. Exiting from the plot will

return you to the value limit and date range submenus sequentially.

If you have an IBM compatible graphics printer connected to your PC,

a hard copy of the plot may be printed by using the DOS utility

GRAPHICS.COM.  Simply load the GRAPHICS.COM program before starting

OTVIEW, and when the plot is displayed on the screen, press the

'Print Screen' key.  Refer to your DOS reference manual for more

information on GRAPHICS.COM.

After much experimentation, it was determined that a minimum screen

resolution of 640x348 is necessary to display a graph that is

legible.  This means that the plot function cannot be used if you

have one of the older Color Graphic Adapter (CGA) cards since their

maximum resolution is 640x200.  The OTVIEW plot function has been

successfully tested on VGA, EGA, MCGA, Hercules, AT&T, and IBM 8514

type graphic adapters with an appropriate monitor.  If a suitable

adapter is not detected, or it cannot be switched to a video mode of

acceptable resolution, then this function will display an error

message.

One thing to keep in mind as you are analyzing these graphs is that

the horizontal axis is normalized with respect to the relative time

the readings were taken, not an absolute time scale.  In other words,

the individual readings are plotted in an even, linear spacing, and

the date scale on the horizontal axis is labeled according to the

time of the corresponding data values, and not the other way around.

If you take the same number of readings every day on a consistent

basis (more or less), then this should not present a problem.

However, if there are wide gaps in time between groups of readings,

or they are 'bunched up' in one or more ranges of dates, then the

graph may not necessarily display an accurate portrayal of your blood

glucose level throughout a continuous, linear time frame.

If you plan to print out and maintain a record or archive of the

plots for different time periods, then one tip we have found useful

is to use the same date range (i.e. 2 weeks, one month, etc.) and

value limits for each plot.  This makes it much more easy to compare

separate plots with one another, and allows you to lay graphs out

side by side and see a pictorial representation of your blood glucose

level over a long period of time.

8.4	SAVE ONE TOUCH DATA TO DISK

This selection will allow you to save the current data (including

annotations) to disk file. A default name of 'OTmmddyy' (or

'OTddmmyy' depending on the date format selected) is already entered

for you, but you can use any legal eight character DOS file name that

you wish.  As stated above, all ONE TOUCH II data files use a file

extension of '.DAT', and they are stored in whatever the current

default directory was when you started OTVIEW.  If a file by that

name already exists, then a warning message is displayed and you are

prompted to either overwrite the existing disk file, or chose another

name.  Error messages will also be displayed for any other disk error

detected during the save (Drive Not Ready, Drive Write Protected, Not

Enough Room, etc.).

One advantage of OTVIEW is that the data is stored as a standard text

file in exactly the same format in which you see it printed or

displayed on the screen, rather than in some binary or non-printable

form such as a data base file or word processor file.  This means

that the OTVIEW data file can be copied or uploaded to any other

computer at a doctor's office or hospital, or even fed into a FAX

modem for transmission to your medical facility.  Everything that

your doctor needs to examine your blood glucose level performance is

included in this file: Patient name, dates and times, overall

statistics, and individual readings.  Data files may also be brought

up under a standard word processor for review and editing.  In

addition, a separate utility is also under development which will

convert OTVIEW data files into comma delimited format (CDL) files, so

that this information may be imported directly into many commercial

spreadsheet and data base applications, allowing for more elaborate

graphing, plotting, and statistical analysis to be performed.  This

utility will be distributed in the near future to registered OTVIEW

users at no charge.

Also note that the save function is not available in the

demonstration version OTDEMO.

8.5	APPEND DATA TO DISK FILE

Once you have downloaded a set of readings from the ONE TOUCH II, the

Append option will be displayed in order to allow you to add the new

readings into an already existing data file.  Just select which disk

file you wish to append the data to, and if it is a valid OTVIEW data

file, the program will insert the new data into it and tell you how

many new readings have been added.  You may then review the data and

add notes, or print or plot the information, but as stated above, the

updated entries are not automatically saved to disk.  You must do

this yourself when you are finished, and if you attempt to exit the

program without first saving it, OTVIEW will display a prompt

reminding you to do this before terminating the program.

As previously mentioned, entries in the data files are stored with

the most recent reading first and the oldest ones last.  When you

tell OTVIEW to append to add new readings to a data file, the program

reads the date and time of the most recent entry in the file, and

then scans the downloaded values to determine if any of them are

newer than this.  If no new entries are detected, an error message

will be displayed, otherwise OTVIEW will show you the number of new

entries added to the beginning of the file.  Values downloaded from

the meter with a date and time tag older than the most recent entry

in the file will not be saved, in order that an accurate ordering of

entries by date and time can be preserved.  For this to be possible,

it is necessary that the meter settings for blood glucose units, date

format, and time format must match those of the data file to which

the new readings will be appended.  If not, an error message stating

that configuration parameters do not match those in the data file

will be displayed.

New readings may be added to a data file until it reaches a maximum

size of 250 entries.  If the file is already this full, then a disk

file full error message will be shown.  In addition, if there is not

enough room in the data file to store all the newly downloaded

readings, then a message will be displayed asking if you would like

for OTVIEW to save only the oldest of the newly downloaded readings

up to the maximum disk file size of 250, or if you want to abort the

operation and create a new disk file.  From actual experience, it has

been determined that this limit of 250 entries does not really

present that much of a problem.  Data files much larger than this

begin to become awkward when trying to review and analyze the

readings, and inconvenient to print or plot.  A typical scenario is

to have one data file per month, and add the new readings to it every

five to seven days.  At this rate, data files can handle over eight

readings per day, which should be more than sufficient !

When an append operation is performed, the summary information in the

file header will also be updated.  The date, time, serial number, and

minimum and maximum check strip values will be modified to reflect

the most recent values of these parameters received from the meter.

The patient name will also be updated if it has been changed since

the file was originally created or last appended.  In addition, new

values for minimum, maximum, and average values will be calculated

based on all readings now in the file.

If the append operation has successfully added all new readings to

the disk file, then the user will be asked if he wants to clear the

meters memory upon exiting the program.  It is your decision whether

or not to maintain old readings in the meter after they have been

saved to a disk file.  However, previous experience has shown that it

is much easier and more convenient to review, manipulate, and

communicate information obtained from the meter when it is saved as

text in a disk file rather than being kept in the meter, where you

must manually scroll through every value for each reading one at a

time.  The data is also much more secure since backup copies of the

data files can easily be made.  It should not be necessary to remind

you to always keep backup of all your data files, preferably on

floppy disks !

8.6	RETURN TO MAIN MENU

When you are finished with these functions, this selection will

return you to the main OTVIEW menu.  If you have downloaded data from

the meter and have not yet saved or appended it to a disk file, the

program will ask you whether or not you want to save the data before

exiting this screen.

9	TROUBLESHOOTING

OTVIEW is designed to be a simple, easy to use tool that should take

very little time in order to become acquainted with how it operates.

The prompts and messages are self explanatory, and when used per the

directions and guidelines as outlined in this document, you should

have little or no trouble in running the program.  The only area

which may appear to be a bit tricky is getting the meter to talk

correctly to your computer.  We have included here a few tips we have

picked up in our experiences with developing this software, and they

should help you to resolve any problems which you might encounter.

When using this program with your ONE TOUCH II meter on a daily

basis, it is not uncommon to receive an occasional Time-out or

Checksum error.  This is particularly true when sending a number of

different commands to the meter such as when you are configuring

multiple operating parameters, and is also commonly seen when

clearing the meter's memory.  Whenever you have different computers

executing different software at different speeds and trying to talk

to each other in a spontaneous manner, communications problems are

almost certain to be seen.  Normally this does not mean that there is

any problem with either the meter or your computer.  Commands sent to

the meter which result in errors are automatically retried several

times by OTVIEW before an error message is displayed.  If a Checksum

or Time-out error message is presented, just make sure that the meter

is still on and flashing "INSERT STRIP", and press enter to retry the

operation.  The ONE TOUCH II meter has a built in timer, so if it is

turned on and not used (or if no communications through the data port

are detected), it will automatically turn itself off after about one

minute.

If your are unable to establish communications with the meter at all,

there may be several possible reasons.  Make sure that the connectors

on each end are inserted tight, and the 25 pin connector on the

computer end should be screwed down into its socket.  Make sure you

are plugged into the correct serial port, not a parallel port by

mistake, and that this matches the port number that you are selecting

on the OTVIEW menu.  If you still have problems, then try a different

serial port, or better yet a port on a different computer, if you

have access to one.  To verify that there is no trouble with the

OTVIEW program itself, try communicating with the meter using a

standard terminal program, such as Procomm.  Configure it for the

correct serial port and baud rate, 8 bits, no parity, and flow

control off.  When you turn the meter on, a line should be displayed

showing the current values for the operating parameters, and then

continuous 'INSERT' 'STRIP' messages will be received.  If you type

in the command 'DMP', the readings from the meter's memory should be

displayed on the screen.

In the event that there is still no data seen from the meter, the

reason might be that it is configured to use a data communications

mode other than RS-232.  To check for this and to set this parameter

to RS-232 if it is not already, follow these steps:

1.	Turn the meter off.

2.	Press and hold the 'M' (memory) button on the rear of

the meter.

3.	Turn the meter on.

4. 	Release the 'M' button. The value of the BEEP/NOBEEP

parameter should now be

seen on the display.

5.	Press the 'M' button seven times to display the value

of the Data Management Mode.

6.	If this does not show 'RS-232', press the 'C' (code)

button until 'RS-232' is displayed.

7.	Turn the meter off.

8. 	Start OTVIEW and attempt to establish communications

with the meter.

10   COPYRIGHT INFORMATION

This program is being made available on an 'as is' basis, and carries

no warranties express or implied.  The authors shall in no way be

held liable for any damages resulting from the use of this program or

the media on which it is distributed, including, without limitation,

misrepresentation of data, loss of information, damage to equipment,

or any other incidental or consequential damages.

Neither the authors, nor Merritt Island Technologies, Incorporated,

nor the SPACECON BBS are affiliated with LifeScan, Inc., or Johnson

and Johnson, Inc.. One Touch II (r) is a registered trademark of

LifeScan, Inc.  MS-DOS, and Windows are registered trademarks of

Microsoft Corporation.  OS/2 is a registered trademark of IBM Corp.

OTVIEW, OT_VIEW, and OTDEMO are proprietary names for exclusive use by

Merritt Island Technologies, Inc. of Merritt Island, Florida, USA.

11	SAMPLE OUTPUT

This is the format of the output from OTVIEW taken from a set of

actual blood glucose readings.  The data file OTSAMPLE included with

this program also shows what the screen and data file formats look

like.

DATE:  8/30/93   TIME: 11:56:00 PM   NAME: Jane Doe

UNITS: MG/DL     CHK MIN:  73     CHK MAX:  98     SER NO: CHS0196AA

MIN VAL:  31     MAX VAL:  405    AVG VAL:  126

DAY    DATE       TIME      VALUE  EVENT   FLAGS          NOTES

---    ----       ----      -----  -----  -------  --------------------

TUE   4/20/93   9:23:00 PM    81     0    CHK

TUE   4/20/93   9:22:00 PM    31     0

TUE   4/20/93   9:21:00 PM    81     0    CON

SUN   4/18/93   1:20:00 PM    59     0

SUN   4/18/93   9:36:00 AM   172     0

SAT   4/17/93   8:51:00 PM   211     0             Ate Dessert

SAT   4/17/93   3:24:00 PM    46     0

SAT   4/17/93  12:50:00 PM   222     0

SAT   4/17/93   8:58:00 AM    53     0             Fasting

SAT   4/17/93   1:41:00 AM   101     0

SAT   4/17/93   1:13:00 AM    31     0    CHK

FRI   4/16/93  10:05:00 PM   202     0

FRI   4/16/93   9:00:00 PM   405     0             Infusion set bad

FRI   4/16/93   1:12:00 PM    65     0

FRI   4/16/93  11:07:00 AM    56     0

FRI   4/16/93   6:44:00 AM    60     0             Still feel bad

THU   4/15/93   9:18:00 PM   194     0

THU   4/15/93   2:56:00 PM    53     0

THU   4/15/93  10:27:00 AM    55     0    CON

THU   4/15/93   6:38:00 AM    92     0

WED   4/14/93   8:33:00 PM   143     0

WED   4/14/93   3:15:00 PM    51     0

WED   4/14/93   1:50:00 PM    58     0             After Lunch

WED   4/14/93  11:25:00 AM   228     0

WED   4/14/93   6:49:00 AM    49     0             Before Breakfast

TUE   4/13/93   9:25:00 PM   209     0

TUE   4/13/93   4:28:00 PM    70     0

TUE   4/13/93   1:31:00 PM   114     0

TUE   4/13/93  10:47:00 AM    58     0             After Exercise

TUE   4/13/93   8:24:00 AM    60     0

TUE   4/13/93   6:54:00 AM   147     0

MON   4/12/93   8:17:00 PM   176     0

MON   4/12/93   3:04:00 PM   104     0

MON   4/12/93  10:58:00 AM    45     0

MON   4/12/93   6:23:00 AM   258     0             stressed out at work

SUN   4/11/93   8:20:00 PM    90     0

SUN   4/11/93   5:09:00 PM    23     0

SUN   4/11/93   1:59:00 PM    78     0

SUN   4/11/93  12:52:00 PM    44     0

SUN   4/11/93   9:45:00 AM   108     0             skipped breakfast

SAT   4/10/93   7:27:00 PM   125     0

- END OF SAMPLE EXTRACT -

-EOF-