The "IP:" Pipe Device

_____________________

-- Pete Goodeve --

August 1990

Overview

________

"IP:" is a handler that may be mounted in your system like any other

AmigaDOS device, to provide you with both standard "pipe" type facilities

and a number of features not available elsewhere.  In particular, as a

pipe, but unlike other pipes available for the Amiga, it has "unbuffered

pass-through"; in other words when a writer process sends a packet it

is immediately available to the reader.  (The manual description of the

1.3 pipe claims that it also does this, but in fact it does not -- the

reader gets no data until either the buffer is full or the writer closes

its end of the pipe.)  This feature makes it possible for processes to

actually interact via these pipes: sending a command line, for example.

will get an immediate response from the recipient.

In addition, an IP channel need not close when the program at either

end goes away.  It will remain open as long as there are ANY processes

(readers or writers) referencing it.  Normally it will send an end-of-file

to the reader (usually causing it to exit) when the last writer closes

its reference to the channel, but it is easy to inhibit this so that the

reader will remain active and waiting for input, whether or not there are

any writers, until a specific QUIT request is sent (see below).

Further, although only one reader may be attached at a time, you can have

multiple writer processes sending output to a single IP channel at the

same time -- a "Funnel", rather than a Pipe.

There is a special convention for opening both an input AND an output

channel with the same specification, so that you can for example attach

a Shell process to piped input and output instead of a window.  As IP

pipes don't buffer, this becomes a workable arrangement.

The IP device is actually based on the "PPIPC" protocol, and the "channel

names" are actually IPCPorts, so it provides a general mechanism for

programs that use this scheme to send text or other unstructured data to

other programs that don't know about PPIPC.  It also means that control

information may be sent (via IPCMessage) to an IP channel; the only

such that is publicly available at this time is the 'QUIT' message

mentioned above, but other capabilities can be provided when appropriate

programs are available.  (A separate program to send the QUIT is provided.)

Because all data is passed via publicly named IPCPorts, you can easily

interpose programs to extend performance in various ways.  Possibilities

(not actually written yet!) include things like a "tee", to make data

simultaneously available amongst several processes, and a "tank" that

would dynamically buffer data to decouple writer and reader more

completely (allowing a certain number of unread packets to accumulate

before blocking the writer).

Requirements and Installation

_____________________________

The IP device needs a number of elements to be set up for its correct

operation.  Most fundamentally it requires 'ppipc.library' to be present

in LIBS:, and of course it needs the handler code itself ('IP-Handler')

in the L: directory.  To allow it to be mounted, it will need a suitable

entry in DEVS:MountList, and for permanent installation you'll also want

a "Mount IP:" command in your Startup-Sequence.  For convenience, you will

probably want "IPCQuit" in C:.

This can all be done for you with minimum fuss -- but with your permission

at each step -- by the installation icon "Install_IP_Device".  This runs a

script that goes through all the above steps, keeping you informed and

asking permission at each stage.  Once you have installed the library, you

can start up the device without actually installing anything else, if you

like, by answering "NO" to the query regarding installation of the handler,

and then "YES" to the question about a temporary trial.

If you prefer to do it yourself by hand, "Install_IP_Device" is actually a

directory (only accessible through the Shell or CLI) containing all the

necessary pieces.  (It is completely self-contained, by the way; you can

copy the whole system to any desired location simply by dragging the icon.)

You don't really need to know anything about PPIPC to use this device,

but if you want full details (and I definitely encourage this!) the

complete package is on Fish Disk 290.

The code itself should run under any WorkBench from about 1.2 on (including

2.0 -- it has been tested there without encountering any problems).  The

installation script uses a couple of command options not available under

1.2 (and has not been tested there) so it may not work completely, but

probably should go most of the way; it is perfectly happy under 2.0.

Use as a standard Pipe

______________________

Once the IP device is installed and mounted as above, you use it just

like other AmigaDOS pipes.  Each channel (that is simultaneously open)

must have its own name, specified in the usual Amiga device:name format,

for example 'IP:xyz' or 'ip:MyChan'.  (These are not true "filenames"

of course.  In particular, there is no "directory structure"; '/'

separators have a different meaning -- see later.)  As distinct from

most AmigaDOS names, the name part is case-sensitive: 'xyz' and 'XYZ'

are different channels.  This is because the channels are actually

IPCPorts, where case is important.

To pass data between two processes, you simply open a particular channel

for writing by one and reading by the other.  Thus as a trivial example:

RUN LIST >IP:xyz

TYPE IP:xyz

or equivalently:

RUN TYPE IP:xyz

LIST >IP:xyz

Either reader or writer may be started first; it will block on its first

I/O request until the other end is ready.  (It is important that at least

the first command be invoked by RUN -- or of course from a separate CLI

window if you prefer -- because it will not continue until the connection

is made.)

Once made, the connection will remain open until the writer closes it,

at which point an end-of-file is sent to the reader, which can in turn

close its end.  This is the conventional sequence, but it may differ at

the option of either program.  If the reader chooses not to exit when

sent an EOF, the channel will remain open for further input from other

processes (further details in the next section).

On the other hand, the reader might terminate on some signal other than

EOF, in which case another reader could attach itself to the channel to

process further output from the writer.  Note, however, that any output

sent to a IP channel which has no current reader -- except when a

connection is being first established -- is DISCARDED; a write error is

actually also returned to the sender, but the majority of programs

unfortunately ignore this.

Use as a "Funnel"

________________

As mentioned you can have as many processes writing to an IP channel

as you want.  These can be either running in parallel or concatenated

sequentially, as appropriate; in the latter case you must inhibit automatic

closing of the channel as described below.  Data is passed by packet as

written by each sender (usually a line at a time, though some programs --

such as COPY -- may use packets of a different size);  if more than one

process is writing at once, packets from each will be interleaved in the

order that they arrive.

Writer processes may attach and detach themselves on a channel as they

require, without disturbing the reader, until the last writer closes,

at which time the reader gets an end-of-file, causing it too to close

(usually).  An EOF, by the way, is -- as normal in AmigaDOS -- an empty

data packet.

If you wish to keep the channel open on a semi-permanent basis, you can

suppress this automatic EOF.  To do so, you prefix the channel name that

you open for reading by "!":

TYPE IP:!xyz

The exclamation mark is NOT a part of the channel name: writers still

normally access it as "IP:xyz"  (for writing, the mark has a different

meaning: "exclusive write" -- see below).

If you do open a reader in this mode, when you eventually want to close it

you will have to do so by some other method: either terminate it directly

somehow -- by some recognized command, say -- or explicitly cause an EOF by

sending the channel a QUIT message.  Using the IPCQuit command (a separate

program provided with the package), you specify the name of the channel(s)

you wish to terminate, WITHOUT the "IP:" prefix:

IPCQUIT xyz

Exclusive Write Mode

____________________

You can prevent a writer process that intends to open a particular channel

from actually opening it if it already exists by specifying "exclusive

write".  The exclamation mark is again used to specify this:

LIST >IP:!xyz

If ANY process (reader or writer) currently has this channel open, the

request will be rejected.  Note that this mode only affects the specific

request in which it is used.  Any later request that does not include

the "!" will connect normally.

A caution:  the channel is actually opened momentarily to make the check;

if there happens to be just a single reader (in non-funnel mode) on the

channel at that time, it will get an EOF as a result.  This scheme should

only be used where the write end is always opened first.

This mode will probably be of use mainly in things like ARexx scripts

that may have several simultaneous multitasking incarnations.  The script

can loop through a sequence of generated channel names until it successfully

opens one, which it then knows is unique and safe to use.

Bidirectional Data

__________________

There is an option for IP channel name specification by which you may

specify both input and output channel at the same time.  This is ONLY

useful for programs that normally expect a "CON:" window specification at

that point -- in other words it is really only useful for NEWCLI or

NEWSHELL.

The convention for this is to specify the input and output channel names

-- which must of course be different -- separated by a slash ("/")

character:

NEWSHELL IP:inchan/outchan

The exclamation mark modifier may be used on either part, but it is

superfluous on the input side because the Shell and CLI ignore EOFs

on that channel.  In fact it is probably undesirable since it would

prevent programs running under the shell from receiving an EOF either!

"Exclusive Write" could well be useful on the output side, though, as

NEWSHELL will fail (with return code 20) if the connection fails, so

whatever script has invoked it could detect the condition.

Clogged Pipes

_____________

A word or two may be useful on what to do if a program at one end or other

of your pipe fails to behave as expected, and leaves the other end hanging.

In general all you'll need to restore normality is an available Shell/CLI

window from which to give a command or two.

If you start a reader successfully, but the writer process never gets

going, the reader will just keep waiting.  If you just made some typing

error in the writer command line, you can just try again with a corrected

line.  If you can't do this, and as long as you didn't open it as a

maintained "funnel" (with "!"), all it needs is some process to first open

a writer then close it, to send an EOF and let it terminate. A simple way

would be:

ECHO >IP:xyz ""

More convenient still is to send a QUIT message, and this also works

for funnels as explained above:

IPCQUIT xyz

If it's the reader side that fails, the writer will be the one to be

left hanging in the air, waiting for somewhere to send its output.  The

best way to clear this is to start some other process to take the output

from that channel.  You could either type it to the screen, or if it is

voluminous or non-text, send it off to NIL:

TYPE IP:xyz to NIL:

Then of course there's the situation where you mistyped the channel name

in one of the two commands, so you have TWO processes going nowhere and

talking to no one.  The most direct solution is to clear one or both

processes as described above, but if you're really desperate -- or just

want to play -- you could simply start a THIRD process, to form a bridge

between the two loose ends, with a command like the following!

TYPE IP:xyz to IP:xzz

Use TYPE rather than COPY, by the way, because the latter does some odd

things when it opens a destination -- checking for existence and so on --

with multiple opens and closes that will result in your destination channel

getting shut before it ever gets started.  For this reason you should also,

if you ever want to COPY to an IP channel, either start the COPY command

FIRST or use maintained funnel mode on the receiver.

One other problem that might crop up if you have two programs communicating

with each other by a pair of IP pipes is circular lock-up.  If one of the

two reads an input line from one pipe, processes it, and puts some output

on the other, it will then hang until that data is read by the other program

-- the one that is sending IT input.  Each program therefore must be sure

to read ALL the current output of the other at each cycle before attempting

to send any of its own, or the other end will never reach to point of

reading it, and there will be a lock up.  If this does occur, judicious

application of IPCQuit should break the lock.

Packet Management

_________________

As this is a DOS device, data is passed into, and taken out of, an IP

channel as standard DOS packets.  The (maximum) amount of data a packet can

contain is determined by the program doing the I/O.

When a packet is written to an IP channel, the whole packet contents is

always passed (as an IPCMessage) to the IPCPort acting as the rendezvous.

There is no copying at this stage -- just a pointer is passed; the sender

is suspended until the data has been accepted by the reader.

If the packet supplied by the reader to receive the data is large enough

to take it all, the data is copied to the receiving packet and the sender's

packet is released.  If the receiving packet is too small, as much data

as possible will be copied, and the sender will remain suspended until

further reads have dealt with all its data.

Symmetrically, of course, if a reader sends a packet (makes a read request)

to the IP device when there is no packet from a writer waiting, it will

be suspended until one arrives.

PPIPC Specifications

____________________

It is a little ironic that while one of the specific advantages claimed for

PPIPC is its concern for the structure of data, this application essentially

passes unformatted byte streams.  However this is of course dictated by

the expectations of the programs that will connect to the IP device.

The application does make good use of the "safe connection" aspect of

PPIPC, though.  This section describes the format of IPCMessages sent

and recognized by the IP device.  Knowledge of the PPIPC protocol will

be needed -- see Fish Disk 290.

IPCPorts:               Each IP device channel uses an IPCPort to pass

its data through. The name of a channel's port is

the string following the "IP:" device identifier prefix (and remember

case is significant).  Any other PPIPC-capable device can communicate

with this port, without having to know that it is an IP channel

(provided that its messages have suitable IDs).

A request to open an IP channel for reading will attach that IP

process to the port as a Server (therefore there may be only one

reader at a time).  A request to write will attach it as a Client:

the first connection attempt is made with LoadIPCPort(), so that if

it is not already served and there is a Broker running, the appropriate

startup action can be taken; if this fails, the connection is simply

made with GetIPCPort(), BUT -- if there is no server -- the 'IPP_LOADING'

flag is also set so that the first message sent will not bounce.  This

assumes that eventually a server WILL arrive!

When an AmigaDOS 'close file' request is sent, a writer will simply

drop its port and quit.  The reader first 'shuts' the port, ensures

that any unprocessed messages are returned with 'IPC_REJECT' set,

and then quits.  The Server will detect the disappearance of its

last Client and -- unless opened as a "funnel" -- will automatically

send an EOF (null packet) to its reader process.

IPCMessages Sent:       An IP output connection itself will send only

one kind of message.  Its ipc_Id field is 'DATA'

and it has one IPCItem whose ii_Id field is also 'DATA'.  ii_Size is

set to the number of bytes in the packet received from the writer, and

ii_Ptr points to that packet data buffer.  No assumptions are made as

to the nature of the data block.  The message must of course be replied

when processed by the reader.

IPCMessages Received:   The reader (Server) end can handle several other

message formats besides the one above.  It expects

ipc_Id to be one of 'DATA', 'TEXT', 'CMND', or 'QUIT'.  (A fifth ID

'SETP' is also recognized.  It is used to set up some other parameters

that are used by an as yet unreleased application.  It will not be

described further at the moment.)

All messages are processed in the order that they arrive.  The first

three message IDs are treated identically.  'QUIT' causes an EOF

(null packet) to be sent to the reader.  Any data message should

contain an item with one of three ii_Id's: 'DATA', 'TEXT', or 'LINE'.

In the current implementation only the first item with one of these IDs

will be found in a message -- any preceding irrelevant items, and

anything following the first found, will be ignored.

The contents of a 'DATA' item is passed to the reader's AmigaDOS packet

exactly as received (broken into suitable pieces if it is too big).

Its size is taken from the ii_Size field of the item.

A 'TEXT' item is treated similarly, except that it is assumed to be

a null-terminated (ASCII) string, and its size is calculated up to

the first null byte.  (It is not allowed to exceed ii_Size, however.)

The null byte is NOT copied to the DOS packet!  (Text length is

indicated by the return value in the packet.)

A 'LINE' is a null terminated ASCII line of text WITHOUT the

terminating newline.  The missing newline is added to the packet

before passing it on.

Normally the Server will reply all messages that it receives, but

a Client (this won't be an IP process naturally) may also send one that

is not to be replied, IF it ensures that the following is true: a) the

data being passed is in the user data area of the message itself, b)

the ReplyPort field is NULL.  (For completeness, 'IPC_TRANSFER' should

be set in ipc_Flags, but this is not currently checked.)  If an IP Server

does receive a message with no replyport, it ASSUMES that it can safely

delete it, and does so without further fuss.

+ + + + +

Coda

____

I suspect many people's reaction to the IP Device may be "Cute... but so

what?".  Its utility may not be immediately obvious, but think about it

and you may find that it's exactly what you need.  It is just another

building block that in certain places will turn out to be a perfect fit.

Most straightforwardly you should be able to replace all use of the 1.3

pipes (provided that you don't rely on buffering to pass data from one

program to another under the same CLI).  I have been frustrated by the

erratic behaviour of the 1.3 pipes -- they will fail to open for no

apparent reason, or irretrievably lock up.  Any "lock-up" of the IP

device should be easily clearable (see "Clogged Pipes", above), and as

far as I can tell it behaves completely reliably.

In addition you get the non-buffered immediate response, and the ability to

concatenate output from more than one program, which are not available

from other pipes.  I haven't included any examples or demos here, because

although the IP dvice is now central to my environment, the scripts I

use are also tailored too much to my needs to be much use to others.

And, quite frankly, most of the appplications I envision for it I haven't

even tried yet.

One such use is as a link in a "SuperShell".  You could have a Rexx script

(or Tcl or something -- let's not restrict ourselves) that keeps an IP

channel open to a Shell process.  Commands would be examined by the script

and processed in some really intelligent fashion before being passed to the

Shell for execution.  (ARexx can of course send command strings to AmigaDOS,

but a new process is opened for each one, so you can't do something as

simple as change a directory.)

The "tee" and "tank" modules mentioned in the overview will add to the

capabilities when they arrive.  In the meantime, see what you can do

with the current ones.

If you have any comments, requests or suggestions, you can contact me

at the address below.

+ + + + +

The IP Device and this manual are Copyright 1990 by Peter Goodeve.

There is no restriction on their non-commercial use or copying.

If you wish to incorporate the IP device in a commercial product or

distribution, please contact the author.  (By 'commercial', I mean

a distribution for which more than a nominal materials and copying

fee is charged.)

+ + + + +

Pete Goodeve

3012 Deakin Street #D

Berkeley, Calif. 94705

U. S. A.

net: pete@violet.Berkeley.EDU