NOTE: This file describes how to modify a Mouse Systems M3 serial mouse

so that it will directly replace the normal Amiga mouse.  Although this

requires a little tinkering with the mouse, I think it is preferable because

it it TOTALLY compatible with ALL software and does not tie up a serial port.

Of course, I discovered this AFTER writing the OptMouse serial mouse program.

Ed Hanway

149 Scotch Pine Dr.

Rochester, NY 14616

Inside my Mouse Systems M3 serial mouse is a row of 10 holes with pin 1

closer to the center of the mouse. Wired as follows, it works perfectly

on my Amiga:

Mouse	Amiga

1 ----- 9

2 ----- 5

3 ----- 6

4 ----- 1

5 ----- 3

6 ----- 4

7 ----- 8

8 ----- 7

9 ----- 2

10 NC

All you need is a length of nine-conductor wire, a DB-9S connector, and

minimal soldering skills and you're set.

This probably doesn't apply to other Mouse Systems mice, like the M2 and M4,

and may not even be true for other revisions of the M3. Be warned.