This document describes the operation and construction of a one megabyte

memory board with optional battery backed up clock for the Amiga. You can

also build a 512k memory by only populating one bank. I do not mention

elsewhere that the software needs to be modified to match the new limits if

you do this.

The memory system consists of two banks of 256k words each. The banks are

selected by the row address strobe (RAS) which is qualified by an address

bit (A19) to select the bank. The upper and lower data strobes are used to

generate seperate column address strobes (CAS) for the upper and lower

bytes. Refresh is accomplished by activating the row address strobes for

both banks while supplying a refresh address. Refresh cycles occur, if

possible, when the memory is not being accessed (because some other resource

is being addressed) or when necessary by holding off the processor to force

time for the refresh cycle. All timing is derived from the 68000 clocks.

This allows a fully synchronous design so that there are no glitches in

timing possible. This also means that the processor must wait one clock

cycle when it accesses this memory. The processor clock is simply too slow

to go through the necessary states without the wait. If one eliminated the

wait the system would be 20% faster. This would require a delay line or an

asynchronous design. The cost of these alternatives is more than I was

willing to pay. Also, the design would be harder to duplicate because of the

more delicate timing.

The clock is a Motorola MC146818. It keeps track of the date and time

including daylight savings and leapyear. A nicad battery is charged by the

Amiga when it is powered up. This battery runs the clock when the Amiga is

off. As long as the Amiga is on 1% of the time, the clock will remain

accurate.

This memory board does not autoconfigure. This means that the Amiga doesn't

know about it until it is told. A program called AddMem is distributed on

the assembler or C disks. This program is needed to tell the Amiga that the

memory exists. I have not written a clone of this program. I have written a

memory test program to test the board. It is included in this release.

At this time, a number of programs, including Deluxe Paint, do not work

properly with external memory. I hope that as more Amigas get added memory

the suppliers of these programs will fix the problems. Mostly, graphics

objects cannot reside in external memory. If a graphics object is a program

constant, the program must be chip memory resident. There is no way to do

this within a language.  When a program allocates memory, it must specially

ask for chip memory if needed because the operating system will allocate

from external memory if it can. If you want to use one of the programs that

does not work with external memory, simply do not use the AddMem command.

There is no need to remove the card. Amiga has released the ATOM program to

developers which is used to tell the loader that some hunks must be loaded

into chip memory. If you can get this program, it will fix the above

problems.

The hardware clock isn't used by the Amiga during normal operation since it

keeps its own internal clock. I have written a program to read the hardware

clock and set the system clock (this is more trouble than I wanted). This

program is also included in this release.

The easiest way to use these new resources is to modify your

s/startup-sequence file to include:

AddMem 400000 500000

Clock

as added commands at the beginning. I also usually copy the entire system

disk to ram: and reassign all the logicals to the copy in ram:. The system

is very fast when running entirely out of ram.

I wirewrapped the memory since I expected to make changes. It took three

tries before the control circuit worked well. There were a number of pins

that were essentially undocumented and I had little choice but to try things

until they worked. Because this design has no support from Amiga, I cannot

guarantee that it works with your machine. On the other hand, I design

memories (and other things) for a living. I know what I'm doing. The design

is very forgiving of small timing changes (another reason to keep it slow)

and I do not expect anyone to have problems. I have no intention of

supplying a PC board. I don't mind if someone else takes on that project as

long as it is non-commercial. (If you are interested in a commercial design

let me know, I would do many things differently if it were a high quantity

commercial product. This design is bent in the direction of ease of

duplication with no special parts.)

There are four additional files I will mention here. The mechanical

description is in mechanical.doc. This includes the connector and board info

and the chip socket locations. Some comments on construction are also

included. The parts list is in parts.lst. The parts given will work.

Substitutions are possible, but at your own risk. The schematic is in

mem.sch and clock.sch (this section can be built at a later time.) This is

not the most beautiful method of drawing schematics, but it allows this

project to be electronically documented so it can reach a lot of people

without the 10th generation xerox problem. The schematic files look best

if printed at eight lines per inch.