This is the documentation file for the VT100 terminal emulator by Dave

Wecker (V2.6 DBW 870227). Comments/suggestions/bugs/problems/praise

should be sent to:

Dave Wecker at	ENET:	COOKIE::WECKER

ARPA:	wecker%cookie.dec.com@decwrl.dec.com

USENET:	{decvax|decwrl}!cookie.dec.com!wecker

SNAIL:	Dave Wecker

115 Palm Springs Drive

Colorado Springs, CO  80908

MANY pieces of code/suggestions have been sent in..

thanks to all!

Program startup:

----------------

1> vt100 [initfile]

- At startup, the program will search for an initialization

file to execute. It will first look for the specified

"initfile", then VT100.INIT (in the current directory)

and finally S:VT100.INIT. The format for the init file

is described later in this document.

- The init file controls the setting of initial defaults

and screen and macro definitions.

- If none of the files (listed above) are found, the

built-in defaults (defined in VT100.C as variables,

beginning with "p_") are used.

- All commands are either menu or script based. Scripts

are described below.

Menus (Commands in parenthesis are keyboard bindings: Right-Amiga-chr):

-----------------------------------------------------------------------

File 	  			- file transfers

Ascii Capture		- Begin/end a script of the current session

Ascii Send		- Type a file to the host

Xmodem Receive	(A-V)	- Receive a file using XMODEM protocol

Xmodem Send	(A-^)	- Send    a file using XMODEM protocol

Kermit Get	(A-G)	- Receive files from a host KERMIT SERVER

Kermit Receive	(A-R)	- Receive files from a host KERMIT

Kermit Send	(A-S)	- Send    files to   a host KERMIT [SERVER]

Kermit Bye	(A-B)	- Terminate a host KERMIT SERVER

Comm Setup			- Setup communications

Baud Rate		- Set the terminal baud rate

300,

1200,	(A-L)

2400,	(A-H)

4800,

9600

Parity			- Type of parity

NONE, 	(A-X)

MARK,

SPACE,

EVEN,	(A-E)

ODD	(A-O)

Xfer Mode

Image	(A-I)	- Send files verbatim (for UNIX hosts or

binary files)

Text	(A-T)	- Send CR LF as line terminator and strip

CR on received files (VMS text).

Convert		- Should KERMIT convert fnames to lower case

Script 	  			- Script commands

Execute file		- Start up an asynchronous script file

Abort Execution		- Terminate a script file

Utility   			- Utility commands

Send Break	(A-.)	- send a break to the host

Hang Up			- close line (not implemented yet)

Change Dir	(A-D)	- change the local directory (for transfers)

Clear Scrn		- clear the screen (initial state)

Echo			- turn on/off half duplex mode

Wrap		(A-W)	- turn on/off long line wrapping mode

Num Key		(A-K)	- turn on/off numeric keypad mode

App Cur		(A-C)	- turn on/off application cursor mode

BS<->DEL	(A-Z)	- swap backspace and delete keys

Keypad mapping (in application keypad mode):

--------------------------------------------

AMIGA		VT100		comments

-------		-------		---------------------------

0-9	==	0-9

.	==	.

ENTER	==	ENTER		(basically, flip the bottom

-	==	,		 2 keys up to get a VT100)

HELP	==	-		(only free key around)

f1-f4	==	PF1-PF4		(or any rebinding you do)

arrows	==	arrows

Note:	Right AMIGA key in conjunction with a period (".")

will send a break to the host.

CTRL in conjunction with an at-sign ("@") a two ("2") or a

space (" ") will send a NULL to the host.

CTRL in conjunction with a six  ("6") will send a CTRL-^

CTRL in conjunction with a dash ("-") or question mark ("?")

will send a CTRL-_ to the host.

Multi file Xfers:

-----------------

The VT100 emulator supports multiple file transfers. This is

specified by using a comma (",") between file names when using XMODEM

or KERMIT. (NOTE: host XMODEM's normally CANNOT support multiple file

transfers).

When specifying a file name to recieve by default the directory path

is stripped off of the filename when sent to the host but is kept for the

local file spec. eg:

receive file: ram:file.txt,df1:newfile.bin,$

will ask the server for file.txt and put it in ram:, and get

newfile.bin and put it on df1: (see explanation of "$" below). If you

do a single file transfer you will get another prompt for the remote

name e.g.:

receive file: ram:file.txt

remote file name [file.txt]:  userdisk1:wantfile.txt

The same rules apply to sending multiple files therefore if you are

doing multi file transfers make sure the host server is connected to

the desired directory.

In addition KERMIT supports wildcards:

* = any number of characters

? = any single character

Examples:

send:	*.c,*.h,*.doc

get:	*.c,*.h,$

KERMIT receive is now smart enough to use the host filename so no

filename needs to be specified on the AMIGA's side (see the CONVERT option).

Initialization and Script file operation:

-----------------------------------------

An initialization file (as described in the "Program Startup" section)

may contain any of the commands shown below that have the word INIT in

their description below. Commands that are available from scripts have

the word SCRIPT in the descriptions below. All commands may be abbreviated

to 3 letters and are case insensitive.

The script file can be invoked by selecting 'execute file' from the

script menu. At any time you can abort the script file by selecting

'Abort Execution'. You may also invoke a script from a function key if

the first character of the function key definition is the KEYSCRIPT

character (e.g., define F5 as "~df1:foo.script").

During the time script file is running the terminal emulation is still

active and you may type simulataneous to the script file. This may be

desired if your script file is WAITing for a string or is DELAYing for

a period of time etc.

Initialization and Script file Commands:

----------------------------------------

#	Commented line					(INIT,SCRIPT)

Format:

# This line is a comment

------------------------------------------------------------

APPCUR	Set the application cursor mode			(INIT,SCRIPT)

Format:

APPCUR	ON/OFF or YES/NO

------------------------------------------------------------

ASCII_SEND Send an ascii file to the host.		(SCRIPT)

Format:

(same format as CAPTURE)

------------------------------------------------------------

BACKGROUND Define a background color			(INIT)

Format:

BACKGROUND hex		three digit hex number

Example:

BACKGROUND F00		bright red background

------------------------------------------------------------

BAUD 	Set baud rate					(INIT,SCRIPT)

Format:

BAUD rate		Sets the baud rate for send/receive

Example:

BAUD 2400		Sets the baud rate at 2400 baud

------------------------------------------------------------

BEEP	Beep at the console				(SCRIPT)

Format:

BEEP

------------------------------------------------------------

BOLD 	Define a color for bold				(INIT)

Format:

(same as BACKGROUND)

------------------------------------------------------------

BREAK	Set the break time (for an SB command)		(INIT,SCRIPT)

Format:

BREAK value		Value is in micro-seconds

Example:

BREAK 750000

------------------------------------------------------------

BUFFER 	Set transmission buffer size			(INIT)

Format:

BUFFER n		Number of bytes to buffer

Example:

BUFFER 512

------------------------------------------------------------

CAPTURE	To start/stop ascii file capture.		(SCRIPT)

Format:

CAPTURE	file		Start ascii capturing

CAPTURE			End ascii capturing

------------------------------------------------------------

CD 	To change the local directory			(SCRIPT)

Format:

CD	newdir		set a new directory for file transfers

Example:

CD	DF1:foo/bar	set the directory as specified

------------------------------------------------------------

CONVERT	Tell KERMIT whether or not to convert filenames	(INIT,SCRIPT)

Format:

CONVERT	ON/OFF or YES/NO

Example:

CONVERT	ON		Filenames will be down cased

------------------------------------------------------------

CURSOR 	Define a color for the cursor			(INIT)

Format:

(same as BACKGROUND)

------------------------------------------------------------

DELAY 	Suspends script file for a specified time	(SCRIPT)

Format:

DELAY 	n		Suspends execution for n seconds

Example:

DELAY	2		Suspends for 2 seconds

------------------------------------------------------------

DEPTH 	Define the depth of the window/screen		(INIT)

Format:

DEPTH n		Number of planes in window/screen

Example:

DEPTH 1		Minimum depth

DEPTH 2		Same as Workbench

------------------------------------------------------------

ECHO	Turn on/off local echo				(INIT,SCRIPT)

Format:

ECHO	ON/OFF or YES/NO

Example:

ECHO	ON		Half duplex mode

------------------------------------------------------------

EXIT	Ends execution of the current script file.	(INIT,SCRIPT)

Format:

EXIT			Exit the current script/init file

EXIT VT100		Exit vt100 program (from SCRIPT only)

EXIT newscript		Exit this file and start up newscript

Example:

EXIT DF1:FOO.BAR	Exit the current file and start FOO.BAR

------------------------------------------------------------

F	Define a function key				(INIT,SCRIPT)

Format:

F n string		Define Function key n to be string

Example:				(see SEND for string format)

F 1 "dir^M"		Define F1 is the string dir<cr>

F 11 "help"		Define Shifted F1 as the string help

F 20 ^C			Define Shifted F10 as a control-C

------------------------------------------------------------

FOREGROUND Define a color for the foreground		(INIT)

Format:

(same as BACKGROUND)

------------------------------------------------------------

GOTO	Jumps to a different part of the script file.	(SCRIPT)

Format:

GOTO label		Jumps to a line beginning with label:

Jumps may be forward or backward.

Example:

FOO:			Sets up a label

...

GOTO FOO		Jumps to FOO

------------------------------------------------------------

INTERLACE Turn on/off interlace				(INIT)

Format:

INTERLACE ON/OFF or YES/NO

Example:

INTERLACE ON		Use interlacing

------------------------------------------------------------

KB  	Send a BYE packet to a host KERMIT server.	(SCRIPT)

Format:

KB 			Shut down server.

------------------------------------------------------------

KEYSCRIPT Define a new keyscript character		(INIT,SCRIPT)

Format:

KEYSCRIPT hex		New character in hex

Example:

KEYSCRIPT 7E		Use "~" as the new character

------------------------------------------------------------

KG  	Gets files from host.				(SCRIPT)

Format:

(same format as KS)	 Get from server

------------------------------------------------------------

KR  	Receives a file from kermit host.		(SCRIPT)

Format:

(same format as KS)	 Not from a server

------------------------------------------------------------

KS  	Sends files via kermit to the host.		(SCRIPT)

Format:

KS file			Send one file

KS file1,file2,...	Send multiple files

KS file1,file2,...,$	Send multiple files and shut down server

Example:

KS foo.bar		sends foo.bar (note no quoting is used)

KS foo1,foo2,foo3	sends three files

KS foo1,foo2,foo3,$	sends three files and shuts down server

------------------------------------------------------------

LINES	Define number of lines in the window		(INIT)

Format:

LINES n

Example:

LINES 24		Maximum for non-interlace

LINES 48		Maximum for interlaced

------------------------------------------------------------

MODE	Set a transfer mode for KERMIT to use		(INIT,SCRIPT)

Format:

MODE type		type of transfers to perform

Example:

MODE IMAGE		image mode transfers

MODE CRLF		<CR><LF> text transfers (VMS Kermit).

------------------------------------------------------------

NUMKEY	Numeric keypad mode				(INIT,SCRIPT)

Format:

NUMKEY	ON/OFF or YES/NO

Example:

NUMKEY	ON		Keypad is pure numbers

------------------------------------------------------------

ON	Peforms a command every time string is received	(SCRIPT)

Format:

ON      "string"  cmd	Execute cmd when string is received.

Only one ON string may be installed at a

time.

If cmd is a GOTO and we were previously

WAITing for a string the WAIT is aborted and

execution resumes at the new label.

If cmd is not SEND and we were previously

DELAYing, then the DELAY is aborted and the

cmd is executed, followed by the next command

after the DELAY.

If cmd is a SEND and we were previously

DELAYing, then the DELAY is continued.

Example:

ON  "LOSS CARRIER" GOTO RESTART

If modem drops carrier, try to redial

ON  "--more--" SEND " "

Send a space every time --more-- is received

------------------------------------------------------------

PARITY	Sets the parity					(INIT,SCRIPT)

Format:

PARITY	type		Set the parity type

Example:

PARITY	NONE		no parity

PARITY	MARK		mark parity

PARITY	SPACE		space parity

PARITY	ODD		odd parity

PARITY	EVEN		even parity

------------------------------------------------------------

SB	Sends a break character to the host		(SCRIPT)

Format:			Note that any pending character to send

SB				is aborted by this call

------------------------------------------------------------

SCREEN	Define the screen type				(INIT)

Format:

SCREEN type		type of screen to use

Example:

SCREEN WORKBENCH	use the workbench screen

SCREEN CUSTOM		use a custom screen

------------------------------------------------------------

SEND 	Sends a string or character to the host.	(SCRIPT)

Format:

SEND    "string"	Sends a string to the host. Beginning and

ending double quotes (") are required. A

carat (^) may be used to send control chars.

Two carats transmits a carat character.

SEND    chr           	Sends a single character.

SEND    ^chr   	    	Sends a single control character. The chr

is NOT case sensitve

Example:

SEND	"mail"		Send the string mail

SEND    "dir^M"		Send the string dir followed by a <CR>

SEND	a		Send the letter a

SEND	^C		Send a control C

SEND	"abc^^def"	Send the string abc^def

SEND	^^		Send a control-uparrow

SEND	"		Send the '"' character

------------------------------------------------------------

SWAP 	Swap the meanings of backspace and delete keys	(INIT,SCRIPT)

Format:

SWAP ON/OFF or YES/NO

Example:

SWAP NO		Use standard definitions

------------------------------------------------------------

VOLUME	Set the BELL volume				(INIT)

Format:

VOLUME n

Example:

VOLUME 0		Use a visual bell

VOLUME 64		Use a loud audible bell

------------------------------------------------------------

WAIT 	Suspends until a certain string is received.	(SCRIPT)

Format:

WAIT	"string"	Same rules for string as SEND

WAIT			Enter an endless wait. Usually used after

some "ON" commands have been set up. Can

still be aborted via the script menu.

Example:

WAIT    "User:"    	Waits for the string User:

------------------------------------------------------------

WBCOLORS Force usage of workbench colors		(INIT)

Format:

WBCOLORS ON/OFF or YES/NO

Example:

WBCOLORS YES		Workbebch colors will be used for all

------------------------------------------------------------

WRAP	Set long line wrapping				(INIT,SCRIPT)

Format:

WRAP	ON/OFF or YES/NO

Example:

WRAP	ON		Long lines will wrap

------------------------------------------------------------

XR  	Receives a file via XMODEM.			(SCRIPT)

Format:

(same format as KS)

------------------------------------------------------------

XS  	Sends a file via XMODEM.			(SCRIPT)

Format:

(same format as KS)

------------------------------------------------------------

----------------------------

Initialization file example:

----------------------------

#####################################################################

#

#	VT100 sample initialization file

#	v2.6 870222 DBW	- Dave Wecker standard defaults

#

# Hash mark at the beginning of a line denotes a comment.

# White space (space(s) or tab(s)) delimit fields.

# Case ignored except for function key bindings.

#

# All items in this file overide variables of the same name in VT100.C

# (all variables in vt100.c have a "p_" prepended to them)

#

#####################################################################

#

APPCUR		ON		# Application keypad mode is being used

BACKGROUND	000		# Colors are in hex RGB from 000 to FFF

BAUD		2400		# Anything after required fields is ignored

BOLD		a00		# Color for bold highlighting (in custom)

BREAK		750000		# Break time in micro-seconds

BUFFER		512		# 512 <= Input buffer size <= 2048

CONVERT		ON		# KERMIT should downcase host names

CURSOR		00a		# Color for cursor (in custom screen)

DEPTH		1		# number of bit planes to use (1 or 2)

ECHO		OFF		# Full duplex mode in use

FOREGROUND	950		# Colors are only used on the custom screen

INTERLACE	ON		# ON for CUSTOM or interlaced workbench

KEYSCRIPT	7E		# Hex value for script introducer

LINES		48		# normal <= 24 interlaced <= 48

MODE		CRLF		# IMAGE or CRLF (for KERMIT transfers)

NUMKEY		ON		# The keypad should be numeric

PARITY		NONE		# NONE (= 8 bit), MARK, SPACE, ODD or EVEN

SCREEN		CUSTOM		# may be CUSTOM or WORKBENCH

SWAP		OFF		# Don't Swap the Back-space and Delete keys

VOLUME		64		# Beep Volume (0 = Visual Beep)

WBCOLORS	YES		# ignore custom colors and use defaults

WRAP		OFF		# Auto wrap ON or OFF

#

# Function bindings (strings to type when any of F1 - F10 are pressed)

#	f <num>		= function key

#	f <num>+10	= shifted function key

#

# The string specified must be the same format as the SEND command:

#	^	= control next character

#	^^	= up arrow

#

# Sample control characters:

#	^[	= escape	^M	= carriage return

#	^J	= line feed	^L	= form feed

#

# If the first character of the string is a script introducer

# (KEYSCRIPT) then the string is interpreted as a script filename

# to be executed when the key is pressed.

#

# Examples of bindings:

#

f 1	"^[OP"			# f1-f4 = PF1 - PF4 on a VT100

f 2	"^[OQ"

f 3	"^[OR"

f 4	"^[OS"

#

# f5,6,7 = scripts to execute (assuming that KEYSCRIPT = '~' = 0x7E)

#

f 5	"~df1:vt100_source/dialwork.script"

f 6	"~df1:vt100_source/sendvt100.script"

f 7	"~df1:vt100_source/getpics.script"

#

f 8	"MAIL^M"		# Reads my mail (note embedded <CR>)

f 9	"NOTE^M"		# Reads conferences

f 11	"$2400!"		# dials the phone to work

f 12	"$bbs1!"		# dials the phone to billboard 1

f 13	"$bbs2!"		# dials the phone to billboard 2

f 14	"$bbs3!"		# dials the phone to billboard 3

#

# all done with init, now execute script as startup sequence

#

exit df1:vt100_source/dialwork.script

--------------------

Script file example:

--------------------

###################################################################

# Script to dial work (dialwork.script)

#	v2.6	870222	DBW

###################################################################

#

# Make sure that we have all the parameters we want

#

DELAY	2

BAUD	2400

PARITY	NONE

MODE	CRLF

BREAK	750000

SB

#

# First get the modem's attention:

#

Start:

DELAY 1

ON "Ready" GOTO Dial

SEND ^B

DELAY 2

GOTO Start

#

# Now dial the 2400 baud line to work:

#

Dial:

ON "Attached" GOTO Login

SEND "$2400!"

DELAY 30

GOTO Start

#

# We got attached, so keep hitting return until the Gandalf terminal

# handler wakes up:

#

Login:

ON "enter" GOTO Gandalf

DELAY 1

SEND ^M

GOTO Login

#

# Now connect from the Gandalf to the terminal server (ts2):

# (when it asks for a password I need to type the password

# manually here)

#

Gandalf:

DELAY 2

SEND "ts2^M"

WAIT "class start"

#

# Keep sending <CR>'s until the LAT prompts for a username:

#

WaitLat:

DELAY 2

ON "username>" GOTO Lat

SEND ^M

GOTO WaitLat

#

# Tell the LAT that it's me, and connect to the "cookie cluster"

# (my host systems). Tell the cluster my user name.

# (when it asks for a password I need to type the password

# manually here)

#

Lat:

SEND "wecker^M"

DELAY 1

SEND "connect cookie^M"

WAIT "Username:"

SEND "WECKER^M"

WAIT "at home"

SEND "^M^Mn^M"

#

# Got through all the LOGIN garbage, so let's do some work.

#

WAIT "$ "

#

# All done so stop:

#

EXIT