* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

*                                                                       *

*                                                                       *

*                     *   *   D C 1 0   *   *                           *

*                                                                       *

*                 F L I G H T S I M U L A T I O N                       *

*                                                                       *

*                  Amiga Version 0.1f, 9 april 1988                     *

*                                                                       *

*               copyright <C> 1988 by Jan Arkesteijn                    *

*                                                                       *

*                                                                       *

*                                                                       *

*                         *  CONTENTS:  *                               *

*                                                                       *

*                                                                       *

*                 1 INTRODUCTION                                        *

*                                                                       *

*                 2 INSTRUMENTPANEL                                     *

*                                                                       *

*                 3 CONTROLS                                            *

*                   3.1  Aircraft conrols                               *

*                   3.2  Select navigation facilities                   *

*                   3.3  Special controls                               *

*                                                                       *

*                 4 BASIC FLIGHT MANOUVRES                              *

*                   4.1  Pre take-off actions                           *

*                   4.2  Take-off                                       *

*                   4.3  Climb                                          *

*                   4.4  Cruise                                         *

*                   4.5  Turning                                        *

*                   4.6  Descent                                        *

*                   4.7  Approach                                       *

*                   4.8  Final approach                                 *

*                   4.9  Landing                                        *

*                   4.10 Overshoot                                      *

*                                                                       *

*                 5 NAVIGATION                                          *

*                   5.1  Navigation systems                             *

*                   5.2  Navigation principles                          *

*                   5.3  Flightplanning                                 *

*                                                                       *

*                 6 PRACTICE-FLIGHTS                                    *

*                   6.1  Flight Birmingham-London                       *

*                   6.2  Landing on Miami Intl                          *

*                                                                       *

*                   REFERENCES                                          *

*                                                                       *

*                                                                       *

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

1.0    INTRODUCTION

------------

In this simulation program the flight of a DC-10 is simulated.

In the real world such aircraft are normally flown on instruments only,

therefore this simulation is a pure instrument flight simulation.

There are numerous versions of the DC-10 with different engine types,

take-off weights etc. This program is based on the long range inter-

continental version, the DC-10-30.

The specification of the aircraft as used in the simulation is :

Powerplant:three GE CF6-50C1 turbofans, each with a max. thrust of 234 kN

Dimensions:span 50m/165ft,length 55m/181ft,wingarea 338sqm/3650sqft

Maximum take-off weight : 250,000 kg / 550,000 lb

Maximum landing weight  : 180,000 kg / 400,000 lb

Maximum fuel load       : 105,000 kg / 130,000 liter

Maximum payload         :  45,000 kg / 100,000 lb

Minimum runway length for a take-off with 250 tons : 3200 m / 10,500 ft

Minimum runway length for a landing  with 180 tons : 1800 m / 6000 ft

Range with maximum payload : 4000 nm (including reserves)

Range with maximum fuel    : 5000 nm (including reserves)

The air conditions for this simulation have been taken from the

International Standard Atmosphere (ISA). Windforce and winddirection

in the simulation change with the location of the aircraft and with

the altitude.

In par. 2 the instrumentpanel is explained. Par. 3 gives a survey of the

control facilities. Par. 4 gives a short introduction in the handling of the

aircraft during the different flight phases. Par. 5 explains the working of

the navigationsystems and contains some information on navigation principles.

The overall accuracy of the navigation aids is extremily good. Even the

official real world navigation charts, as issued by the Civil Aviation

Authorities (CAA) of the relevant countries (GB, NL, B), can be used in

this simulation. Par 6 gives some example flights. At the end a reference

list of books and articles about the subject is added.

After starting the program DC10.BAS a menu appears with 3 options :

1) Making a flightplan

Chapter 5.3 explains how to make a flightplan. If the flightplan is

ready the computer goes to option 2.

2) Take-off and fly

For beginning pilots this is the best choice to get experience in the

handling of the aircraft and the navigation procedures.

After loading of DC10EUR.BAS you have to choose your take-off runway.

See chapter 5.1.3 for details.

Because the program uses functions from the 'exec.library' it is

necessary to have the file 'exec.bmap' in your 'libs' directory.

This file can be found in the 'Extras_1.2:basicdemos' directory.

Because not all runways are equal in length there will be a difference

in the maximum take-off weight depending on the selected runway.

Keep this in mind when entering the payload and the fuelload. The next

screen gives the take-off data. After hitting of the enter key the

instrumentpanel will show up. Chapter 2 includes a detailed explanation

about its lay-out.

To prevent VDU-beeping click the left mouse button if the pointer is

outside the window. This is possible by bringing the pointer to the

righthand windowborder and clicking there.

3) Practice landings

A special version of the program, called DC10MIA.BAS, is available to

build up experience in 'blind' landings. Chapter 6.2 gives the details.

2.0    INSTRUMENTPANEL

---------------

The instrumentpanel is devided in two main blocks :

The upper part gives, in principle, all information about flying the plane.

The lower part is totally dedicated to navigation.

The upper part is divided in 5 boxes refferred to as A,B,C,D,E

The lower part is also divided in 5 parts named F,G,H,I,J

For each box A to J a short explanation is given:

A) From top to bottom the following information is given in this box:

- Time elapsed from take-off clearance

- AUW  : All up weight in tons

- FUEL : Fuel load in tons

- FFHR : Fuel flow in tons per hour

- M    : Machnumber. This is the airspeed divided by the speed of sound.

If the aircraft approaches M=1 then shockwaves are formed around

the wing. This means a strong increase in drag and at the utmost

loss of lift (high speed stall) For the wingdesign of the DC10

this will start from about M=0.9, so the maximum operating

machnumber is limited to M=0.88

- TAS  : True airspeed in kt. As it says, this is the true speed of the

aircraft in respect to the air.

- WD   : Wind. The first figure is the direction from which the wind

blows, the second one is the windspeed in knots.

- GS   : Groundspeed. The speed of the aircraft in respect to the ground.

B) IAS: Indicated airspeed

This is the airspeed measured with a 'Pitot tube'. Essentially it

measures the dynamic pressure. This is the pressure on a diaphragm caused

by the speed with which it moves through the air. This pressure depends

also on the airdensity. The same applies to the liftforce which keeps

the aircraft in the air. It is therefore that the IAS is so important,

because wingstalling is primarily fixed by this speed.

In the righthand side of this box the approach of critical speedvalues

is indicated :

V 1  : If an engine fails during the take-off run and IAS < V1 then the

aircraft may be safely stopped. If an engine fails if IAS > V1 then

the take-off has to be continued because the aircraft cannot be

brought to a standstill before the end of the runway.

V R  : If during the take-off run the IAS reaches VR (rotate speed) then

pitch up to increase the angle of attack. Then the liftforce will

exceed the weight and the aircraft gets off the ground.

V 2  : Lowest safe flying speed during take-off and climb-out

(slats 16 degr. and flaps 15 degr.)

V AT : Target speed at treshold. This is the lowest safe flying speed

during final approach (sl 25 degr, fl 55 degr) This includes

margins for special conditions like '1 engine out'. The value

without the margins is called V ref.

V MO : Max. operating speed. For DC-10 below 24000 ft the V MO= 380 kt.

M MO : Max. operating machnumber. For DC-10 above 24000 ft M MO= .88

At the bottom of this box the VSI (vertical speed indicator) gives the

climbspeed (positive) or descentrate (negative) in feet per minute.

Values exceeding + or - 9999 are not indicated

C) The artificial horizon

The most important instrument is the artificial horizon. This instrument

visualizes the attitude and bankangles in respect to the horizontal

plane. In the simulation the small aircraft (--O--) in the centre is

fixed and the horizon moves. In the real world it is of course the

opposite.

At the top the exact value of the attitude (in degrees), the bankangle

(in degrees) and the rate of turn (in degrees per second) is shown.

During the ground run the direction can be changed by steering with the

nosewheel. In the second line the angle between the fuselage centreline

and the real direction of the nosewheel is shown during the take-off run

and landing run.

On the same line you will get a warning if you appraoch the stallspeed

during flight.

At the bottom line the altitude (ALT) in feet is reported.

D) Engines

In this part the information about the engines is given. N1 is the

fanspeed in % and range from 25 % (idle) to 110 %.

To prevent overloading the following restrictions apply at the upper end :

* At sealevel

- For normal take-off the upper limit is 100 % for 5 minutes at most.

- Maximum continuous N1=96 % (climb power).

- In emergency situations an absolut maximum of 106 % for 1 minute.

With increasing high the airtemperature decreases so the upperlimits

increase :

* At 30000 ft

- Maximum continuous N1=106 %

- Absolute maximum N1=110 % for 5 minutes

For easy calculation a linear interpolation can be used for N1 at other

heighs.

If you do not hold these limits you will get a flashing alert lamp.

At the utmost even an engine failure can be the result.

If an engine has been shut down  SD  is shown.

If reverse thrust is activated  *R* is shown.

E) This section gives the status of the 'extendable' parts.

From top to down you find :

- SL : The slat setting in degrees. During the take-off and intermediate

approach slats are set to 16 degr. During the final approach and

landing this has to be 25 degr.

- FL : Flap setting in degrees. During take-off and the intermediate

approach a value of 15 degr. has to be selected. During the final

approach and landing full flaps or 55 degr. has to be set.

The following restrictions must be taken into account :

o Flaps cannot be extended if slats are in.

o Flap extension is only possible below certain speedvalues.

- AB : Airbrakes.

- WB : Wheelbrakes.

- LG : Landinggear, UP=retracted, DN=down=extended, *=going up/dn.

F) In the upper part of this box the exact data of the Automatic Direction

Finding (ADF) equipment is given. For a short explanation of how the ADF

system works see par. 5.1.1 . The first line of part F gives the station

identifier. The second line gives the bearing (BG). This is the direction

in which you will find the radiobeacon if you take (magnetic) north as

the reference direction. The third line gives the relative bearing (RBG).

In this simulation program this is defined as the direction in which you

will find the radiobeacon if you take the course or the groundtrack of

the aircraft as the reference direction.

The last but one line contains the information about the runway to which

the ILS and its DME equipment is tuned.

The last line gives the remaining runway length if the aircraft is on the

runway or the distance to the touchdown point on the selected runway if

the aircraft is flying.

G) RMI

This is the Radio Magnetic Indicator (RMI). This instrument incorporates

both compass and ADF representation. It contains two 'needles'. The light

blue dot represents  the compass. The yellow dot shows the bearing data

from the ADF equipment. The exact data from the compass is given as the

heading (HDG). In this data the difference between geographical and

compass bearing is taken into account. In other words locationdependent

magnetic deviation is included.

H) ILS crosspoint meter

During the approach and landing the Instrument Landing System (ILS) is

the most important navigation aid. It gives guidance in the horizontal

as well as in the vertical plane (see par. 5.1.3)

The display is a kind of crosspoint meter. The crosspoint of the two

needles shows the location of the 'beam' in respect to the aircraft.

Although there is no mark in the centre, the aircraft is supposed to be

dead centre.

At the bottom the exact data is given. At the left the angle in degrees

at the localizer transmitter between the centreline of the runway and the

real location of the aircraft is given. The arrow points to the beam.

At the right the angle in degrees at the glidepath transmitter between

the nominal glideslope of 3 degrees and the real location of the aircraft

is given. Also here the arrow points to the beam.

If you are not within the range of the meter (5 degr. from the runway

centreline or 1 degr. from the nominal glideslope) only the direction

of the beam will be indicated and there will be no exact data.

If at the decision heigh (see par 4.8) the deviation in either direction

is more then roughly 0.5 degr then you better go around, because it is

far from certain that you will land the aircraft safely.

I) Mapdisplay of selected VOR/DME beacons.

The  +  sign in the middle is supposed to be the aircraft. The upward

pointing part is the nose. At the top the real flying direction over

the ground called track (TRK) is given. The difference between heading

and track is caused by the wind. The VOR/DME stations (see par 5.1.2)

are shown with the track as the reference direction. The angle between

track and beacon is defined as the relative bearing (RBG).

This instrument is also called  VOR-tracking intstrument.

On the display the distance between aircraft and beacon is proportional

to the real distance. In principle the distance scale is linear.

Two ranges can be selected. If long range (L) has be selected then the

maximum distance is 125 nm. If short range (S) is chosen then the

displayed range is restricted to 25 nm.

J) In this part the exact data from the VOR/DME equipment is given.

The standard presentation is : distance in nm / station identifier /

radial. For example  23 SPL 057  means : the aircraft is at a distance

of 23 nm from a VOR/DME station with station code SPL (=Schiphol) and

the angle at this station between magnetic north and the location of the

aircraft is 57 degr. RBG is again relative bearing.

3.0    CONTROLS

--------

3.1    AIRCRAFT CONTROLS

The chosen approach is that you have to press a letter to choose a function

(the letter to press is normally the first letter of that function) and

simultaeously press up arrow (ua) or down arrow (da) to realise a increase

or decrease.

Some of the controls are capable of fine adjustment. If you press the space

bar (sb) in addition to the two other keys the stepsize of the change will

be 1/5th.

T + ua/da + (sb) -throttle-      open/close throttle(s) 10 (2) percent

P + ua/da + (sb) -pitch-         pitch up/down 2.5 (.5) degrees

R + ra/la + (sb) -roll-          roll to the right/left 5 (1) degree(s)

N + ra/la + (sb) -nosewheel-     turn nosewheel to the right/left .5(.1) degr

K + 0            -kick-off       bring heading (almost) parallel to runway-

drift-     centreline just before landing (below 15 ft)

G + da/ua        -gear-          initiate extension/retraction of landinggear

S + da/ua        -slats-         set/retract slats in 2 steps (0-16-25)

F + da/ua        -flaps-         set/retract flaps in 7 steps

(0-5-10-15-25-35-45-55)

B + da/ua        -breaks-        activate/release wheelbrakes

A + da/ua        -airbrakes-     extend/release airbrakes

E + da/ua        -engine reverse-activate/cancel thrust reverse

1 + ua/da        -engine 1-      start/shut down engine 1   ) 1,2 and 3 on

2 + ua/da        -engine 2-      start/shut down engine 2   ) the numerical

3 + ua/da        -engine 3-      start/shut down engine 3   ) pad only

M +da            -mass-          dump fuel as not to exceed maximum

landingweight

3.2    SELECTION OF NAVIGATION FACILITIES

A total of about 70 groundfacilities (NDB, VOR/DME, Loc. and ILS) are

available. The table below shows how these can be selected.

Country      GB     GB     GB     NL    NL/B     B           GB/NL/B

Type        VOR    VOR    NDB    VOR    NDB    VOR    LOC      ILS

KEYS         U      V      W      X      Y      Z      L        D

1        BCN    GAM    BPK    EEL    EHN    BUB     OA    ASD 19R

2        BIG    HON    CAM    HDR    ENK    BUN           ASD 24

3        BNN    IBY    CON    HSD    LAK    CIV     WP    ASD 27

4        BUR    LAM    DUN    MAS    NYK    COA     GX    BMH 15

5        BTN    LON    KNI    PAM    ROT    KOK           BXL 25L

6        CFD    MAY     LA    RKN    STD    LNO     GY    LGW 08

7        CLN    MID    LIC    RTM    THN    NIK           LHR 10R

8        DET    OCK    NCR    SPL    DEN    SPI           MCR 06

9        DTY    SFD     NH    SPY    GAA            NW    MST 22

0        DVR    WAL    WOD           ONT            RR    RTD 24

For example press U and 1 simultaneously to select the VOR/DME station BCN

(Bracon) or D and 0 for the destination RWY RTD 24 (Rotterdam).

If you want BCN on VOR/DME receiver 2 press  U + 1 + sb, with only U + 1

VOR/DME receiver 1 is tuned to BCN.

In selecting radiobeacons do not use the numerical pad, it will not work.

Two other controls for the navigation equipment should be mentioned :

V + ua/da  -VOR-tracking- select long/short range on the VOR-tracking device

I + ua/da  -ILS-          switch the ILS equipment on/off

3.3    SPECIAL CONTROLS

There are two control possibilities which are not true to life, however they

can be very helpfull, especially the first one because it  gives the

opportunity to calm down in critical situations.

C + da/ua   -computer-    pause/continue

Q + da      -quit-        restart program

4.0    BASIC FLIGHT MANOUVRES

---------------------

The take-off and climb procedures as described in this chapter are based on

the maximum take-off weight of 250 tons. The desription of the approach and

landing procedures is based on the maximum landing weight of 180 tons.

At other weights sometimes a better performance is realised by deviating

slightly from the described procedures.

4.1    PRE TAKE-OFF ACTION

Before you can take-off you have to start the engines <1+ua>,<2+ua>,<3+ua>.

Set slats to 16 degr <S+da>, set flaps to 15 degr <F+da> and tune in to the

appropriate VOR/DME stations and NDB.

4.2    TAKE-OFF

4.2.1  Normal take-off

If cleared for take-off press <T+ua+(sb)> untill the thrust reaches the

maximum take-off value with N1=100%. If necessary correct the heading with

the nosewheel steering. See par. 5.1.3 for runway centreline directions.

If the speed (IAS in kt) reaches VR press <P+ua> to pitch up to 15 degrees

for lift-off and climb out at V2+10 kt.

If the altitude is at least 20 ft retract the undercarriage <G+ua>.

At about 2000 ft pitch down to 9 degrees <P+da+sb> (reduce climbspeed to

about 500 ft/min) and reduce N1 to 96% to prolonge engine life <T+da+sb>.

Let the aircraft speed up. If the speed reaches V2+20 retract flaps to

10 degrees <F+ua>. If after some speed increase the rate of climb exceeds

1000 ft/min then retract flaps to 5 degrees. If again after some speed

inrease the climbrate exceeds 1000 ft/min then retracts the flaps fully.

If the speed reaches V2+60 then pitch down to 8 degrees and retract slats.

The table below shows the influence of the take-off weight on the afore-

mentioned speeds.

TOW      V1    VR    V2    V2+20  V2+60

(tons)   (kt)  (kt)  (kt)   (kt)   (kt)

130     115   123   130    150    190

150     124   132   139    159    199

170     133   141   148    168    208

190     142   150   157    177    217

210     150   158   165    185    225

230     158   166   173    193    233

250     160!  173   180    200    240

4.2.2  Noise abatement take-off

The standard instrument departure (SID) routes are often called mimimum

noise routes. Under certain conditions however this may still give to much

disturbance. Under these circumstances a possible procedure for a noise

abatement take-off could be:

Up to an altitude of 2000 ft normal take-off (see par 4.2.1). If 2000 ft is

reached pitch down to about 10 degrees and reduce N1 to about 75 % .

The actual setting should give a speed of V2+10 kt IAS, and a climbrate of

about 400 ft/min. After 5 minutes from the start of the take-off climbpower

(N1=96%) can be selected. Then set pitch to 9 degrees. At reaching V2+20

retract flaps to 10 degrees. For further flap and slat retraction procedure

see par. 4.2.1

It is important to note that this procedure is abondened if an engine

failure occurs. Then of course the procedure of par 4.2.3 must be followed.

4.2.3  Take-off with engine failure

If an engine fails during the take-off run before V1 is reached, then the

take-off must be abondened. Press <E+da> to activate thrust reverse, <B+da>

for wheelbrakes, <A+da> for airbrakes and <T+da!!!> to increase N1.

If an engine fails after passing V1 then the take-off has to be continued.

At reaching VR pitch up to 12 degrees and climb out at V2. Above 20 ft

retract the gear.

If an engine fails during climb out pitch down to 12 degrees and maintain V2

At reaching 1000 ft pitch slowly down to 9 degrees <P+da+sb>. At V2+20

retract flaps to 10 degrees. If the climbrate exceeds 500 ft/min pitch

slowly down to 7 degrees. If the climbrate again exceeds 500 ft/min retract

flaps to 5 degrees. If after some speedincrease the climbrate again exceeds

500 ft/min retract flaps fully. At reaching V2+60 pitch down to 6 degrees

and retract slats. To prevent overheating of the live engines set N1=96%.

4.3    CLIMB

4.3.1  Normal climb

For air traffic control (ATC) reasons and risk of bird impact the speed is

restricted to 275 kt IAS below 10000 ft. So after cleaning up pitch is kept

at 8 degrees and N1 stays 96% untill the speed reaches 275 kt. Then N1 is

reduced to roughly 90%. To maintain this speed N1 must then again gradually

be increased with increasing heigh.

After passing 10000 ft pitch down to 4 degrees and set or keep N1 to 96% .

At about 16000 ft pitch down to 3 degrees and set N1 to 100% .

At about 23000 ft pitch down to 2 degrees and keep N1 to 100% .

The real procedure has to result in a rather constant speed Vcl=325

(+ or -10 ) kt IAS above 10000 ft.

4.3.2  Climb with 1 engine out

Below 10000 ft there is again a speedlimit of 275 kt. Therefore after the

cleaning up the pitch is kept at 6 degrees and N1 stays 96% untill the speed

reaches 275 kt. Then maintain this speed by adjusting N1.

Between 7000 and 10000 ft the pitch is 5 degrees. The speed is kept constant

by adjusting the throttles.

After passing 10000 ft set N1 to 96% and pitch slowly down to 3 degrees.

At about 15000 ft set N1 to 100% and pitch down to 2 degrees.

The real procedure has to result in a rather constant speed Vcl=310

(+ or -10) kt above 10000 ft.

4.4    CRUISE

If the cruise heigh is almost reached pitch slowly down and reduce thrust.

To maintain a certain flightlevel select a certain pitch and set speed or

thrust accordingly (see examples in the annex to this chapter).

Do not exceed the maximum operating speed (below 24000 ft V MO = 380 kt,

above 24000 ft M MO = .88)

For the economy cruise above 30000 ft a speed equal to M=.82 is recommended.

If the aircraft is very heavy then the first part of a long distance cruise

has to be flown at about 30000 ft (or 18000 ft with 1 engine out). When the

fuel burns up the aircraft looses weight and higher and more economical

cruise heighs are possible. The last part is often flown at 37000 ft for

example (or 27000 ft with 1 engine out).

4.5    TURNING

4.5.1  Turning on the ground

To correct the direction during the take-off run or the landing run the

nosewheel can be turned by pressing the N-key and the appropriate arrow

<N+ra/la+(sb)> . For runway directions see par 5.1.3

4.5.2  Turning in the air

A turn in the air can only be realised with banking. Ruder is not

implemented. The roll actions are controlled with the R-key and the

appropriate arrow <R+ra/la+(sb)> . If the new heading is almost reached roll

back to horizontal. As in real life bankangles are restricted to 25 degrees.

4.6    DESCENT

Normally the descent should be started if the distance to the landing point,

measured along the flightpath, satisfies the eqation d=3.2*h (d in nm and

h in 1000th of ft). This means a descentpath of 3 degrees.

In noise sensitive areas however sometimes steeper descentpathes (up to 6

degrees) are used, to delay the start of the descent.

For practical purpose, especially during the final approach, the following

rule of thumb is usefull for a descentpath of 3 degrees : the rate of

descent in ft/min should be about 5.3 * the GROUNDspeed in kt.

The actual descent starts by pitching down a few degrees and reducing thrust

and when necessary using the airbrakes <A+da>. Especially in the (steep)

descent you should be aware not to exceed V MO or M MO.

In many cases there is a speed limit of 250 kt IAS below 10000 ft prescribed

by air traffic control.

For an emergency descent (for example if there are problems with the

pressurization) reduce thrust to idle <E+ua>, extend the airbrakes and

pitch down to -12 degrees.

4.7    APPROACH

At about 20 nm to go (and a speed of about 250 kt) extend the slats to 16

degrees. Then reduce to Vref+100. If everything is fine slow down further

and after each speedloss of 10-15 kt extend the next step of the flaps.

Maintain Vref+50 with the slats at 16 and the flaps at 15 degrees up to

about 10 nm to go.

Then extend  the landinggear and if not already done switch on the ILS

equipment.

At about 8 nm to go you must be roughly in line with the runway and at an

altitude of 2500 ft. Set slats to 25 degrees and slow down further.

After each speedloss of roughly 10 kt extend the next step of the flaps.

With slats and flaps fully extended maintain V AT + 10 kt.

During all the preceding operations thrust, attitude and bankangles should

be selected depending on the deviation from the glideslope (see examples in

the annex)  Keep in mind : Stick for speed and power for descentrate !!!

In the table below the influence of the landing weight on the different

speedvalues is visable.

LW       Vref    Vref+10    Vref+50    Vref+100

(tons)     (kt)    (kt)       (kt)       (kt)

120       111     121        161         211

140       120     130        170         220

160       128     138        178         228

180       136     146        186         236

4.8    FINAL APPROACH

4.8.1  Final approach with all engines working

At the outer marker (OM), the final approach begins. You must be on the

glidepath with sufficient accuracy and slats are at 25 degrees and flaps at

55 degrees and the speed is V AT + 10 kt IAS (=Vref+10). Continue the

descent to 500 ft. Below 500 ft reduce speed to V AT (=Vref). If at the

decision heigh (DH) of 100 ft the deviation from the glidepath is too large

then an overshoot (OS) should be carried out (see par 4.10.1).

4.8.2  Final approach with 1 engine out

At the outer marker (OM) the final approach begins. you must be on the

glidepath with sufficient accuracy and slats are at 25 degrees and flaps at

55 degrees and the speed is V AT + 10 (=Vref+15). Continue the descent to

500 ft Below 500 ft reduce speed to V AT (=Vref+5). In this case the

decision heigh is 200 ft. If the deviation from the glidepath at this heigh

is too large then an overshoot (OS) should be carried out (see par 4.10.2).

Remark :

Under light weight conditions (below roughly 150 tons) it is possible to

land and when necessary to make an overshoot with just 1 engine operating.

(DH=300 ft, V AT=Vref+10, slats at 25 degr.,flaps at 35 ! degr. and at

150 tons and below 500 ft the attitude is 4 degr., N1=82%, IAS=133 kt)

At higher weights, up to the maximum landing weigh of 180 tons, you can

- in principle - also land on 1 engine, however it is not possible to go

around because of lack of climb performance.

4.9 LANDING

At about 50 ft the flare-out begins. Pitch up slowly about 2 degrees

<P+ua+sb>. At about 20 ft close throttles <E+ua>. At about 10 ft push off

drift <K+0>. Aim for a vertical speed at touch down of -100 ft/min.

After touch down, pitch down <P+da> untill the nosewheel is also on the

ground and start braking. First start braking on the wheels <B+da> then

activate thrust reverse <E+da> and increase the fanspeed <T+da!!!>.

If heading correction is necessary use the nosewheel steering <N+ra/la+(sb)>.

If runway length permits, cancel thrust reverse <E+ua> if the speed is

below 80 kt. Bring the aircraft to a standstill before the end of the

runway.

4.10   OVERSHOOT

4.10.1 Overshoot with all engines working

If for some reason an overshoot is necessary then pitch up to 10 degrees and

set the engines to climbpower (N1=96%). If a positive rate of climb is

established retract the undercarriage.

If the altitude is above 500 ft and the speed at least Vref+10 start flap-

retraction from 55 to 15 degrees, which each next step after a further

speedincrease of about 5 kt.

With slats at 16 degrees and flaps at 15 degrees and a speed of V2+10 or

above pitch up to 15 degrees. Switch off the ILS and climb to 2000 ft and

see par. 4.2.1 for further information about speeding up and cleaning up.

4.10.2 Overshoot with 1 engine out

If for some reason an overshoot is necessary set the life engines to the

maximum take-off power of N1=100% . Then pitch up to 7 degrees. If a

positive rate of climb is established retract the landinggear.

If the altitude is above 500 ft and the speed at least Vref retract the

flaps to 45 degrees and pitch up to 8 degrees. After each speedincrease

of 5 kt retract the next step of the flaps and pitch up 1 degree further.

With slats at 16 degrees and flaps at 15 degrees and an attitude of 12

degrees climb to 1000 ft at V2 or above. For speeding up and cleaning up

see par. 4.2.2

Remark :

If the all up weight is not above 150 tons then an overshoot is possible

with only 1 engine operating.

If an overshoot is necessary set the live engine to the maximum take-off

thrust with N1=100% . Pitch up to 6 degrees (level flight) and retract gear.

If the speed is at least Vref+10 retract flaps from 35 to 25 degrees and

pitch up to 8 degrees. If the speed is at least Vref+15 retract flaps to

15 degrees and pitch up to 10 degrees.

With slats at 16 degrees and flaps at 15 degrees and an attitude of 10

degrees climb to 1000 ft at V2 or above. Clean up above 1000 ft.

ANNEX TO BASIC FLIGHT MANOUVRES

A guide to throttle, pitch, slat and flap setting

A) All engines working

s e t t i n g      r e s u l t

FLIGHT PHASE          N1 pitch  sl  fl   alt   IAS   VSI   Remarks

1 TAKE-OFF+CLIMB OUT                                       T.O.W.=250 tons

Normal take-off   100  15.0  16  15   1000  191   2350  V2+10,LG:UP

Noise abatement    76  10.0  16  15   2400  190    500  V2+10

2 CLIMB

Up to 10000 ft     90   8.0   0   0   6000  275   2240  IAS<275,N1=90-96%

Up to 16000 ft     96   4.0   0   0  15000  325   1850  Vcl=325 +/-10

Up to 23000 ft    100   3.0   0   0  20000  332   1480

Up to cruise alt. 100   2.0   0   0  25000  334    840  245 tons

3 CRUISE

At  6000 ft (1)    58   4.0   0   0   6000  270      0  IAS<275,248 tons

At  6000 ft (2)    49   3.0   0   0   6000  248      0  IAS<250,180 tons

At 12000 ft        60   1.0   0   0  12000  307      0  200 tons

At 24000 ft        75   1.0   0   0  24000  301      0  200 tons

At 29000 ft        88   1.5   0   0  29000  315      0  240 tons

At 37000 ft        94   2.0   0   0  37000  262      0  180 tons

4 DESCENT                                                  180 tons

Down to 24000 ft   25  -2.0   0   0  28000  291  -2150  from 37000 ft

Down to 10000 ft   25  -3.0   0   0  18000  324  -2160

Down to  6000 ft   25   0.0   0   0   7000  250  -1400  IAS<250,AB:ON/OFF

5 APPROACH                                                 Straight in appr.

Down to 4500 ft    25   1.0  16   0   5000  234  -1250  Vref+100,17nm out

Down to 3000 ft    35   1.5  16  15   3300  186  -1080  Vref+ 50,11nm out

Down to 1200 ft OM 55   0.0  25  55   1200  144   -730  Vat+10,LG:DN

6 FINAL APPROACH                                           AB:ON,DH=100 ft

Down to 500        63   0.0  25  55    600  144   -690  Vat+10

Down to  50        61   3.0  25  55    100  134   -670  Vat=Vref

7 LANDING

Touch down         25   5.0  25  55      0  130   -200  L.W.=180 tons

B) One engine out

s e t t i n g      r e s u l t

FLIGHT PHASE          N1  pitch sl  fl   alt   IAS   VSI   Remarks

1 TAKE-OFF+CLIMB OUT                                       T.O.W.=250 tons

With eng. failure 100  12.0  16  15    700  182    790  V2,LG:UP

2 CLIMB

Up to  7000 ft     92   6.0   0   0   4000  274   1100  IAS<275,N1=90-96%

Up to 10000 ft     92   5.0   0   0   9000  274    700  IAS<275,N1=90-96%

Up to 15000 ft     96   3.0   0   0  13000  306    630  Vcl=310 +/-10

Up to cruise alt  100   2.0   0   0  17000  322    440  245 tons

3 CRUISE

At  6000 ft        62   3.0   0   0   6000  249      0  IAS<250,180 tons

At 12000 ft        72   1.0   0   0  12000  293      0  180 tons

At 18000 ft        92   1.5   0   0  18000  314      0  240 tons

At 27000 ft        98   0.5   0   0  27000  304      0  180 tons

4 DESCENT                                                  180 tons

Down to 10000 ft   25  -3.0   0   0  20000  324  -2240  from 27000 ft

Down to  6000 ft   25   0.0   0   0   7000  250  -1430  IAS<250,AB:ON/OFF

5 APPROACH                                                 Straight in appr.

Down to 4500 ft    25   1.0  16   0   5000  234  -1300  Vref+100,17nm out

Down to 3000 ft    45   1.5  16  15   3300  186  -1100  Vref+ 50,11nm out

Down to 1200 ft OM 71  -1.0  25  55   1200  148   -740  Vat+10,LG:DN

6 FINAL APPROACH                                           AB:OFF,DH=200 ft

Down to 500 ft     71  -1.0  25  55    600  148   -710  Vat+10=Vref+15

Down to  50 ft     71   2.0  25  55    100  138   -630  Vat=Vref+5

7 LANDING

Touch down         25   4.0  25  55      0  133   -200  L.W.=180 tons

5.0    NAVIGATION

----------

5.1    NAVIGATION SYSTEMS

5.1.1  ADF

The Automatic Direction Finder (ADF) is an instrument which, when correctly

tuned to a ground radio station is capable of indicating the relative and/or

magnetic bearing of that station. The ground stations are called Non

Directional Baecons (NDB) A locator (L) is a low-powered NDB used primarily

as an aid to bring the aircraft to a suitable position for an ILS approach.

The correct tuning of a station is checked by identifying the two or three

letter Morse code which is superimposed on the signal.

The bearing of a station is found by nulling the received signal with an

electro-mechanical (electric motor) or electronical steered directional

sensitive aerial.

The three main items (identification code, bearing and relative bearing) of

the ADF are given in the upper part of box F of the instrumentpanel (par 2).

Box G is a kind of Radio Magnetic Indicator (RMI). The light-blue colored

dot represents the compass needle, the yellow one shows the bearing data

from the ADF equipment.

In the table below the the NDB's available in the program are listed.

Identifier   Full name    Country    Type

BPK        Brookmanspark     GB       NDB

CAM        Cambridge         GB       NDB

CON        Congleton         GB       NDB

DUN        Dunsfold          GB       NDB

KNI        Knighton          GB       NDB

LA        Lyneham           GB       NDB

LIC        Lichfield         GB       NDB

MCR        Manchester        GB       NDB

NH        Norwich           GB       NDB

WOD        Woodley           GB       NDB

EHN        Eindhoven         NL       NDB

ENK        Enkhuizen         NL       NDB

LAK        Lake              NL       NDB

NYK        Nyke              NL       NDB

ROT        Rotterdam         NL       NDB

STD        Stad              NL       NDB

THN        Thorn             NL       NDB

DEN        Dender             B       NDB

GAA        Gatta              B       NDB

ONT        Kleine Brogel      B       NDB

OA                          NL         L for ASD 19R

GX                          GB         L for BMH 15

GY                          GB         L for LGW 08

NW        Nieuwstad         NL         L for MST 22

RR                          NL         L for RTD 24

WP        Weesp             NL         L for ASD 27

To make your own chart with these beacons use the longitude and latitude

information in the program.

5.1.2  VOR/DME

The Very high frequency Omnidirectional Range (VOR) is a system which

indicates very accurate the direction (radial), as seen from the VOR ground

station, of the aircraft position. Like a compass 360 radials are used with

magnetic north being 0 or 360.

The working of a VOR can be explained by a simile.

For the ground radio station you should think of a light house with a

rotating beam with known rotation rate. When the beam passes magnetic north

a reference signal is send to all directions, for example by a flashing red

lamp. From the time difference between the reference signal and the moment

that the rotating beam crosses the aircraft and the known rotation rate the

direction of the radial can be calculated.

The real implementation with radio transmitters on the ground and receivers

in the aircraft is of course very complicated.

Like NDB stations also VOR stations have three letter identification codes

in Morse superimposed on the signal.

The Distance Measuring Equipment (DME) does exactly what it says: it

measures the distance between the aircraft and a known point on the ground.

A socalled Interrogater in the aircraft transmits coded pulses to the

Responder on the ground. The Responder reacts by sending back a signal to

the aircraft. From the time lapse between transmission from and reception in

the aircraft and the well known speed of radiowaves the distance between

aircraft and Responder can be calculated.

By using different pulse patterns the Responder can accept signals from up to

100 aircraft simultaneously.

In the simulation program distance is given as groundtrack distance, so it

is indepent of the altitude of the aircraft.

Very often the ground equipment for VOR and DME is placed together  on the

same location. In this case the combined information of one VOR/DME station

is sufficient for position fixing.

The information from the two VOR/DME receivers appears in box J (par 2).

Left of the station identifier the distance in nm is given. At the right the

VOR radial is given

In the table below the VOR/DME stations available in the program are listed.

Identifier  Full name   Country

BCN         Brecon         GB

BIG         Biggin         GB

BNN         Bovingdon      GB

BUR         Burham         GB

BTN         Barton         GB

CFD         Cranfield      GB

CLN         Clacton        GB

DET         Detling        GB

DTY         Daventry       GB

DVR         Dover          GB

GAM         Gamston        GB

HON         Honily         GB

IBY         Ibsley         GB

LAM         Lambourne      GB

LON         London         GB

MAY         Mayfield       GB

MID         Midhurst       GB

OCK         Ockham         GB

SFD         Seaford        GB

WAL         Wallasey       GB

EEL         Eelde          NL

HDR         Den Helder     NL

HSD         Haamstede      NL

MAS         Maastricht     NL

PAM         Pampus         NL

RKN         Rekken         NL

RTM         Rotterdam      NL

SPL         Schiphol       NL

SPY         Spykerboor     NL

BUB         Brussels        B

BUN         Bruno           B

CIV         Chievres        B

COA         Costa           B

KOK         Koksy           B

LNO         Olno            B

NIK         Nicky           B

SPI         Sprimont        B

To make your own chart with these beacons use the longitude and latitude

information in the program.

5.1.3  ILS

The Instrument Landing System (ILS) is purily designed as a let down aid

and guides aircrafts down from a few thousend feet onto the runway. This

aid makes landings possible almost indepent of the visibility conditions.

The ILS consists of a localizer transmitter (loc tx) near the upwind end

of the runway. The directional diagram of the aerial system and the

modulation of this transmitter is such that with a receiver in the aircraft,

after some processing, the deviation from the runway centreline can be read

on the instrumentpanel. A second transmitter, called glidepath transmitter

(gp tx), is located near the touch down point on the runway. The direction

diagram of the aerial system and the modulation of this transmitter is such

that with a receiver in the aircraft, after some processing, the deviation

from the glidesope (normally 3 degrees) is presented on the instrumentpanel.

The processed signals of both receivers are fed in an instrument called

crosspoint meter. This instrument has two needles, a vertical one moving

left/right and indicating the deviation from the centreline and a horizontal

one moving up/down and indicating the deviation from the glideslope.

For the interpretation of the instrument indication see chapter 2.H .

The combined guidance in a horizontal and vertical sense will guide the

aircraft smoothily down.

Information about distance to the touch down point is partly obtained from

marker beacons. The outer marker (OM) is normally about 4 nm from touch

down. At the outer marker the altitude should be about 1300 ft. A middle

marker (MM) at roughly 0.6 nm from the treshhold should be crossed at about

200 ft. The inner marker is normally not present at civil airports.

In the table below the runways available in the program are listed.

Runway ident   length   magnetic bearing   country    town

(ft)       (degrees)

ASD 19R         10800         187            NL     Amsterdam

ASD 24          10600         242            NL     Amsterdam

ASD 27          11300         271            NL     Amsterdam

BMH 15           7000         153            GB     Birmingham

BXL 25L         10500         254             B     Brussels

LGW 08           8200         085            GB     London-Gatwick

LHR 10R         11000         097            GB     London-Heathrow

MCR 06           8100         059            GB     Manchester

MST 22           8200         216            NL     Maastricht

RTD 24           7200         241            NL     Rotterdam

To make your own charts with these runways use the longitude and latitude

information for the glidepath transmitters in the program.

5.2    NAVIGATION PRINCIPLES

The main purpose of this section is to give some guidelines to pilots

interested in precision flying.

The basic operation in navigation procedures is the change of the flying

direction. For maximum control during all these procedures you must have

some insight in the relation between speed, bankangle, turnradius and rate

of turn. Therefore some background is given about the true banked flat turn.

A true banked turn is a turn purily caused by banking without any sideslip.

Because in this DC-10 program the vertical tail effect is not simulated the

only way of turning is the true banked turn. Based on an analysis of the

forces working on an aircraft during such a turn the following practical

formulas can be deduced :

R = 0.0000146 * V * V / tan B

T = 1090 * tan B / V

With R is turnRadius in nm, V is TAS in kt, B is the Bankangle in degrees

and T is the rate of Turn in degr/sec. Note that lift and weight are no

parameters in these formulas.

Take for example a speed of 180 kt TAS and bankangle of 15 degrees, then

the turnradius R=1.8 nm and the rate of turn T=1.6 degr/sec.

From the foregoing it is clear that an aircraft need (a lot of) space and

time to change its direction. If you have to take up another bearing after

crossing of a radiobeacon the smoothest way is to start your turn some

distanc before the crossing. The same applies if you have to join a

particular track to or from a radiobeacon. You should start to pick up your

new bearing before reaching the track. This anticipation of the turn is

called lead. For the first situation lead distance (LD) is the most relevant

parameter, in the second one lead angle (LA) is most helpfull.

Some simple formulas will be given to calculate the value of these

parameters for almost all practical situations.

First the mathematical relation between lead distance in nm (LD),

turnradius in nm (R) and bearing change in degrees (BC) is given :

LD = R * tan(BC/2)

For example at 300 kt TAS and a bankangle of 15 degrees the turnradius is

R = 4.9 nm, so if a bearingchange of 45 degrees is necessary then LD=2 nm.

In order to pick up your new bearing as smooth as possible start the turn

about 2 nm before you cross the beacon.

Two remarks should be added :

- If a VOR/DME station is approached then the DME data will give the right

starting point for the turn. If a NDB is approached it is not always that

simple. However if you fly from a VOR/DME to a NDB and you know the

distance between the beacons (from your flightplan) then the turn at the

NDB should be started if the distance between the aircraft and the VOR/DME

station is equal to the distance between the VOR/DME and the NDB minus the

lead distance

- In these calculations windeffects are not taken into account, so in

practice the patterns will not be flown as accurate as the calculation

suggests, but they are still a good basis.

Lead angle is a little more complicated. If you have to track in or out on a

certain bearing/radial to or from a beacon then it is helpfull to know at

which bearing/radial you have to start the turn. This of course depends on

the turnradius (R), the magnitude of the bearingchange (BC)and the distance

(D) between aircraft and the beacon. A mathematical approximation is :

LA = 114.6 * (R/D) * sin(BC/2) * sin (BC/2)

For LA < 10 degr the accuracy is better then  1 % .

For LA < 45 degr the accuracy is better then 10 % .

Do not use this formula for LA values above 45 degrees

An example will be taken from an approach to RWY MIA 09R (see chapter 6.2).

If you approach WP1 turn left to home at MI on a track of 87 degrees.

The lead angle for the anticipation of this turn will be calculated.

At 7000 ft and a speed of 250 kt IAS the TAS is 273 kt. With a bankangle of

25 degrees the turnradius R=2.3 nm. Using the chart, the distance between the

beacon and the aircraft at the start of the turn can be estimated . D=29 nm.

The nominal bearing before the start of the turn is 158 degrees, so the

bearingchange is 158-87= 71 degrees. Then LA= 3 degrees

So you have to start your turn if the bearingindicater of MI shows 90 degr.

In the simulation program also the data about the actual wind, groundspeed

and track is available. In modern airliners this is deduced from data from

Doppler radar equipment, the Inertial Navigation System (INS) or the Omega

system. Especially the data on track is very helpfull because it is often

much simpler to find the necessary compensation for windeffects.

Comparing this with the situation in the past without these direct data

makes clear that flying a prescribed pattern is a little easier nowadays.

For more information about navigation procedures some of the books in the

reference list are very helpfull.

5.3    FLIGHTPLANNING

Associated with a flightplan is a lot of documentation and calculation on

fuel requirements, take-off data, weather reports, navigation data etc.

In this chapter only some aspects related to navigation will be discussed,

especially the use of the flightplanning possibility in the first part of

the program.

After choosing option 1 a survey of the radiobeacons and runways available

in the program are presented on the screen.

In order to make a simple and straightforward flightplan just enter the

numbers for the required aids separated by a  >  sign.

For example entering  80>43>7  gives distance and bearing from runway RTD 24

to HSD and further on to CLN. So if you cross HSD you know at forehand what

new bearing you have to take up to CLN.

Airways are not always straight from beacon to beacon. Sometimes there are

waypoints (WP) between them where a change of direction is necessary.

This flightplanning program has features build in to calculate also such

routeplans.

For example a route from HON (aid nr 12) to BNN (aid nr 3) via a waypoint

37 HON 147 can be brought in by entering 12>37@147>3. So if you enter

the distance and bearing from the first aid to the waypoint then the

computer gives you the complete route. Chapter 6.1 contains an example

how an complete flightplan can be composed using this possibility.

As shown, the waypoint information is brought in starting with the distance

and the bearing is put after the @-sign.

Some restriction to the distance data has to be taken into acount.

Maximum distance is 99 nm, above 10 nm it must be an integer, below 10 nm

an accuracy of 0.1 nm is possible. The bearing data must be in the range

from 000 to 360 degrees and must always consist of 3 figures. The program

accepts only waypoints between a longitude of 7 deg W and 10 deg E.

6.0    PRACTICE-FLIGHTS

----------------

6.1    FLIGHT BIRMINGHAM-LONDON

In this chapter a flight from Birmingham to London Heathrow is described.

If the first part of the program is loaded choose option 1 to make a flight-

plan. A standard approach route to London Heathrow from the north-west is

via VOR/DME DTY and BNN, tracking out on BNN 222 to 15 BNN 222 and after

the final turn (lefthand turn of 125 degr) land on LHR 10R guided by the ILS

For the first part of the route it is proposed to keep the runway track

until reaching the airway between VOR/DME stations HON and DTY.

To get a complete and accurate flightplan enter the following line :

74>8@153>9>3>15@222>1.3@192>1.3@160>1.3@127>77   (see also par 5.3)

The resulting flightplan is shown below.

FLIGHTPLAN     BMH - LHR

nm    degr

BMH-WP2        8.0   153   (WP=waypoint)

WP2-DTY       22     125

DTY-BNN       34     150

BNN-WP5       15     222

WP5-WP6        1.3   192   )

WP6-WP7        1.3   160   ) Final turn

WP7-WP8        1.3   127   )

WP8-LHR        9.6   097   (to LHR 10R, 11000ft/097 degr)

----

Total distance 93 nm

If the flightplan does not need any change press the N-key to load the real

simulation program.

After loading of DC10EUR.BAS choose BMH 15 (7000 ft/153degr) as the take-off

runway by entering 4. Before entering the payload and the fuelload note the

maximum take-off weight for this runway length. Enter 45 tons as the payload

and 20 tons as the fuelload for this trip.

The take-off data for this flight is given on the next screen and also here:

T.O.W. = 180 tons, V1 = 138 kt, V R = 145 kt, V2 = 153 kt.

Start flap retraction at 173 kt and slat retraction at 213 kt.

If the instrumentpanel has been set up, prepare for the take-off :

Start the engines, set slats to 16 degrees and flap to 15 degrees, select

VOR/DME HON and DTY and select GX on the ADF to make it easy to keep runway-

track after the take-off.

If you are ready for the take-off, increase the fanspeed N1. With a take-off

weight of 180 tons and a runway length of 7000 ft it is not necessary to use

the maximum take-off power. Increase N1 to 95% .

If at the start of the take-off roll the aircraft track (heading) is not the

same as the runway track of 153 degrees correct this with the nosewheel-

steering, otherwise you will end up in the mud aside the runway.

Keep an eye on the proper working of the engines, because if one fails

before reaching V1 you have to stop the aircraft because you cannot reach

your take-off speed before the end of the runway.

Supposing everything goes fine pitch up at reaching V R. The aircraft is

very light so pitch up to 18 degrees. Retract gear above 20 ft and keep

runway heading until you reach 500 ft. Then correct for sidewindeffects and

keep a track of 153 degrees.

At reaching 2000 ft pitch down to 11 degrees and reduce N1 to 85% . Retract

flaps and slats after reaching the relevant speeds. Above 5000 ft set pitch

to 9 degrees.

Abeam HON turn left (bank 15 degr) and track in on DTY 305 (track:125 degr).

Above 8000 ft pitch down to 7 degrees and set pitch to 4 degr after passing

10000 ft, N1 still keeping at 85% .

With a total weight of 180 tons and a route distance of nearly 100 nm a

cruiseheigh of 12000 ft is most suitable. Reach this heigh before DTY.

After reaching of 11500 ft reduce pitch to 2 degr and throttle back to 65% .

Level-off at 12000 ft witch a pitchsetting of 1 degr, N1 at 56% and a speed

just above 290 kt.

If you approach DTY select VOR/DME BNN instead of HON. At roughly 1 nm

before DTY (1 DTY 305) start turning to the right. After crossing DTY track

in on BNN 330 (track:150 degr,track out on DTY 150).

For the descent a descentpath angle of 5 degr is chosen for the first part

down 5000 ft. Below 5000 ft a descentpath of 3 degr is used.

At 15 BNN 222 you must be down at 2500 ft, so you have to loose first

7000 ft on a 5 degr descentpath and then 2500 on a 3 degr path before the

final turn. The start of the descent is then  (7000/6078*tan5) +

(2500/6078*tan3) = 13.2+7.8 = 21 nm before 15 BNN 222  (1 nm = 6078 ft).

Therefore start the descent just before 6 BNN 330 by reducing thrust to idle

and pitching down to 4 degr below horizontal. Below 10000 ft there is a speed

restriction of 250 kt, so start braking as soon as the descent has started.

There is an easy way to check of you go down on a 5 degr path :

The vertical speed in ft/min must be equal to 6078*tan5/60=8.9 * GROUNDspeed

in kt. For example at 250 kt IAS at 8000 ft the TAS is 276 kt and supposing

a tailwind of 20 kt the resulting groundspeed is 296 kt. In this case the

VSI must show something near to -2630 ft/min for a 5 degr descent.

Because the speed cannot be chosen freely you have to control the descent-

path with the pitchangle. So if the speed come down to 250 kt IAS pitch up

slowly to 2 degr below horizontal to keep on the right descentpath.

Cross BNN at 8700 ft track out on BNN 222 (track:222 degr) and aim for

5000 ft at 7 BNN 222.

If not already done select LHR 10R as the destination runway and tune in to

WOD on the ADF as an secondary aid.

After arriving at 5000 ft near 7 BNN 222 pitch up 2 degr to reduce the

descentpath from 5 to 3 degr. At a weight just below 180 tons, clean wings

and a speed near 250 kt IAS level pitch gives a nice 3 degr descentpath.

Before the final turn the speed must be reduced to Vref+50 (182 kt) so

extend the slats to 16 degr and start braking before 10 BNN 222.

Keep level pitch and use the flap extension to keep the aircraft on a 3 degr

descentpath despite the decreasing speed. For a 3 degr descentpath the

vertical speed in feet/min must be equal to 6078*tan3/60 = 5.3 * GROUNDspeed

in kt.

Extend flaps to 25 degr. Because of the flap extension the drag increases

so keep watching your speed and release airbrakes and open throttles when

necessary.

The final turn takes place at a constant altitude of 2500 ft. For a flat

turn started at 15 BNN 222 with slats at 16 degr, flaps at 25 degr and a

speed of about 182 kt the bankangle is 15 degr, the pitch is 3 degr and

N1 about 59% .

If the final turn is started switch on the ILS for LHR 10R (11000ft/097degr)

If you approach the runway track then track in first on the localizer.

Pick up the glideslope from 2500 ft at 8 nm to go. Do this by pitching down

to level again and control the descentpath with the throttle only.

Extend the landinggear now.

Before the outer marker slats and flaps must be extended fully, so reduce

thrust to idle and extend the airbrakes if you like. Set slats to 25 degr.

At the same way as before the final turn keep level pitch and if the speed

decreases use flap extension to keep the aircraft on the glideslope.

If slats and flaps are fully extended release the airbrake and set N1 to 55%

to maintain a speed of V AT + 10 (142 kt, all engines working).

With level pitch this speed will keep you on the glideslope.

Cross the outer marker (OM on the screen) at an altitude of about 1300 ft.

At an altitude of 800 ft reduce N1 with 10% . If the speed decreases keep

on the glideslope by pitching up slowly. If the pitch is 3 degr up and the

speed is V AT (132 kt) then increase N1 again to 55% . Keep the pitch

constant and correct for glideslope deviations with your throttle only.

At the decision heigh (DH=100 ft, all engines working) you have to decide

wether or not to go ahead and land or to overshoot and go around. If at the

decision heigh the deviation on the ILS crosspoint meter is less then

0.5 degr in both directions (especially in a horizontal sense it is very

stringent) then a safe landing is possible.

At 50 ft start the flare by pitching up 1.5 degrees (0.5 degr/sec), at 20 ft

close throttles, below 10 ft kick-off drift and pitch up another 0.5 degr.

After touch down pitch down until the nosewheel is also on the ground and

start braking making use of the wheel brakes as well as thrust reverse.

Keep the aircraft on the runway using the nosewheel steering and stop it

before the end of the runway.

If at the decision heigh the conclusion had to be : do not land but go

around then, supposing all engines are working, pitch up to 10 degr and

increase N1 to 95% . If the aircraft starts climbing retract the gear.

Above 500ft and at a speed of at least Vref+10 (142 kt) start flap

retraction from 55 to 15 degr, with each next step after a further speed-

increase of about 5 kt.

With slats at 16 and flaps at 15 degr and a speed of V2+10 (160 kt) or above

pitch up to 15 degr. Switch off the ILS and climb to 1500 ft on the runway-

track. (097 degr) Then turn to the right and track in on VOR/DME OCK.

Above 2000 ft pitch down to 10 degr, reduce N1 to 85% and retract slats and

flaps at the prescribed speeds. Keep your speed below 250 kt IAS

Cross OCK at 3000 ft and track out on OCK 306. At 14 OCK 306 the final turn

starts. Before that point you must have come down to 2500 ft, reduced speed

to Vref+50 and slats extended to 16 and flap to 25 degr. After the final turn

pick up the ILS beam for another try.

6.2    LANDING ON MIAMI INTL

This DC-10 program is purily an instrument flight simulation . For most

pilots therefore the most difficult part in flying with DC-10 will be the

approach to and landing on a selected runway. To build up experience on

this part of flying a special version of the program has been developed,

called DC10MIA.BAS

In this chapter in principle only the differences with and the additions

to the general part of the documentation are discussed.

6.2.1  Autopilot options

For beginning pilots it not easy to land an aircraft, certainly not if it

has to be done on instrument information only. Therefore it is thought to be

very helpfull to have a variable level of assistance by an automatic pilot,

especially during the final approach and landing. So if you get more

experience you need less and less support by the automatic pilot until you

can do it all alone.

The autopilot build in in DC10MIA.BAS for this purpose has 5 options :

- Autoland

- Autothrottle + autoflare

- Autothrottle

- Autoflare

- No autopilot

In the autoflare option all pitch up and down is done by the autopilot.

In the autothrottle option thrust control including reverse thrust and any

braking action is done by the autopilot. If the autoland option is chosen

everything is done by the autopilot, a socalled 'hands-off' landing.

In the autothrottle + autoflare option the only action to be done by the

pilot is to roll to the left/right to keep the aircraft on the beam in

horizontal sense, kick-off drift at or below 10 ft and after the landing

keep the aircraft on the runway using the nosewheel steering.

An automatic pilot can not take control from any random start situation.

To be precise, the autoland instruction will only be accepted if the

following conditions are met simultaneously :

- The plane is above the outer marker (OM on the screen)

- The deviation from the glidepath is not more then 2 degr horizontally and

0.5 degr vertically

- The track must be within 5 degr from the runway centreline (087/267 degr)

- The landinggear must be down  and the slats and flaps fully extended

- The attitude must be level (pitch: 0 degr)

- The speed must be within 5 kt from V AT + 10 kt  (Vat+5 < IAS < Vat+15)

- All engines working

On the same basis the autothrottle and/or autoflare instruction(s) will only

be accepted if the following conditions are met simultaneously :

- The altitude must be between 500 and 700 ft

- The deviation from the glidepath is not more then 1 degr horizontally and

0.5 degr vertically

- The track must be within 5 degr from the runway centreline

- The landinggear must be down and the slats and flaps fully extended

- The attitude must be 3 degr up

- The speed must be within 5 kt from V AT  ( Vat-5 < IAS < Vat+5 )

( V AT on the screen)

- All engines working

If any of the autopilot instructions is accepted there will always be a

check at an altitude between 20 and 30 ft if it is likely to become a safe

landing. If not, then an automatic overshoot action is started (AOS on the

screen). After this has been completed the autopilot will be disconnected

(AOS dissappears). From there on the pilot must take over.

6.2.2  Area navigation

In the terminal area of Miami International airport 2 VOR/DME stations are

in service: MIA (Miami) and BSY (Biscayne Bay) and one combined locater/

outer marker: MI (Keynes) The locations can be found in the (very inaccurate)

chart given below.

With the old navigation equipment the flexibility in approach routes is very

restricted. With modern computerized navigation equipment new possibilities

are available. One of them is called RNAV or better Vector computer. This is

one of the many forms of area navigation. It gives the possibility of route-

plans outside the airways system.

The RNAV equipment makes use of the existing VOR/DME groundfacilities.

The computer in the aircraft gives the capability of 'moving' a VOR/DME

station from its actual location to a 'ghost' position (WayPoint) of the

pilots choice. So if the VOR and DME receivers in the aircarft are tuned to

MIA and the computer is instructed to move this 17.8 nm on a bearing of

231 degr from this station (to WP1) then the RNAV equipment will give the

the distance and bearing from the aircraft to WP1. You will get the

impression as if WP1 is an indepent facility.

A simplified form of RNAV is implemented in the program. It is not possible

to choose your waypoints freely, only 10 pre-progammed waypoints are

available. The location of these waypoints (WP0-9) are also (roughly)

indicated in the drawing below. The accurate 'moving' data is :

WP0: 20.0 MIA 279  WP1: 17.8 MIA 231  WP2: 12.4 MIA 205  WP3: 11.0 MIA 183

WP4: 18.0 MIA 214  WP5:  5.2 BSY 293  WP6:  4.8 BSY 061  WP7:  7.9 BSY 072

WP8: 10.4 BSY 044  WP9:  8.3 BSY 028

With this information and the bearing from the locater/outer marker MI it

must be possible to intercept the ILS beam before you have come down to

2500 ft. From this point on the ILS guides you down to the runway.

The selection of the runway you are going to land on (09R =087 degr or

27L =267 degr) must of course primarily be based on the groundwind direction.

WP0   *                                           MIAMI (FLA/USA)

(WESTO)                                             WESTO ONE RNAV ARR.

MIA                TO RWY 09R/27L

+                      5 nm

|-------|

start  o

WP2     WP3           RWY      MI      WP9    WP8

WP1  *            *       *            ===       O       *      *

WP4  *                               *             *      *

WP5     +     WP6   WP7

BSY

6.2.3 Differences in control facilities

It will be clear that all facilities for pilotcontrol of the aircraft itself

are unchanged (see chapter 3)

All European NDB-, VOR/DME- and ILS groundfacilities have been deleted.

The following possiblities are added :

HELP + L     -autopilot-     autoland

HELP + T                     autothrottle

HELP + F                     autoflare

HELP + da                    disconnect autopilot

D + 1        -destination-   select RWY MIA 09R on the ILS and DME

D + 2                        select RWY MIA 27L on the ILS and DME

W + 0..9     -waypoint-      select WP0 to WP9 on the RNAV equipment

REFERENCES

* For practical information :

- D.P.Davies: 'Handling the big jets'

Third edition, Civil Aviation Aythority, London 1971

- J. Belson: 'A300 in the air'

Flight International, 11 dec 1976

- H. Varley, Ed: 'The fliers handbook'

Panbooks Ltd, London 1978

- G.D.P. Worthington: 'Airline instrument flying'

Pitman & Sons, London 1968

- N.H. Birch and A.E.Branson: 'Flightbriefing for pilots part 3:

radio aids to air navigation'

Fourth edition, Pitman publishing, London 1979

- D.J. Clausing: 'The aviator's guide to modern navigation'

TAB BOOKS Inc, USA 1987

- C. McAllister: 'Aircraft alive: aviation and airtraffic for enthusiasts'

Batsford, London 1980

- M.Horseman: 'Air Europe six seven zero'

Aircraft Illustrated, april + may + july 1981

- J.W.R. Taylor, Ed: 'The lore of flight'

Crescent books, New York 1978

- A.C. Kermode: 'Flight without formulae'

Fourth edition, Pitman publishing, London 1970

* For theoretical background :

- E. Torenbeek: 'Synthesis of subsonic airplane design'

Delft University Press 1976

- T.I. Ligum: 'Aerodynamics and flight dynamics of turbojet aircraft'

NASA Technical Translation ....

- D.M. McRae: 'The aerodynamic development of the wing of the A 300 B'

Areonautical Journal, july 1973

- W. McIntosh and W.K. Impress: 'Prediction and analysis of the low speed

stall characteristics of the Boeing 747'

AGARD Lectures Series No LS-74, march 1975

- J.G. Callaghan: 'Aerodynamic prediction methods for aircraft at low

speed with mechanical high lift devices"

AGARD Lectures Series No LS-67, may 1974

- J. Williams: 'Airfield performance prediction methods for transport

and combat aircraft'

AGARD Lectures Series No LS-56, april 1972

- H. Friedel: 'Flight manouvre and climb performance prediction'

AGARD Lectures Series No LS-56, april 1972

- I.E. Traeger: 'Aircraft gasturbine technoligy'

McGrawhill 1970

- R. von Mises: ' Theory of flight'

Dover publication, New York 1945, 1959

- E.W. Anderson: 'The principles of navigation'

Hollis and Carter, London 1966

- M. Kayton and W.R. Fried, Ed: 'Avionics navigation systems'

Wiley & Sons, New York ....

- Anon.: 'Air navigation today and in the year 2000'

Proceedings of the National Aerospace Symposium, 25-27 april 1978

in Atlantic City

The Institute of Navigation, Washington