SLOWSETUP.DOC

Matthew Dillon

891 Regal Rd.

Berkeley, Ca. 94708

pacbell.PacBell.COM!sorinc!overload!dillon

GETTING UUCP UP AND RUNNING FOR REAL

This UUCP distribution in general is not for the uninitiated.  That is,

it is not user friendly.  This particular document outlines a general

installation procedure.  Be warned that there are a LOT of things you

will have to do to bring up UUCP on your machine.

This document shows a TYPICAL setup.  You will most likely want to

modify the example to fit your own needs.  I cover all the points

though.

(1) You made backups of the distribution floppies, right?  DO NOT

MODIFY THE DISTRIBUTION FLOPPIES.  The instructions below are

long and some are complex and you may wish to refer to the

original distribution diskettes if you accidently mess something

up.

(2A) Installing UUCP on a single floppy system.

Well, it *is* possible, but difficult.	you need to make as much

space as possible available on the main floppy.

The second distribution disk contains only the source and manual

pages.	I suggest you shuffle things around a bit.  You already

have a copy of the first distribution diskette. I suggest you

format a new disk and put the MAN directory of the second

distribution disk on it.  Note that there is a MAN directory

on the boot disk as well but this one contains only dmail.help

which you need to gain online help from dmail.

The following executables are required on the boot floppy and

should not be moved off.  You should move as many of the remaining

executables somewhere else as you can.

Required Executables in C

c/dmail

c/getty

c/rmail

c/uucico

c/rnews

c/uucp

c/unbatcher

c/dme	    (unless you configure your own editor)

c/sendmail

c/uux

c/uuxqt

You will want to change the ASSIGNments in s/startup-sequence

as follows:

Assign UUSPOOL:	UUCP:SPOOL

Assign UUMAIL:	UUCP:MAIL

Assign UUNEWS:	UUCP:MAIL/NEWS

These directories MUST be in nonvoltile storage or your USENET

node will be unreliable.

(2B) Installing UUCP on a duel floppy system.

In this case very little shuffling is required.  I suggest

the second floppy be used for UUSPOOL:, UULIB:, UUMAIL:, UUNEWS:,

GETTY:, and UUPUB:.

Format a new floppy (this will be your second floppy) and create

directories as follows:

MakeDir df1:spool

MakeDir df1:mail

MakeDir df1:mail/news

MakeDir df1:pub

MakeDir df1:lib

Modify the Assignments in s/startup-sequence of the boot floppy

as follows:

Assign UUSPOOL:	df1:spool

Assign UULIB:	df1:lib

Assign UUMAIL:	df1:mail

Assign UUNEWS:	df1:mail/news

Assign GETTY:	UULIB:

Assign UUPUB:	df1:pub

Copy df0:lib to df1:lib and either delete or rename df0:lib to

prevent confusion.  What we have done is move all dynamic storage

to df1: .. files that will change a lot or require a lot of disk

space.	Specifically, UULIB: was moved to DF1: because the

sequence number file is kept in UULIB: and this is updated for

each mail message sent.

(2B) Installing UUCP on a Hard Drive

Make a directory called UUCP on your hard drive somewhere.  And

Assign UUCP: to it.   Copy the contents of both distribution

floppies to your harddrive:

(put first disk in DF0:)

1> Copy df0: UUCP: ALL CLONE

(put second disk in DF0:)

1> Copy df0: UUCP: ALL CLONE

It is easiest to keep all the directories in one place.  Note

that a MAN directory exists on both distribution floppies, simply

merge them together.

Merge the startup-sequence on the boot floppy into your master

startup-sequence.  Make sure you get all the Assigns and that

they are properly reassigned to the appropriate directory on

the harddisk.  Most people normally make a master UUCP: assign

as a reference point to other assigns.

Be sure that UUSPOOL:, UUMAIL:, UUNEWS: and UUPUB: are assigned

to your hard disk instead of ram.

You will most likely want to run DCron and Getty from your

startup-sequence.  Be sure that UUCP:C is assigned *before*

these programs are run so they pick up the appropriate path.

If Getty interferes with any terminal program you might use

you may be forced to BREAK it.	Remember to restart it to

enable incomming UUCP calls again.

DCron is an extremely useful utility to run, especially in

systems with hard disks.  The only thing you have to watch out

for is to be sure to BREAK it (kill it) before shutting off your

machine.  DCron writes to its log file and runs other programs

based on the date and you don't want to accidently turn off your

machine when DCron has just started up some other program that

may write to your hard disk.

In the context of UUCP, DCron is normally used to automatically

poll systems in the wee hours while you are asleep, to trim

log files automagically, and maybe disable the modem's speaker

while you are asleep (at least, that is what I use it for).

Most people with HDs running UUCP leave their Amiga's on 24 hours

a day.

Other System Directories

Merge the mountlist into your master mountlist and copy any

devices from l that you do not already have into your

master l directory.

merge or copy uucp:s/crontab to s:crontab and uucp:s/.edrc to

s:.edrc .

You can ignore files in uucp:system, they are standard system

files.

----	--  -	-   -	-   -	-  --	----

(3)     Other startup-sequence efficiency considerations

UUCP relies heavily on the Amiga's multitasking capabilities.

Most programs execute other programs.  Many programs in UUCP:C

may be made resident as shown by the pure bit being set.  You

will probably at least want to make C:RUN resident if you haven't

already.

Depending on available RAM you may want to make certain other

progams resident.  Here are some suggestions:

uucp:c/dme	    ; run by dmail to edit mail

uucp:c/uuxqt	    ; run by uucico to handle received files

uucp:c/rmail	    ; run by uuxqt to handle received mail

uucp:c/sendmail     ; run by dmail to handle outgoing mail

(note that uucp:c/sendmail and uucp:c/rmail are the same

executable just renamed)

STACK:	All programs should run with a stack of 8192.  This

has been tested for all programs except RNews and ANews which

may require more.

RNEWS:	RNews currently takes a huge amount of memory and

will run only on machines with at least 1.5MB of ram.  Even

with 1.5MB you may need to run rnews manually instead of having

it automatically run by uuxqt.	In this case rename the rnews

executable to something else and create a dummy script file

in its place (named as 'rnews') with the script bit set.

(4)     System requirements

Be sure you are running at least version 1.3 of the operating

system.  The main consideration is that you are using at least

the 1.3 serial device.

The NULL: device is required, this is in the l directory of

the boot floppy as already discussed.

The UUSER: device is not required unless you want to start

getting fancy with the Getty.  This is mainly a programmer's

tool.

(5)     Getty

Normally one runs a Getty in his startup-sequence on the internal

serial port (from which you wish to receive calls).  Currently

only one Getty may be run PER SERIAL DEVICE NAME, and using other

units of a given serial device will interfere with Getty's operation.

This is due to the difficulty of implementation in Getty and will

eventually be fixed (to fix the problem I have to patch into

the OpenDevice and CloseDevice vectors).

So for now you can really only run Getty on the internal serial

device.

Getty does its best to not interfere with comm programs and

the like, but if a comm program refuses to run because it

can't get exclusive access to the serial port you may have

to kill the Getty by BREAKing it from a CLI.

Refer to the manual page MAN:Getty for more information (for

floppy based systems all manual pages are on the second

distribution diskette).

(6)     SHELL-STARTUP

Assuming you are using the 1.3 Shell, the S:Shell-Startup file

normally contains at least a Path command and Stack command to

properly setup a newshell's path and stack.  Be sure the stack is

at least 8192 for this distribution of UUCP and add the UUCP:C (or

wherever you have placed the distribution executables) to your path

list.  Older UUCP distributions required a huge stack, this has

been fixed.

(7)                   SITE-SETUP CONTROL FILES

The following is a description of control and log files required

for UUCP and how you must modify them to install your system.

(GETTY: is normally Assign'd to UULIB:)

GETTY:Getty-Header

This file contains the header line that is printed before a

login: prompt and is typically only two or three lines long.

The header usually contains your node name and the version of

the OS you are running under so people calling from term

programs know who you are.

(Getty can be used to run things other than uucico)

Modify accordingly.

GETTY:Passwd

The password file is used by Getty to authenticate login

requests.  In the context of UUCP, remote UUCICOs that call

your machine up will provide a login and password.	Getty

handles this and if the login is valid and the password matches

will run the appropriate program found in the password file.

Getty also deals with such features as automatic disconnect on

timeout and logs times for all incomming and outgoing calls.

In a serious system you should first remove all the junk

entries that come with the distribution.   You need an entry

for each remote machine you want to be able to call you.  You

do not necessarily need to give them logins that corrospond to

their machine names though this is how it is usually done.

Other programs in the future will be able to be run from

a Getty.

Later versions of UUCP will implement MAILER-DAEMON type

stuff like 'unknown user', in which case the password file

will be used to define which user names are valid on the

system.

GETTY:LOGFILE

The logfile is appended to by Getty whenever an incomming

call occurs or somebody takes over the serial port.  It

logs connect and disconnect times for incomming calls and

usage for external programs.

UULIB:Domain

In nearly every case you will not need this file.

Those of you who wish to do domain routing through

their machine or have more than one default for

unknown destinations should refer to the manual

page MAN:Domains.

UULIB:.Signature

.Signature is used by the Mail program and normally appended

to outgoing mail.

Modify accordingly.

UULIB:Seq

The Seq file should exist but otherwise not be modified by

the user (except for setting it to a "1" once a year).  This

file is atomically updated and used to provide unique filenames

and mail IDs for outgoing mail.

UULIB:Security

This file contains a list of allowed directories remote UUCP

transfers may read and write to.  Refer to the Security manual

page.  This file never contains the UUSPOOL: directory which

is implicitly read-write.

You need not modify this file now but should keep it in mind.

It protects your system in as much as I have been able to

close the loopholes.  Loopholes might still exist.

UULIB:news.distribution

This file contains a list of news distribution idents for

various areas and must normally be modified according to

where you are.. you need to be a UUCP guru to do this.  Most

people can just leave it alone.

UULIB:Config

This file contains configuration information for many

executables in UUCP:C and should be modified accordingly.

Be sure you modify at least the following entries.	REFER TO

THE 'Assigns' MANUAL PAGE.

NodeName

UserName

RealName

NewsFeed    (not required if you don't care about news)

Organization

TimeZone

DefaultNode

**** Take some time to pick a node name for yourself.  It

should be no more than 8 characters long.  Some UUCP sites

barf at node names longer than 6 chars so if you are not

sure of your neighbors pick one no more than 6 characters

long.	Your site name should be in lower case.

UULIB:L.Sys

This file contains site information for sites that may call

you up or you may call up.

You must modify this file appropriately before you can run UUCP,

refer to the 'L.Sys' manual page.

UULIB:Aliases

This is a mail aliases file with aproximately the same

power as /usr/lib/aliases on UNIX systems.	You should

refer to the 'Aliases' manual page and modify this file as

appropriate for various defaults like 'postmaster'... to

whatever user name you pick for yourself.

UUCP:S/Crontab

This is a functional Crontab file for use with the DCron program.

This file is normally moved to S: (it is useless in UUCP:S).

UUSPOOL:LOGFILE

UUCICO and other programs log errors and other things to

this file.	Normally you use DCron to clean up this and

other log files once daily to keep them from getting to large.

(8)     TESTING / DMAIL

You will next want to test your UUCP configuration.  If testing

with a friend's Amiga running the same distribution simply

email to him, then one of you run uucico -s<remotesystem>

just like in QuickSetup.DOC, except this time you are using

the system name he picked instead of 'test1' or 'test2'.

If the initial tests in QuickSetup.DOC worked and the system

is not working now check the following (common problems):

(a) the login and password in your L.Sys file (that uucico uses

to login to the remote machine) matches the login and

password in the remote machine's Getty:Passwd file.

(b) There is an entry for your node in the remote machine's

L.Sys file and vise versa

(c) The telephone number is correct

(d) the node names are all in lower case

(e) you constructed the email address correctly (nodename!user)

I suggest you get UUCP running with a friend before getting it

running with a UNIX system as it is easier to test it with a

friend.

Assuming you email to the user name he picked for his UULIB:Config

'UserName' entry and he did the same thing to you, entering

'dmail' without any arguments will bring up the mail shell

and allow you to read the message after transfer via UUCico

is complete.  You can check that the email actually arrived

into the right mail box in UUMAIL: by simply getting a directory

of UUMAIL:

READ the manual page for DMail for information on the

construction of mail paths to other systems and users.