This text contains some background technical info.  It is not a manual,

but it fills in some data regarding the problems with disk control and

how things are implemented. (And I love to talk about it after all this

work)

INTRODUCTION

The rotational speed of an Amiga disk is 300 rpm.  The time of one

revolution is .2 seconds.  Data is sampled at .5 MHz.  So a track on an

Amiga floppy disk contains 100000 bits.  This can vary a little due to

motor speed variations.  Alas we cannot put any desired pattern in these

bits.  Therefore data is converted to patterns which are allowed.  This

conversion, called MFM-encoding, has an efficiency of 50 percent.  For

every bit of data two bits of the magnetic track are used.  We can

picture an encoded sector as follows:

4 bytes inter sector gap

4 bytes sync pattern

8 bytes sector ID

32 bytes label

8 bytes checksum over sector ID and label

8 bytes checksum over data block

1024 bytes data block

Therefore the size of a sector is 1088*8=8704 bits. 100000/8704=11,

remainder 4256. Thus we can store 11 sectors.  We have 4256 bits left we

donot use for storage.  Instead these bits allow us to accept a motor

speed increase of 4.25 %. If we allow the same speed decrease, we must

be able to supply a non used space of roughly 8500 bits or 1100 bytes.

In fact, Trackdisk is prepared for an unused space of 1660 bytes. (Why

that much?)

Trackdisk does not write separate sectors as found on other systems.

Instead it writes out a complete track, actually reformatting the track.

That is why blocks can be so tight together. (On the other systems you

need gaps between the sectors and between sector ID and data block to

allow soft and hardware to synchronize) If Trackdisk writes out a track,

it begins with the 1660 bytes gap, followed by the sectors.  Although

the sectors are written out in order, the first sector can be any of the

eleven.  The last sector is written over the begin of the gap, thus

effectively removing old data and patterns.  Although Trackdisk allows

for synchronisation with the disk index, this is normaly not used;

Trackdisk just starts writing when it is ready to do so.

Reading is something else.  If Trackdisk should wait for the gap to

pass, average halve a tracktime is done nothing.  Therefore reading is

started, ignoring where the head is in the track.  To ensure that all

sectors are read, the readsize must be one sector larger than the number

of available sectors.  And the gap must be read too.  This results in a

readsize of 12*1088+1660=14716 bytes or 117728 bits, 18% more than a

physical track.  Usually the read is started somewhere within a sector,

continues until the gap, the gap itself and enough sectors after the gap

to cover the whole track.  Reading and writing is done by DMA (Direct

Memory Access) and word wide (16 bits). Therefore data read from disk

can be in memory shifted up to 15 bits.  The part behind the gap has no

exact relation in position and shift to the first read part.  To recover

the track, a search is started for a sync pattern.  If found, this

informs us about the shift.  Knowing the shift, the following sector id

can be decoded.  This sector id has the following form and are all

bytes:

$ff (formatbyte)

tracknumber

sectornumber

number of sectors until gap

The last item "number of sectors until gap" tells us how large the part

is with the current shift.  This part is shifted and moved to the begin

of the data part of the trackbuffer.  Then a second search for a

syncword is done, starting behind the the first part.  The remaining

sectors are found and moved and shifted tight behind the first part.

Now there is in the buffer a block of 11 sectors and after updating the

last items of the sector id's, "number of sectors until gap", the track

is ready to be written again.

But before that a lot can go wrong.  If some syncword or intersector gap

is distorted, or one of the 22 checksums is wrong, Trackdisk stops and

returns an error.  If the error concerns a syncword or a gap, or if a

faulting checksum belongs to a sector ID, it is likely that all data can

be recovered (exept for the label, but that is not used by Dos). And if

the checksum belongs to a data block, but this data block is not

requested, then the other blocks could be read without a problem.  Of

course a track with an error must not be written back to disk.

SALVE FUNCTION

The salve function of TrackSalve reads data from disk in such a large

buffer, that there will be an uninterrupted array of sectors in this

buffer.  If one sector can be found with a good checksum over its

header, all data to calculate the position of the other sectors is

available.  The data and label blocks are copied to the original buffer,

this time with sector 0 in the first position.  If the checksum of the

current block is correct, a corresponding bit is cleared in a word that

is initiated with 11 ones.  Such a word exists as well for the data as

the label blocks.  There is a byte that contains normally the number of

the first sector, but it will contain an error number if the track is

bad.  The Trackdisk errors start at 20, so if the byte is larger than

10, the track is bad.  Somewhere within trackdisk this is tested and as

result the tracknumber is set to -1, meaning "trackbuffer not valid".

Therefore any subsequent read request will result in a seek and several

read retries of the track.  If the salve function is active, this

invalidation of the tracknumber is prevented.  Instead the first sector

byte is tested at a read request and if this indicates an error, the

bitpattern with bad blocks are tested.  As soon as the request concerns

a bad block, the error value and the number of transferred bytes is set

and fixed, but transfers continues until the number of requested bytes

are copied.  There is no difference between the originial behaviour of

trackdisk and the salve function other than the bonus of the extra

transferred data (IO_ERROR contains the error and IO_ACTUAL contains the

number of bytes without error). If label transfers is switched on,

IO_ACTUAL presents the number of sectorbytes transferred until an error

is encountered in a label OR sector.  Some specialised recovery program

can use this info to recover more data at the cost of accuracy, which

may be acceptable in some cases.  A write to the same track still will

set this tracknumber to -1, and the normal retry scheme will be used.

The salve function is rather introvert and will at a priority of +5

disturb the feeling of the Amiga.  So the priority is lowered to -2

during track salvage.

MFM FLAW

How heavy the tests are on the track read from disk, there is a serious

gap (to my opinion) in these itegrity checks.  As stated earlier, not

every pattern is allowed in the magnetic track.  A MFM encoded data

block exist of true data bits and timing bits.  The databits are always

valid:  these are not coded, but the timing bits, these are to be found

between the data bits, are meaningless as data.  They are just there to

prevent too much and to less subsequent magnetic transitions.  Too much

is beyond the bandwith of the medium and too less may lead to loss of

synchronisation and gives noise a chance.  The used recording technique

is a transition on a one and no transition on a zero.  MFM coding

defines patterns where a one always is followed by a zero, and there are

never more than three subsequent zeros.  What happens if there are two

subsequent ones?  Or more than three zeros?  The system is not that

sensitive that this immediately will lead to bad reads of the data

containing bits.  And Trackdisk maintains checksums over encoded data,

or..? But ONLY over the data bits, NOT over the timing bits!  If a

timing bit is read wrong from disk, this will not be detected.  The

sector is copied to a new position in the trackbuffer, and if an other

sector is changed, it will be written back to disk with the faulting

timing bit!  If the sector is not updated (eg belonging to a program)

this faulting bit wil stay forever.  Slowly all timing bits can

disappear due to the retry scheme of trackdisk.  At a moment the track

will be unreadable.  Especially the roottrack is sensitive for this

phenomen.

TrackSalve has modied Trackdisk in such way that just before a track is

written to disk, all timing bits are set according to the MFM standard.

This takes very little time because it is all done by the blitter.  Now

that it is ensured that the track is MFM updated before it goes to disk,

the encoding routines can ignore these bits, and that saves cpu time.

Because the encoding routines were rewritten, it was easy to allow non

chip memory for IO buffers.  Non-chip buffers are copied by the cpu at

the most efficient way (cribbed from exec's CopyMem()), although

CopyMem() is used if it is replaced.

BUGS IN TRACKDISK

I knew of two bugs and found a third.  These all are repaired in

TrackSalve.

Raw read and write had problems because IO_LENGTH was compared with

$8000 instead of #$8000. The difference is that IO_LENGTH should be

compared with the value 8000 hex.  Instead it was compared with some

value that was at memory location 8000 hex.

In a multitasking environt the hardware must be shared between the

concurrent tasks.  There can be up to four drives connected to the

Amiga.  For each drive a Trackdisk task is started (all executing the

same code but with separate data area's). These four drives are

controlled by one piece of hardware (some custom chip I forgot the name

but I know its address). The TD-tasks use it to control their particlar

drives.  The tasks ask permission to the disk.resource to access this

chip directly by a call named GetUnit(). If the task is ready with it,

this is told to the disk.resource by a call named GiveUnit(). Another

TD-task that perhaps was waiting for it, gets access permission.  There

is a nasty bug in Trackdisk which could disturb the permission scheme.

Because a branch was too far, a call to GetUnit() was missed if that

drive was empty.  The task asumed permission, did some access and called

GiveUnit(). This is not as bad as it looks, because the time of violated

hardware access is very short, and was very short after a permitted

access.  But if another TD-task had got permission (and the more drives

the more likely this will happen), the turnips are coocked.  The other

tasks commands to the hardware are possibly destroyed and to make

matters worse, it looses its permission to access by the first tasks

call to GiveUnit(). So now this ignorent task is violating the rules.

The patch done by the setpatch program modifies disk.resource in such

way that only GiveUnits() are valid from tasks that did the GetUnit().

The access violation was not repaired.

During the tests with special Trackdisk IO programs, some strange

behaviour of Trackdisk came up.  Dos is very precise in its IO with

Trackdisk, but a Trackdisk tester is not.  That must explain why this

was not disturbing more general.  If a write to a protected disk was

done, this disk became unreadable for all other tracks than that was

just written to.  Reads via Dos resulted in a Read/Write error requester

although retry selection solved the problem (CMD_CLEAR). A new bug in

Trackdisk?  Yes!  If you write a sector to a protected disk, first the

track is read into the trackbuffer, followed by a copy of the sector

into this buffer, replacing the original, and a flag is set, meaning

"trackbuffer changed". If you want to read a sector from another track,

trackdisk tries to flush (write to disk) its trackbuffer.  The disk is

protected and the attempt will fail.  So you cannot read another track!

But it is even worse if you want to read from the track currently in the

buffer.  You think the data is coming from disk, but it might be the

never updated sector!  What is the problem?  Trackdisk does not check

the protect status before letting a CMD_WRITE modify the trackbuffer.

AUTO UPDATE FUNCTION

There were times when I used Sectorama (I still use this nice program

from the hand of David Joiner), I did not understand why sometimes a

modified and written sector would not stick on disk.  Now I know this is

because Trackdisk needs some help.  It must be told to flush its

buffers, although this is automaticly done if some data is requested

from another track.  Trackdisk also does not switch off its drive motor.

If not be told to switch it off, the motor runs forever.  Dos knows

about Td's lack of updating and does some special commands:  CMD_UPDATE

and TD_MOTOR.  Dos does this well within five seconds.  But programs

that directly access Td, may not be aware and leave the motor running

and the buffers unflushed.

There are a lot of timers in Trackdisk, so why does not Td do it by

it self?  I do not know, but if you have the function switched on,

Td flushes a modified trackbuffer and switch the motor off after

five seconds idling.

NOCLICK FUNCTION

The presence of a disk in a drive is reflected by a line called CHNG.

This line is asserted if there is no disk in the drive.  It stays that

way until a disk in the drive and the head is stepped (and the drive is

selected of course). I asume this is done this way to give the

opportunity to acknowledge an disk extraction.  If the line directly

reflects the disk presence state, an extraction can be missed.  Well, it

is a way, I know some other, and I know too that the sound of an empty

drive generally is experienced to be annoying.  The step direction is

each time reversed, so the head will stay about the same position.  This

reverse is done by a CPU instruction "bchg", bitchange.  Therefore is

was very simple to implement the noclick function, just change the

instruction into "bset", bit set.  Then the head steps to the outside of

the disk and once arrived, it stops because it is blocked there.  This

can be by a pillow or by an electrical switch, the latter effectively

preventing stepping.  When Kickstart got in rom, this was not that easy

anymore.  TrackSalve executes Trackdisk in ram, and the patch is simple

again.  But should you have noclick on every drive?  Some drives still

make noise, this time not as regular as well in time as in sound.  Even

more disturbing!  Apart from the fact that I believe that these drives

are not suitable for this method.

VERIFY FUNCTION

A drawback of this whole track read/write system is that a simple verify

is not possible.  It is not enough to verify the just written sector,

because ten more sectors are written to disk.  These sectors have

probably no connection with the writing program, and a more general

warning must be made.  It is the particular Trackdisk task that does a

request and the user must reply on it.  The AutoRequest() has been

proven unreliable in low memory situations.  It returns "Cancel" if the

function could not be carried out, where it should return "Retry". The

same is true for an Alert(). Therefore TrackSalve build its own

requester.

My mouse is often hard to find, and if found, I have to search for a

surface for it (the screen would be nice but it's too steep). So I like

keyboard shortcuts.  This is well implemented in the system requester.

If you build your own requester, suddenly all input is blocked, and I

found no way (other than an special inputhandler, which is beyond I am

willing to implement in a patch) to receive shortcuts.  What is a

requester other than a window with some gadgets?  Then I can receive

everything I like.  But this time Dmouse does not recognize the

requester as a requester.  Sigh.    It doesn't get any easier with

Intuition.

Do do the verify, the track is read back into another buffer and then

compared with the original.  This is broken in two parts, because the

data in the verify buffer probably begins with another sector.  The

comparision is done by the blitter, xor-ing both buffers.

The motor is switched off before the requester is displayed.  The

writing program however is not notified and may not switch off a motor

of a source.  The standard diskcopy is a good example.  If you use

diskcopy without its verify function, but with TrackSalve verifying

(which is faster), and a verify error happens, the destination drive

will stop turning.  Diskcopy does not know and does not switch off its

source drive.  This is a good reason to enable the auto update function

in TrackSalve, which will stop the source's motor.

BUGS?

The preceding release of TrackSalve (1.0) had a bug in it, although it

was extremely unlikely that the buggy code would be executed.  This

unlikelyness was the reason that the bug slipped through the the

debugging phase.  It brought me to an improved debugging administration

and every single branch is executed under debug control, how trivial a

branch might look.  Pieces of code that could behave unexpectable

because of special combinations of parameters, have been run under

simulators producing all possible combinations.  This time I would like

to qualify the debugging as "extreme". Again:  This program is extremely

debugged!

CONCLUSION

With this patch Trackdisk becomes more efficient in its CPU use,

although slightly slower (well under 2%) in its writing, because of the

MFM update.  A lot features are added, which are fully transparant.

Trackdisk has come closer to what I think a floppy disk device driver

should be.  Better requireres a complete rewrite.  (Sorry Commodore,

but disk drivers are my favorite pastime, yours too?).

D.W.Reisig

Woudweeren 10

1151 AV  Broek in Waterland

Holland