PROBLEMS.DOC

Matthew Dillon

891 Regal Rd.

Berkeley, Ca. 94708

USA

uunet.uu.net!overload!dillon

CED

Those of you using the CED editor need to use the sticky option.

Unfortunately, CED requires that STICKY be the last argument so

what you really need to do is set the editor command in UULIB:Config

to execute a script file that runs CED with the filename and STICKY

option.

MODEM AND GETTY

There are several possible sources of problems in setting up

UUCP.  The major problem areas in order of likelyhood are

listed below:

(1) The modem is expecting a protocol that Getty has not

been told to use (XON/XOFF, 7WIRE, none)

If the modem uses 7-wire use the -7 option for Getty, else it is

assumed the modem uses no protocol.  Normally you specify -7 but

the modem will probably work just fine whether you specify it or

not.

(2) The modem does not hangup when DTR is dropped

All modern modems hangup when DTR is dropped.  Usually a dip switch

enables the option.  If it is impossible to drop a connection by

dropping DTR you can use the -d0 option to make Getty use the +++

sequence.  This only works if you do not specify a dumb modem in

the -M option.

(3) The modem does not understand simple AT commands or does

not generate a CONNECT message.

If the modem does understand dropping DTR but does not understand

simple AT commands or generate a CONNECT message use -c0 -Md (no

connect msg and dumb modem). In this case the baud rate must be

known and specified with one or more -B options.  When no CONNECT

message is available Getty does not know what the connect baud rate

actually is.  It will begin by trying the first -B option and

switch to the next whenever a line-break is received.

(4) Baud rate problems

The default modem type when no -M option is specified for Getty is

a hays modem.  If you have a multimodem you can use -Mm and the

baud adjust options for Getty.	Normally you specify a -B option

for each baud rate the modem is capable of connecting at.  If the

modem generates a CONNECT message you need only specify one -B

option that is used to reset the modem and Getty will automatically

handle the CONNECT messages.

If your modem always talks to your Amiga at a given baud rate no

matter what the CONNECT message says then use a single -B option to

specify that baud rate then the -A option which tells getty to

ignore any baud rate specified in the CONNECT message.

OUTGOING CALLS

Unfortunately Getty handles only incomming calls.  UUCico deals without

outgoing calls itself.  Currently you must specify the proper baud rate

in the L.Sys file for uucico to work properly. UUCico currently ignores

any connect message.

Disconnecting will work the same way Getty handles it.  If there is no

Getty running disconnecting works by dropping DTR.

NEWS

RNews supposedly takes less memory now.  It still takes quite a

hunk and you may run out, which will cause queue files to be

left in your spool directory and possible temporary files in

your news directory.

MAIL ADDRESSES

If you have problems queuing mail check your L.sys file.  Try emailing

to the adjacent node directly. E.G. if you can connect to 'foo' try

emailing a test message to 'foo!postmaster'.  If you do not wind

up with three queue files in UUSPOOL: then your L.sys file

probably has no entry for foo.  Whenever you email to somebody three

files should end up being added to UUSPOOL: and whenever you UUCico

to a given machine the spool files in question should be deleted as

they are sent.  When UUCico receives mail from the remote machine it

will download the remote queue files to UUSPOOL: then uuxqt them.

That is, the remote queue files should will be placed temporarily in

UUSPOOL:, unpacked with rmail and placed in UUMAIL:, then deleted.

The most common of all problems is an address like this:

a!b!c!d!e!user%g

In UUCP land this means:	a->b->c->d->e->g->user

In ARPA land this means:	g->a->b->c->d->user

To get around the problem you must determine the proper sequence

and convert the address to SOLELY bang-paths (!).