SetRamsey V1.02  by Nic Wilson

------------------------------

This program is freely distributable with no conditions

except that the program and this text file must remain

intact without any modification.

This program was written while I was testing the reason

why STATIC COLUMN MODE was not being set by the computer

as stated by the docs.  This program allows you to test

the current settings of RAMSEY and change them if you wish.

If you try to set Static Column Mode or Burst Mode on, then

a test of your FAST RAM is done to ensure that it is infact

Static Column Mode (SCM) RAM.

REQUIREMENTS

------------

Amiga 3000, Kickstart V2.0 or V1.3

FUNCTION

--------

Allows manipulation of the RAMSEY ram controller modes in an

Amiga 3000.

USAGE

-----

SetRamsey STATICON, STATICOFF, BURSTOFF, BURSTON, WRAPON, WRAPOFF

REFRESHON n, REFRESHOFF

or	SetRamsey ?   Gives USAGE string

SetRamsey     Gives current settings of RAMSEY and USAGE string.

STATICON	-	Tests and enables Static Column Mode if RAM passes test

STATICOFF	-       Turns Static Column Mode off

BURSTON         -       Tests RAM and turns SCM & BURST ON if RAM passes test

BURSTOFF	-       Turns Burst mode off

WRAPON		-       Allows all 4 longwords of a burst to take place

WRAPOFF		-       Burst will only continue A3,A2 are increasing

Burst will not be allowed to WRAP A3,A2=00

REFRESHON n     -       Sets refresh to value specified by n

n must equal 154, 238, 380

Defaults to 238 if n is not given.

REFRESHOFF      -	Turns refresh off

DO NOT USE UNLESS YOU HAVE A SPECIFIC REASON.

OTHERWISE IT WILL CAUSE A LOCK OR GURU.

TECHNICAL INFO

--------------

STATIC COLUMN MODE

This mode requires static column mode DRAMS.  When ram is first accessed

RAS is held low after the cycle completes.  This leaves the current RAM

page 'open' and the chip will act like static ram on this page.  Any data

in this page can be accessed by changing the column address only.  Since

the column address access time (tAA) is much less than the RAS access

time (tRAC), subsequent accesses to this page of data can be done faster.

As long as RAS is held low (10 usecs max) RAMSEY will allow the CPU to

access RAM on this page in only 3 clocks (16 & 25 MHZ).

BURST MODE

In this mode, RAMSEY will respond to *CBREQ (cache burst request) input

from the 68030 and allow burst accesses to FAST RAM.  Each burst cycle

takes 2 clocks each (up to 3.  Burst mode requires the use of static

column mode DRAMS.

WRAP MODE

RAMSEY allows for additional control over burst cycles via the WRAP option.

The 68030 will always request 4 longword (4 x 32 bit ) values during a

burst sequence.  However if the initial longword is not aligned on a quad

long word boundary (A3,A2 not equal to 0,0), the 68030 will read data which

is behind the first data it asked for.  Since it is less likely that this

data will actually be needed, RAMSEY can stop the burst sequence after the

data with A3,A2 = 1,1 is accessed.

All enquiries to

Nic Wilson Software

138d South Street

TOOWOOMBA QLD 4350

Phone (076) 358522  Work hours or FAX