PROGRAM

MMUSR

FUNCTION

To display various information on a supplied address,

according to the current 68040 MMU setup.  Information

regarding the translated address, write protect status,

whether the translation is being performed by the transparent

translation registers or by the ATC tables, Cache modes for

the address etc.

SOURCE       Written in 100% assembler using Macro68

AUTHOR       Nic Wilson

Nic Wilson Software

138d South Street

TOOWOOMBA QUEENSLAND 4350

Australia

EMAIL cbmvax!cbmaus!wilson!nic

or  cbmehq!cbmaus!wilson!nic

TELEPHONE +61 76 358539 After Hours Australian Time

+61 76 358384 Work  Hours Australian Time

FAX       Same as Work Telephone

COPYRIGHT    This program, source code & document files are Copyright

Nic Wilson Software 1991.

DISTRIBUTION As per the Set040.doc file, as this program is only to

be distributed as part of the Set040 program archive.

REQUIREMENTS

68040 CPU

OTHER INFO

This program was written and tested on an Amiga

A3000 with a Progressive Peripherals & Software

68040 card.

USAGE        MMUSR <-switch> <address>

<-switch> = -i for instruction space

-d for data space

This switch will supply information that matches on

the required data or instruction space.

<address> = hexadecimal, decimal or binary address

if hex it must be preceded with a '$' EG. $F80000

if dec it requires no prefix          EG. 16252928

if bin it must be preceded with a '%' EG. %111110000000000000000000

IMPROVEMENTS/BUGS

All suggestions will be carefully considered.

All bugs will be fixed A.S.A.P.

NOTES

I wrote this program to test the MMU set up I was doing

for my Set040 program.  It would be handy for anyone

who is working with the MMU on a 68040 processor.

REFERENCES

MC68040UM/AD Motorola 32-bit Processors Manual

Published by Motorola Literature Distribution

P.O. Box 20912; Phoenix Arizona 85036