** communication protocol:                                               **

** pioneer: Computer activates CTS (pin5) of rs-232 port,                **

** (amiga 7-line handshake)                                              **

** then sends a command sequence and expects 'R' and carriage return(CR) **

** back. ASCII digits used for addresses,etc. Returns frame# as ASCII    **

** digits. Player is string oriented and reverse-polish (arg then verb). **

**                                                                       **

COMMANDS        CHARS    HEX     DECIMAL              COMMENT

----------------------------------------------------------------------------

Door Open       OP                                  open the door

Reject          RJ                                  stop disc rotation

Start           SA                                  start disc rotation

Play            PL                                  (address)PL

Pause           PA

Still           ST                                  still frame

Step Forward    SF

Step Reverse    SR

Scan Forward    NF

Scan Reverse    NR

MultiSpeed FWD  MF                                  (address)MF

MultiSpeed REV  MR                                  (address)MR

Speed           SP                                  integer SP

Search          SE                                  address SE

Stop Marker     SM                                  address SM

Frame           FR                                  set frame mode

Time            TM                                  set time code mode

Chapter         CH                                  set chapter mode

Audio Control   AD                                  integer AD  0=off,1=Ch1,2=Ch2,3=stereo

Video Control   VD                                  integer VD  0=off,1=on

Display Control DS                                  integer DS  0=off,1=on

Clear           CL                                  clear entry or mode

Frame #         ?F                                  get frame number

Time code #     ?T                                  get time code number

Chapter #       ?C                                  get chapter number

Player active?  ?P                                  P00=door open,P01=park,P05=still

Disc status     ?D                                  5 bytes and CR returned

(and a few others....)

SERIAL PORT CONNECTION:

computer   player

--------   ------

TxD 2      3 RxD

RxD 3      2 TxD

CTS 5      4 DTR

GND 7      1 GND

------------------

For more info on your type of Pioneer player:

Pioneer Communications of America, Inc.

Engineering and Technical Support Dept.

Sherbrooke Office Center

600 East Crescent Avenue

Upper Saddle River, NJ 07458-1827