#[33mAVT Modes#[31m

The star set of AFSK image transfer modes in this system are, naturally

enough, the AVT modes.

These are a set of low through high resolution image modes that currently

absolutely define the state of the art in SSTV. These modes offer many

advantadges, summarized against the older SSTV modes as follows:

Ratings: X/No, or: Great->Good->Fair->Poor->Bad->Rotten->Dead

#[33mAVT#[31m  - Std - #[33mRobot#[31m - FrmSeq - #[33mFAX#[31m - LinSeq - #[33mVolker_Wrasse#[31m

#[33mColor#[31m - B&W - #[33mColor#[31m - Color  - #[33mB&W#[31m - Color  - #[33mColor#[31m

#[33mFormats: #[31m

128x128 col:  #[33mX#[31m      No      #[33mX#[31m       X      #[33mNo#[31m     X        #[33mX#[31m

256x240 col:  #[33mX#[31m      No      #[33mX#[31m       X      #[33mNo#[31m     X        #[33mX#[31m

320x200 col:  #[33mX#[31m      No      #[33mNo#[31m      No     #[33mNo#[31m     No       #[33mNo#[31m

1024x1200 B&W:  #[33mNo#[31m     No      #[33mNo#[31m      No     #[33mX#[31m      No       #[33mNo#[31m

#[33mImage Stability#[31m

Thru Multipath: #[33mGreat#[31m   Bad    #[33mBad#[31m    Rotten  #[33mGood#[31m   Rotten   #[33mBad#[31m

Thru Splatter: #[33mGreat#[31m  Rotten #[33mRotten#[31m  Rotten  #[33mGood#[31m   Rotten  #[33mRotten#[31m

Thru QRN: #[33mGreat#[31m   Bad   #[33mRotten#[31m   Bad    #[33mGreat#[31m  Rotten   #[33mGood#[31m

Thru QRM: #[33mGreat#[31m  Rotten #[33mRotten#[31m  Rotten  #[33mGood#[31m   Rotten  #[33mRotten#[31m

Thru Hetrodynes: #[33mGreat#[31m   Dead   #[33mDead#[31m    Dead   #[33mGreat#[31m   Dead    #[33mDead#[31m

Thru Talk: #[33mGreat#[31m   Dead   #[33mDead#[31m    Dead   #[33mGreat#[31m   Dead    #[33mDead#[31m

Low signal: #[33mGreat#[31m   Poor   #[33mDead#[31m    Poor   #[33mGood#[31m    Dead    #[33mDead#[31m

#[33mImage Quality#[31m

Col Accuracy: #[33mGreat#[31m   No     #[33mFair#[31m    Great   #[33mNo#[31m     Good    #[33mGood#[31m

Col Resolution: #[33m256k#[31m    16     #[33m256k#[31m    4096    #[33m64#[31m     4096    #[33m4096#[31m

Pel Accuracy: #[33mGreat#[31m  Good    #[33mGood#[31m     Bad   #[33mGreat#[31m  Rotten   #[33mGood#[31m

AutoStart: #[33mGreat#[31m   No     #[33mPoor#[31m     No    #[33mGreat#[31m    No      #[33mNo#[31m

AutoMode: #[33mGreat#[31m   No     #[33mPoor#[31m     No    #[33mGreat#[31m    No      #[33mNo#[31m

The AVT modes were specifically designed to deal with the problems encountered

on the HF bands - high noise of all types, periods of fading that result in a

complete absence of signal, talking, splatter, and so on. Like facsimile,

AVT images are self-clocked; This means the computer system times the image

completely, rather than relying on "sync" signals which become damaged

under interfering signal conditions.

AVT images are sent in 2 phases. First, a #[33mdigital#[31m signal is sent

that is similar to radioteletype. This signal contains information about

the timing of the image, the resolution, and information that is crucial to

the reliable reconstruction of color - information that is lacking in

all other SSTV modes.

After the digital "header", the image information is transmitted. If you

listen to the signal, you will note a complete lack of "sync" pulses. In

fact, there is no 1200 Hz information in these images at all. The entire

transmission uses frequencies between 1500 and 2300 Hz, including the

digital header.

This can have an additional benefit, one that makes AVT mode images even

better. You can now apply 800 hz wide filtering to the incoming signal,

instead of the 1100 Hz (1200-2300) required for std SSTV, or the

1200 Hz (1100-2300) required for Robot mode SSTV. This is about a 25%

reduction in bandwidth, and results in a comparable reduction in interfering

signals.

Now, looking at the chart above, you might think, "Well, these guys say

AVT is so great... why should I need to filter the signal?"

The answer is fairly simple. Although AVT mode images will not lose color

tracking, pixel positioning, or line positioning information once started,

under ANY circumstances, they are subject to one problem - when interfering

signals become so strong that the incoming AVT video is covered up, in

that position in the image there will be noise. Likewise, when the signal

fades past usability. The hardware is fairly sensitive, so that signal

fading isn't as much of a problem as it is with other equipment, but it's

still impossible to copy an image when there is no audio coming thru.

So, if you reduce the passband width of the incoming RF (prefereably) or

audio, the types of signals that can interfere in this manner are

reduced considerably. Signals at the standard sync frequency will no

longer have an effect on the image. For those of you with pass band tuning

and IF shift controls, this is easily accomplished - simply adjust the

control until the low aduio tones in the receiver dissapear. Others may

wish to purchase optional filters for their rigs.

#[33mNarrow-band AVT Images:#[31m

As if all of this wasn't enough, There is a provision in the system for

narrow-bandwidth image transmission. This format reduces the signal from

it's nominal 1500 Hz - 2300 Hz swing to one of about 1700 Hz to 2100 Hz.

This allows you to use a 400 Hz width filter, again enhancing the ability

of the image to get thru under lousy conditions. The control for this mode

is in the "SET" window. When selected, Narrow bandwidth AVT is in effect.

Please note that because of the narrow bandwidth, some degradation of the

color accuracy may be experienced. Only 32 grey levels can be discriminated

by the hardware with this frequency swing, so the overall accuracy does

drop to some extent. However, since the Amiga's display hardware can

currently only show you 4096 colors, in general images should be quite

good. 4096 colors is equivalent to 16 grey levels per memory (red, green,

and blue, 16*16*16=4096) and the AVT hardware, even in narrow bandwidth

mode, is discriminating 32, for a total of 32k colors, 32*32*32=32768.

There is no question, however, that the 64 grey level range used in the

wide band AVT does provide better color accuracy. The best thing to do

is to use it and get that "feel" for the two modes. Then you will be

able to decide which mode to use, and when, by ear.

#[33mTuning AVT modes#[31m

Again, AVT images have significant advantadges over more primitive color

modes. If you are off frequency, AVT images do not lose their color

accuracy; instead, they become darker or lighter. This can be "fixed"

after the image has been received, by using the brighness and histogram

capabilities available in the RoboSynth window.

For an AVT mode to start, you should be tuned reasonably close to the

other signal - in general, any tuning position where the other station

sounds reasonably "normal" should be good enough. Please note however

that the narrow-bandwidth AVT signals, though much better at getting

thru under heavy QRM and so on, are more slightly finicky about being

properly tuned.

When tuning to another AVT signal, use the BLK control in the setttings

window to transmit a tone at exactly the black frequency. Have the other

station use the 'Scope to tune such that the signal causes the Scope to

trace across #[33mexactly#[31m on the black graticule. Once this has

been done, you may wish to do the same, if you have RIT (receive

incremental tuning) capability - you do NOT wish to retune your transceiver

if you do not, or your transmit frequency will change, rendering the

careful adjustment performed by the other staion useless.

These adjustments only need be performed when using SSB modes to

communicate. FM and PM modes will not require precise tuning, as the

received signal does not change frequency with tuning.

#[33mFailure to receive the header:#[31m

If the header is not recieved correctly, the system will not begin

reception of the image. In general, this should not be a problem. The

AVT system's sensitivity and discrimination as regards to the

reception of the digital header is extremely high. 16 bit checksum

information in the header ensures that if it is garbled, the system will

not accept the signal - yet, the system is so accurate on header

reception, you will find that it will be able to hear images that are

better than 95% noise, and start them correctly!

Although the AVT mode information IS in the header, this version of

the program does not use it - that means you must sleect the mode

manually, using the mode select buttons on the control panel.

Currently, the three AVT modes are: 24, 90, and 94 second images. These

are selected just to the right of the AVT button in the control panel.

Select the mode (speed) first, then press AVT, then press either the

RX or the TX button, as appropriate.

#[33mNote for owners of AmMigas with "fast" memory:#[31m

The 24 AVT mode will leave the control panel "up front" during reception

if you have not specified the "chip" option at program startup.

There is a small green indicator at about the top center of the control

panel - it is unlabeled, directly above the "H" and "L" RGB display

controls. During reception of the digital header portion of the AVT

image, this indicator turns on (becomes bright green) when (if!) the

system is able to successfully aquire the digital header. The information

isn't of any particular use to you, but does let you kind of relax,

knowing that the image is going to start when the header completes.