The AVT program offers a variety of startup options that may be utilized

either from the WorkBench or the CLI.

Before we get to these, we should point out one of the programs

contained in the tools drawer:

#[33mNextScreen#[31m

This is an EXTERMELY useful program to have running in your system while

using the AVT system - or any other software, for that matter. It changes

the function of the Left-Amiga-M key such that the NEXT screen is brought

to the rear, instead of the active screen. This is very important in the

AVT system, because there may be as many as 3 screens active just from

the AVT program itself... then there's the workbench, and what if you

have a graphics screen up? Trust us - you NEED this utility!

To use it, under 1.2, you'll also need the program 'runback', also in

the tools drawer. Put them both in your c: directory, and place this line

in your startup-sequence file:

#[33mrunback c:nextscreen#[31m

To use it under 1.3, just place the nextscreen program in your

c: directory, and put this line in your startup-sequence:

#[33mrun >nil: nextscreen#[31m

Now, on to the program options:

You can always get a list of these options when you are in the CLI mode

by typing:

#[33m

HAM ? <cr>

#[31m

Which produces this list:

#[33m

program options:  []=option  <>=required

ham [path <sstv:raw/path>] [ipath <SSTV:iff/path>] [fpath <FAX:iff/path>]

[mem <1-16>] [version] [4meg] [chip] [?] [trim <value>] [ypo <height>]

Option format for WorkBench tooltypes use:

PARMS=[chip]|[4meg]|[version]

PATH=path:to/save/sstv/files/in

IPATH=path:to/save/SSTV_HAM/iff/files/in

FPATH=path:to/save/FAX_hi-res/iff/files/in

TRIM=value  ( -255 -> +255 )

MEM=value  (    1 ->   16 )

YPO=value  (  200 ->  240 )

#[31m

The notation used for the CLI options  ( [], <> ) is defined as follows:

a term surrounded by the square brackets is an option, plain and simple.

You can use it or not, as you see fit according to your needs. The

[version] term is an example of this.

A term surrounded by the angle brackets MUST be included. In this version

of the program, only terms that go along with options are absolutely

required. For instance, the term [mem] is an option - but if you look at

the list of options printed by the program, you will see that it isn't

written that way - it's written: " [mem <1-16>] ", which means this:

You can type "mem" or not, as you choose. But if you DO choose to type

mem, you MUST supply a number from 1 to 16 to go along with it.

In the future, if there is a required parameter, it will be in the

option list as " <option> " without and square brackets surrounding

it.

Here are the details on the specific options available:

CLI:  #[33m[path <sstv:raw/path>]#[31m

WB:   #[33mPATH=sstv:raw/path#[31m

If you type:

ham path piclib <cr>

The program will look for "raw" sstv files in a directory called

"piclib" - and it will expect to find that directory in the

current directory. If you type:

ham path df1:piclib <cr>

It will expect to find the "piclib" directory on the DF1: drive.

CLI:  #[33m[ipath <SSTV:iff/path>]#[31m

WB:   #[33mIPATH=SSTV:iff/path#[31m

If you type:

ham ipath ifflib <cr>

The program will save "IFF" sstv files in a directory called

"ifflib" - and it will expect to find that directory in the

current directory. If you type:

ham path df1:ifflib <cr>

It will expect to find the "ifflib" directory on the DF1: drive.

CLI:  #[33m[fpath <FAX:iff/path>]#[31m

WB:   #[33mFPATH=FAX:iff/path#[31m

If you type:

ham ipath ifflib <cr>

The program will save "IFF" facsimile files in a directory called

"ifflib" - and it will expect to find that directory in the

current directory. If you type:

ham path df1:ifflib <cr>

It will expect to find the "ifflib" directory on the DF1: drive.

CLI:  #[33m[mem <1-16>]#[31m

WB:   #[33mMEM=1#[31m

If you type:

ham mem 2 <cr>

The program will attempt to allocate memory for TWO full function

SSTV image memories. If there is not enough memory, you will only

get one; likewise, if you typed:

ham mem 16 <cr>

and there was not enough memory for 16 image buffers, you would only

get as many as there was memory for. It takes 192k per image

buffer - so for all sixteen, you need just over 3 megabytes of memory

on top of what the program itself requires.

CLI: #[33m[version]#[31m

WB:  #[33mPARMS=version#[31m

This option acts differently in the CLI than it does in the WB

startup. In the CLI, if you include this in the command line, the

program will print the verison number and then immediately terminate.

If the version keyword is used in the ToolTypes in the ICON for

the WB startup, the program will print the version in the small

window it opens on the WB and then start normally.

Use this option when writing us in reference to any aspect of the

system's performance - we need it to understand the situation

fully.

CLI:  #[33m4meg#[31m

WB:   #[33mPARMS=4meg#[31m

This option is to be used ONLY if you have a 4 mhz crystal in the

AVT interface. You should NOT have such a crystal; production versions

of the AVT are all shipped with a 4.194304 Mhz crystal installed.

This option was required during development for technical reasons,

and remains for the same reasons. This is not for use by the

normal user - only for those in the Beta test program.

CLI:  #[33m[chip}#[31m

WB:   #[33mPARMS=chip#[31m

This option MUST be used by those who have Amiga 500's or 2000's

with only 1 megabyte of RAM. This option enables special screen

managment routines within the AVT software, without which many

of the SSTV and FAX modes will not function properly.

The specific problem that the "chip" option was developed to address

is that of DMA contention between the Amiga's screen display

hardware and the CPU in a machine that has no memory that is

devoted to the CPU only (this is called "fast" memory in Amiga

terms). During receive situations, when the program is invoked with

the chip option, A "ghost" screen that is a black and white screen

is brought to the front. This is not a HAM (Hold-and modify) screen,

but a standard Amiga color register-based screen. These special

screen manipulations ensure that the CPU has full access to the

system's memeory, and so has enough "horsepower" to handle the

data that is coming in at an extremely high rate of speed from

the receive interface.

CLI:  #[33m[trim <value>]#[31m

WB:   #[33mTRIM=value#[31m

This option is also a development option; it "tunes" the receive

interface parameters by writing data to the PALS on the AVT

interface board. This has the effect of shifting the grey levels

(or in the case of Robot color, the tint).

The default from either the CLI or the WB is 27, and this is also

in the ICON as supplied by us. If you experiment with this option

(Is there any doubt?) Remember to set it back to 27 when you are

done.

It's a very fine tune control - you won't see a lot of difference

per step. The available range is about -1000 to 1000.

CLI:  #[33m[ypo <height>]#[31m

WB:   #[33mYPO=height#[31m

This option controls the severity of the overscan screen; This is

also a development option. The default, from both CLI and WB,

is 240, which is a "severe" overscan screen in vertical terms;

You can set the screen all the way back to 200, though you will miss

much of the image on many hi-res SSTV images. We don't recommend

playing with this one, in general.

A final note on the WorkBench #[33mPARMS#[31m command:

You may have noticed that there are a number of terms you

can use with the PARMS= tooltype. You can use one or more of

these terms by typing a vertical bar (found at the upper right

of your keyboard) between them:

#[33mPARMS=version|chip|4meg#[31m

Take a look in the supplied ICON is you'd like to see an example

of this.