Programs NISTIMEW and NISTIME-32bit

These programs will connect to one of the time servers operated by the

National Institute of Standards and Technology (NIST), and will compare the

time of the clock in your computer with the atomic time-scale operated by

NIST.  The program will display the time difference and will offer you the

option of adjusting your clock by the displayed amount.

Both of these programs are designed to run in a Windows environment.

Program NISTIMEW is a 16-bit version that is suitable for Windows 3.1 or

Windows 95/98 (see additional comments below).  Program NISTIME-32bit is a

32-bit version of the same program that is primarily intended for Windows NT

or Windows 95/98/2000/Me.  All of the versions are likely to run in any

version of Windows, but there may be some problems if the 32-bit version is

run in Windows 3.1 (even if Win32S is installed) and the 16-bit version cannot

set the clock on a Windows NT system (even if it is run with system

privileges). In addition, the 16-bit version may not run in some versions

of Win 9x because the network interface supports only 32-bit applications.

We would recommend using the 16-bit version in Windows 3.1 and some early

versions of Win 9x. It is smaller than the 32-bit version, and this advantage

is likely to be most important on smaller systems.  It offers most of the

the same capabilities as the 32-bit program. Change to the 32-bit program

if you get network errors when using the 16-bit program in a Win 9x environment.

(The errors may be "protocol not supported" or "invalid parameter in

network call"). Use the 32-bit program for most later versions of Win 98,

and for Win 2000 and Me.

The programs are compiled with a default list of server names and ip

addresses.  You may update this list at any time using the menu option

File | Update server list

This will copy the file nist-srv.lst from the NIST archive to your system.

The file will be stored in the system directory (usually c:\windows or

c:\winnt). This file contains a list of all of the time servers operated

by NIST.  It is a simple text file which you may examine or print using

the standard utilities. The program checks for the existence of this

file each time it starts, and uses the contents to set the server menu

options. You may edit this file provided you preserve its format; the

program will probably malfunction if you change the format of the

entries. See the comments in the file for more information.

You can also use this file to support communicating with our systems

through a proxy server. See README.PROXY in this directory for more

information about doing this.

1.  NISTIMEW

You can configure the program to run in any time zone and with or

without the adjustment for daylight saving time enabled.  These parameters

will be set automatically from your user environment as defined by the

control-panel settings, and you should check to be sure that these are

correct before starting the program. In Windows 95/98, for example, you can

examine or change these settings using

Start | Settings | Control Panel | Date/Time | Time Zone

They can also be configured from inside the program using the menus; changing

either the time zone or the daylight saving time flag using the menus will

override the values defined by the system environment, but this is not

recommended under normal circumstances -- the change is only local to the

program and will not be propagated to the rest of the system.

The program is designed to run on any configuration that supports

Windows (v3.1 or later) and using a network interface that is compatible with

the Winsock standard (v1.1 or later).  It will probably run in any 32-bit

environment but will be unable to set the clock on a Windows NT system.

However, it is not compatible with the network interface supplied with

most versions of Windows after Win 98.

If you are using MSDOS or Windows 3.1, and if you have software that runs

in this environment and which depends on the definition of the local time zone,

then it may be useful to define the environment variable TZ if you have

not already done so.  This will provide a system-wide definition of the time-

zone and whether or not your system should be adjusted for daylight saving

time if necessary.  The syntax of the definition statement is:

SET TZ= XXXyZZZ

where:

XXX is the 3-character name of your time zone: EST for Eastern

Standard Time, etc.

y is the offset of your standard time from UTC (Greenwich Time) in

hours, measured West from Greenwich. This value can have a negative

sign if necessary but will be positive for all US locations.

ZZZ is the 3-character name of your daylight time if you observe

daylight saving time; otherwise it is blank.

Thus:

SET TZ = EST5EDT		Eastern Time, daylight saving on

SET TZ = CST6			Central time, daylight saving off

SET TZ = MST7MDT		Mountain time, daylight saving on

etc.

You can add this statement to your AUTOEXEC.BAT file so that TZ is defined

automatically when your system starts. This procedure is not necessary

for Windows 95 or Windows NT since these system provide the same adjustment

from the control panel.  They are also optional for Windows 3.1 -- the

same functionality can be achieved using the menu selections inside of

the program.

Note that turning daylight saving time on simply enables the correction

for the Summer months -- it is never applied during the Winter months no

matter how you set this switch.  The boundaries between Summer and Winter are

based on the US transitions dates of the first Sunday in April and the last

one in October.

2.  NISTIME-32bit

This program reads both the time zone offset and the daylight saving time

flag from the system, and these choices cannot be changed by the program.

Apart from this difference, the two versions are almost the same from the

user point of view.

Since this version gets the time zone offset and the daylight saving

time flag from the system, it is important that these parameters be set

correctly.  You can set or check these settings using

Start | Settings | Control Panel | Date/Time | Time Zone

and you should be sure that these settings are correct before you run

the program.

Although this program will probably run in a 16-bit Windows environment

if win32s is installed, it may not work properly in that environment.

Among other difficulties, the program may not be able to determine the local

time zone offset and may do erratic things as a result.

Using either version, the overall uncertainty in the time comparison

should be no worse than +/- 0.5 seconds under all circumstances, and will be

substantially less than this value in most cases.

Installation of either version

The program files are named nistimew.exe and nistime-32bit.exe.  These

are the only files that are needed initially.  Either one can be copied to

any directory on your system.

In Windows 3.1 You can run either version by selecting the Program Manager

command "Run ..." (on the File menu) and entering the command line:

<path>\<program name>

where <path> is the name of the directory where you have placed the .exe file.

If you have copied nistimew.exe to c:\time, for example, you would enter:

c:\time\nistimew

You can also install an icon for this program using the "New" option on the

program manager File menu.  When you select this option, complete the command

line box with the path to the program file and its name.  Using the example

above, you would fill in this box with the string:

c:\time\nistimew

The other boxes are optional; you may use them to add a text string to the

icon, etc.  Once you have installed the program in this way, you can run it by

double-clicking on its icon in the usual way.

In Windows 95 or Windows NT,  you can run the program using the command

Start | Run | <path>\<program name>

where <path> and <program name> are as above. You can also run the program

by double-clicking on its name in Windows explorer or by creating a short-

cut to it on your desk-top.  To create the short-cut, hold down the left

mouse button and drag the program from the Windows explorer display to

your desktop.

If you add the word "Once" to the command line of either program (without the

quotes and using any mixture of upper- and lower-case letters), then the

program will query the NIST server once each time it is started, set your

clock (if necessary), and exit after the clock has been set.  For example,

if the program is located in directory c:\time, you could use the command:

Start | Run | c:\time\nistimew once

If you have defined a desk-top short-cut to the program, you could also

add once to the short-cut. To do this, right-click on the short-cut, open

Properties | Short cut, add the word once to the target line and click OK.

This change affects the short-cut only; it will not have any effect if

the program is started in some other way.

Note that the "once" option is never added using the periodic query

dialog of the program. That dialog only accepts a numerical value that sets

the interval between multiple calibration cycles.

Each time the program is started, it will use the configuration you have

saved (if this file exists) or the default configuration if no previously

saved configuration can be found.  Look at "HELP | Getting Started" for more

information.

For additional information, please consult the various help messages.

You may address questions or comments to:

Network Time Service

Time and Frequency Division

National Institute of Standards and Technology

325 Broadway

Boulder, Colorado 80303

e-mail: time@time.nist.gov

Software Version: 23 February 2001