"Preparing for Ultra C"                     A step-by-step guide for setting up                 your system to run Ultra C compiled programsIntroduction============For a long time, OS-9 users were used to getting files from BBS's andFTP sites containing executables, bursting the archive, and runningthe executable programs.  Because many of these executables were compiledwith Microware's long-used C 3.2 compiler, it was expected that modulessuch as math and cio had to be loaded into memory.  While this is stillthe case, new requirements are demanded by Ultra C compiled programs.Ultra C is Microware's current ANSI compilant C/C++ compiler.  As Ultra Cbecomes more widely used, programs which have been compiled with it areappearing everywhere.  These programs rely on a different set of modulesto be in memory than programs which were born of the old C 3.2 compiler.If proper steps have not been taken to run Ultra C compiled programs on asystem, strange errors can occur.The modules referenced here can be found in the UCCMODS directory ofthe archive.Floating Point Support======================Unlike the old C 3.2 compiler which depended on a trap module called'math' for floating point processing, Ultra C places floating pointinstructions directly into the programs that it compiles.  While thishas its advantages for systems equipped with a hardware floating pointunit (or FPU), it can cause havoc for those who don't have a hardwareFPU and haven't made the necessary system preparations.If your OS-9 system DOES HAVE a hardware FPU, then skip this section;OTHERWISE, you will need to install the supplied software emulationmodule (fpu) if you have not already done so.  If you have alreadyinstalled the software fpu module at an earlier time, you will wantto check if the included fpu has a higher edition number and replaceyour current fpu module with it.The software fpu module is called when a floating point instruction isfetched by the CPU, and will emulate the math instruction in software.If a system without a hardware or software fpu were to execute a programwith floating point instructions, you would see the OS-9 error #000:111appear, and the process would immediately die.To install the software fpu module, you need to do two things:	1. Modify your init module to reference the fpu module	2. Place the fpu module in your bootfileThe fpu module MUST be in your bootfile.  It cannot merely be loaded oncethe system is up and running.  Additionally, your system's init modulemust reference the fpu module by name.  The init module has space for an'extension list' which lists modules in the bootfile that may calledduring the OS-9 boot process.  The fpu module must be listed as one ofthese modules.The simplest way to modify the init module is to use a binary file editor(dEd works nicely) to edit the init module file.  Locate the string 'os9p2',and you've found the extension list.  If you don't use os9p2, then great!Just overwrite it with the string: fpu  and be sure to NULL terminate it.If you have os9p2, then you'll need to make sure you have enough room inthe init module's system extension list to add the fpu module name.  Ifnot, then you will need to remake your init module or find some way toextend it.Ultra C and CSL===============CSL is to Ultra C-compiled programs what CIO is to C 3.2 compiledprograms.  That is, CSL (C Shared Library) is a trap module which someprograms (depending on how they were compiled) may need to run.  Thesimplest method is to place CSL in the bootfile, but you can also placeit in the execution directory, where it will be found and loaded by thefirst program that needs it.Don't throw away your CIO trap module though.  Old programs that werecompiled with C 3.2 may still need to use it, so keep both CIO and CSLin memory.Two versions of CSL are included.  csl.68k is for 68000 based systems,and csl.020 is for 68020 systems, including the CPU32.  Install themodule which pertains to your system.RBF Device Descriptors======================Another change that is necessary is setting the append bit in all of yourRBF device descriptors.  At offset $37 in an OS-9 RBF device descriptor isthe 'mode' byte.  You need to make sure that bit 4 is set in that byte.Typically, the value is $a7, so it should be changed to $b7.  If thisbit is not set, then programs compiled with Ultra C that open files inappend mode will receive E_BMODE errors.Once these changes have been made, your system is now ready to runUltra C compiled programs.  If you have any questions, please contactme at boisy@os9er.waukee.ia.us, or BOISY on Delphi.Boisy G. Pitre