A Small fix for the PRINTER driver     I was experimenting with a way to make use of the cassette motor controlunder OS9. It could be used for a variety of reasons. One item I was playingwith was a way to turn the hard drives off and on under program control.(by using a larger relay to switch the 110V source) so during idle timeswhen my BBS isn't doing anything to shut the drives off.      I wrote a small program in assembly that will allow you to toggle thecassette motor on or off. However one small problem came up was when I wentto use the printer. sending anything to the printer caused the motor toturn off. So i went to investigate the problem. Using disasm I dissasembledthe printer driver and discovered it clears $FF21 which bit 3 controls themotor. Clearing this register turns the motor off. So basically the fix isto check to see what the status of the motor is and not to disturb it.     Well before you go cracking out Ipatch you must check one thing first.The PRINTER driver has a delay value (set by TunePort) that could be different. At offset $0023 is where it is stored. Mine is $18 hex.an ident of my OLD module will verify Header for:  PRINTER Module size: $017A    #378Module CRC:  $CC3EA4 (Good) Hdr parity:  $A0 Exec. off:   $0024    #36Data Size:   $0029    #41Edition:     $0C      #12Ty/La At/Rv: $E1 $81 Dev Dvr mod, 6809 obj, re-en, R/O After the fix the new module will beHeader for:  PRINTER Module size: $0182    #386Module CRC:  $C55F59 (Good) Hdr parity:  $58 Exec. off:   $0024    #36Data Size:   $0029    #41Edition:     $0D      #13Ty/La At/Rv: $E1 $81 Dev Dvr mod, 6809 obj, re-en, R/O to make sure you have the delay set right use: tuneport /p   to find out what your delay is. then do this:tuneport /p -s=24  to set it to 24 for patching. Just crank out Ipatch and go to it. after your done if your delay is not 24 reset it to your value using the -s option of tuneport.Technical stuff:Ok for those who wonder what I did here it is..taking a clip from the dissassembly with the fixesL0036    pshs  cc         orcc  #$50         ldx   #$FF20** Patch code not to clear $FF21 This causes the motor* relay to turn off. this is not desired* old code shown below**        clr   $01,x** New code starts here*         lda   1,x  load PIA register         anda  #$08   clear all but bit 3         sta   1,x  store it back** old code resumes*         ldd   <$2C,y         std   <$22,u         lda   #$FE         sta   ,x** Another area which it messes with the Motor bit* Must make sure we leave it alone* old bit pattern %00110110 or $36* motor bit -----------|* old code shown**        lda   #$36*        sta   $01,x** Add new code here*         lda   1,x load a with PIA register         anda  #$8 zero out all but bit3         ora   #$36 bit pattern desired plus bit3         sta   1,x store it back to PIA** old code resumes*         lda   ,x         ldd   <$26,y         lbsr  L0138         puls  ccThats all there is to it!Also included is the motor command. The assembly language version is a smallversion. you only can toggle it on or off. Just read the doc file that goeswith it. The Basic09 version is larger but more descriptive. Only note thereis if your not using shell+ to use the parenthesis. such as:motor ("off")    motor("on")  motor ("?")   motor("h")shell+ can do this:motor off        motor on     motor ?       motor hMike Guzzi - CIS: 76576,2715  Call the Astral Plane BBS 717-586-2771 300/1200/2400 baud 24hrs 8/N/1