DMODE	  - Manipulate RBF device parameters on the options area of a 			device descriptor.Syntax    : dmode <RBF descriptor> [<RBF parameter list>]Function  : Change RBF device parameters in a descriptor            The device is detached, changed, and re-attached            If it isn't in use this should re-initialise it.Parameters:     drv      = Drive number               stp      = Step rate                  typ(hex) = Device type                dns(hex) = Media density              cyl      = Number of cylinders        sid      = Number of sides            vfy(hex) = Verify disk writes         sct      = Sectors per track          tos      = Sectors on track 0         sas      = Segment allocation         ilv      = Interleave factor          tof      = Track base offset          sof      = Sector base offset         siz      = Sector size (bytes)        ctl(hex) = Control word               try      = Retry count                prk      = Hard disk park cyl         lsn      = Logical sector offset      tot      = Total cylinders            rts(hex) = Rpm, xfr rate              xfr      = Maximum transfer      Execution:	This program has no options.  It is the RBF equivalent of XMODE for SCF 	devices.  If it is run with just a device name specified it will report 	the current parameter settings in the descriptor:		dmode /dd	Parameters which assign specific meanings to particular bits or values	are reported in hex and the meaning of the current setting is displayed.	Parameters in an RBF descriptor may be reset by specifying them on the	command line in the form key=n, key=0xh or key=$h. NOTE: if you're using	the EFFO shell you'll need to escape the dollar sign, so a hex parameter	should be input as key=0xhh or key=\$hh. Parameters shown in decimal	must be set in decimal; those shown in hex (and flagged (hex) in a help	display) must be set in hex:		dmode /d1 stp=3 vfy=$01	sets the step rate to 3 and turns disk verify after write off.  See the	OS9 technical manual for the meanings of any parameters you don't 	understand.	When parameters are set the descriptor's module CRC is reset.  dmode is	an easy way to create new descriptors from an existing one provided that	the new descriptor is to use the original device driver and RBF file 	manager.  It is intended to ease the task of reading an unknown disk 	format, especially in conjunction with LABEL, which can report the	options with which a disk was formatted provided that LSN 0 can be read	with an existing descriptor.	Dmode detaches the device once, modifies it, and re-attaches it. If	the device is not already in use this will force it to be	re-initialised. The effect is similar to the command sequence:		deiniz /dev; moded dev; load dev; iniz /dev	except that the changes are only applied to the copy of the descriptor	that is RAM resident.