LaTerm 						 Version 1.1        	*******	 MM/1 KWindows Application  *******					(c) 1994 Alan Weston						Quick Help File						---------------LaTerm emerged out of a desire to have a terminal program which made useof as many external routines as possible.  Thus allowing the potentialuser to tailor most of the options.  Furthermore, key strokes have beenminimized and a point and click KWindows interface has been used. KWindows is required to run this application. As of this writing, windiov.52 & 48 were used for all the testing.Users will be required to designate and make use of individual externalroutines to handle such things as up/download protocols, editor, list,shell, etc.1. Getting Started------------------Quick startup:-------------Set up environment variables:setenv TERM vscsetenv PORT /t3 (or /whatever)The following files must be in appropriate locations:file                location----------------------------------LaTerm				execution pathLaDial				exec pathconfer				exec pathsend_break			exec pathla.sys				/dd/sysdial.sys			/dd/sysexternal routines   exec path (or as defined by setenv PATH)Type: latermNote: merge LaTerm and LaDial together for quicker response of autodialerUsage/Options/Parameters:------------------------Usage:  LaTerm [-s <script> -p <port>]Options:        s: execute a script file on startup        p: serial port -- defaults to MODEM environment variable Usage:   LaDial [-p<port>]         LaDial [-s<n>]Options:		 port: optional port assignment, e.g. /t3         s: run script where n: entry number of site to autodial         -if port is not specified then use environment variable MODEM.         -s option requires env. var. MODEM         -optional environment variable LAPLAY can be used to call an         external alarm to notify of connect.Usage: confer [pipe] (hsize) (vsize)       Open one way conference line to pipe.       ** Requires LaTerm ** Usage: send_break [port], e.g. send_break /t32. Main Menu Functions:----------------------Files------Chd							change directory on current device	ChDrive						change driveList File					call external file list programOpen Log 					open log/capture file default:"la.log"Pause Log					pause log file - reopen with Open Log optionRename Log					change default log nameClose Log					close log file Editor						invoke user specified editorPrint						print file (forks "print")Shell						fork shell in windowQuit						quit LaTermSetup-----8-N-1 						8 bit word length, no parity, 1 stop bit 8-E-1 						8 bit word length, even parity, 1 stop bit 8-O-1 						8 bit word length, odd parity, 1 stop bit 7-N-1 						7 bit word length, no parity, 1 stop bit 7-E-1						7 bit word length, even parity, 1 stop bit 7-O-1                       7 bit word length, odd parity, 1 stop bit Local Echo 					echo stdin on/off for half/full duplexHandshake 					switch between XON/XOFF (software) and 							RTS/CTS (hardware) handshaking Termcap 					not implemented in this versionBaud 						change serial bps rateUser Defines				dial.sys settings (excluding macro keys)Port Options				view &/or edit port path descriptorTerm Options				view &/or edit window path descriptorThe Port and Term Options provide direct editing of the various optionsthat make up the path descriptors for the screen and serial port.  Thelist box display allows for selection with a simple click of the mouse. A prompt then appears at the appropriate location to take decimal input(up to 3 characters). The output is displayed in hexadecimal. The pathdescriptor is updated upon exit using the close box. Users should refer to sgbuf.h or the Microware manuals for details. Do not edit unless you know what will ensue -- this is an easy way to crash your MM/1.Access ------Transmit					send an ASCII file with optional line delay (line							delay is disabled in script mode)Blind Mode					disables screen output -- handy for capturing							messages at high baud rates when the screen cannot							keep up. Always provides faster throughput.SendBreak					send a true breakConference					toggle conference modeDialer						call external dialer LaDialRun Script					execute user defined script fileHangup						send hangup string to modemReinit Modem				send modem initialization string (INI in dial.sys)Host Mode					not implemented in this versionReceive/Send------------KermitBin					kermit binary transfers KermitASC					kermit ASCII transfersXmodem						xmodemXmodem 1K					xmodem 1KYmodem						ymodemZmodem						zmodemCis-B+						Compuserve CIS B+LaTerm------Help						Short cut key help fileAbout...3. Format of "la.sys" file:--------------------------RKB=kermit -ril /t3^M			kermit binary receiveRKA=kermit -rl /t3^M			kermit ascii receiveRXM=xydown^M					xmodem receiveRXK=xydown^M					xmodem 1K receive RYM=xydown^M					ymodem receiveRZM=rz^M						zmodem receiveRCB=bplus						CIS-B+ receiveSKB=kermit -sil /t3				kermit binary send	SKA=kermit -sl /t3				kermit ascii sendSXM=sz -X						xmodem sendSXK=sx -k						xmodem 1k sendSYM=sb  						ymodem sendSZM=sz  						zmodem sendSCB=bplus						CIS-B+ sendEDT=umacs						user specified editorCAP=la_log						default log capture filenameSHL=sh -nlh=20					shell LST=less						user specified list program MAC=/dd/sys/scripts				script file locationDIR=/dd/temp					working directoryINI=ATZ0^M						modem initialization stringHNG=+++~ATH^M					hang-up stringAL0=whatever you want^M			user definable macrosAL1=							 "		"		"AL2=AL3=AL4=AL5=AL6=AL7=AL8=AL9=Parsing requires that no spaces be present around = sign.  All the filetransfer protocols have a 30 character limit.  EDT and LST are limitedto 10 characters.  SHL, CAP, MAC, DIR have a 20 character limit.  Userdefinable macros Al0-9 and INI have a 40 character limit.  These sizesare also detailed in la.sys. The use of ^M forces carriage return append.Trailing spaces should be added where appropriate (e.g. after sendtransfer protocols). A tilde ~ can be used to insert 1 second pauses inthe hang-up string.5. Script Files---------------The script files are designed to link with the supplied autodialer (LaDial).The linkage requires knowledge of the entry number as provided in thedial.sys file.Format of script files:#n							<- entry number n from dial.sys%n							<- wait n seconds (range 1-9)     ^C                          <- Send control character e.g. ctrl-C~<CR>						<- Wait for string after tilde ~logon^M						<- carriage return must be forced;OPEN logfile 				<- capture file "logfile" (or default) open;STOP						<- capture file closed;WAIT						<- capture file paused;CONT						<- continue capture;TRAN filename				<- ascii transmit file;BLON						<- turn on blind mode;BLOF 						<- turn off blind mode;HANG						<- hangup;SEND:n file1 file2...		<- send using menu option n (e.g. n=3:xmodem);RECE:n						<- receive (e.g. n=6:zmodem);IDLE:n						<- suspend script execution until modem input								has stopped for at least n seconds (n=1-9)First column identifiers:(~) The tilde precedes a string to be searched.(%) The percent sign precedes a time delay in seconds.  (#) The pound sign precedes the entry number to dial.  (^) The carot represent CTRL, 1 control character per line. (;) The semicolon provides miscellaneous features.  Multiple log files are implemented. If no identifier is present in the first column, the string is sent.Remarks:-------o	Mouseclick aborts dial. o	Keyboard input aborts macro execution.  o	Searching should be used when a substantial delay in reception is	likely to occur. For cases where the wait is no longer than a couple	seconds, a pause will work best. This is due to immediate buffered input 	data which doesn't get searched for the matching pattern. Search patterns	are best kept short. o	identifier lines must have carriage returns right after the last letter,	spaces may cause errors. 6. Implicit Functions:---------------------o	box in upper left hand corner: same as Quito	click on blank portion of menu bar: clears/resets screeno	rts/cts is used by default to control flowo	default setup: 8-N-1 2400 bps, hardware handshake set to current driver	setting, echo off, log file off, default terminal environment. o	capture file has ~4K buffero	subsequent uses of "la.log" file will cause overwrite, so user will	need to rename it before or after use.o	if serial read errors occur, a beep is sounded.o	short key options are available as depicted with underlined menu characters	as well as in help display.o	carrier detect starts the on-line clock in hours:minutes, loss of carrier	turns the clock off.7. Conference Mode:------------------The conference mode option starts up a separate process in a separatewindow.  Therefore, changing window will cause the conference window todisappear.  It will reappear by reselecting it.  The conference modeopens a separate path to the modem port and will stay active even afterexiting from LaTerm.  Conference mode is toggled via the access menu orthe short cut key press.  The short cut key is only active when the mainscreen is selected.  Then reset the screen work area by clicking on themenu bar. Attention:---------Since conference mode changes the working area, if the cursor is at thescreen bottom, screen output will get trapped outside of the workingarea.  This can be avoided by simply clearing the screen prior toconference mode.  It was not desired to force a screen clear and cursorreturn to home (i.e. top of screen), in order to provide more flexibility.8. Automation-------------Since script files can be assigned on startup from the command line, automatedoperation is implementable. Cron

provides the ability to start operationat any time and date according to definitions in the crontab file.The following is an example crontab file (/dd/sys):30 23 * * * setenv MODEM /t3;setenv LAPLAY bell;cx /h1/c/cmds;LaTerm -s cisThis entry autostarts LaTerm any day at 11:30 pm and LaTerm executesscript file 'cis'. Note that since cron start a new shell, you may haveto pass environment settings for both LaTerm and LaDial. 9. Status Bar-------------The status bar provides current configuration and status information. From left to right the display information includes:/t3 2400 8N1 | Echo | RTS/CTS | Log:la_log Off | (Blind) (Confer) 00:00:00   												^		^		^												blind, conference modes & ^	 ^	  ^		^		^			^		^		time online |   |	  |		|		|			|		 \_ log file status |   |	  |		|		|		 	 \_ log file name (1st 10 characters) |   |	  |		|		 \_ handshake status (RTS/CTS or XON/XOFF) |   |	  |		 \_ terminal echo status |	 |	   \_ word length/parity/stop bits |	  \_ bpsportWhen switching to another screen, upon return the status bar will notreappear.  In such instances, if needed, simply sweep the mouse pointeracross the top line. 10. Tips:--------o	LaTerm bumps its priority up to 140 on startup, this appears to be	sufficient in most cases.  o 	Use moded to increase the input and output buffer sizes for the serial ports	that you plan on using. A 1024 byte size seems to work well on both input 	and output buffers. Some users have better throughput with input set at 	2048.o	Communication speeds at 9600 baud and above are difficult for the screen to 	keep up with. So for capturing message bases and text on the fly, use	the "Blind" mode. This way incoming data is dumped straight to a file	minimizing loss of data. Simply monitor RD modem status and turn the	blind mode off when no more data is coming in.o	For high speed communication, if possible use a port that supports RTS/CTS	hardware flow control. Furthermore, most high speed modems will buffer	incoming data if configured accordingly - look for DCE flow control	options (on my modem this is \G1).11. Registration:----------------To register your copy and receive the next update, send $35 (payable to author)to:Tantricomc/o	Alan Weston RR2 Box 322-WCarbondale, IL 62901For support:Internet:		weston@sysa.c-engr1.siu.eduDelphi:			ALWESTONCompuserve:		76234,350012. Technical Detail--------------------On startup LaTerm boosts the process priority to 140, this helps toreduce lost characters.  Configuration information is gathered from/dd/sys/la.sys and DIR is adopted as current directory. The signal handler deals with mouse signals as well as others. Thesignals encountered are: carrier signals, alarm signals, as well as somefeedback from LaDial.Port settings that are conserved on startup include: parity, whereasbaud is reset to 2400.  Term settings that are conserved on startupinclude: parity and baud. Xon and Xoff signals are reset. The RTS lineis enabled by default. If hardware handshaking is enabled and the portis iniz'ed, errors will occur during RTS/CTS enable/disable operationswithin LaTerm -- this shouldn't be a problem. LaTerm is designed tooperate with a port that hasn't been initialized.The conference mode is handled by an external routine which makes use ofa named pipe for communication with LaTerm. The program confer simplytakes user input and then dumps it in /pipe/confer. LaTerm then monitorsthe pipe to see if data is available, etc. In this incarnation LaTerm forks confer/list/edit and creates an orphan. These processes remain on the LaTerm's linked list of processdescriptors thus filling up the process table.  For right now this isconsidered better than waiting until the child terminates. Furthermore, during LaDial access if LaTerm receives a signal (e.g.  amouse click) LaDial will become an orphan and I/O during dialing will bedisrupted.