UNIX KERMIT********This document is formatted as an ordinary, plain text ASCII disk file.  Typesetcopies are available  in  the  Kermit  User  Guide  from  Columbia  University.Changes should be made to CKUKER.MSS.********Program:    Frank  da  Cruz,  Bill Catchings, Jeff Damens, Columbia University;            Herm Fischer, Encino CA; contributions by many others.Language:   CDocumentation:            Frank da Cruz, Herm FischerVersion:    4E(068)Date:       January 24, 1988C-Kermit is an implementation of Kermit, written modularly and transportably inC. The protocol state transition table is written in wart, a  (non-proprietary)lex-like  preprocessor for C. System-dependent primitive functions are isolatedinto separately compiled modules so that the program should be easily  portableamong  Unix systems and also to non-Unix systems that have C compilers, such asVAX/VMS, Data General AOS/VS, Apollo Aegis, the Apple Macintosh, and  the  Com-modore  Amiga.   This document applies to Unix implementations of C-Kermit, andin most ways also to the VMS, Data General, and other implementations.Unix Kermit Capabilities At A Glance:  Local operation:                   Yes  Remote operation:                  Yes  Login scripts:                     Yes (UUCP style)  Transfer text files:               Yes  Transfer binary files:             Yes  Wildcard send:                     Yes  File transfer interruption:        Yes  Filename collision avoidance:      Yes  Can time out:                      Yes  8th-bit prefixing:                 Yes  Repeat count prefixing:            Yes  Alternate block checks:            Yes  Terminal emulation:                Yes  Communication settings:            Yes  Transmit BREAK:                    Yes (most versions)  Support for dialout modems:        Yes  IBM mainframe communication:       Yes  Transaction logging:               Yes  Session logging:                   Yes  Debug logging:                     Yes  Packet logging:                    Yes  Act as server:                     Yes  Talk to server:                    Yes  Advanced server functions:         Yes  Local file management:             Yes  Command/Init files:                Yes  UUCP and multiuser line locking:   Yes  Long packets:                      Yes  Sliding Windows:                   No  File attributes packets:           No  Command macros:                    No  Raw file transmit:                 NoAll numbers in the C-Kermit documentation are decimal unless noted otherwise.C-Kermit provides traditional Unix command line operation  as  well  as  inter-active  command  prompting and execution.  The command line options provide ac-cess to a basic subset of C-Kermit's capabilities; the interactive command  setis far richer.On  systems  with  dialout  modems,  C-Kermit's  command  file and login scriptfacilities provide a counterpart  to  UUCP  for  file  transfer  with  non-UNIXoperating  systems, including the use of scheduled (e.g. late night) unattendedoperation.1.1. The Unix File SystemConsult your Unix manual for details about the file system under  your  versionof  Unix.  In general, Unix files have lowercase names, possibly containing oneor more dots or other special characters.   Unix  directories  are  tree-struc-tured.  Directory levels are separated by slash ("/") characters.  For example,    /usr/foo/bardenotes the file bar in the directory /usr/foo.  Alphabetic case is significantin  Unix  file and directory names, i.e. "a" is a different file (or directory)from "A".  Wildcard or "meta" characters allow groups of files to be specified."*" matches any string; "?" matches any single character.When  C-Kermit  is  invoked  with  file arguments specified on the Unix commandline, the Unix shell (Bourne Shell, C-Shell, K-Shell,  etc)  expands  the  metacharacters itself, and in this case a wider variety is available.  For example,    kermit -s ~/ck[uvm]*.{upd,bwr}]is expanded by the Berkeley C-Shell into a list of all the files in the  user'shome  directory  (~/) that start with the characters "ck", followed by a singlecharacter "u", "v", or "m", followed by zero or more characters, followed by  adot,  followed  by one of the strings "upd" or "bwr".  Internally, the C-Kermitprogram itself expands only the "*" and "?" meta characters.Unix files are linear (sequential) streams of 8-bit bytes.  Text files  consistof 7-bit ASCII characters, with the high-order bit off (0), and lines separatedby the Unix newline character, which is linefeed (LF, ASCII 10).  This  distin-guishes  Unix text files from those on most other ASCII systems, in which linesare separated by a carriage-return linefeed sequence (CRLF, ASCII 13,  followedby  linefeed,  ASCII  10).  Binary files are likely to contain data in the highbits of the file bytes, and have no particular line or record structure.When transferring files, C-Kermit will convert between  upper  and  lower  casefilenames  and  between LF and CRLF line terminators automatically, unless toldto do otherwise.  When binary files must be transferred, the  program  must  beinstructed  not  to  perform LF/CRLF conversion (-i on the command line or "setfile type binary" interactively; see below).1.2. File TransferIf C-Kermit is in local mode, the screen (stdout)  is  continously  updated  toshow  the  progress  of the file transer.  A dot is printed for every four datapackets, other packets are shown by type:    I Exchange Parameter Information    R Receive Initiate    S Send Initiate    F File Header    G Generic Server Command    C Remote Host Command    N Negative Acknowledgement (NAK)    E Fatal Error    T Indicates a timeout occurred    Q Indicates a damaged, undesired, or illegal packet was received    % Indicates a packet was retransmittedYou may type certain "interrupt" commands during file transfer:    Control-F:  Interrupt the current File, and go on to the next (if any).    Control-B:  Interrupt the entire Batch of files, terminate the transaction.    Control-R:  Resend the current packet    Control-A:  Display a status report for the current transaction.These interrupt characters differ from the ones used in other Kermit  implemen-tations  to  avoid conflict with commonly used Unix shell interrupt characters.With Version 7, System III, and System V  implementations  of  Unix,  interruptcommands must be preceeded by the 'connect' escape character (e.g. normally-\).Ctrl-F and Ctrl-B are effective only during the transfer of data  (D)  packets,and cannot be used to interrupt a transfer that has not yet reached that stage.    CAUTION:  If Control-F or Control-B is used to cancel an incoming file,    and a file of the same name previously existed, and the "file  warning"    feature  is  not  enabled, then the previous copy of the file will dis-    appear.EMERGENCY EXIT:  When running Unix Kermit in remote mode, if you have started aprotocol operation (sending or receiving a file, server command wait, etc), youwill not be able to communicate with the terminal in the normal way.   In  par-ticular,  you  cannot  stop  the  protocol  by typing the normal Unix interruptcharacters, since the terminal has been put in "raw mode".    If  you  need  toregain  control  quickly  -- for instance, because the protocol is stuck -- youcan type two Control-C's directly to the Unix Kermit program  ("connect"  firstif necessary):      Control-C Control-CThis will cause the program to exit and restore the terminal to normal.1.3. Command Line OperationThe  C-Kermit command line syntax conforms to the Proposed Syntax Standards forUnix System Commands put forth by Kathy Hemenway and Helene  Armitage  of  AT&TBell Laboratories in Unix/World, Vol.1, No.3, 1984.  The rules that apply are:   - Command names must be between 2 and 9 characters ("kermit" is 6).   - Command names must include lower case letters and digits only.   - An option name is a single character.   - Options are delimited by '-'.   - Options  with  no  arguments  may  be  grouped  (bundled)  behind one     delimiter.   - Option-arguments cannot be optional.   - Arguments immediately follow options, separated by whitespace.   - The order of options does not matter.   - '-' preceded and followed by whitespace means standard input.A group of bundled options may end with an option that has an argument.The following notation is used in command descriptions:fn      A Unix file specification, possibly containing the  "wildcard"  charac-        ters  `*'  or  `?' (`*' matches all character strings, `?'  matches any        single character).fn1     A Unix file specification which may not contain `*' or `?'.rfn     A remote file specification in the remote system's  own  syntax,  which        may denote a single file or a group of files.rfn1    A remote file specification which should denote only a single file.n       A decimal number between 0 and 94.c       A  decimal  number between 0 and 127 representing the value of an ASCII        character.cc      A decimal number between 0 and 31, or else  exactly  127,  representing        the value of an ASCII control character.[ ]     Any field in square braces is optional.{x,y,z} Alternatives are listed in curly braces.C-Kermit  command  line options may specify any combination of actions and set-tings.  If C-Kermit is invoked with a command line that specifies  no  actions,then  it  will  issue  a  prompt  and begin interactive dialog.  Action optionsspecify either protocol transactions or terminal connection.-s fn   Send the specified file or files.    If  fn  contains  wildcard  (meta)        characters,  the  Unix shell expands it into a list.  If fn is '-' then        kermit sends

from standard input, which may come from a file:              kermit -s - < foo.bar        or a parallel process:              ls -l | grep christin | kermit -s -        You cannot use this mechanism to send terminal typein.  If you want  to        send  a  file  whose  actual name is "-" you can precede it with a path        name, as in             kermit -s ./--r      Receive a file or files.  Wait passively for files to arrive.-k      Receive (passively) a file or files, sending them to  standard  output.        This option can be used in several ways:        kermit -k            Displays the incoming files on your screen;  to  be  used  only  in            "local mode" (see below).        kermit -k > fn1            Sends the incoming file or files to the named file, fn1.   If  more            than  one  file  arrives,  all  are  concatenated together into the            single file fn1.        kermit -k | command            Pipes the incoming data (single or multiple files) to the indicated            command, as in                 kermit -k | sort > sorted.stuff-a fn1  If you have specified a file transfer option, you may give an alternate        name for a single file with the -a ("as") option.  For example,             kermit -s foo -a bar        sends  the file foo telling the receiver that its name is bar.  If more        than one file arrives or is sent, only the first file  is  affected  by        the -a option:              kermit -ra baz        stores the first incoming file under the name baz.-x      Begin server operation.  May be used in either local or remote mode.Before  proceeding, a few words about remote and local operation are necessary.C-Kermit is "local" if it is running on PC or workstation that  you  are  usingdirectly, or if it is running on a multiuser system and transferring files overan external communication line -- not your job's controlling terminal  or  con-sole.   C-Kermit is remote if it is running on a multiuser system and transfer-ring files over its own controlling  terminal's  communication  line  (normally/dev/tty), connected to your PC or workstation.If  you  are  running C-Kermit on a PC, it is normally used in local mode, withthe "back port" designated for file transfer and terminal connection.   If  youare  running C-Kermit on a multiuser (timesharing) system, it is in remote modeunless you explicitly point it at an external line for file  transfer  or  ter-minal connection.  The following command sets C-Kermit's "mode":-l dev  Line  --  Specify a terminal line to use for file transfer and terminal        connection, as in             kermit -l /dev/ttyi5When an external line is being used, you will also need some additional optionsfor successful communication with the remote system:-b n    Baud  --  Specify the baud rate for the line given in the -l option, as        in             kermit -l /dev/ttyi5 -b 9600        This option should always be included with the  -l  option,  since  the        speed of an external line is not necessarily what you expect.-p x    Parity  --  e,o,m,s,n  (even, odd, mark, space, or none).  If parity is        other than none, then the 8th-bit prefixing mechanism will be used  for        transferring  8-bit  binary  data, provided the opposite Kermit agrees.        The default parity is none.-t      Specifies half duplex,  line  turnaround  with  XON  as  the  handshake        character.The  following  commands may be used only with a C-Kermit which is local eitherby default or else because the -l option has been specified.-g rfn  Actively request a remote server to send the named file or  files;  rfn        is a file specification in the remote host's own syntax.  If fn happens        to contain any special shell characters, like  space,  '*',  '[',  etc,        these must be quoted, as in             kermit -g x\*.\?        or             kermit -g "profile exec"-f      Send a 'finish' command to a remote server.-c      Establish  a  terminal  connection  over  the specified or default com-        munication line, before any protocol transaction takes place.  Get back        to   the   local  system  by  typing  the  escape  character  (normally        Control-Backslash) followed by the letter 'c'.-n      Like -c, but after a protocol transaction takes place; -c  and  -n  may        both  be used in the same command.  The use of -n and -c is illustrated        below.If the other Kermit is on a remote system, the -l and -b options should also beincluded with the -r, -k, or -s options.Several other command-line options are provided:-i      Specifies that files should be sent or received exactly "as is" with no        conversions.  This option is necessary for transmitting  binary  files.        It may also be used in Unix-to-Unix transfers (it must be given to both        Unix Kermit programs), where it will improve performance by circumvent-        ing  the  normal  text-file conversions, and will allow mixture of text        and binary files in a single file group.-w      Write-Protect -- Avoid filename collisions for incoming files.-e n    Extended packet length -- Specify that C-Kermit is allowed  to  receive        packets  up  to length n, where n may be between 10 and some large num-        ber, like 1000, depending on the system.  The  default  maximum  length        for  received  packets is 90.  Packets longer than 94 will be used only        if the other Kermit supports, and agrees  to  use,  the  "long  packet"        protocol extension.-q      Quiet  --  Suppress screen update during file transfer, for instance to        allow a file transfer to proceed in the background.-d      Debug -- Record debugging information in the file debug.log in the cur-        rent  directory.    Use  this option if you believe the program is mis-        behaving, and show the resulting log to your local Kermit maintainer.-h      Help -- Display a brief synopsis of the command line options.The command line may contain no more than one protocol action option.Files are sent with their own names, except that lowercase letters  are  raisedto upper, pathnames are stripped off, certain special characters like (`~') and(`#') are changed to `X', and if the file name begins with a period, an `X'  isinserted  before  it.    Incoming files are stored under their own names exceptthat uppercase letters are lowered, and, if -w  was  specified,  a  "generationnumber"  is  appended  to  the name if it has the same name as an existing filewhich would otherwise be overwritten.  If the -a option is included,  then  thesame  rules  apply to its argument.  The file transfer display shows any trans-formations performed upon filenames.During transmission, files are encoded as follows:   - Control characters are converted to prefixed printables.   - Sequences of repeated characters are collapsed via repeat counts,  if     the other Kermit is also capable of repeated-character compression.   - If  parity  is  being used on the communication line, data characters     with the 8th (parity) bit on are  specially  prefixed,  provided  the     other  Kermit  is  capable of 8th-bit prefixing; if not, 8-bit binary     files cannot be successfully transferred.   - Conversion is done between Unix newlines and carriage-return-linefeed     sequences unless the -i option was specified.Command Line Examples:    kermit -l /dev/ttyi5 -b 1200 -cn -rThis command connects you to the system on the other end of ttyi5 at 1200 baud,where you presumably log in and run Kermit with a 'send' command.    After  youescape  back,  C-Kermit  waits  for a file (or files) to arrive.  When the filetransfer is completed, you are reconnected to the remote system so that you canlogout.    kermit -l /dev/ttyi4 -b 1800 -cntp m -r -a fooThis  command  is like the preceding one, except the remote system in this caseuses half duplex communication with mark parity.  The first file  that  arrivesis stored under the name foo.    kermit -l /dev/ttyi6 -b 9600 -c | tekThis  example  uses  Kermit to connect your terminal to the system at the otherend of ttyi6.  The C-Kermit terminal connection does not provide any particularterminal   emulation,   so   C-Kermit's   standard   i/o  is  piped  through  a(hypothetical) program called tek, which performs (say) Tektronix emulation.    kermit -l /dev/ttyi6 -b 9600 -nfThis command would be used to shut down a remote server and then connect to theremote system, in order to log out or to make further use of it.  The -n optionis invoked after -f (-c would have been invoked before).    kermit -l /dev/ttyi6 -b 9600 -qg foo.\* &This command causes C-Kermit to be invoked in the background, getting  a  groupof files from a remote server (note the quoting of the `*' character).  No dis-play occurs on the screen, and the keyboard is  not  sampled  for  interruptioncommands.    This  allows other work to be done while file transfers proceed inthe background.    kermit -l /dev/ttyi6 -b 9600 -g foo.\* > foo.log < /dev/null &This command is like the previous one, except the  file  transfer  display  hasbeen  redirected  to  the  file foo.log.  Standard input is also redirected, toprevent C-Kermit from sampling it for interruption commands.    kermit -iwxThis command starts up C-Kermit as a server.  Files  are  transmitted  with  nonewline/carriage-return-linefeed conversion; the -i option is necessary for bi-nary file transfer and recommended for Unix-to-Unix transfers.  Incoming  filesthat have the same names as existing files are given new, unique names.    kermit -l /dev/ttyi6 -b 9600This  command  sets  the  communication  line  and  speed.   Since no action

isspecified, C-Kermit issues a prompt and enters an interactive dialog with  you.Any  settings  given on the command line remain in force during the dialog, un-less explicitly changed.    kermitThis command starts up Kermit interactively with all default settings.The next example shows how Unix Kermit might be used to send an  entire  direc-tory  tree  from  one Unix system to another, using the tar program as Kermit'sstandard input and output.  On the orginating system, in this case the  remote,type (for instance):    tar cf - /usr/fdc | kermit -is -This  causes  tar to send the directory /usr/fdc (and all its files and all itssubdirectories and all their files...) to standard output instead of to a tape;kermit  receives  this as standard input and sends it as a binary file.  On thereceiving system, in this case the local one, type (for instance):    kermit -il /dev/ttyi5 -b 9600 -k | tar xf -Kermit receives the tar archive, and sends it via standard output  to  its  owncopy of tar, which extracts from it a replica of the original directory tree.A  final example shows how a Unix compression utility might be used to speed upKermit file transfers:    compress file | kermit -is -     (sender)    kermit -ik | uncompress          (receiver)Exit Status Codes:Unix Kermit returns an exit status of zero, except when a fatal  error  is  en-countered,  where  the  exit  status  is set to one.  With background operation(e.g., `&' at end of invoking command line) driven by scripted interactive com-mands  (redirected  standard  input  and/or take files), any failed interactivecommand (such as failed dial or script attempt) causes the fatal error exit.1.4. Interactive OperationC-Kermit's interactive command prompt is "C-Kermit>".    In  response  to  thisprompt, you may type any valid interactive C-Kermit command.  C-Kermit executesthe command and then prompts you for another command.   The  process  continuesuntil you instruct the program to terminate.Commands  begin  with a keyword, normally an English verb, such as "send".  Youmay omit trailing characters from any keyword, so  long  as  you  specify  suf-ficient  characters  to  distinguish  it  from  any other keyword valid in thatfield.  Certain commonly-used keywords (such as "send",  "receive",  "connect")also  have special non-unique abbreviations ("s" for "send", "r" for "receive","c" for "connect").Certain characters have special functions during  typein  of  interactive  com-mands:    ?   Question  mark, typed at any point in a command, will produce a message        explaining what is possible or expected at that point.    Depending  on        the  context, the message may be a brief phrase, a menu of keywords, or        a list of files.    ESC (The Escape or Altmode  key)  --  Request  completion  of  the  current        keyword  or filename, or insertion of a default value.  The result will        be a beep if the requested operation fails.    DEL (The Delete or Rubout key) -- Delete the previous  character  from  the        command.  You may also use BS (Backspace, Control-H) for this function.    ^W  (Control-W) -- Erase the rightmost word from the command line.    ^U  (Control-U) -- Erase the entire command.    ^R  (Control-R) -- Redisplay the current command.    SP  (Space) -- Delimits fields (keywords, filenames, numbers) within a com-        mand.  HT (Horizontal Tab) may also be used for this purpose.    CR  (Carriage Return) -- Enters the command for execution.   LF  (Linefeed)        or FF (formfeed) may also be used for this purpose.    \   (Backslash)  --  Enter  any  of  the above characters into the command,        literally.  To enter a backslash, type two backslashes in a  row  (\\).        A  backslash  at  the  end of a command line causes the next line to be        treated as a continuation line; this is useful for readability in  com-        mand files, especially in the 'script' command.    ^Z  (Control-Z)  --  On  systems (like Berkeley Unix, Ultrix) with job con-        trol, suspend Kermit, i.e. put it into the  background in  such  a  way        that  it  can  be  brought  back into the foreground (e.g. with an 'fg'        shell command) with all its settings intact.You may type the editing characters (DEL, ^W, etc) repeatedly,  to  delete  allthe  way  back to the prompt.  No action will be performed until the command isentered by typing carriage return, linefeed, or formfeed.  If you make any mis-takes,  you  will receive an informative error message and a new prompt -- makeliberal use of `?' and ESC to feel your way through the commands.   One  impor-tant command is "help" -- you should use it the first time you run C-Kermit.A command line beginning with a percent sign "%" is ignored.  Such lines may beused to include illustrative commentary in Kermit command dialogs.Interactive C-Kermit accepts commands from files as well as from the  keyboard.When you start C-Kermit, the program looks for the file .kermrc in your home orcurrent directory (first it looks in the home directory, then  in  the  currentone)  and  executes any commands it finds there.  These commands must be in in-teractive format, not Unix command-line format (the initialization file is  notprocessed if you invoke Kermit with command-line action arguments, such that itdoes not enter interactive dialog).  A "take" command is also provided for  useat any time during an interactive session, to allow interactive-format commandsto be executed from a file; command files  may  be  nested  to  any  reasonabledepth.Here is a brief list of C-Kermit interactive commands:              %  Comment              !  Execute a Unix shell command, or start a shell.            bye  Terminate and log out a remote Kermit server.          close  Close a log file.        connect  Establish a terminal connection to a remote system.            cwd  Change Working Directory.           dial  Dial a telephone number.      directory  Display a directory listing.           echo  Display arguments literally.           exit  Exit from the program, closing any open files.         finish  Instruct a remote Kermit server to exit, but not log out.            get  Get files from a remote Kermit server.           help  Display a help message for a given command.            log  Open a log file -- debugging, packet, session, transaction.           quit  Same as 'exit'.        receive  Passively wait for files to arrive.         remote  Issue file management commands to a remote Kermit server.         script  Execute a login script with a remote system.           send  Send files.         server  Begin server operation.            set  Set various parameters.           show  Display values of 'set' parameters.          space  Display current disk space usage.     statistics  Display statistics about most recent transaction.           take  Execute commands from a file.The 'set' parameters are:    block-check  Level of packet error detection.          delay  How long to wait before sending first packet.         duplex  Specify which side echoes during 'connect'.    escape-character  Prefix for "escape commands" during 'connect'.           file  Set various file parameters.   flow-control  Communication line full-duplex flow control.      handshake  Communication line half-duplex turnaround character.     incomplete  Disposition for incompletely received files.           line  Communication line device name.   modem-dialer  Type of modem-dialer on communication line.         parity  Communication line character parity.         prompt  The C-Kermit program's interactive command prompt.        receive  Parameters for inbound packets.          retry  Packet retransmission limit.           send  Parameters for outbound packets.          speed  Communication line speed.       terminal  Terminal parameters.The 'remote' commands are:            cwd  Change remote working directory.         delete  Delete remote files.      directory  Display a listing of remote file names.           help  Request help from a remote server.           host  A command to the remote host in its own command language.          space  Display current disk space usage on remote system.           type  Display a remote file on your screen.            who  Display who's logged in, or get information about a user.Most of these commands are described adequately in the Kermit User Guide or theKermit book.  Special aspects of certain Unix  Kermit  commands  are  describedbelow.                              THE 'SEND' COMMANDSyntax:  send fn  - or -  send fn1 rfn1Send  the file or files denoted by fn to the other Kermit, which should be run-ning as a server, or which should be given the 'receive' command.  Each file issent  under  its own name (as described above, or as specified by the 'set filenames' command).  If the second form of the 'send' command is used, i.e.   withfn1 denoting a single Unix file, rfn1 may be specified as a name to send it un-der.  The 'send' command may be abbreviated to 's', even though 's'  is  not  aunique abbreviation for a top-level C-Kermit command.The wildcard (meta) characters `*' and `?' are accepted in fn.  If `?' is to beincluded, it must be prefixed by `\' to override its normal function of provid-ing  help.    `*'  matches any string, `?' matches any single character.  Othernotations for file groups, like `[a-z]og', are  not  available  in  interactivecommands  (though  of  course they are available on the command line).  When fncontains `*' or `?' characters, there is a limit to the number  of  files  thatcan  be  matched,  which  varies from system to system.  If you get the message"Too many files match" then you'll have

to make a more judicious selection.  Iffn was of the form     usr/longname/anotherlongname/*then  C-Kermit's  string  space  will  fill  up rapidly -- try doing a cwd (seebelow) to the path in question and reissuing the command.Note -- C-Kermit sends only from the current or specified directory.   It  doesnot traverse directory trees.  If the source directory contains subdirectories,they will be skipped.  By the same token, C-Kermit does not create  directorieswhen  receiving files.  If you have a need to do this, you can pipe tar throughC-Kermit, as shown in the example on page 3, or under System III/V Unix you canuse cpio.Another  Note  --  The 'send' command does not skip over "invisible" files thatmatch the file specification; Unix systems  usually  treat  files  whose  namesstart  with  a  dot (like .login, .cshrc, and .kermrc) as invisible.  Similarlyfor "temporary" files whose names start with "#".                             THE 'RECEIVE' COMMANDSyntax:  receive  - or -  receive fn1Passively wait for files to arrive from the other Kermit, which must  be  giventhe 'send' command -- the 'receive' command does not work in conjunction with aserver (use 'get' for that).  If fn1 is specified,  store  the  first  incomingfile under that name.  The 'receive' command may be abbreviated to 'r'.                              THE 'GET' COMMAND:Syntax:  get rfn        or: get                rfn                fn1Request a remote Kermit server to send the named file or files.  Since a remotefile specification (or list)  might  contain  spaces,  which  normally  delimitfields  of  a C-Kermit command, an alternate form of the command is provided toallow the inbound file to be given a new name: type 'get' alone on a line,  andyou  will  be prompted separately for the remote and local file specifications,for example    C-Kermit>get     Remote file specification: profile exec     Local name to store it under: profile.execAs with 'receive', if more than one file arrives as a result of the 'get'  com-mand,  only the first will be stored under the alternate name given by fn1; theremaining files will be stored under their own names if possible.  If a `?'  isto be included in the remote file specification, you must prefix it with `\' tosuppress its normal function of providing help.If you have started a multiline 'get' command, you may escape from  its  lower-level prompts by typing a carriage return in response to the prompt, e.g.    C-Kermit>get     Remote file specification: foo     Local name to store it under: (Type a carriage return here)    (cancelled)    C-Kermit>                             THE 'SERVER' COMMAND:The 'server' command places C-Kermit in "server mode" on the currently selectedcommunication line.  All further commands must arrive as valid  Kermit  packetsfrom  the  Kermit  on  the  other  end of the line.  The Unix Kermit server canrespond to the following commands:Command                Server Response  get                    Sends files  send                   Receives files  bye                    Attempts to log itself out  finish                 Exits to level from which it was invoked  remote directory       Sends directory lising  remote delete          Removes files  remote cwd             Changes working directory  remote type            Sends files to your screen  remote space           Reports about its disk usage  remote who             Shows who's logged in  remote host            Executes a Unix shell command  remote help            Lists these capabilitiesThe Unix Kermit server cannot always respond properly to a  BYE  command.    Itwill  attempt to do so using "kill()", but this will not work on all systems orunder all conditions because of the complicated process structures that can  beset up under Unix.If  the  Kermit  server  is  directed at an external line (i.e. it is in "localmode") then the console may be used for other work if you have 'set  file  dis-play  off'; normally the program expects the console to be used to observe filetransfers and enter status queries or interruption commands.  The  way  to  getC-Kermit  into  background operation from interactive command level varies fromsystem to system (e.g. on Berkeley Unix you would halt the program with ^Z  andthen  use the C-Shell 'bg' command to continue it in the background).  The morecommon method is to invoke the program with the desired command line arguments,including "-q", and with a terminating "&".When  the  Unix  Kermit server is given a 'remote host' command, it executes itusing the shell invoked upon login, e.g.  the  Bourne  shell  or  the  BerkeleyC-Shell.                  THE 'REMOTE', 'BYE', AND 'FINISH' COMMANDS:C-Kermit  may itself request services from a remote Kermit server.  In additionto 'send' and 'get', the following commands may also be sent from C-Kermit to aKermit server:    remote cwd [directory]        If the optional remote directory specification is included, you will be        prompted  on a separate line for a password, which will not echo as you        type it.  If the remote system does not require  a  password  for  this        operation, just type a carriage return.    remote delete rfn       delete remote file or files.    remote directory [rfn]  directory listing of remote files.    remote host command     command in remote host's own command language.    remote space            disk usage report from remote host.    remote type [rfn]       display remote file or files on the screen.    remote who [user]       display information about who's logged in.    remote help             display remote server's capabilities.    bye and finish:        When connected to a remote Kermit  server,  these  commands  cause  the        remote  server  to  terminate;  'finish' returns it to Kermit or system        command level (depending on the implementation or how the  program  was        invoked); 'bye' also requests it to log itself out.                        THE 'LOG' AND 'CLOSE' COMMANDS:Syntax: log {debugging, packets, session, transactions} [ fn1 ]C-Kermit's  progress  may be logged in various ways.  The 'log' command opens alog, the 'close' command closes it.  In addition, all open logs are  closed  bythe 'exit' and 'quit' commands.  A name may be specified for a log file; if thename is omitted, the file is created with a default name as shown below.log debugging    This produces a voluminous log of the internal workings of C-Kermit, of use    to Kermit developers or maintainers in tracking down suspected bugs in  the    C-Kermit  program.   Use of this feature dramatically slows down the Kermit    protocol.  Default name: debug.log.log packets    This  produces  a  record of all the packets that go in and out of the com-    munication port.  This log is of use to Kermit maintainers who are tracking    down  protocol  problems  in either C-Kermit or any Kermit that C-Kermit is    connected to.  Default name:  packet.log.log session    This  log  will  contain a copy of everything you see on your screen during    the 'connect' command, except for local messages or interaction with  local    escape commands.  Default name:  session.log.log transactions    The transaction log is a record of all the files that were sent or received    while  transaction  logging  was  in  effect.   It includes time stamps and    statistics, filename transformations, and records of any  errors  that  may    have occurred.  The transaction log allows you to have long unattended file    transfer sessions without  fear  of  missing  some  vital  screen  message.    Default name:  transact.log.The 'close' command explicitly closes a log, e.g. 'close debug'.Note:    Debug  and Transaction logs are a compile-time option; C-Kermit may becompiled without these logs, in which case it will run faster, it will take  upless space on the disk, and the commands relating to them will not be present.                        LOCAL FILE MANAGEMENT COMMANDS:Unix  Kermit  allows some degree of local file management from interactive com-mand level:directory [fn]    Displays  a listing of the names, modes, sizes, and dates of files matching    fn (which defaults to `*').  Equivalent to `ls -l'.cwd [directory-name]    Changes  Kermit's  working  directory  to  the one given, or to the default    directory if the directory name is omitted.  This command affects only  the    Kermit process and any processes it may subsequently create.space    Display information about disk space and/or quota in the current  directory    and device.! [command]    The command is executed by the Unix shell.  If  no  command  is  specified,    then  an  interactive  shell  is  started;  exiting from the shell, e.g. by    typing Control-D or 'exit', will return you to C-Kermit command level.  Use    the  `!'  command  to  provide  file  management or other functions not ex-    plicitly provided by C-Kermit  commands.    The  `!'  command  has  certain    peculiarities:       - C-Kermit attempts to use your preferred, customary (login) shell.       - At least one space must separate the '!' from the shell command.       - A  'cd'  (change  directory) command executed in this manner will         have no effect -- use the C-Kermit 'cwd' command instead.                        THE 'SET' AND 'SHOW' COMMANDS:Since Kermit is designed to allow diverse systems to communicate, it  is  oftennecessary  to  issue  special  instructions  to  allow  the program to adapt topeculiarities of the another system or the communication path.  These  instruc-tions are accomplished by the 'set' command.  The 'show' command may be used todisplay current settings.  Here is a brief synopsis

of  settings  available  inthe current release of C-Kermit:block-check {1, 2, 3}    Determines the level of per-packet error  detection.    "1"  is  a  single-    character  6-bit  checksum,  folded  to include the values of all bits from    each character.   "2"  is  a  2-character,  12-bit  checksum.    "3"  is  a    3-character,  16-bit  cyclic  redundancy check (CRC).  The higher the block    check, the better the error detection and correction  and  the  higher  the    resulting  overhead.   Type 1 is most commonly used; it is supported by all    Kermit implementations, and it has proven adequate in  most  circumstances.    Types  2  or  3  would  be used to advantage when transferring 8-bit binary    files over noisy lines.delay n    How  many  seconds  to  wait before sending the first packet after a 'send'    command.  Used in remote mode to give you time to escape back to your local    Kermit and issue a 'receive' command.  Normally 5 seconds.duplex {full, half}    For use during 'connect'.  Specifies  which  side  is  doing  the  echoing;    'full' means the other side, 'half' means C-Kermit must echo typein itself.escape-character cc    For use during 'connect' to get C-Kermit's attention.  The escape character    acts  as a prefix to an 'escape command', for instance to close the connec-    tion and return to C-Kermit or Unix  command  level.    The  normal  escape    character  is Control-Backslash (28).  The escape character is also used in    System III/V implementations  to  prefix  interrupt  commands  during  file    transfers.file {display, names, type, warning}    Establish various file-related parameters:    display {on, off}        Normally  'on';  when in local mode, display progress of file transfers        on the screen (stdout), and listen to the keyboard (stdin)  for  inter-        ruptions.    If  off (-q on command line) none of this is done, and the        file transfer may proceed in the background oblivious to any other work        concurrently done at the console terminal.    names {converted, literal}        Normally converted, which  means  that  outbound  filenames  have  path        specifications  stripped, lowercase letters raised to upper, tildes and        extra periods changed to X's, and an X inserted in front  of  any  name        that  starts  with  period.    Incoming  files  have  uppercase letters        lowered.  Literal means that none of these conversions are done; there-        fore,  any  directory  path  appearing in a received file specification        must exist and be write-accessible.  When literal naming is being used,        the  sender  should not use path names in the file specification unless        the same path exists on the target system and is writable.    type {binary, text} [{7, 8}]        The file type is normally text, which means that conversion is done be-        tween Unix newline  characters  and  the  carriage-return/linefeed  se-        quences  required by the canonical Kermit file transmission format, and        in common use on non-Unix systems.  Binary means to transmit file  con-        tents  without  conversion.   Binary (`-i' in command line notation) is        necessary for binary files, and desirable in  all  Unix-to-Unix  trans-        actions to cut down on overhead.        The  optional  trailing parameter tells the bytesize for file transfer.        It is 8 by default.  If you specify 7,  the  high  order  bit  will  be        stripped from each byte of sent and received files.  This is useful for        transferring text files that may have extraneous high order bits set in        their  disk  representation  (e.g.   Wordstar or similar word processor        files).    warning {on, off}        Normally  off,  which means that incoming files will silently overwrite        existing files of the same name.  When on (`-w' on command line) Kermit        will check if an arriving file would overwrite an existing file; if so,        it will construct a new name for the arriving file, of the form  foo~n,        where foo is the name they share and n is a "generation number"; if foo        exists, then the new file will be called foo~1.  If foo and  foo~1  ex-        ist,  the  new file will be foo~2, and so on.  If the new name would be        longer than the maximum length for a filename, then characters would be        deleted  from  the  end first, for instance, thelongestname on a system        with a limit of 14 characters would become thelongestn~1.            CAUTION:  If Control-F or Control-B is used to cancel an incom-            ing  file,  and a file of the same name previously existed, and            the "file warning" feature is not enabled,  then  the  previous            copy of the file will disappear.flow-control {none, xon/xoff}    Normally xon/xoff for full duplex flow control.  Should be set to 'none' if    the  other  system cannot do xon/xoff flow control, or if you have issued a    'set handshake' command.  If set to xon/xoff, then handshake should be  set    to  none.    This  setting applies during both terminal connection and file    transfer.  Warning: This command may have no effect on  certain  Unix  sys-    tems,  where Kermit puts the communication line into 'rawmode', and rawmode    precludes flow control.incomplete {discard, keep}    Disposition for incompletely received files.  If an incoming file is inter-    rupted or an error occurs during transfer, the part that  was  received  so    far  is  normally  discarded.   If you "set incomplete keep" then such file    fragments will be kept.handshake {xon, xoff, cr, lf, bell, esc, none}    Normally  none.  Otherwise, half-duplex communication line turnaround hand-    shaking is done, which means Unix Kermit will not reply to a  packet  until    it  has received the indicated handshake character or has timed out waiting    for it; the handshake setting applies only during file transfer.    If  you    set handshake to other than none, then flow should be set to none.line [device-name]    The device name for the communication line to be used for file transfer and    terminal connection, e.g. /dev/ttyi3.  If you specify a device name, Kermit    will be in local mode, and you should remember to issue any other necessary    'set'  commands,  such as 'set speed'.  If you omit the device name, Kermit    will revert to its default mode of operation.   If  you  specify  /dev/tty,    Kermit  will  enter  remote  mode  (useful when logged in through the "back    port" of a system normally used as a local-mode workstation).    When  Unix    Kermit  enters  local  mode, it attempts to synchronize with other programs    (like uucp) that use external communication lines  so  as  to  prevent  two    programs  using  the  same  line  at  once;  before  attempting to lock the    specified line, it will  close  and  unlock  any  external  line  that  was    previously  in  use.   The method used for locking is the "uucp lock file",    explained in more detail later.modem-dialer {direct, hayes, racalvadic, ventel, ...}    The type of modem dialer on the communication line.  "Direct" indicates ei-    ther there is no dialout modem, or that if the line requires carrier detec-    tion  to  open,  then  'set  line'  will hang waiting for an incoming call.    "Hayes", "Ventel", and the others indicate  that  'set  line'  (or  the  -l    argument)  will  prepare  for  a  subsequent  'dial'  command for the given    dialer.  Support for new dialers is added from time to time, so  type  'set    modem  ?'  for  a  list of those supported in your copy of Kermit.  See the    description of the 'dial' commandparity {even, odd, mark, space, none}    Specify  character  parity for use in packets and terminal connection, nor-    mally none.  If other than none, C-Kermit will  seek  to  use  the  8th-bit    prefixing  mechanism  for transferring 8-bit binary data, which can be used    successfully only if the other Kermit agrees; if  not,  8-bit  binary  data    cannot be successfully transferred.prompt [string]    The given string will be substituted  for  "C-Kermit>"  as  this  program's    prompt.    If the string is omitted, the prompt will revert to "C-Kermit>".    If the string is enclosed in doublequotes, the quotes will be stripped  and    any leading and trailing blanks will be retained.send parameter    Establish parameters to use when sending packets.  These will be in  effect    only for the initial packet sent, since the other Kermit may override these    parameters during the protocol parameter exchange (unless noted below).    end-of-packet cc        Specifies the control character needed by the other Kermit to recognize        the end of a packet.  C-Kermit sends this character at the end of  each        packet.    Normally  13  (carriage return), which most Kermit implemen-        tations require.  Other Kermits require no  terminator  at  all,  still        others may require a different terminator, like linefeed (10).    packet-length n        Specify the maximum packet length to  send.    Normally  90.    Shorter        packet  lengths  can be useful on noisy lines, or with systems or front        ends or networks that have small buffers.  The shorter the packet,  the        higher  the  overhead,  but the lower the chance of a packet being cor-        rupted by noise, and the less time  to  retransmit  corrupted  packets.        This  command  overrides the value requested by the other Kermit during        protocol initiation unless the other Kermit requests a shorter length.    pad-character cc        Designate  a  character  to send before each packet.  Normally, none is        sent.  Outbound padding

is sometimes necessary for  communicating  with        slow half duplex systems that provide no other means of line turnaround        control.  It can also be used to send special characters to  communica-        tions  equipment  that  needs  to  be put in "transparent" or "no echo"        mode, when this can be accomplished in by feeding it a certain  control        character.    padding n        How many pad characters to send, normally 0.    start-of-packet cc        The  normal Kermit packet prefix is Control-A (1); this command changes        the prefix C-Kermit puts on outbound packets.  The  only  reasons  this        should  ever be changed would be: Some piece of equipment somewhere be-        tween the two Kermit programs will not pass through  a  Control-A;  or,        some  piece  of of equipment similarly placed is echoing its input.  In        the latter case, the recipient of such an echo can  change  the  packet        prefix for outbound packets to be different from that of arriving pack-        ets, so that the echoed packets will be ignored.  The  opposite  Kermit        must also be told to change the prefix for its inbound packets.    timeout n        Specifies the number of seconds you want the other Kermit to wait for a        packet before timing it out and requesting retransmission.receive parameter    Establish parameters to request the other Kermit to use when sending  pack-    ets.    end-of-packet cc        Requests the other Kermit to terminate its packets with  the  specified        character.    packet-length n        Specify the maximum packet length to that you want the other Kermit  to        send,  normally  90.  If you specify a length of 95 or greater, then it        will be used if the other Kermit supports, and agrees to use, the  Ker-        mit  protocol  extension  for  long packets.  In this case, the maximum        length depends upon the systems involved, but there would  normally  be        no  reason for packets to be more than about 1000 characters in length.        The 'show parameters' command displays C-Kermit's current  and  maximum        packet lengths.    pad-character cc        C-Kermit normally does not need to have incoming packets preceded  with        pad characters.  This command allows C-Kermit to request the other Ker-        mit to use cc as a pad character.  Default cc is NUL, ASCII 0.    padding n        How many pad characters to ask for, normally 0.    start-of-packet cc        Change the prefix C-Kermit looks for on inbound packets  to  correspond        with what the other Kermit is sending.    timeout n        Normally, each Kermit partner sets its packet timeout interval based on        what the opposite Kermit requests.  This command allows you to override        the normal procedure and specify a timeout interval for Unix Kermit  to        use  when waiting for packets from the other Kermit.  If you specify 0,        then no timeouts will occur, and Unix Kermit will wait forever for  ex-        pected packets to arrive.speed {0, 110, 150, 300, 600, 1200, 1800, 2400, 4800, 9600}    The baud rate for the external communication line.  This command cannot  be    used  to  change the speed of your own console terminal.  Many Unix systems    are set up in such a way that you must give this command after a 'set line'    command  before  you  can  use  the line.  'set baud' is a synomym for 'set    speed'.terminal    Used for specifying terminal parameters.  Currently, 'bytesize' is the only    parameter provided, and it can be set to 7 or 8.  It's 7 by default.                              THE 'SHOW' COMMAND:Syntax: show {parameters, versions}The "show" command with the  default  argument  of  "parameters"  displays  thevalues  of  all  the  'set'  parameters  described  above.    If you type "showversions", then C-Kermit will display the version numbers and dates of all  itsinternal  modules.  You should use the "show versions" command to ascertain thevintage of your Kermit program before reporting problems to Kermit maintainers.                           THE 'STATISTICS' COMMAND:The statistics command  displays  information  about  the  most  recent  Kermitprotocol transaction, including file and communication line i/o, timing and ef-ficiency, as well as what encoding options were  in  effect  (such  as  8th-bitprefixing, repeat-count compression).                        THE 'TAKE' AND 'ECHO' COMMANDS:Syntax: take fn1     echo [text to be echoed]The  'take' command instructs C-Kermit to execute commands from the named file.The file may contain any interactive C-Kermit commands, including 'take';  com-mand  files  may be nested to any reasonable depth, but it may not contain textto be sent to a remote system during the 'connect' command.  This means that  acommand file like this:    set line /dev/tty17    set speed 9600    connect    login myuserid    mypassword    etcwill  not  send "login myserid" or any of the following text to the remote sys-tem.  To carry on a canned dialog, use the 'script' command, described later.The '%' command is useful for including comments in take-command files.  It mayonly be used at the beginning of a line.The  'echo'  command  may  be used within command files to issue greetings, an-nounce progress, ring the terminal bell, etc.  The 'echo' command should not beconfused  with  the  Unix  'echo'  command,  which can be used to show how metacharacters would be expanded.  The Kermit echo command simply displays its textargument (almost) literally at the terminal; the argument may contain octal es-capes of the form "\ooo", where o is an octal digit (0-7), and there may be  1,2, or 3 such digits, whose value specify an ASCII character, such as "\007" (or"\07" or just "\7") for beep, "\012" for newline, etc.  Of course,  each  back-slash  must  be must be entered twice in order for it to be passed along to theecho command by the Kermit command parser.Take-command files are in exactly the  same  syntax  as  interactive  commands.Note  that  this  implies  that  if you want to include special characters likequestion mark or backslash that you would have to  quote  with  backslash  whentyping  interactive  commands,  you must quote these characters the same way incommand files.  Long lines may be continued by ending them with a single  back-slash.Command files may be used in lieu of command macros, which have not been imple-mented in this version of C-Kermit.  For instance, if you commonly connect to asystem  called  'B' that is connected to ttyh7 at 4800 baud, you could create afile called b containing the commands    % C-Kermit command file to connect to System B thru /dev/ttyh7    set line /dev/ttyh7    set speed 4800    % Beep and give message    echo \\007Connecting to System B...    connectand then simply type 'take b' (or 't b' since no other commands begin with  theletter  't')  whenever  you wish to connect to system B. Note the comment linesand the beep inserted into the 'echo' command.For connecting to IBM mainframes, a number  of  'set'  commands  are  required;these, too, can be conveniently collected into a 'take' file like this one:    % Sample C-Kermit command file to set up current line    % for IBM mainframe communication    %    set parity mark    set handshake xon    set flow-control none    set duplex halfNote  that no single command is available to wipe out all of these settings andreturn C-Kermit to its default startup state; to do that, you can  either  res-tart the program, or else make a command file that executes the necessary 'set'commands:    % Sample C-Kermit command file to restore normal settings    %    set parity none    set handshake none    set flow-control xon/xoff    set duplex fullAn implicit 'take' command is executed upon your  .kermrc  file  when  C-Kermitstarts  up,  upon  either  interactive or command-line invocation.  The .kermrcfile should contain 'set' or other commands you want to be  in  effect  at  alltimes.   For instance, you might want override the default action when incomingfiles have the same names as existing files -- in that case, put the command     set file warning onin your .kermrc file.  On some non-Unix systems  that  run  C-Kermit,  the  in-itialization file might have a different name, such as kermit.ini.Errors  encountered during execution of take files (such as failure to completedial or script operations) cause termination of the current take file,  poppingto  the  level  that  invoked  it (take file, interactive level, or the shell).When kermit is executed in the background, errors during execution  of  a  takefile are fatal.Under  Unix,  you  may  also  use  the  shell's  redirection mechanism to causeC-Kermit to execute commands from a file:      kermit < cmdfileor you can even pipe commands in from another process:      cmdprocess | kermit                            THE 'CONNECT' COMMAND:The 'connect'  command  ('c'  is  an  acceptable  non-unique  abbreviation  for'connect')  links  your terminal to another computer as if it were a local ter-minal to that computer, through the device specified in the  most  recent  'setline' command, or through the default device if your system is a PC or worksta-tion.  All characters you type at your keyboard are sent out the  communicationline  (and  if  you have 'set duplex half', also displayed on your screen), andall characters arriving at the communication port are displayed on the  screen.Current  settings  of  speed, parity, duplex, and flow-control are honored, andthe data connection is 7 bits wide unless you have given the command 'set  ter-minal  bytesize 8'.  If you have issued a 'log session'

command, everything yousee on your screen will also be recorded to your session log.  This provides  away  to  "capture"  files  from  remote systems that don't have Kermit programsavailable.To get back to your own system, you must type the escape  character,  which  isControl-Backslash  (^\)  unless  you have changed it with the 'set escape' com-mand,  followed  by  a  single-character  command,  such  as  'c'  for   "closeconnection".  Single-character commands include:  c     Close the connection  b     Send a BREAK signal  0     (zero) send a null  s     Give a status report about the connection  h     Hangup the phone  ^\    Send  Control-Backslash  itself  (whatever  you have defined the escape        character to be, typed twice in a row sends one copy of it).Uppercase and control equivalents for (most of)  these  letters  are  also  ac-cepted.    A  space  typed  after  the  escape character is ignored.  Any othercharacter will produce a beep.The connect command simply displays incoming characters on the screen.   It  isassumed  any  screen  control sequences sent by the host will be handled by thefirmware or emulation software in your terminal or PC.    If  special  terminalemulation is desired, then the 'connect' command can invoked from the Unix com-mand line (-c or -n), piped through a terminal emulation filter, e.g.      kermit -l /dev/acu -b 1200 -c | tek                              THE 'DIAL' COMMAND:Syntax: dial telephone-number-stringThis command controls dialout modems; you should have  already  issued  a  "setline"  and  "set  speed"  command  to  identify the terminal device, and a "setmodem" command to identify the type of modem to be used for dialing.    In  the"dial"  command, you supply the phone number and the Kermit program feeds it tothe modem in the appropriate format and then interprets dialer return codes andmodem  signals  to  inform  you whether the call was completed.  The telephone-number-string may contain imbedded modem-dialer commands,  such  as  comma  forHayes  pause, or `&' for Ventel dialtone wait and `%' for Ventel pause (consultyour modem manual for details).At the time of this writing, support is included for the following modems:   - AT&T 7300 Internal Modem   - Cermetek Info-Mate 212A   - Concord Condor CDS 220   - DEC DF03-AC   - DEC DF100 Series   - DEC DF200 Series   - General DataComm 212A/ED   - Hayes Smartmodem 1200 and compatibles   - Penril   - Racal Vadic   - US Robotics 212A   - VentelSupport for new modems is added to the program from time to time; you can checkthe current list by typing "set modem ?".The  device  used  for  dialing out is the one selected in the most recent "setline" command (or on a workstation, the default line if no "set  line"  commandwas  given).   The "dial" command calls locks the path (see the section on linelocking below) and establishes a call on an exclusive basis.  If it is  desiredto  dial  a  call and then return to the shell (such as to do kermit activitiesdepending on standard in/out redirection), it is necessary to place the  dialedcall  under  one  device  name  (say, "/dev/cua0") and then escape to the shellwithin Kermit on a linked device which is separate from the dialed  line  (say,"/dev/cul0").    This  is the same technique used by uucp (to allow locks to beplaced separately for dialing and conversing).Because modem dialers have strict requirements to override  the  carrier-detectsignal most Unix implementations expect, the sequence for dialing is more rigidthan most other C-Kermit procedures.Example one:    kermit -l /dev/cul0 -b 1200    C-Kermit>set modem-dialer hayes    hint: abbreviate set m h    C-Kermit>dial 9,5551212    Connected!    C-Kermit>connect                   hint: abbreviate c    logon, request remote server, etc.    ^\c                                escape back    C-Kermit> ...    C-Kermit>quit                      hint: abbreviate qthis disconnects modem, and unlocks line.Example two:    kermit    C-Kermit>set modem-dialer ventel    C-Kermit>set line /dev/cul0    C-Kermit>dial 9&5551212%    Connected!    C-Kermit> ...Example three:    kermit    C-Kermit>take my-dial-procedure    Connected!    file my-dial-procedure:    set modem hayes    set line /dev/tty99    dial 5551212    connectIn general, C-Kermit requires that the modem provide the "carrier detect"  (CD)signal  when  a  call is in progress, and remove that signal when the call com-pletes or the line drops.  If a modem switch setting is available to force  CD,it  should  normally  not  be in that setting.  C-Kermit also requires (on mostsystems) that the modem track the computer's "data terminal ready"  (DTR)  sig-nal.    If  a  switch  setting is available to simulate DTR asserted within themodem, then it should normally not be in that setting.    Otherwise  the  modemwill  be unable to hang up at the end of a call or when interrupts are receivedby Kermit.For Hayes  dialers, two important switch settings are #1 and  #6.    Switch  #1should be normally be UP so that the modem can act according to your computer'sDTR signal.  But if your computer, or particular implementation of Kermit, can-not control DTR, then switch 1 should be DOWN.  Switch #6 should normally be UPso carrier-detect functions properly (but put it DOWN if you have trouble  withthe  UP  position).    Switches  #2  (English versus digit result codes) and #4(Hayes echoes modem commands) may be in either position.If you want to interrupt a dial command in progress (for instance, because  youjust  realize  that you gave it the wrong number), type a Control-C to get backto command level.                             THE 'SCRIPT' COMMAND:Syntax: script expect send [expect send] . . ."expect" has the syntax: expect[-send-expect[-send-expect[...]]]The 'script' command carries on a "canned dialog"  with  a  remote  system,  inwhich data is sent according to the remote system's responses.  The typical useis for logging in to a remote system automatically.C-Kermit's script facility operates in a manner similar to that  commonly  usedby  the  Unix UUCP system's "L.sys" file entries.  A login script is a sequenceof the form:      expect send [expect send] . . .where expect is a prompt or message to be issued by the remote site,  and  sendis  the  string (names, numbers, etc) to return, and expects are separated fromsends by spaces.  The send may also be the keyword EOT, to send  Control-D,  orBREAK, to send a break signal.  Letters in sends may be prefixed by `~' to sendspecial characters, including:    ~b  backspace    ~s  space    ~q  `?'(trapped by Kermit's command interpreter)    ~n  linefeed    ~r  carriage return    ~t  tab    ~'  single quote    ~~  tilde    ~"  double quote    ~x  XON (Control-Q)    ~c  don't append a carriage return    ~o[o[o]]  an octal character    ~d  delay approx 1/3 second during send    ~w[d[d]]  wait specified interval during expect, then time outAs with some UUCP systems, sent strings are followed by ~r unless they  have  a~c.Only  the last 7 characters in each expect are matched.  A null expect, e.g. ~0or two adjacent dashes, causes a short delay before proceeding to the next sendsequence.  A null expect always succeeds.As  with  UUCP, if the expect string does not arrive, the script attempt fails.If you expect that a sequence might not arrive, as with UUCP,  conditional  se-quences may be expressed in the form:      -send-expect[-send-expect[...]]where dashed sequences are followed as long as previous expects fail.  Timeoutsfor expects can be specified using ~w; ~w with no arguments waits 15 seconds.Expect/send transactions can be easily be  debugged  by  logging  transactions.This  records  all  exchanges,  both expected and actual.  The script executionwill also be logged in the session log, if that is activated.Note that `\' characters in login scripts, as in any other C-Kermit interactivecommands,  must  be doubled up.  A line may be ended with a single `\' for con-tinuation.Example one:Using a modem, dial a UNIX host site.   Expect  "login"  (...gin),  and  if  itdoesn't come, simply send a null string with a ~r.  (Some Unixes require eitheran EOT or a BREAK instead of the null sequence,  depending  on  the  particularsite's "logger" program.)  After providing user id and password, respond "x" toa question-mark prompt, expect the Bourne shell "$" prompt (and send return  ifit  doesn't  arrive).   Then cd to directory kermit, and run the program called"wermit", entering the interactive connect state after wermit is loaded.    set modem ventel    set line /dev/tty77    set baud 1200    dial 9&5551212    script gin:--gin:--gin: smith ssword: mysecret ~q x $--$ \     cd~skermit $ wermit    connectNote that 'set line' is issued after 'set modem',  but  before  'set  baud'  orother line-related parameters.Example two:Using  a  modem,  dial the Telenet network.  This network expects three returnswith slight delays between them.  These are sent following null expects.    Thesingle return is here sent as a null string, with a return appended by default.Four returns are sent to be safe before looking  for  the  prompt.    Then  theTelenet  id and password are entered.  Then Telenet is instructed to connect toa host site (c 12345).  The host has a data switch that  asks  "which  system";the  script responds "myhost" (if the "which system" prompt doesn't appear, theTelenet connect command is reissued).  The script waits for an "@" prompt  fromthe  host,  then  sends  the user ID ("joe") and password ("secret"), looks foranother "@" prompt, runs Kermit, and in response to the Kermit's prompt  (whichends  in  ">"),  gives  the commands

"set parity even" and "server".  Files arethen exchanged.  The commands are in a take file; note the continuation of  the'script' command onto several lines using the `\' terminator.    set modem hayes    set line /dev/acu    set speed 1200    set parity mark    dial 9,5551212    script ~0 ~0 ~0 ~0 ~0 ~0 ~0 ~0 @--@--@ id~saa001122 = 002211 @ \        c~s12345 ystem-c~s12345-ystem myhost @ joe~ssecret @ kermit \        > set~sparity~seven > server    send some.stuff    get some.otherstuff    bye    quitSince  these  commands may be executed totally in the background, they can alsobe scheduled.  A typical shell script, which might be scheduled by cron,  wouldbe as follows (csh used for this example):    #    #keep trying to dial and log onto remote host and exchange files    #wait 10 minutes before retrying if dial or script fail.    #    cd someplace    while ( 1 )            kermit < /tonight.cmd >> nightly.log &            if ( ! $status ) break            sleep 600    endFile  tonight.cmd  might  have two takes in it, for example, one to take a filewith the set modem, set line, set baud, dial, and script, and a second take  ofa  file  with  send/get  commands  for  the  remote  server.  The last lines oftonight.cmd should be a bye and a quit.                              THE 'HELP' COMMAND:Syntax: help   or: help keyword   or: help {set, remote} keywordBrief help messages or menus are always available at interactive command  levelby  typing  a question mark at any point.  A slightly more verbose form of helpis available through the 'help' command.  The 'help' command with no  argumentsprints  a  brief  summary of how to enter commands and how to get further help.'help' may be followed by one of the top-level C-Kermit command keywords,  suchas  'send', to request information about a command.  Commands such as 'set' and'remote' have a further level of help.  Thus you may type 'help',  'help  set',or  'help  set parity'; each will provide a successively more detailed level ofhelp.                        THE 'EXIT' AND 'QUIT' COMMANDS:These two commands are identical.  Both of them do the following:   - Attempt to insure that the terminal is returned to normal.   - Relinquish access to any communication line assigned via 'set line'.   - Relinquish any uucp and multiuser locks on the communications line.   - Hang up the modem, if the communications line supports data  terminal     ready.   - Close any open logs or other files.After  exit  from C-Kermit, your default directory will be the same as when youstarted the program.  The 'exit' command is issued implicitly whenever C-Kermithalts normally, e.g. after a command line invocation, or after certain kinds ofinterruptions.1.5. UUCP Lock FilesUnix has no standard way of obtaining exclusive  access  to  an  external  com-munication  line.    When you issue the 'set line' command to Unix Kermit, Unixwould normally grant you access to the line  even  if  some  other  process  ismaking  use  of  it.    The  method adopted by most Unix systems to handle thissituation is the "UUCP lock  file".    UUCP,  the  Unix-to-Unix  Copy  program,creates  a  file  in  its  directory  (usually /usr/spool/uucp, on some systems/etc/locks) with a name like LCK..name, where name is the device name, for  in-stance tty07.Unix Kermit uses UUCP lock files in order to avoid conflicts with UUCP, tip, orother programs that follow this convention.  Whenever you attempt to access  anexternal  line using the 'set line' command or `-l' on the command line, Kermitlooks in the UUCP directory for a lock file corresponding to that device.   Forinstance, if you 'set line /dev/ttyi6' then Kermit looks for the file     /usr/spool/uucp/LCK..ttyi6If it finds this file, it gives you an error message and a directory listing ofthe file so that you can see who is using it, e.g.    -r--r--r--  1 fdc        4 May  7 13:02 /usr/spool/uucp/LCK..ttyi6In this case, you would look up user fdc to find out how soon the line will be-come free.This  convention  requires  that  the  uucp  directory be publicly readable andwritable.  If it is not, the program will issue an appropriate warning message,but will allow you to proceed at your own risk (and the risk of anyone else whomight also be using the same line).If no lock file is found, Unix Kermit will attempt create one, thus  preventinganyone  who subsequently tries to run Kermit, UUCP, tip, or similar programs onthe same line from gaining access until you release the line.  If Kermit  couldnot  create  the  lock  file (for instance because the uucp directory is write-protected), then you will receive a warning message  but  will  be  allowed  toproceed  at  your  -- and everyone else's -- risk.  When Kermit terminates nor-mally, your lock file is removed.Even when the lock directory is writable and readable,  the  locking  mechanismdepends  upon  all  users using the same name for the same device.  If a devicehas more than one path associated with it, then a lock can be  circumvented  byusing an alias.When a lock-creating program abruptly terminates, e.g. because it crashes or iskilled via shell  command,  the  lock  file  remains  in  the  uucp  directory,spuriously  indicating  that  the line is in use.  If the lock file is owned byyourself, you may remove it.  Otherwise, you'll have to get the  owner  or  thesystem manager to remove it, or else wait for a system task to do so; uucp sup-ports a function (uuclean) which removes these files after a predetermined  age-- uucp sites tend to run this function periodically via crontab.Locking  is  not needed, or used, if communications occur over the user's loginterminal line (normally /dev/tty).It may be seen that line locking is fraught with peril.  It is included in UnixKermit  only because other Unix communication programs rely on it.  While it isnaturally desirable to assure exclusive access  to  a  line,  it  is  also  un-desirable  to  refuse  access  to a vacant line only because of a spurious lockfile, or because the uucp directory is not appropriately protected.1.6. C-Kermit under Berkeley or System III/V Unix:C-Kermit may be interrupted at command level or during file transfer by  typingControl-C.    The  program will perform its normal exit function, restoring theterminal and releasing any lock.  If a protocol transaction was in progress, anerror  packet  will  be  sent  to  the opposite Kermit so that it can terminatecleanly.C-Kermit may be invoked in the background ("&" on shell commmand line).   If  abackground  process is "killed", the user will have to manually remove any lockfile and may need to restore the modem.    This  is  because  the  kill  signal(kill(x,9)) cannot be trapped by Kermit.During execution of a system command ('directory', 'cwd', or `!'), C-Kermit canoften be returned to command level by typing a single Control-C.  (With  SystemIII/V,  the  usual  interrupt  function  (often  the  DEL  key)  is replaced byControl-C.)Under Berkeley Unix only: C-Kermit may also be interrupted by  ^Z  to  put  theprocess  in  the  background.   In this case the terminal is not restored.  Youwill have to type Control-J followed by "reset" followed by  another  Control-Jto get your terminal back to normal.Control-C, Control-Z, and Control-\ lose their normal functions during terminalconnection and also during file transfer when the controlling tty line is beingused for packet i/o.If  you are running C-Kermit in "quiet mode" in the foreground, then interrupt-ing the program with a console interrupt like Control-C will  not  restore  theterminal  to  normal conversational operation.  This is because the system callto enable console interrupt traps will cause the program to block if it's  run-ning  in  the background, and the primary reason for quiet mode is to allow theprogram to run in the background without blocking, so that  you  can  do  otherwork in the foreground.If C-Kermit is run in the background ("&" on shell commmand line), then the in-terrupt signal (Control-C) (and System III/V quit signal) are  ignored.    Thisprevents  an interrupt signal intended for a foreground job (say a compilation)from being trapped by a background Kermit session.1.7. C-Kermit on the DEC Pro-3xx with Pro/Venix Version 1The DEC Professional 300 series are PDP-11/23 based personal computers.   VenixVersion  1  is a Unix v7 derivative.  It should not be confused with Venix Ver-sion 2, which is based on ATT System V; these comments apply to Venix Version 1only.    C-Kermit  runs in local mode on the Pro-3xx when invoked from the con-sole; the default device is /dev/com1.dout.  When connected to a remote  system(using  C-Kermit's  'connect'  command), Pro/Venix itself (not Kermit) providesVT52 terminal emulation.  Terminal operation at high speeds  (like  9600  baud)requires  xon/xoff  flow  control, which unfortunately interferes with applica-tions such as the EMACS that use Control-Q and Control-S as commands.When logging in to a Pro-3xx (or any workstation) through the "back  port",  itmay  be  necessary  to  give  the  command  "set line /dev/tty" in order to getC-Kermit to function correctly in remote mode (on a system in which it normallyexpects to be operating in local mode).1.8. C-Kermit under VAX/VMSC-Kermit  can  be  built using VAX-11 C to run under VMS.  Most of the descrip-tions in this manual hold true, but it should be noted that as of this  writingthe  VMS  support  is not thoroughly tested, and no explicit support exists forthe various types of VMS files and their attributes.The C-Kermit init file for VMS is called KERMIT.INI.1.9. C-Kermit on the Macintosh and other SystemsThe "protocol kernel" of C-Kermit is

also used by Columbia's Macintosh  Kermit.The  user  and  system  interface  is  entirely  different, and is covered in aseparate document.There is also a Kermit for the Commodore Amiga based on C-Kermit,  as  well  asversions for MS-DOS, Data General operating systems, etc.1.10. C-Kermit Restrictions and Known Bugs   1. Editing  characters:    The program's interactive command interrupt,      delete, and kill characters are Control-C,  Delete  (or  Backspace),      and  Control-U,  respectively.   There is currently no way to change      them to suit your taste or match those used by your shell,  in  case      those are different.   2. Flow control:  C-Kermit attempts to use XON/XOFF flow control during      protocol operations, but it also puts the  communication  line  into      "rawmode".   On many systems, rawmode disables flow control, so even      though you may have "set flow xon/xoff", no  flow  control  will  be      done.  This is highly system and Unix-version dependent.   3. High  baud  rates:  There's no way to specify baud rates higher than      9600 baud.  Most Unix systems don't supply symbols for them  (unless      you  use  EXTA, EXTB), and even when they do, the program has no way      of knowing whether a specific port's serial i/o controller  supports      those rates.   4. Modem  controls:   If a connection is made over a communication line      (rather than on the controlling terminal line), and  that  line  has      modem  controls,  (e.g.  data  terminal  ready and carrier detection      implementation), returning to the shell level  will  disconnect  the      conversation.    In  that case, one should use interactive mode com-      mands, and avoid use of piped shell-level operation (also  see  'set      modem-dialer' and 'dial' commands.)   5. Login Scripts:  The present login scripts implementation follows the      Unix conventions of uucp's "L.sys" file, rather than the normal Ker-      mit "INPUT/OUTPUT" style.   6. Dial-out  vs  dial-in  communications  lines:    C-Kermit requires a      dial-out or dedicated line for the "set line" or "-l" options.  Most      systems  have  some  lines  dedicated  to dial-in, which they enable      "loggers" on,  and  some  lines  available  for  dial-out.    Recent      releases  of  Unix (ATT & Berkeley) have mechanisms for changing the      directionality of a line.   7. Using C-Kermit on Local Area Networks:   C-Kermit  can  successfully      operate  at  speeds  up to 9600 baud over LANs, provided the network      buffers are big enough to accommodate Kermit packets.      When computers are connected to LAN's through asynchronous  terminal      interfaces,  then the connection should be configured to do XON/XOFF      flow control between the network interface and the computer,  rather      than  passing  these  signals  through transparently.  This can help      prevent Kermit from overrunning the LAN's buffers if they are  small      (or if the LAN is congested), and will can also prevent the LAN from      overrunning a slow Kermit's buffers.      If the network hardware cannot accept 100 characters at a time,  and      flow  control  cannot  be done between the network and the computer,      then Kermit's "set send/receive packet-length" command can  be  used      to shorten the packets.   8. Resetting terminal after abnormal termination or kill: When C-Kermit      terminates abnormally (say, for example, by a kill command issued by      the  operator)  the  user  may need to reset the terminal state.  If      commands do not seem  to  be  accepted  at  the  shell  prompt,  try      Control-J  "stty  sane"  Control-J  (use  "reset" on Berkeley Unix).      That should take the terminal out of "raw  mode"  if  it  was  stuck      there.   9. Remote  host  commands  may  time-out  on  lengthy  activity:  Using      "remote host" to instruct the C-Kermit server to invoke  Unix  func-      tions  (like  "make")  that might take a long time to produce output      can cause timeout conditions.  10. XOFF deadlocks:  When connecting back to  C-Kermit  after  a  trans-      action, or after finishing the server, it may be necessary to type a      Control-Q to clear up an  XOFF  deadlock.    There's  not  much  the      program can do about this...1.11. How to Build C-Kermit for a Unix SystemThe  C-Kermit  files,  as  distributed from Columbia, all begin with the prefix"ck".  You should make a directory for these files  and  then  cd  to  it.    Amakefile  is  provided  to  build  C-Kermit for various Unix systems (there areseparate makefiles for VMS and the Macintosh).  As  distributed,  the  makefilehas  the  name  "ckuker.mak".  You should rename it to "makefile" and then type"make xxx", where xxx is the symbol for your system, for instance "make bsd" tomake  C-Kermit for 4.x BSD Unix.  The result will be a program called "wermit".You should test this to make sure it works; if it does, then you can rename  itto "kermit" and install it for general use.  See the makefile for a list of thesystems supported and the corresponding "make" arguments.1.12. Adapting C-Kermit to Other SystemsC-Kermit is designed for portability.  The level of portability is indicated inparentheses  after  the module name: "C" means any system that has a C compilerthat conforms to the description in "The C Programming Language" by Kernighan &Ritchie  (Prentice-Hall, 1978).  "Cf" is like "C", but also requires "standard"features like printf and fprintf, argument passing via argv/argc, and so on, asdescribed in Kernighan & Ritchie.  "Unix" means the module should be useful un-der any Unix implementation; it requires features such  as  fork()  and  pipes.Anything  else  means  that  the  module is particular to the indicated system.C-Kermit file names are of the form:ck<system><what>.<type>where the part before the dot is no more than 6 characters long, the part afterthe dot no more than 3 characters long, and:<type> is the file type:  c:    C language source  h:    Header file for C language source  w:    Wart preprocessor source, converted by Wart (or Lex) to a C program  nr:   Nroff/Troff text formatter source  mss:  Scribe text formatter source  doc:  Documentation  hlp:  Help text  bld:  Instructions for building the program  bwr:  A "beware" file - list of known bugs  upd:  Program update log  mak:  Makefile<system> is a single character to tell what system the file applies to:  a:    Descriptive material, documentation  c:    All systems with C compilers  d:    Data General  h:    Harris computers (reserved)  i:    Commodore Amiga (Intuition)  m:    Macintosh  p:    IBM PC, PC-DOS (reserved)  u:    Unix  v:    VAX/VMS  w:    Wart<what> is mnemonic (up to 3 characters) for what's in the file:  aaa:  A "read-me" file, like this one  cmd:  Command parsing  con:  Connect command  deb:  Debug/Transaction Log formats, Typedefs  dia:  Modem/Dialer control  fio:  System-depdendent File I/O  fns:  Protocol support functions  fn2:  More protocol support functions  ker:  General C-Kermit definitions, information, documentation  mai:  Main program  pro:  Protocol  scr:  Script command  tio:  System-dependent terminal i/o & control and interrupt handing  usr:  User interface  us2:  More user interface  us3:  Still more user interfaceExamples:ckufio.c        File i/o for Unixckmtio.c        Terminal i/o for Macintoshckuker.mss      Scribe source for for Kermit User Guide chapterckuker.nr       Nroff source file for Unix C-Kermit man pageThe following material discusses each of the C-Kermit modules briefly.ckcmai.c, ckcker.h, ckcdeb.h (Cf):    This is the main program.  It contains declarations  for  global  variables    and a small amount of code to initialize some variables and invoke the com-    mand parser.  In its distributed form, it assumes  that  command  line  ar-    guments  are passed to it via argc and argv.  Since this portion of code is    only several lines long, it should be easy to replace for systems that have    different  styles of user interaction.  The header files define symbols and    macros used by the various modules of  C-Kermit.    ckcdeb.h  is  the  only    header  file  that  is included by all the C-Kermit modules, so it contains    not only the debug format  definitions,  but  also  any  compiler-dependent    typedefs.ckwart.c (Cf), ckcpro.w (C):    The ckcpro module embodies the Kermit protocol state table and the code  to    accomplish  state switching.  It is written in "wart", a language which may    be regarded as a subset of the Unix "lex" lexical analyzer generator.  Wart    implements  enough  of lex to allow the ckprot module to function.  Lex it-    self was not used because it is proprietary.  The protocol module  ckcpro.w    is  read by wart, and a system-independent C program is produced.  The syn-    tax of a Wart program is illustrated  by  ckcpro.w,  and  is  described  in    ckwart.doc.ckcfns.c (C):    The module contains all the Kermit protocol  support  functions  --  packet    formation,  encoding,  decoding, block check calculation, filename and data    conversion, protocol parameter negotiation, and high-level interaction with    the communication line and file system.  To accommodate small systems, this    module has been split into two -- ckcfns.c and ckcfn2.c.ckutio.c:    This module contains the system-dependent primitives for communication line    i/o, timers, and interrupts for the various versions of Unix.  Certain  im-    portant  variables  are  defined  in  this  module, which determine whether    C-Kermit is by default remote or  local,  what  the  default  communication    device is, and so forth.  The tio module maintains

its own private database    of file descriptors and modes for the console terminal and the file  trans-    fer  communication  line so that other modules (like ckcfns or the terminal    connect module) need not be concerned with them.  The variations among Unix    implementations  with  respect  to  terminal  control and timers are accom-    modated via conditional compilation.ckufio.c:    This  module  contains  system-dependent  primitives for file i/o, wildcard    (meta character) expansion, file existence and access checking, and  system    command execution for the various versions of Unix.  It maintains an inter-    nal database of i/o "channels" (file pointers in this case) for  the  files    C-Kermit  cares about -- the input file (the file which is being sent), the    output file (the file being received), the various logs, the screen, and so    forth.  This module varies little among Unix implementations except for the    wildcard expansion code; the directory structure of  4.2bsd  Unix  is  dif-    ferent  from  that of other Unix systems.  Again, variation among Unix sys-    tems is selected using conditional compilation.ckuusr.h, ckuusr.c, ckuus2.c, ckuus3.c (Unix):    This is the "user interface" for C-Kermit.  It includes the command parser,    the screen output functions, and console input functions.  The command par-    ser comes in two pieces -- the traditional Unix command line decoder (which    is quite small and compact), and the interactive keyword parser  (which  is    rather large).  This module is fully replacable; its interface to the other    modules is very simple, and is explained at the  beginning  of  the  source    file.    The  ckuusr  module  also  includes  code  to execute any commands    directly which don't require the Kermit protocol -- local file  management,    etc.    The  module  is rated "Unix" because it makes occasional use of the    system() function.    Note that while ckuusr is logically one module, it has been split  up  into    three C source files, plus a header file for the symbols they share in com-    mon.  This is to accommodate small systems that cannot handle big  modules.    ckuusr.c  has  the  command  line and top-level interactive command parser;    ckuus2.c has the help command and strings; ckuus3 has the  set  and  remote    commands along with the logging, screen, and "interrupt" functions.ckucmd.c, ckucmd.h (Cf):    This is an interactive command parsing package developed for C-Kermit.   It    is written portably enough to be usable on any system that has a C compiler    that supports functions like printf.    The  file  name  parsing  functions    depend  upon primitives defined in the fio module; if these primitives can-    not be supplied for a certain system, then the filename  parsing  functions    can  be deleted, and the package will still be useful for parsing keywords,    numbers, arbitrary text strings, and so forth.  The style of interaction is    the same as that found on the DECSYSTEM-20.ckucon.c (Unix):    This is the connect module.  As supplied, it should operate in any Unix en-    vironment,  or  any  C-based environment that provides the fork() function.    The module requires access to global variables  that  specify  line  speed,    parity,  duplex,  flow  control,  etc,  and  invokes functions from the tio    module to accomplish the desired settings and input/output,  and  functions    from  the  fio module to perform session logging.  No terminal emulation is    performed, but since standard i/o is used for  the  console,  this  may  be    piped  through a terminal emulation filter.  The ckucon function may be en-    tirely replaced, so long as the global settings are honored by its replace-    ment.    PC implementations of C-Kermit may require the ck?con module to do    screen control, escape sequence interpretation, etc, and may also  wish  to    write special code to get the best possible performance.ckudia.c (Unix):    This is the dialer module.  As supplied, it handles Hayes, Ventel,  Penril,    Racal-Vadic, and several other modems.ckuscr.c (Unix):    This is the login script  module.    As  supplied,  it  handles  uucp-style    scripts.Moving C-Kermit to a new system entails:   1. Creating  a  new ck?tio module in C, assembler, or whatever language      is most appropriate for system programming on the new  system.    If      the  system  is  Unix-like,  then  support  may  be added within the      ckutio.c module itself using conditional compilation.   2. Creating a new ck?fio module, as above.   3. If the system is not Unix-like, then a new ckuusr module may be  re-      quired, as well as a different invocation of it from ckcmai.   4. If  the  distributed connect module doesn't work or performs poorly,      then it may be replaced.  For instance, interrupt-driven i/o may  be      required, especially if the system doesn't have forks.Those  who  favor  a  different  style  of  user/program  interaction from thatprovided in ckuusr.c may replace the entire module, for instance with one  thatprovides a mouse/window/icon environment, a menu/function-key environment, etc.A few guidelines should be followed to maintain portability:   - Keep  variable and function names to 6 characters or less.  Don't use     identifiers that are distinguished  from  one  another  only  by  al-     phabetic case.   - Keep  modules  small.    For instance, on a PDP-11 it is necessary to     keep the code segment of each module below 8K in order to  allow  the     segment  mapping  to  occur which is necessary to run programs larger     than 64K on a non-I-and-D-space machine.   - Keep strings short; many compilers have restrictive maximum  lengths;     128  is the smallest maximum string constant length we've encountered     so far.   - Keep (f,s)printf formats short.  If these exceed  some  compiler  de-     pendent maximum (say, 128) memory will be overwritten and the program     will probably core dump.   - Do not introduce system dependencies into ckcpro.w or ckcfn*.c.   - If a variable is a character, declare as CHAR, not  int,  to  prevent     the  various sign extension and byte swapping foulups that occur when     characters are placed in integer variables.   - Remember that different systems may use different  length  words  for     different  things.  Don't assume an integer can be used as a pointer,     etc.   - Don't declare static functions; these can wreak  havoc  with  systems     that do segment mapping.   - In  conditional  compilations expressions, use #ifdef and #ifndef and     not #if, which is not supported by some compilers.  Also,  don't  use     any  operators  in these expressions; many compilers will fail to un-     derstand expressions like #ifdef FOO | BAR.  Also, don't put trailing     tokens on #else's or #endif's (use /* comments */).   - Don't define multiline macros.In  general,  remember that this program will have to be compilable by old com-pilers and runnable on small systems.