XDC - External Device Compiler - and

DDL - Device Definition Language

Contents:

1  Introduction:

2  External Devices

2.1  Combinatorial circuits

2.2  Asynchronous sequential logic systems

2.3  Synchronous sequential logic systems

3  Description of XDC and DDL

3.1  Call of XDC

3.2  Syntax of DDL

3.2.1  Keywords

3.2.2  Syntaxdiagrams

4  Error messages

5  Acknowledgements

1  Introduction:

XDC and DDL allow to define external devices for the register trans-

fer net simulator Sim (V4.0 and higher). With XDC it is possible to

increase the number of devices that can be simulated. It is also pos-

sible to increase the speed of the simulation if parts of a register

transfer net are combined in new, complexer components. New devices

can be combinatorial circuits, asynchronous, and synchronous sequen-

tial logic systems.

2  External Devices

Fundamental it is possible to define any given device. With synchro-

nous sequential logic systems it is necessary to consider the sequen-

tial handling of the devices (see 2.3).

2.1  Combinatorial circuits

Example of an AND device with four inputs:

device x_and;

input  in1,in2,in3,in4;

output out;

{

out = in1 & in2 & in3 & in4;

}

2.2  Asynchronous sequential logic systems

Example of an asynchronous automat:

Automat graph:

___                   ___

+--- /   \       1/0       /   \ ---+

0/0 |    | 0 | --------------> | 1 |    | 1/0

+--> \___/                 \___/ <--+

A                     |

|                     |

0/0 |                     | 0/0

|                     |

|                     V

___                   ___

+--- /   \       1/1       /   \ ---+

1/1 |    | 3 | <-------------- | 2 |    | 0/0

+--> \___/                 \___/ <--+

Definition:

device auto;

input  in;

output out;

state  s;

{

switch (s) {

case 0:

if (in & 1)

s = 1;

break;

case 1:

if (~in & 1)

s = 2;

break;

case 2:

if (in & 1) {

s = 3;

out = 1;

}

break;

case 3:

if (~in & 1) {

s = 0;

out = 0;

}

break;

}

}

2.3  Synchronous sequential logic systems

With sysnchronous sequential logic systems the value of the input

leads have to be buffered. Due to the forced sequencing of the devices

the input values must not be assigned directly to the outputs. If this

is done, a second register that is connected to the output of the first

will read the new but wrong value.

Example of a positive-edge-triggered D-Register:

device x_reg;

input  c,d;

output q;

state  s;

{

if (c & 1)

q = s;

else

s = d;

}

3  Description of XDC and DDL

XDC transforms the device definition to C source code which must be

compiled afterwards. Up to now only the AZTEC compiler is tested.

Generally spoken, the compiler must generate code using 16-bit-

integers with LARGE DATE (AZTEC: +D) memory model. For the LATTICE

compiler the file xcomp have to be changed.

3.1  Call of XDC

For the usage of XDC a AZTEC compiler and linker is needed. The direc-

tory, in which XDC lies, must contain the following files:

xcomp, sim.h, sim_msg.h

XDC is started by the following statement:

Execute xcomp <device>

The file, containing the device definition, must have the name

<device>.x. .x must not be given on the call of xcomp.

xcomp generates a file <device>. This file is the new external device.

This means this new device can be used in a VLI. The input and output

lead name have to be given in the sequence of their definition in

<device>.x. The file <device> should be copied to the directory that

contains the simulator.

3.2  Syntax of DDL

3.2.1  Keywords

The DDL uses the following keywords:

BREAK    CASE     DEFAULT  DEVICE   ELSE

IF       INPUT    OUTPUT   STATE    SWITCH

The keyword can be written in capitals or non-capitals (also mixed).

3.2.2  Syntaxdiagrams

file:

--> device_definition -->

device_definition:

--> DEVICE --> identifier --> ';' --> declaration_list --+

|

+------------------------<---------------------------+

|

+--> '{' --> statement_list --> '}' -->

comment:

--> '/*' ---+----------------+--> '*/' -->

A                |

+-- character <--+

declaration_list:

--> input_declaration_list --> output_declaration_list --+

|

+-------------------------<--------------------------+

|

+--> state_declaration_list --+-->

|                             A

+--------------->-------------+

input_declaration_list:

--+--> input_declaration --+-->

A                        |

+------------<-----------+

input_declaration:

--> INPUT --> identifier_list --> ';' -->

output_declaration_list:

--+--> output_declaration --+-->

A                         |

+-------------<-----------+

output_declaration:

--> OUTPUT --> identifier_list --> ';' -->

state_declaration_list:

--+--> state_declaration --+-->

A                        |

+------------<-----------+

state_declaration:

--> STATE --> identifier_list --> ';' -->

identifier_list:

--+--> identifier --+-->

A                 |

+------ ',' <-----+

identifier:

--> letter --+------->------+-->

A              |

+<-- letter <--+

|              |

+---- digit <--+

letter:

--+--> 'A' ---+-->

|     .     A

|     .     |

|           |

+--> 'Z' -->+

|           |

+--> 'a' -->+

|     .     |

|     .     |

|           |

+--> 'z' -->+

|           |

+--> '_' ---+

digit:

--+--> '0' --+-->

|     .    A

|     .    |

|          |

+--> '9' --+

statement_list:

--+--> statement --+-->

A                |

+--------<-------+

statement:

--+--> switch_statement ------>+-->

|                            A

+--> if_statement ---------->+

|                            |

+--> assignment_statement -->+

|                            |

+----------> ';' ------------+

assignment_statement:

--> identifier --> '=' --> expression --> ';' -->

expression:

--+--> expression -------------------------------+-->

|                                              A

+--> '~' --> expression ---------------------->+

|                                              |

+--> constant -------------------------------->+

|                                              |

+--> identifier ------------------------------>+

|                                              |

+--> '(' --> expression --> ')' -------------->+

|                                              |

+--> expression --> operator --> expression ---+

operator:

--+--> '>' --->+-->

|            A

+--> '<' --->+

|            |

+--> '>=' -->+

|            |

+--> '<=' -->+

|            |

+--> '==' -->+

|            |

+--> '!=' -->+

|            |

+--> '&' --->+

|            |

+--> '|' --->+

|            |

+--> '^' --->+

|            |

+--> '+' --->+

|            |

+--> '-' --->+

|            |

+--> '>>' -->+

|            |

+--> '<<' ---+

if_statement:

--> IF --> '(' --> expression --> ')' --> if_else_statement_list--+

|

+-----------------------------<-------------------------------+

|

+--> ELSE --> if_else_statement --+-->

|                                 A

+----------------->---------------+

if_else_statement_list:

--+--> statement -----------------------+-->

|                                     A

+--> '{' --> statement_list --> '}' --+

switch_statement:

--> SWITCH --> '(' --> expression --> ')' --> switch_statement_list -->

switch_statement_list:

--> '{' --+--> case_statment --+---+--> default_statement --+--> '}' -->

A                    |   |                        A

+----------<---------+   +------------>-----------+

case_statement:

--> CASE --> constant --> ':' --+--> statement_list --+--+

|                     A  |

+---------->----------+  |

|

+--------------------------<-------------------------+

|

+--> BREAK --> ';' --+-->

|                    A

+--------------------+

default_statement:

--> DEFAULT --> ':' --+--> statement_list --+---+--> BREAK --+-->

|                     A   |            A

+---------->----------+   +------>-----+

constant:

--+--> hexadecimal_constant --+-->

|                           A

+--> decimal_constant-------+

hexadecimal_constant:

+----------<---------+

V                    |

--+--> '0x' --+---+---+--> digit --+---+-->

|           A       |            A

+--> '0X' --+       +--> 'A' --->+

|     .      |

|     .      |

|            |

+--> 'F' --->+

|            |

+--> 'a' --->+

|     .      |

|     .      |

|            |

+--> 'f' ----+

decimal_constant:

--+--> '0' -----------------------------+-->

|                                     A

+--> positive_digit --+--> digit --+--+

A            |

+------<-----+

positive_digit:

--+--> '1' --+-->

|     .    A

|     .    |

|          |

+--> '9' --+

4  Error messages

There are three classes of errors:

a) parse errors without description

b) parse errors with description

'=' expected

'(' expected

')' expected

'{' expected

'}' expected

';' expected

':' expected

constant expected

lead name expected

c) semantical errors

BREAK outside of SWITCH

CASE outside of SWITCH

DEFAULT outside of SWITCH

no CASE behind DEFAULT

missing CASE

duplicate CASE

duplicate DEFAULT

already defined lead name

undefined lead

device name differs from file name

no memory

Error should be corrected from the start to the end because there are

often succeeding errors. Succeeding error occur mostly with the clas-

ses a) and b), seldom with c).

5  Acknowledgements

Sim, XDC, and DDL owe to Klaus-Dieter Renner whose idea and design of

a register transfer net simulator and a device definition language

gives the stimulation for my programs, and to Michael Koch for beta-

testing and his suggestions of new features.