SPICE Version 2G User's Guide

_____ _______ __ ______ _____

(10 Aug 1981)

A.Vladimirescu, Kaihe Zhang,

A.R.Newton, D.O.Pederson, A.Sangiovanni-Vincentelli

Department of Electrical Engineering and Computer Sciences

University of California

Berkeley, Ca., 94720

Acknowledgement:  Dr.  Richard Dowell and  Dr.   Sally  Liu

________________

have  contributed  to  develop the present SPICE version.  SPICE

was originally developed by Dr.  Lawrence  Nagel  and  has  been

modified extensively by Dr.  Ellis Cohen.

SPICE is a general-purpose circuit simulation  program  for

nonlinear  dc,  nonlinear  transient,  and  linear  ac analyses.

Circuits may contain resistors,  capacitors,  inductors,  mutual

inductors,  independent  voltage and current sources, four types

of dependent sources, transmission lines, and the four most com-

mon semiconductor devices:  diodes, BJT's, JFET's, and MOSFET's.

SPICE has built-in models for  the  semiconductor  devices,

and the user need specify only the pertinent model parameter va-

lues.  The model for the BJT is based  on  the  integral  charge

model  of Gummel and Poon;  however, if the Gummel- Poon parame-

ters are  not  specified,  the  model  reduces  to  the  simpler

Ebers-Moll model.  In either case, charge storage effects, ohmic

resistances, and a current-dependent output conductance  may  be

included.   The  diode model can be used for either junction di-

odes or Schottky barrier diodes.  The JFET model is based on the

FET  model  of Shichman and Hodges.  Three MOSFET models are im-

plemented;  MOS1 is described by a square-law  I-V  characteris-

tic, MOS2 is an analytical model, while MOS3 is a semi-empirical

model.  Both MOS2 and MOS3 include second-order effects such  as

channel  length  modulation, subthreshold conduction, scattering

limited   velocity   saturation,    small-size    effects    and

charge-controlled capacitances.

CHAPTER 1

TYPES OF ANALYSIS

1.1  DC Analysis

The dc analysis portion of SPICE  determines  the  dc  operating

point  of  the  circuit  with  inductors  shorted and capacitors

opened.  A dc analysis is automatically  performed  prior  to  a

transient  analysis  to  determine  the transient initial condi-

tions, and prior to an ac small-signal analysis to determine the

linearized,  small-signal  models for nonlinear devices.  If re-

quested, the dc small-signal value of a transfer function (ratio

of  output variable to input source), input resistance, and out-

put resistance will also be computed as a part of the  dc  solu-

tion.   The dc analysis can also be used to generate dc transfer

curves:  a specified independent voltage or  current  source  is

stepped  over a user-specified range and the dc output variables

are stored for each  sequential  source  value.   If  requested,

SPICE  also  will determine the dc small-signal sensitivities of

specified output variables with respect to  circuit  parameters.

The  dc analysis options are specified on the .DC, .TF, .OP, and

.SENS control cards.

If one desires to see the small-signal models for nonlinear

devices  in  conjunction  with  a  transient  analysis operating

point, then the .OP card must be provided.  The dc  bias  condi-

tions  will  be identical for each case, but the more comprehen-

sive operating point information is not available to be  printed

when transient initial conditions are computed.

1.2  AC Small Signal Analysis

The ac small-signal portion of SPICE computes the ac output

variables  as  a  function of frequency.  The program first com-

putes the dc operating point of the circuit and determines line-

arized,  small-signal models for all of the nonlinear devices in

the circuit.  The resultant linear circuit is then analyzed over

a user-specified range of frequencies.  The desired output of an

ac small- signal analysis is usually a transfer  function  (vol-

TYPES OF ANALYSIS                                               PAGE 1-2

tage gain, transimpedance, etc).  If the circuit has only one ac

input, it is convenient to set that  input  to  unity  and  zero

phase,  so  that  output  variables  have  the same value as the

transfer function of the output variable  with  respect  to  the

input.

The generation of white noise by resistors and  semiconduc-

tor  devices can also be simulated with the ac small-signal por-

tion of SPICE.  Equivalent noise source  values  are  determined

automatically  from the small-signal operating point of the cir-

cuit, and the contribution of each noise source is  added  at  a

given summing point.  The total output noise level and the equi-

valent input noise level are determined at each frequency point.

The output and input noise levels are normalized with respect to

the square root of  the  noise  bandwidth  and  have  the  units

Volts/rt  Hz  or  Amps/rt  Hz.   The output noise and equivalent

input noise can be printed or plotted in  the  same  fashion  as

other  output variables.  No additional input data are necessary

for this analysis.

Flicker noise sources can be simulated in  the  noise  ana-

lysis  by  including  values for the parameters KF and AF on the

appropriate device model cards.

The  distortion  characteristics  of  a  circuit   in   the

small-signal  mode  can  be  simulated  as  a  part  of  the  ac

small-signal analysis.  The analysis is performed assuming  that

one or two signal frequencies are imposed at the input.

The frequency range and the noise and  distortion  analysis

parameters  are specified on the .AC, .NOISE, and .DISTO control

lines.

1.3  Transient Analysis

The transient analysis portion of SPICE computes the  tran-

sient   output   variables   as   a  function  of  time  over  a

user-specified time interval.  The initial conditions are  auto-

matically  determined  by  a dc analysis.  All sources which are

not time dependent (for example,  power  supplies)  are  set  to

their  dc  value.   For  large-signal  sinusoidal simulations, a

Fourier analysis of the output waveform can be specified to  ob-

tain  the  frequency domain Fourier coefficients.  The transient

time interval and the Fourier analysis options are specified  on

the .TRAN and .FOURIER control lines.

TYPES OF ANALYSIS                                               PAGE 1-3

1.4  Analysis at Different Temperatures

All input data for SPICE is assumed to have  been  measured

at  27 deg C (300 deg K).  The simulation also assumes a nominal

temperature of 27 deg C.  The circuit can be simulated at  other

temperatures by using a .TEMP control line.

Temperature appears explicitly in the exponential terms  of

the BJT and diode model equations.  In addition, saturation cur-

rents have a built-in temperature dependence.   The  temperature

dependence of the saturation current in the BJT models is deter-

mined by:

IS(T1) = IS(T0)*((T1/T0)**XTI)*exp(q*EG*(T1-T0)/(k*T1*T0))

where k is Boltzmann's constant, q is the electronic charge,  EG

is the energy gap which is a model parameter, and XTI is the sa-

turation current temperature exponent (also a  model  parameter,

and  usually equal to 3).  The temperature dependence of forward

and reverse beta is according to the formula:

beta(T1)=beta(T0)*(T1/T0)**XTB

where  T1  and  T0  are  in  degrees  Kelvin,  and  XTB   is   a

user-supplied  model parameter.  Temperature effects on beta are

carried out by appropriate adjustment to the values of BF,  ISE,

BR,  and  ISC.  Temperature dependence of the saturation current

in the junction diode model is determined by:

IS(T1) = IS(T0)*((T1/T0)**(XTI/N))*exp(q*EG*(T1-T0)/(k*N*T1*T0))

where N is the emission coefficient, which is a model parameter,

and the other symbols have the same meaning as above.  Note that

for Schottky barrier diodes, the value of the saturation current

temperature exponent, XTI, is usually 2.

Temperature appears explicitly in the value of junction po-

tential, PHI, for all the device models.  The temperature depen-

dence is determined by:

PHI(TEMP) = k*TEMP/q*log(Na*Nd/Ni(TEMP)**2)

where k is Boltzmann's constant, q is the electronic charge,  Na

is  the acceptor impurity density, Nd is the donor impurity den-

sity, Ni is the intrinsic concentration, and EG  is  the  energy

gap.

Temperature appears explicitly in the value of surface  mo-

bility, UO, for the MOSFET model.  The temperature dependence is

determined by:

UO(TEMP) = UO(TNOM)/(TEMP/TNOM)**(1.5)

TYPES OF ANALYSIS                                               PAGE 1-4

The effects of temperature on resistors is modeled  by  the

formula:

value(TEMP) = value(TNOM)*(1+TC1*(TEMP-TNOM)+TC2*(TEMP-TNOM)**2))

where TEMP is the circuit temperature, TNOM is the nominal  tem-

perature,  and  TC1 and TC2 are the first- and second-order tem-

perature coefficients.

CHAPTER 2

CONVERGENCE

Both dc and transient solutions are obtained by  an  itera-

tive process which is terminated when both of the following con-

ditions hold:

1.  The nonlinear branch  currents  converge  to  within  a

tolerance  of  0.1  percent or 1 picoamp (1.0E-12 Amp),

whichever is larger.

2.  The node voltages converge to within a tolerance of 0.1

percent  or  1  microvolt  (1.0E-6  Volt), whichever is

larger.

Although the algorithm used in SPICE has been found  to  be

very reliable, in some cases it will fail to converge to a solu-

tion.  When this failure occurs, the program will print the node

voltages  at  the last iteration and terminate the job.  In such

cases, the node voltages that are printed  are  not  necessarily

correct or even close to the correct solution.

Failure to converge in the dc analysis is usually due to an

error  in  specifying  circuit  connections,  element values, or

model parameter values.  Regenerative switching circuits or cir-

cuits  with  positive feedback probably will not converge in the

dc analysis unless the OFF option is used for some of  the  dev-

ices in the feedback path, or the .NODESET card is used to force

the circuit to converge to the desired state.

CHAPTER 3

INPUT FORMAT

The input format for SPICE is  of  the  free  format  type.

Fields  on  a card are separated by one or more blanks, a comma,

an equal (=) sign, or a left or right parenthesis;  extra spaces

are  ignored.  A card may be continued by entering a + (plus) in

column 1 of the following card;  SPICE continues reading  begin-

ning with column 2.

A name field must begin with a letter  (A  through  Z)  and

cannot  contain any delimiters.  Only the first eight characters

of the name are used.

A number field may be an integer field (12, -44), a  float-

ing  point  field (3.14159), either an integer or floating point

number followed by an integer exponent (1E-14, 2.65E3),  or  ei-

ther  an  integer  or a floating point number followed by one of

the following scale factors:

T=1E12   G=1E9    MEG=1E6   K=1E3     MIL=25.4E-6

M=1E-3   U=1E-6   N=1E-9    P=1E-12   F=1E-15

Letters immediately following a number that are  not  scale

factors  are  ignored, and letters immediately following a scale

factor are ignored.  Hence, 10, 10V, 10VOLTS, and 10HZ  all  re-

present  the  same number, and M, MA, MSEC, and MMHOS all repre-

sent the same scale factor.  Note  that  1000,  1000.0,  1000HZ,

1E3, 1.0E3, 1KHZ, and 1K all represent the same number.

CHAPTER 4

CIRCUIT DESCRIPTION

The circuit to be analyzed is described to SPICE by  a  set

of  element cards, which define the circuit topology and element

values, and a set of control cards, which define the model  par-

ameters  and the run controls.  The first card in the input deck

must be a title card, and the last card must  be  a  .END  card.

The  order  of  the  remaining  cards  is  arbitrary (except, of

course, that continuation cards must immediately follow the card

being continued).

Each element in the circuit is specified by an element card

that  contains  the element name, the circuit nodes to which the

element is connected, and the values of the parameters that  de-

termine  the  electrical  characteristics  of  the element.  The

first letter of the element name  specifies  the  element  type.

The format for the SPICE element types is given in what follows.

The strings XXXXXXX, YYYYYYY, and ZZZZZZZ denote  arbitrary  al-

phanumeric  strings.   For  example,  a resistor name must begin

with the letter R and can contain from one to eight  characters.

Hence, R, R1, RSE, ROUT, and R3AC2ZY are valid resistor names.

Data fields that are enclosed in lt and gt signs '< >'  are

optional.   All indicated punctuation (parentheses, equal signs,

etc.) are required.  With respect to branch  voltages  and  cur-

rents,  SPICE uniformly uses the associated reference convention

(current flows in the direction of voltage drop).

Nodes must be nonnegative integers but need not be numbered

sequentially.   The  datum  (ground) node must be numbered zero.

The circuit cannot contain a loop of voltage sources and/or  in-

ductors  and  cannot  contain a cutset of current sources and/or

capacitors.  Each node in the circuit must have  a  dc  path  to

ground.   Every  node  must have at least two connections except

for transmission line nodes (to permit unterminated transmission

lines)  and MOSFET substrate nodes (which have two internal con-

nections anyway).

CHAPTER 5

TITLE CARD  COMMENT CARDS AND  END CARD

5.1  Title Card

Examples

________

POWER AMPLIFIER CIRCUIT

TEST OF CAM CELL

This card must be the first card in the  input  deck.   Its

contents are printed verbatim as the heading for each section of

output.

5.2  END Card

Examples

________

.END

This card must always be the last card in the  input  deck.

Note that the period is an integral part of the name.

5.3  Comment Card

General Form

_______ ____

* <any comment>

Examples

________

* RF=1K      GAIN SHOULD BE 100

* MAY THE FORCE BE WITH MY CIRCUIT

The asterisk in the first column indicates that  this  card

is  a comment card.  Comment cards may be placed anywhere in the

circuit description.

CHAPTER 6

ELEMENT CARDS

6.1  Resistors

General form

_______ ____

RXXXXXXX N1 N2 VALUE <TC=TC1<,TC2>>

Examples

________

R1 1 2 100

RC1 12 17 1K TC=0.001,0.015

N1 and N2 are the two element nodes.  VALUE is  the  resis-

tance  (in  ohms)  and may be positive or negative but not zero.

TC1 and TC2 are the (optional) temperature coefficients;  if not

specified,  zero is assumed for both.  The value of the resistor

as a function of temperature is given by:

value(TEMP) = value(TNOM)*(1+TC1*(TEMP-TNOM)+TC2*(TEMP-TNOM)**2))

6.2  Capacitors and Inductors

General form

_______ ____

CXXXXXXX N+ N- VALUE <IC=INCOND>

LYYYYYYY N+ N- VALUE <IC=INCOND>

Examples

________

CBYP 13 0 1UF

COSC 17 23 10U IC=3V

LLINK 42 69 1UH

LSHUNT 23 51 10U IC=15.7MA

N+ and N- are the positive and negative element nodes, res-

pectively.  VALUE is the capacitance in Farads or the inductance

in Henries.

For the capacitor, the (optional) initial condition is  the

ELEMENT CARDS                                                   PAGE 6-2

initial  (time-zero) value of capacitor voltage (in Volts).  For

the inductor, the (optional) initial condition  is  the  initial

(time-zero)  value of inductor current (in Amps) that flows from

N+, through the inductor, to N-.  Note that the  initial  condi-

tions  (if  any)  apply 'only' if the UIC option is specified on

the .TRAN card.

Nonlinear  capacitors  and  inductors  can  be   described.

General form

_______ ____

CXXXXXXX N+ N- POLY C0 C1 C2 ... <IC=INCOND>

LYYYYYYY N+ N- POLY L0 L1 L2 ... <IC=INCOND>

C0 C1 C2 ...(and L0 L1 L2 ...) are the  coefficients  of  a

polynomial describing the element value.  The capacitance is ex-

pressed as a function of the voltage across  the  element  while

the  inductance  is a function of the current through the induc-

tor.  The value is computed as

value=C0+C1*V+C2*V**2+...

value=L0+L1*I+L2*I**2+...

where V is the voltage across the capacitor and  I  the  current

flowing in the inductor.

6.3  Coupled  Mutual  Inductors

General form

_______ ____

KXXXXXXX LYYYYYYY LZZZZZZZ VALUE

Examples

________

K43 LAA LBB 0.999

KXFRMR L1 L2 0.87

LYYYYYYY and LZZZZZZZ are the names of the two coupled  in-

ductors, and VALUE is the coefficient of coupling, K, which must

be greater than 0 and less than or equal to 1.  Using the  'dot'

convention, place a 'dot' on the first node of each inductor.

6.4  Transmission Lines, Lossless

General form

_______ ____

TXXXXXXX N1 N2 N3 N4 Z0=VALUE <TD=VALUE> <F=FREQ <NL=NRMLEN>><IC=V1,I1,V2,I2>

Examples

________

T1 1 0 2 0 Z0=50 TD=10NS

ELEMENT CARDS                                                   PAGE 6-3

N1 and N2 are the nodes at port 1;  N3 and N4 are the nodes

at  port  2.  Z0 is the characteristic impedance.  The length of

the line may be expressed in either of two forms.  The transmis-

sion delay, TD, may be specified directly (as TD=10ns, for exam-

ple).  Alternatively, a frequency F may be given, together  with

NL,  the  normalized  electrical length of the transmission line

with respect to the wavelength in the line at the  frequency  F.

If  a  frequency is specified but NL is omitted, 0.25 is assumed

(that is, the frequency is assumed to be the  quarter-wave  fre-

quency).   Note that although both forms for expressing the line

length are indicated as optional, one of the two must be  speci-

fied.

Note that this element models only  one  propagating  mode.

If  all  four nodes are distinct in the actual circuit, then two

modes may  be  excited.   To  simulate  such  a  situation,  two

transmission  line  elements  are required.  (see the example in

Appendix A for further clarification.)

The (optional) initial condition specification consists  of

the  voltage and current at each of the transmission line ports.

Note that the initial conditions (if any) apply  'only'  if  the

UIC option is specified on the .TRAN card.

One should  be  aware  that  SPICE  will  use  a  transient

time-step  which  does  not  exceed 1/2 the minimum transmission

line delay.  Therefore very short transmission  lines  (compared

with the analysis time frame) will cause long run times.

6.5  Linear Dependent Sources

SPICE allows circuits to contain linear  dependent  sources

characterized by any of the four equations

i=g*v          v=e*v          i=f*i          v=h*i

where g, e, f, and h  are  constants  representing  transconduc-

tance,  voltage gain, current gain, and transresistance, respec-

tively.

Note:  A more complete description of dependent sources  as

implemented in SPICE is given in Appendix B.

6.6  Linear Voltage Controlled Current Sources

General form

_______ ____

GXXXXXXX N+ N- NC+ NC- VALUE

Examples

________

ELEMENT CARDS                                                   PAGE 6-4

G1 2 0 5 0 0.1MMHO

N+ and N- are the positive and negative nodes,  respective-

ly.  Current flow is from the positive node, through the source,

to the negative node.  NC+ and NC- are the positive and negative

controlling  nodes, respectively.  VALUE is the transconductance

(in mhos).

6.7  Linear Voltage Controlled Voltage Sources

General form

_______ ____

EXXXXXXX N+ N- NC+ NC- VALUE

Examples

________

E1 2 3 14 1 2.0

N+ is the positive node, and N- is the negative node.   NC+

and NC- are the positive and negative controlling nodes, respec-

tively.  VALUE is the voltage gain.

6.8  Linear Current Controlled Current Sources

General form

_______ ____

FXXXXXXX N+ N- VNAM VALUE

Examples

________

F1 13 5 VSENS 5

N+ and N- are the positive and negative nodes,  respective-

ly.  Current flow is from the positive node, through the source,

to the negative node.  VNAM is the  name  of  a  voltage  source

through  which  the controlling current flows.  The direction of

positive controlling current flow is  from  the  positive  node,

through  the source, to the negative node of VNAM.  VALUE is the

current gain.

6.9  Linear Current Controlled Voltage Sources

General form

_______ ____

HXXXXXXX N+ N- VNAM VALUE

Examples

________

HX 5 17 VZ 0.5K

N+ and N- are the positive and negative nodes,  respective-

ELEMENT CARDS                                                   PAGE 6-5

ly.  VNAM is the name of a voltage source through which the con-

trolling current flows.  The direction of  positive  controlling

current  flow  is from the positive node, through the source, to

the negative node of VNAM.  VALUE  is  the  transresistance  (in

ohms).

6.10  Independent Sources

General form

_______ ____

VXXXXXXX N+ N- <<DC> DC/TRAN VALUE> <AC <ACMAG <ACPHASE>>>

IYYYYYYY N+ N- <<DC> DC/TRAN VALUE> <AC <ACMAG <ACPHASE>>>

Examples

________

VCC 10 0 DC 6

VIN 13 2 0.001 AC 1 SIN(0 1 1MEG)

ISRC 23 21 AC 0.333 45.0 SFFM(0 1 10K 5 1K)

VMEAS 12 9

N+ and N- are the positive and negative nodes,  respective-

ly.   Note  that voltage sources need not be grounded.  Positive

current is assumed to flow from the positive node,  through  the

source,  to  the  negative  node.   A current source of positive

value, will force current to flow out of the  N+  node,  through

the  source, and into the N- node.  Voltage sources, in addition

to being used for circuit excitation,  are  the  'ammeters'  for

SPICE, that is, zero valued voltage sources may be inserted into

the circuit for the purpose of measuring current.  They will, of

course, have no effect on circuit operation since they represent

short-circuits.

DC/TRAN is the dc  and  transient  analysis  value  of  the

source.   If  the source value is zero both for dc and transient

analyses, this value may be omitted.  If  the  source  value  is

time-invariant  (e.g.,  a  power supply), then the value may op-

tionally be preceded by the letters DC.

ACMAG is the ac magnitude and ACPHASE is the ac phase.  The

source is set to this value in the ac analysis.  If ACMAG is om-

itted following the keyword AC, a value of unity is assumed.  If

ACPHASE  is  omitted, a value of zero is assumed.  If the source

is not an ac small-signal input, the keyword AC and the  ac  va-

lues are omitted.

Any independent source can  be  assigned  a  time-dependent

value  for  transient  analysis.  If a source is assigned a time

dependent value, the time-zero value is used  for  dc  analysis.

There  are  five  independent source functions:  pulse, exponen-

tial, sinusoidal, piece-wise linear,  and  single-frequency  FM.

If  parameters  other  than  source values are omitted or set to

zero, the default values shown will be assumed.  (TSTEP  is  the

printing  increment  and  TSTOP is the final time (see the .TRAN

ELEMENT CARDS                                                   PAGE 6-6

card for explanation)).

6.10.1  Pulse         PULSE(V1 V2 TD TR TF PW PER)

Examples

________

VIN 3 0 PULSE(-1 1 2NS 2NS 2NS 50NS 100NS)

parameters              default values         units

__________              _______ ______         _____

V1 (initial value)                       Volts or Amps

V2 (pulsed value)                        Volts or Amps

TD (delay time)         0.0              seconds

TR (rise time)          TSTEP            seconds

TF (fall time)          TSTEP            seconds

PW (pulse width)        TSTOP            seconds

PER(period)             TSTOP            seconds

A single pulse so specified is described by  the  following

table:

time          value

____          _____

0             V1

TD            V1

TD+TR         V2

TD+TR+PW      V2

TD+TR+PW+TF   V1

TSTOP         V1

Intermediate points are determined by linear interpolation.

6.10.2  Sinusoidal    SIN(VO VA FREQ TD THETA)

Examples

________

VIN 3 0 SIN(0 1 100MEG 1NS 1E10)

parameters                default value   units

__________                _______ _____   _____

VO     (offset)                           Volts or Amps

VA     (amplitude)                        Volts or Amps

FREQ   (frequency)        1/TSTOP         Hz

TD     (delay)            0.0             seconds

THETA  (damping factor)   0.0             1/seconds

The shape of the waveform is  described  by  the  following

table:

time          value

____          _____

0 to TD       VO

TD to TSTOP   VO+VA*exp(-(time-TD)*THETA)*sin(2*pi*FREQ*(time+TD))

ELEMENT CARDS                                                   PAGE 6-7

6.10.3  Exponential  EXP(V1 V2 TD1 TAU1 TD2 TAU2)

Examples

________

VIN 3 0 EXP(-4 -1 2NS 30NS 60NS 40NS)

parameters                  default values   units

__________                  _______ ______   _____

V1   (initial value)                         Volts or Amps

V2   (pulsed value)                          Volts or Amps

TD1  (rise delay time)      0.0              seconds

TAU1 (rise time constant)   TSTEP            seconds

TD2  (fall delay time)      TD1+TSTEP        seconds

TAU2 (fall time constant)   TSTEP            seconds

The shape of the waveform is  described  by  the  following

table:

time           value

____           _____

0 to TD1       V1

TD1 to TD2     V1+(V2-V1)*(1-exp(-(time-TD1)/TAU1))

TD2 to TSTOP   V1+(V2-V1)*(1-exp(-(time-TD1)/TAU1))

+(V1-V2)*(1-exp(-(time-TD2)/TAU2))

6.10.4  Piece-Wise Linear  PWL(T1 V1 <T2 V2 T3 V3 T4 V4 ...>)

Examples

________

VCLOCK 7 5 PWL(0 -7 10NS -7 11NS -3 17NS -3 18NS -7 50NS -7)

Parameters and default values:  Each pair  of  values  (Ti,

Vi)  specifies  that  the value of the source is Vi (in Volts or

Amps) at time=Ti.  The value of the source at  intermediate  va-

lues  of time is determined by using linear interpolation on the

input values.

6.10.5  Single-Frequency FM   SFFM(VO VA FC MDI FS)

Examples

________

V1 12 0 SFFM(0 1M 20K 5 1K)

parameters                default values   units

__________                _______ ______   _____

VO  (offset)                               Volts or Amps

VA  (amplitude)                            Volts or Amps

FC  (carrier frequency)   1/TSTOP          Hz

MDI (modulation index)

FS  (signal frequency)    1/TSTOP          Hz

The shape of the waveform is  described  by  the  following

equation:

value = VO + VA*sine((twopi*FC*time) + MDI*sine(twopi*FS*time))

ELEMENT CARDS                                                   PAGE 6-8

CHAPTER 7

SEMICONDUCTOR DEVICES

The elements that have been described to this  point  typi-

cally  require only a few parameter values to specify completely

the electrical characteristics of the element.  However, the mo-

dels for the four semiconductor devices that are included in the

SPICE program require many  parameter  values.   Moreover,  many

devices in a circuit often are defined by the same set of device

model parameters.  For these reasons, a set of device model par-

ameters is defined on a separate .MODEL card and assigned a uni-

que model name.  The device element cards in SPICE  then  refer-

ence the model name.  This scheme alleviates the need to specify

all of the model parameters on each device element card.

Each device element card  contains  the  device  name,  the

nodes  to  which  the  device is connected, and the device model

name.  In addition, other optional parameters may  be  specified

for each device:  geometric factors and an initial condition.

The area factor used on the diode, BJT and JFET device card

determines the number of equivalent parallel devices of a speci-

fied model.  The affected parameters are marked with an asterisk

under  the  heading  'area'  in  the  model  descriptions below.

Several geometric factors associated with the  channel  and  the

drain  and source diffusions can be specified on the MOSFET dev-

ice card.

Two different forms of initial conditions may be  specified

for  devices.  The first form is included to improve the dc con-

vergence for circuits that contain more than one  stable  state.

If  a  device is specified OFF, the dc operating point is deter-

mined with the terminal voltages for that device  set  to  zero.

After  convergence is obtained, the program continues to iterate

to obtain the exact value for the terminal voltages.  If a  cir-

cuit  has  more  than one dc stable state, the OFF option can be

used to force the solution to correspond to a desired state.  If

a device is specified OFF when in reality the device is conduct-

ing, the program will still obtain the correct solution  (assum-

ing the solutions converge) but more iterations will be required

since the program must independently converge  to  two  separate

solutions.   The  .NODESET  card serves a similar purpose as the

OFF option.  The .NODESET option is easier to apply and  is  the

preferred means to aid convergence.

SEMICONDUCTOR DEVICES                                           PAGE 7-2

The second form of initial conditions are specified for use

with   the   transient   analysis.    These  are  true  'initial

conditions' as opposed to the convergence aids above.   See  the

description  of  the  .IC card and the .TRAN card for a detailed

explanation of initial conditions.

7.1  Junction Diodes

General form

_______ ____

DXXXXXXX N+ N- MNAME <AREA> <OFF> <IC=VD>

Examples

________

DBRIDGE 2 10 DIODE1

DCLMP 3 7 DMOD 3.0 IC=0.2

N+ and N- are the positive and negative nodes,  respective-

ly.   MNAME  is the model name, AREA is the area factor, and off

indicates an (optional) starting condition on the device for  dc

analysis.   If the area factor is omitted, a value of 1.0 is as-

sumed.  The (optional)  initial  condition  specification  using

IC=VD is intended for use with the UIC option on the .TRAN card,

when a transient analysis is desired starting  from  other  than

the quiescent operating point.

7.2  Bipolar Junction Transistors  BJT s

General form

_______ ____

QXXXXXXX NC NB NE <NS> MNAME <AREA> <OFF> <IC=VBE,VCE>

Examples

________

Q23 10 24 13 QMOD IC=0.6,5.0

Q50A 11 26 4 20 MOD1

NC, NB, and NE are the collector, base, and emitter  nodes,

respectively.  NS is the (optional) substrate node.  If unspeci-

fied, ground is used.  MNAME is the model name, AREA is the area

factor, and OFF indicates an (optional) initial condition on the

device for the dc analysis.  If the area factor  is  omitted,  a

value of 1.0 is assumed.  The (optional) initial condition spec-

ification using IC=VBE,VCE is intended for use with the UIC  op-

tion  on  the  .TRAN  card, when a transient analysis is desired

starting from other than the quiescent operating point.  See the

.IC  card  description for a better way to set transient initial

conditions.

SEMICONDUCTOR DEVICES                                           PAGE 7-3

7.3  Junction Field Effect Transistors  JFETs

General form

_______ ____

JXXXXXXX ND NG NS MNAME <AREA> <OFF> <IC=VDS,VGS>

Examples

________

J1 7 2 3 JM1 OFF

ND, NG, and NS are the drain, gate, and source nodes,  res-

pectively.   MNAME  is  the model name, AREA is the area factor,

and OFF indicates an (optional) initial condition on the  device

for  dc analysis.  If the area factor is omitted, a value of 1.0

is assumed.  The  (optional)  initial  condition  specification,

using  IC=VDS,VGS is intended for use with the UIC option on the

.TRAN card, when a transient analysis is desired  starting  from

other than the quiescent operating point (see the .IC card for a

better way to set initial conditions).

7.4  MOSFETs

General form

_______ ____

MXXXXXXX ND NG NS NB MNAME <L=VAL> <W=VAL> <AD=VAL> <AS=VAL>

<PD=VAL> <PS=VAL> <NRD=VAL> <NRS=VAL> <OFF> <IC=VDS,VGS,VBS>

Examples

________

M1 24 2 0 20 TYPE1

M31 2 17 6 10 MODM L=5U W=2U

M31 2 16 6 10 MODM 5U 2U

M1 2 9 3 0 MOD1 L=10U W=5U AD=100P AS=100P PD=40U PS=40U

M1 2 9 3 0 MOD1 10U 5U 2P 2P

ND, NG, NS, and NB are the drain, gate, source, and  bulk  (sub-

strate)  nodes, respectively.  MNAME is the model name.  L and W

are the channel length and width, in meters.  AD and AS are  the

areas  of  the  drain and source diffusions, in sq-meters.  Note

that the suffix U specifies microns (1E-6 m)  and  P  sq-microns

(1E-12  sq-m).  If any of L, W, AD, or AS are not specified, de-

fault values are used.  The user may specify the  values  to  be

used for these default parameters on the .OPTIONS card.  The use

of defaults simplifies input deck preparation, as  well  as  the

editing required if device geometries are to be changed.  PD and

PS are the perimeters of the drain and source junctions, in  me-

ters.  NRD and NRS designate the equivalent number of squares of

the drain and source  diffusions;   these  values  multiply  the

sheet  resistance  RSH specified on the .MODEL card for an accu-

rate representation of the parasitic series drain and source re-

sistance of each transistor.  PD and PS default to 0.0 while NRD

and NRS to 1.0.  OFF indicates an (optional)  initial  condition

on the device for dc analysis.  The (optional) initial condition

SEMICONDUCTOR DEVICES                                           PAGE 7-4

specification using IC=VDS,VGS,VBS is intended for use with  the

UIC  option  on the .TRAN card, when a transient analysis is de-

sired starting from other than the  quiescent  operating  point.

See the .IC card for a better and more convenient way to specify

transient initial conditions.

7.5  MODEL Card

General form

_______ ____

.MODEL MNAME TYPE(PNAME1=PVAL1 PNAME2=PVAL2 ... )

Examples

________

.MODEL MOD1 NPN BF=50 IS=1E-13 VBF=50

The .MODEL card specifies a set of  model  parameters  that

will  be  used by one or more devices.  MNAME is the model name,

and type is one of the following seven types:

NPN    NPN BJT model

PNP    PNP BJT model

D      diode model

NJF    N-channel JFET model

PJF    P-channel JFET model

NMOS   N-channel MOSFET model

PMOS   P-channel MOSFET model

Parameter values are defined  by  appending  the  parameter

name,  as  given below for each model type, followed by an equal

sign and the parameter value.  Model  parameters  that  are  not

given  a  value  are assigned the default values given below for

each model type.

7.6  Diode Model

The dc characteristics of the diode are determined  by  the

parameters  IS  and  N.   An  ohmic resistance, RS, is included.

Charge storage effects are modeled by a transit time, TT, and  a

nonlinear depletion layer capacitance which is determined by the

parameters CJO, VJ, and M.  The temperature  dependence  of  the

saturation  current  is defined by the parameters EG, the energy

and XTI, the saturation current temperature  exponent.   Reverse

breakdown  is  modeled by an exponential increase in the reverse

diode current and is determined by the  parameters  BV  and  IBV

(both of which are positive numbers).

name   parameter                        units   default    example    area

____   _________                        _____   _______    _______    ____

1   IS     saturation current               A       1.0E-14    1.0E-14    *

SEMICONDUCTOR DEVICES                                                   PAGE 7-5

2   RS     ohmic resistance                 Ohm     0          10         *

3   N      emission coefficient             -       1          1.0

4   TT     transit-time                     sec     0          0.1Ns

5   CJO    zero-bias junction capacitance   F       0          2PF        *

6   VJ     junction potential               V       1          0.6

7   M      grading coefficient              -       0.5        0.5

8   EG     activation energy                eV      1.11       1.11 Si

0.69 Sbd

0.67 Ge

9   XTI    saturation-current temp. exp     -       3.0        3.0 jn

2.0 Sbd

10   KF     flicker noise coefficient        -       0

11   AF     flicker noise exponent           -       1

12   FC     coefficient for forward-bias     -       0.5

depletion capacitance formula

13   BV     reverse breakdown voltage        V       infinite   40.0

14   IBV    current at breakdown voltage     A       1.0E-3

7.7  BJT Models  both NPN and PNP

The bipolar junction transistor model in SPICE is an  adap-

tation  of the integral charge control model of Gummel and Poon.

This modified Gummel-Poon model extends the  original  model  to

include several effects at high bias levels.  The model will au-

tomatically simplify to the simpler Ebers-Moll model  when  cer-

tain  parameters are not specified.  The parameter names used in

the modified Gummel-Poon model have been chosen to be more easi-

ly  understood  by  the program user, and to reflect better both

physical and circuit design thinking.

The dc model is defined by the parameters IS, BF, NF,  ISE,

IKF,  and  NE  which  determine the forward current gain charac-

teristics, IS, BR, NR, ISC, IKR, and NC which determine the  re-

verse current gain characteristics, and VAF and VAR which deter-

mine the output conductance for  forward  and  reverse  regions.

Three  ohmic  resistances  RB, RC, and RE are included, where RB

can be high current dependent.  Base charge storage  is  modeled

by  forward  and  reverse  transit times, TF and TR, the forward

transit time TF being bias dependent if desired,  and  nonlinear

depletion  layer  capacitances which are determined by CJE, VJE,

and MJE for the B-E junction , CJC, VJC, and  MJC  for  the  B-C

junction and CJS, VJS, and MJS for the C-S (Collector-Substrate)

junction.  The temperature dependence of the saturation current,

IS, is determined by the energy-gap, EG, and the saturation cur-

rent temperature exponent, XTI.  Additionally base current  tem-

perature  dependence is modeled by the beta temperature exponent

XTB in the new model.

The BJT parameters used in the modified  Gummel-Poon  model

are  listed below.  The parameter names used in earlier versions

of SPICE2 are still accepted.

SEMICONDUCTOR DEVICES                                                   PAGE 7-6

Modified Gummel-Poon BJT Parameters.

name   parameter                               units   default    example

____   _________                               _____   _______    _______

1    IS     transport saturation current            A       1.0E-16    1.0E-15

2    BF     ideal maximum forward beta              -       100        100

3    NF     forward current emission coefficient    -       1.0        1

4    VAF    forward Early voltage                   V       infinite   200

5    IKF    corner for forward beta

high current roll-off                   A       infinite   0.01

6    ISE    B-E leakage saturation current          A       0          1.0E-13

7    NE     B-E leakage emission coefficient        -       1.5        2

8    BR     ideal maximum reverse beta              -       1          0.1

9    NR     reverse current emission coefficient    -       1          1

10   VAR    reverse Early voltage                   V       infinite   200

11   IKR    corner for reverse beta

high current roll-off                   A       infinite   0.01

12   ISC    B-C leakage saturation current          A       0          1.0E-13

13   NC     B-C leakage emission coefficient        -       2          1.5

14   RB     zero bias base resistance               Ohms    0          100

15   IRB    current where base resistance

falls halfway to its min value          A       infinite   0.1

16   RBM    minimum base resistance

at high currents                        Ohms    RB         10

17   RE     emitter resistance                      Ohms    0          1

18   RC     collector resistance                    Ohms    0          10

19   CJE    B-E zero-bias depletion capacitance     F       0          2PF

20   VJE    B-E built-in potential                  V       0.75       0.6

21   MJE    B-E junction exponential factor         -       0.33       0.33

22   TF     ideal forward transit time              sec     0          0.1Ns

23   XTF    coefficient for bias dependence of TF   -       0

24   VTF    voltage describing VBC

dependence of TF                        V       infinite

25   ITF    high-current parameter

for effect on TF                        A       0

26   PTF    excess phase at freq=1.0/(TF*2PI) Hz    deg     0

27   CJC    B-C zero-bias depletion capacitance     F       0          2PF

28   VJC    B-C built-in potential                  V       0.75       0.5

29   MJC    B-C junction exponential factor         -       0.33       0.5

30   XCJC   fraction of B-C depletion capacitance   -       1

connected to internal base node

31   TR     ideal reverse transit time              sec     0          10Ns

32   CJS    zero-bias collector-substrate

capacitance                             F       0          2PF

33   VJS    substrate junction built-in potential   V       0.75

34   MJS    substrate junction exponential factor   -       0          0.5

35   XTB    forward and reverse beta

temperature exponent                    -       0

36   EG     energy gap for temperature

effect on IS                            eV      1.11

37   XTI    temperature exponent for effect on IS   -       3

38   KF     flicker-noise coefficient               -       0

39   AF     flicker-noise exponent                  -       1

40   FC     coefficient for forward-bias

depletion capacitance formula           -       0.5

SEMICONDUCTOR DEVICES                                           PAGE 7-7

7.8  JFET Models  both N and P Channel

The JFET model is derived from the FET  model  of  Shichman

and  Hodges.   The dc characteristics are defined by the parame-

ters VTO and BETA, which determine the variation of  drain  cur-

rent with gate voltage, LAMBDA, which determines the output con-

ductance, and IS, the saturation current of the two  gate  junc-

tions.   Two ohmic resistances, RD and RS, are included.  Charge

storage is modeled by nonlinear depletion layer capacitances for

both  gate  junctions  which  vary as the -1/2 power of junction

voltage and are defined by the parameters CGS, CGD, and PB.

name     parameter                            units    default   example

____     _________                            _____    _______   _______

1   VTO      threshold voltage                    V        -2.0      -2.0

2   BETA     transconductance parameter           A/V**2   1.0E-4    1.0E-3

3   LAMBDA   channel length modulation

parameter                            1/V      0         1.0E-4

4   RD       drain ohmic resistance               Ohm      0         100

5   RS       source ohmic resistance              Ohm      0         100

6   CGS      zero-bias G-S junction capacitance   F        0         5PF

7   CGD      zero-bias G-D junction capacitance   F        0         1PF

8   PB       gate junction potential              V        1         0.6

9   IS       gate junction saturation current     A        1.0E-14   1.0E-14

10   KF       flicker noise coefficient            -        0

11   AF       flicker noise exponent               -        1

12   FC       coefficient for forward-bias         -        0.5

depletion capacitance formula

7.9  MOSFET Models  both N and P channel

SPICE provides three MOSFET device models which  differ  in

the  formulation  of the I-V characteristic.  The variable LEVEL

specifies the model to be used:

LEVEL=1 ->    Shichman-Hodges

LEVEL=2 ->    MOS2 (as described in 1)

LEVEL=3 ->    MOS3, a semi-empirical model(see 1)

The dc characteristics of the MOSFET  are  defined  by  the

device parameters VTO, KP, LAMBDA, PHI and GAMMA.  These parame-

ters are computed by SPICE if  process  parameters  (NSUB,  TOX,

...)  are given, but user-specified values always override.  VTO

is positive (negative) for enhancement mode and negative  (posi-

tive)  for depletion mode N-channel (P-channel) devices.  Charge

storage is modeled by three constant capacitors, CGSO, CGDO, and

CGBO  which  represent  overlap  capacitances,  by the nonlinear

thin-oxide capacitance which  is  distributed  among  the  gate,

source,   drain,   and   bulk  regions,  and  by  the  nonlinear

depletion-layer capacitances for both substrate junctions divid-

ed  into  bottom  and  periphery,  which vary as the MJ and MJSW

power of junction voltage respectively, and  are  determined  by

SEMICONDUCTOR DEVICES                                           PAGE 7-8

the  parameters  CBD, CBS, CJ, CJSW, MJ, MJSW and PB.  There are

two built-in models of the  charge  storage  effects  associated

with  the  thin-oxide.  The default is the piecewise linear vol-

tage-dependent capacitance model proposed by Meyer.  The  second

choice  is  the  charge-controlled capacitance model of Ward and

Dutton (note #1.) The XQC model parameter acts as a flag  and  a

coefficient  at the same time.  As the former it causes the pro-

gram to use Meyer's model whenever larger than 0.5 or not speci-

fied,  and  the  charge-controlled model when between 0 and 0.5.

In the latter case its value defines the share  of  the  channel

charge  associated with the drain terminal in the saturation re-

gion.  The thin-oxide charge storage effects are treated slight-

ly different for the LEVEL=1 model.  These voltage dependent ca-

pacitances are included only if TOX is specified  in  the  input

description and they are represented using Meyer's formulation.

There is some overlap among the parameters  describing  the

junctions,  e.g.   the reverse current can be input either as IS

(in A) or as JS (in A/m**2).  Whereas the first is  an  absolute

value  the second is multiplied by AD and AS to give the reverse

current of the drain and source  junctions  respectively.   This

methodology  has been chosen since there is no sense in relating

always junction characteristics with AD and AS  entered  on  the

device card;  the areas can be defaulted.  The same idea applies

also to the zero-bias junction capacitances CBD and CBS  (in  F)

on  one  hand,  and  CJ (in F/m**2) on the other.  The parasitic

drain and source series resistance can be expressed as either RD

and  RS  (in ohms) or RSH (in ohms/sq.), the latter being multi-

plied by the number of squares NRD and NRS input on  the  device

card.

name     parameter                             units     default  example

____     _________                             _____     _______  _______

1    LEVEL    model index                           -         1

2    VTO      zero-bias threshold voltage           V         0.0      1.0

3    KP       transconductance parameter            A/V**2    2.0E-5   3.1E-5

4    GAMMA    bulk threshold parameter              V**0.5    0.0      0.37

5    PHI      surface potential                     V         0.6      0.65

6    LAMBDA   channel-length modulation

(MOS1 and MOS2 only)                  1/V       0.0      0.02

7    RD       drain ohmic resistance                Ohm       0.0      1.0

8    RS       source ohmic resistance               Ohm       0.0      1.0

9    CBD      zero-bias B-D junction capacitance    F         0.0      20FF

10   CBS      zero-bias B-S junction capacitance    F         0.0      20FF

11   IS       bulk junction saturation current      A         1.0E-14  1.0E-15

12   PB       bulk junction potential               V         0.8      0.87

13   CGSO     gate-source overlap capacitance

per meter channel width               F/m       0.0      4.0E-11

14   CGDO     gate-drain overlap capacitance

per meter channel width               F/m       0.0      4.0E-11

15   CGBO     gate-bulk overlap capacitance

per meter channel length              F/m       0.0      2.0E-10

16   RSH      drain and source diffusion

sheet resisitance                     Ohm/sq.   0.0      10.0

17   CJ       zero-bias bulk junction bottom cap.

per sq-meter of junction area         F/m**2    0.0      2.0E-4

SEMICONDUCTOR DEVICES                                                   PAGE 7-9

18   MJ       bulk junction bottom grading coef.    -         0.5      0.5

19   CJSW     zero-bias bulk junction sidewall cap.

per meter of junction perimeter       F/m       0.0      1.0E-9

20   MJSW     bulk junction sidewall grading coef.  -         0.33

21   JS       bulk junction saturation current

per sq-meter of junction area         A/m**2             1.0E-8

22   TOX      oxide thickness                       meter     1.0E-7   1.0E-7

23   NSUB     substrate doping                      1/cm**3   0.0      4.0E15

24   NSS      surface state density                 1/cm**2   0.0      1.0E10

25   NFS      fast surface state density            1/cm**2   0.0      1.0E10

26   TPG      type of gate material:                -         1.0

+1 opp. to substrate

-1 same as substrate

0  Al gate

27   XJ       metallurgical junction depth          meter     0.0      1U

28   LD       lateral diffusion                     meter     0.0      0.8U

29   UO       surface mobility                      cm**2/V-s 600      700

30   UCRIT    critical field for mobility

degradation (MOS2 only)               V/cm      1.0E4    1.0E4

31   UEXP     critical field exponent in

mobility degradation (MOS2 only)      -         0.0      0.1

32   UTRA     transverse field coef (mobility)

(deleted for MOS2)                    -         0.0      0.3

33   VMAX     maximum drift velocity of carriers    m/s       0.0      5.0E4

34   NEFF     total channel charge (fixed and

mobile) coefficient (MOS2 only)       -         1.0      5.0

35   XQC      thin-oxide capacitance model flag

and coefficient of channel charge

share attributed to drain (0-0.5)     -         1.0      0.4

36   KF       flicker noise coefficient             -         0.0      1.0E-26

37   AF       flicker noise exponent                -         1.0      1.2

38   FC       coefficient for forward-bias

depletion capacitance formula         -         0.5

39   DELTA    width effect on threshold voltage

(MOS2 and MOS3)                       -         0.0      1.0

40   THETA    mobility modulation (MOS3 only)       1/V       0.0      0.1

41   ETA      static feedback (MOS3 only)           -         0.0      1.0

42   KAPPA    saturation field factor (MOS3 only)   -         0.2      0.5

Note #1 A.  Vladimirescu and S.  Liu,  "The  Simulation  of

MOS  Integrated  Circuits  Using  SPICE2",  ERL  Memo  No.   ERL

M80/7,Electronics Research Laboratory, University of California,

Berkeley, Oct.  1980.

CHAPTER 8

SUBCIRCUITS

A subcircuit that consists of SPICE elements can be defined

and  referenced in a fashion similar to device models.  The sub-

circuit is defined in the input deck by a  grouping  of  element

cards;  the program then automatically inserts the group of ele-

ments wherever the subcircuit is referenced.  There is no  limit

on  the  size  or complexity of subcircuits, and subcircuits may

contain other subcircuits.  An example of  subcircuit  usage  is

given in Appendix A.

8.1  SUBCKT Card

General form

_______ ____

.SUBCKT subnam N1 <N2 N3 ...>

Examples

________

.SUBCKT OPAMP 1 2 3 4

A circuit definition is begun with a .SUBCKT card.   SUBNAM

is the subcircuit name, and N1, N2, ...  are the external nodes,

which cannot be zero.  The group of element cards which  immedi-

ately  follow  the .SUBCKT card define the subcircuit.  The last

card in a subcircuit definition is the .ENDS card  (see  below).

Control  cards  may  not  appear within a subcircuit definition;

however, subcircuit definitions may contain anything  else,  in-

cluding other subcircuit definitions, device models, and subcir-

cuit calls (see below).  Note that any device models or  subcir-

cuit definitions included as part of a subcircuit definition are

strictly local (i.e., such models and definitions are not  known

outside the subcircuit definition).  Also, any element nodes not

included on the .SUBCKT card are strictly local, with the excep-

tion of 0 (ground) which is always global.

SUBCIRCUITS                                                     PAGE 8-2

8.2  .ENDS Card

General form

_______ ____

.ENDS <SUBNAM>

Examples

________

.ENDS OPAMP

This card must be the last one for any  subcircuit  defini-

tion.  The subcircuit name, if included, indicates which subcir-

cuit definition is being terminated;  if omitted, all  subcircu-

its  being defined are terminated.  The name is needed only when

nested subcircuit definitions are being made.

8.3  Subcircuit Calls

General form

_______ ____

XYYYYYYY N1 <N2 N3 ...> SUBNAM

Examples

________

X1 2 4 17 3 1 MULTI

Subcircuits are used in SPICE by specifying pseudo-elements

beginning with the letter X, followed by the circuit nodes to be

used in expanding the subcircuit.

CHAPTER 9

CONTROL CARDS

9.1  .TEMP Card

General form

_______ ____

.TEMP T1 <T2 <T3 ...>>

Examples

________

.TEMP -55.0 25.0 125.0

This card specifies the temperatures at which  the  circuit

is  to  be  simulated.   T1, T2, ...  Are the different tempera-

tures, in degrees C.  Temperatures less than -223.0  deg  C  are

ignored.  Model data are specified at TNOM degrees (see the .OP-

TIONS card for TNOM);  if the .TEMP card is omitted, the simula-

tion will also be performed at a temperature equal to TNOM.

9.2  .WIDTH Card

General form

_______ ____

.WIDTH IN=COLNUM OUT=COLNUM

Examples

________

.WIDTH IN=72 OUT=133

COLNUM is the last column read from  each  line  of  input;

the  setting  takes effect with the next line read.  The default

value for COLNUM is 80.  The out parameter specifies the  output

print  width.  Permissible values for the output print width are

80 and 133.

CONTROL CARDS                                                   PAGE 9-2

9.3  .OPTIONS Card

General form

_______ ____

.OPTIONS OPT1 OPT2 ... (or OPT=OPTVAL ...)

Examples

________

.OPTIONS ACCT LIST NODE

This card allows the user to reset program control and user

options  for  specific  simulation purposes.  Any combination of

the following options  may  be  included,  in  any  order.   'x'

(below) represents some positive number.

option        effect

______        ______

ACCT          causes accounting and run time statistics to be printed

LIST          causes the summary listing of the input data to be printed

NOMOD         suppresses the printout of the model parameters.

NOPAGE        suppresses page ejects

NODE          causes the printing of the node table.

OPTS          causes the option values to be printed.

GMIN=x        resets the value of GMIN, the minimum conductance

allowed by the program.  The default value is 1.0E-12.

RELTOL=x      resets the relative error tolerance of the program.  The

default value is 0.001 (0.1 percent).

ABSTOL=x      resets the absolute current error tolerance of the

program.  The default value is 1 picoamp.

VNTOL=x       resets the absolute voltage error tolerance of the

program.  The default value is 1 microvolt.

TRTOL=x       resets the transient error tolerance.  The default value

is 7.0.  This parameter is an estimate of the factor by

which SPICE overestimates the actual truncation error.

CHGTOL=x      resets the charge tolerance of the program.  The default

value is 1.0E-14.

PIVTOL=x      resets the absolute minimum value for a matrix entry

to be accepted as a pivot.  The default value is 1.0E-13.

PIVREL=x      resets the relative ratio between the largest column entry

and an acceptable pivot value. The default value is 1.0E-3.

In the numerical pivoting algorithm the allowed minimum

pivot value is determined by

EPSREL=AMAX1(PIVREL*MAXVAL,PIVTOL)

where MAXVAL is the maximum element in the column where

a pivot is sought (partial pivoting).

NUMDGT=x      resets the number of significant digits printed for

output variable values.  X must satisfy the relation

0 < x < 8.  The default value is 4.  Note:  this option is

independent of the error tolerance used by SPICE (i.e., if

the values of options RELTOL, ABSTOL, etc., are not changed

then one may be printing numerical 'noise' for NUMDGT > 4.

TNOM=x        resets the nominal temperature.  The default value is

27 deg C (300 deg K).

ITL1=x        resets the dc iteration limit.  The default is 100.

ITL2=x        resets the dc transfer curve iteration limit.  The

default is 50.

CONTROL CARDS                                                         PAGE 9-3

ITL3=x        resets the lower transient analysis iteration limit.

the default value is 4.

ITL4=x        resets the transient analysis timepoint iteration limit.

the default is 10.

ITL5=x        resets the transient analysis total iteration limit.

the default is 5000.  Set ITL5=0 to omit this test.

CPTIME=x      the maximum cpu-time in seconds allowed for this job.

LIMTIM=x      resets the amount of cpu time reserved by SPICE for

generating plots should a cpu time-limit cause job

termination.  The default value is 2 (seconds).

LIMPTS=x      resets the total number of points that can be printed

or plotted in a dc, ac, or transient analysis.  The

default value is 201.

LVLCOD=x      if x is 2 (two), then machine code for the matrix

solution will be generated.  Otherwise, no machine code is

generated.  The default value is 2.  Applies only to CDC

computers.

LVLTIM=x      if x is 1 (one), the iteration timestep control is used.

if x is 2 (two), the truncation-error timestep is used.

The default value is 2.  If method=Gear and MAXORD>2 then

LVLTIM is set to 2 by SPICE.

METHOD=name   sets the numerical integration method used by SPICE.

Possible names are Gear or trapezoidal.  The default is

trapezoidal.

MAXORD=x      sets the maximum order for the integration method if

Gear's variable-order method is used.  X must be between

2 and 6.  The default value is 2.

DEFL=x        resets the value for MOS channel length; the default

is 100.0 micrometer.

DEFW=x        resets the value for MOS channel width; the default

is 100.0 micrometer.

DEFAD=x       resets the value for MOS drain diffusion area; the

default is 0.0.

DEFAS=x       resets the value for MOS source diffusion area; the

default is 0.0.

9.4  .OP Card

General form

_______ ____

.OP

The inclusion of this card in  an  input  deck  will  force

SPICE  to  determine  the dc operating point of the circuit with

inductors shorted and capacitors opened.  Note:  a  dc  analysis

is  automatically performed prior to a transient analysis to de-

termine the transient initial conditions, and  prior  to  an  ac

small-signal  analysis to determine the linearized, small-signal

models for nonlinear devices.

SPICE performs a dc operating point analysis  if  no  other

analyses are requested.

CONTROL CARDS                                                   PAGE 9-4

9.5  .DC Card

General form

_______ ____

.DC SRCNAM VSTART VSTOP VINCR SRC2 START2 STOP2 INCR2

Examples

________

.DC VIN 0.25 5.0 0.25

.DC VDS 0 10 .5 VGS 0 5 1

.DC VCE 0 10 .25 IB 0 10U 1U

This card defines the dc transfer curve  source  and  sweep

limits.  SRCNAM is the name of an independent voltage or current

source.  VSTART, VSTOP, and VINCR are the starting,  final,  and

incrementing  values respectively.  The first example will cause

the value of the voltage source VIN to be swept from 0.25  Volts

to  5.0  Volts  in  increments  of  0.25 Volts.  A second source

(SRC2) may optionally be specified with associated sweep parame-

ters.   In  this  case,  the first source will be swept over its

range for each value of the second source.  This option  can  be

useful  for  obtaining  semiconductor device output characteris-

tics.  See the second example data deck in that section  of  the

guide.

9.6  .NODESET Card

General form

_______ ____

.NODESET V(NODNUM)=VAL V(NODNUM)=VAL ...

Examples

________

.NODESET V(12)=4.5 V(4)=2.23

This card helps the program find the dc or initial  transi-

ent  solution  by  making  a preliminary pass with the specified

nodes held to the given voltages.  The restriction is  then  re-

leased  and  the  iteration continues to the true solution.  The

.NODESET card may be necessary for convergence  on  bistable  or

astable  circuits.  In general, this card should not be necessa-

ry.

9.7  .IC Card

General form

_______ ____

.IC V(NODNUM)=VAL V(NODNUM)=VAL ...

Examples

________

CONTROL CARDS                                                   PAGE 9-5

.IC V(11)=5 V(4)=-5 V(2)=2.2

This card is for setting transient initial conditions.   It

has  two different interpretations, depending on whether the UIC

parameter is specified on the .TRAN card.  Also, one should  not

confuse  this card with the .NODESET card.  The .NODESET card is

only to help dc convergence, and does not affect final bias  so-

lution  (except for multi-stable circuits).  The two interpreta-

tions of this card are as follows:

1.  When the UIC parameter is specified on the .TRAN  card,

then  the  node  voltages specified on the .IC card are

used to compute the capacitor, diode,  BJT,  JFET,  and

MOSFET initial conditions.  This is equivalent to spec-

ifying the IC=...  parameter on each device  card,  but

is  much  more  convenient.   The IC=...  parameter can

still be specified and will take  precedence  over  the

.IC values.  Since no dc bias (initial transient) solu-

tion is computed before  the  transient  analysis,  one

should  take  care to specify all dc source voltages on

the .IC card if they are to be used to  compute  device

initial conditions.

2.  When the UIC parameter is not specified  on  the  .TRAN

card,  the dc bias (initial transient) solution will be

computed before the transient analysis.  In this  case,

the  node  voltages  specified  on the .IC card will be

forced to the desired initial values  during  the  bias

solution.  During transient analysis, the constraint on

these node voltages is removed.

9.8  .TF Card

General form

_______ ____

.TF OUTVAR INSRC

Examples

________

.TF V(5,3) VIN

.TF I(VLOAD) VIN

This card defines the small-signal output and input for the

dc  small-signal  analysis.   OUTVAR  is the small-signal output

variable and INSRC is the small-signal input  source.   If  this

card  is  included, SPICE will compute the dc small-signal value

of the transfer function (output/input), input  resistance,  and

output  resistance.   For the first example, SPICE would compute

the ratio of V(5,3) to VIN, the small-signal input resistance at

VIN,  and  the  small-signal  output  resistance measured across

nodes 5 and 3.

CONTROL CARDS                                                   PAGE 9-6

9.9  .SENS Card

General form

_______ ____

.SENS OV1 <OV2 ... >

Examples

________

.SENS V(9) V(4,3) V(17) I(VCC)

If a .SENS card is included in the input deck,  SPICE  will

determine  the  dc  small-signal sensitivities of each specified

output variable with respect to every circuit parameter.   Note:

for large circuits, large amounts of output can be generated.

9.10  .AC Card

General form

_______ ____

.AC DEC ND FSTART FSTOP

.AC OCT NO FSTART FSTOP

.AC LIN NP FSTART FSTOP

Examples

________

.AC DEC 10 1 10K

.AC DEC 10 1K 100MEG

.AC LIN 100 1 100HZ

DEC stands for decade variation, and ND is  the  number  of

points  per  decade.  OCT stands for octave variation, and NO is

the number of points per octave.  LIN stands for  linear  varia-

tion,  and  NP  is the number of points.  FSTART is the starting

frequency, and FSTOP is the final frequency.  If  this  card  is

included  in  the deck, SPICE will perform an ac analysis of the

circuit over the specified frequency range.  Note that in  order

for  this  analysis  to  be meaningful, at least one independent

source must have been specified with an ac value.

9.11  .DISTO Card

General form

_______ ____

.DISTO RLOAD <INTER <SKW2 <REFPWR <SPW2>>>>

Examples

________

.DISTO RL 2 0.95 1.0E-3 0.75

This card controls whether SPICE will compute  the  distor-

tion  characteristic  of the circuit in a small-signal mode as a

CONTROL CARDS                                                   PAGE 9-7

part of the ac small-signal  sinusoidal  steady-state  analysis.

The  analysis  is performed assuming that one or two signal fre-

quencies are imposed at the input;  let the two  frequencies  be

f1 (the nominal analysis frequency) and f2 (=SKW2*f1).  The pro-

gram then computes the following distortion measures:

1.  HD2 - the magnitude of the frequency component 2*f1 as-

suming that f2 is not present.

2.  HD3 - the magnitude of the frequency component 3*f1 as-

suming that f2 is not present.

3.  SIM2 - the magnitude of the frequency  component  f1  +

f2.

4.  DIM2 - the magnitude of the frequency  component  f1  -

f2.

5.  DIM3 - the magnitude of the frequency component 2*f1  -

f2.

RLOAD is the name of the output load  resistor  into  which

all  distortion power products are to be computed.  INTER is the

interval at which the summary printout of the  contributions  of

all  nonlinear devices to the total distortion is to be printed.

If omitted or set to zero, no summary  printout  will  be  made.

REFPWR  is  the reference power level used in computing the dis-

tortion products;  if omitted, a value of 1 mW (that is, 1  dbm)

is used.  SKW2 is the ratio of f2 to f1.  If omitted, a value of

0.9 is used (i.e., f2 = 0.9*f1).  SPW2 is the amplitude  of  f2.

If omitted, a value of 1.0 is assumed.

The distortion measures HD2, HD3, SIM2, DIM2, and DIM3  may

also  be  be  printed and/or plotted (see the description of the

.PRINT and .PLOT cards).

9.12  .NOISE Card

General form

_______ ____

.NOISE OUTV INSRC NUMS

Examples

________

.NOISE V(5) VIN 10

This card controls the noise analysis of the circuit.   The

noise  analysis is performed in conjunction with the ac analysis

(see .AC card).  OUTV is an output  voltage  which  defines  the

summing  point.  INSRC is the name of the independent voltage or

current source which is the noise input reference.  NUMS is  the

summary  interval.   SPICE  will  compute  the equivalent output

CONTROL CARDS                                                   PAGE 9-8

noise at the specified output as well as  the  equivalent  input

noise at the specified input.  In addition, the contributions of

every noise generator in the circuit will be  printed  at  every

NUMS  frequency points (the summary interval).  If NUMS is zero,

no summary printout will be made.

The output noise and the equivalent input noise may also be

printed  and/or  plotted  (see the description of the .PRINT and

.PLOT cards).

9.13  .TRAN Card

General form

_______ ____

.TRAN TSTEP TSTOP <TSTART <TMAX>> <UIC>

Examples

________

.TRAN 1NS 100NS

.TRAN 1NS 1000NS 500NS

.TRAN 10NS 1US UIC

TSTEP  is  the   printing   or   plotting   increment   for

line-printer  output.  For use with the post-processor, TSTEP is

the suggested computing increment.  TSTOP is the final time, and

TSTART is the initial time.  If TSTART is omitted, it is assumed

to be zero.  The transient analysis always begins at time  zero.

In  the  interval  <zero,  TSTART>,  the circuit is analyzed (to

reach a steady state), but no outputs are stored.  In the inter-

val  <TSTART,  TSTOP>,  the  circuit is analyzed and outputs are

stored.  TMAX is the maximum stepsize that SPICE will  use  (for

default,     the     program    chooses    either    TSTEP    or

(TSTOP-TSTART)/50.0, whichever is smaller.  TMAX is useful  when

one  wishes  to  guarantee a computing interval which is smaller

than the printer increment, TSTEP.

UIC (use initial conditions) is an optional  keyword  which

indicates  that  the  user  does not want SPICE to solve for the

quiescent operating point before beginning  the  transient  ana-

lysis.   If  this  keyword  is  specified, SPICE uses the values

specified using IC=...  on the various elements as  the  initial

transient  condition and proceeds with the analysis.  If the .IC

card has been specified, then the node voltages on the .IC  card

are  used  to  compute  the intitial conditions for the devices.

Look at the description on the .IC card for  its  interpretation

when UIC is not specified.

CONTROL CARDS                                                   PAGE 9-9

9.14  .FOUR Card

General form

_______ ____

.FOUR FREQ OV1 <OV2 OV3 ...>

Examples

________

.FOUR 100K  V(5)

This card controls whether SPICE performs  a  Fourier  ana-

lysis  as  a part of the transient analysis.  FREQ is the funda-

mental frequency, and OV1, ..., are  the  output  variables  for

which  the  analysis  is  desired.  The Fourier analysis is per-

formed over the interval <TSTOP-period, TSTOP>, where  TSTOP  is

the  final time specified for the transient analysis, and period

is one period of the fundamental frequency.   The  dc  component

and the first nine components are determined.  For maximum accu-

racy, TMAX (see the .TRAN card) should be  set  to  period/100.0

(or less for very high-Q circuits).

9.15  .PRINT Cards

General form

_______ ____

.PRINT PRTYPE OV1 <OV2 ... OV8>

Examples

________

.PRINT TRAN V(4) I(VIN)

.PRINT AC VM(4,2) VR(7) VP(8,3)

.PRINT DC V(2) I(VSRC) V(23,17)

.PRINT NOISE INOISE

.PRINT DISTO HD3 SIM2(DB)

This card defines the contents of a tabular listing of  one

to  eight  output variables.  PRTYPE is the type of the analysis

(DC, AC, TRAN, NOISE, or DISTO) for which the specified  outputs

are  desired.   The form for voltage or current output variables

is as follows:

V(N1<,N2>)

specifies the voltage difference between nodes N1 and N2.  If N2

(and  the  preceding  comma)  is omitted, ground (0) is assumed.

For the ac analysis, five additional outputs can be accessed  by

replacing the letter V by:

VR  -    real part

VI  -    imaginary part

VM  -    magnitude

VP  -    phase

VDB -    20*log10(magnitude)

CONTROL CARDS                                                  PAGE 9-10

I(VXXXXXXX)

specifies the current flowing in the independent voltage  source

named  VXXXXXXX.  Positive current flows from the positive node,

through the source, to the negative node.  For the ac  analysis,

the  corresponding  replacements for the letter I may be made in

the same way as described for voltage outputs.

Output variables for the noise and distortion analyses have

a different general form from that of the other analyses, i.e.

OV<(X)>

where OV is any of ONOISE  (output  noise),  INOISE  (equivalent

input  noise),  D2, HD3, SIM2, DIM2, or DIM3 (see description of

distortion analysis), and X may be any of:

R  -    real part

I  -    imaginary part

M  -    magnitude (default if nothing specified)

P  -    phase

DB -    20*log10(magnitude)

thus, SIM2 (or SIM2(M)) describes the magnitude of the SIM2 dis-

tortion measure, while HD2(R) describes the real part of the HD2

distortion measure.

There is no limit on the number of .PRINT  cards  for  each

type of analysis.

9.16  .PLOT Cards

General form

_______ ____

.PLOT PLTYPE OV1 <(PLO1,PHI1)> <OV2 <(PLO2,PHI2)> ... OV8>

Examples

________

.PLOT DC V(4) V(5) V(1)

.PLOT TRAN V(17,5) (2,5) I(VIN) V(17) (1,9)

.PLOT AC VM(5) VM(31,24) VDB(5) VP(5)

.PLOT DISTO HD2 HD3(R) SIM2

.PLOT TRAN V(5,3) V(4) (0,5) V(7) (0,10)

This card defines the contents of one plot of from  one  to

eight output variables.  PLTYPE is the type of analysis (DC, AC,

TRAN, NOISE, or DISTO) for which the specified outputs  are  de-

sired.   The  syntax  for  the  OVI is identical to that for the

.PRINT card, described above.

The optional plot limits (PLO,PHI) may be  specified  after

CONTROL CARDS                                                  PAGE 9-11

any  of  the output variables.  All output variables to the left

of a pair of plot limits (PLO,PHI) will  be  plotted  using  the

same lower and upper plot bounds.  If plot limits are not speci-

fied, SPICE will automatically determine the minimum and maximum

values  of all output variables being plotted and scale the plot

to fit.  More than one scale will be used if the output variable

values  warrant (i.e., mixing output variables with values which

are orders-of-magnitude different still gives readable plots).

The overlap of two or more traces on any plot is  indicated

by the letter X.

When more than one output  variable  appears  on  the  same

plot,  the  first  variable specified will be printed as well as

plotted.  If a printout of all variables is desired, then a com-

panion .PRINT card should be included.

There is no limit on the number of  .PLOT  cards  specified

for each type of analysis.

CHAPTER 10

APPENDIX A   EXAMPLE DATA DECKS

10.1  Circuit 1

The following deck determines the dc  operating  point  and

small-signal  transfer  function  of a simple differential pair.

In addition, the ac small-signal response is computed  over  the

frequency range 1Hz to 100MEGHz.

SIMPLE DIFFERENTIAL PAIR

VCC 7 0 12

VEE 8 0 -12

VIN 1 0 AC 1

RS1 1 2 1K

RS2 6 0 1K

Q1 3 2 4 MOD1

Q2 5 6 4 MOD1

RC1 7 3 10K

RC2 7 5 10K

RE 4 8 10K

.MODEL MOD1 NPN BF=50 VAF=50 IS=1.E-12 RB=100 CJC=.5PF TF=.6NS

.TF V(5) VIN

.AC DEC 10 1 100MEG

.PLOT AC VM(5) VP(5)

.PRINT AC VM(5) VP(5)

.END

10.2  Circuit 2

The following deck computes the output characteristics of a

MOSFET device over the range 0-10V for VDS and 0-5V for VGS.

MOS OUTPUT CHARACTERISTICS

.OPTIONS NODE NOPAGE

VDS 3 0

VGS 2 0

M1 1 2 0 0 MOD1 L=4U W=6U AD=10P AS=10P

.MODEL MOD1 NMOS VTO=-2 NSUB=1.0E15 UO=550

APPENDIX A   EXAMPLE DATA DECKS                                        PAGE 10-2

*VIDS MEASURES ID, WE COULD HAVE USED VDS BUT ID WOULD BE NEGATIVE

VIDS 3 1

.DC VDS 0 10 .5 VGS 0 5 1

.PRINT DC I(VIDS) V(2)

.PLOT DC I(VIDS)

.END

10.3  Circuit 3

The following deck determines the dc transfer curve and the

transient pulse response of a simple RTL inverter.  The input is

a pulse from 0 to 5 Volts with delay, rise, and  fall  times  of

2ns  and  a pulse width of 30ns.  The transient interval is 0 to

100ns, with printing to be done every nanosecond.

SIMPLE RTL INVERTER

VCC 4 0 5

VIN 1 0 PULSE 0 5 2NS 2NS 2NS 30NS

RB 1 2 10K

Q1 3 2 0 Q1

RC 3 4 1K

.PLOT DC V(3)

.PLOT TRAN V(3) (0,5)

.PRINT TRAN V(3)

.MODEL Q1 NPN BF 20 RB 100 TF .1NS CJC 2PF

.DC VIN 0 5 0.1

.TRAN 1NS 100NS

.END

10.4  Circuit 4

The following deck simulates a four-bit binary adder, using

several  subcircuits  to  describe various pieces of the overall

circuit.

ADDER - 4 BIT ALL-NAND-GATE BINARY ADDER

*** SUBCIRCUIT DEFINITIONS

.SUBCKT NAND 1 2 3 4

*   NODES:  INPUT(2), OUTPUT, VCC

Q1 9 5 1 QMOD

D1CLAMP 0 1 DMOD

Q2 9 5 2 QMOD

D2CLAMP 0 2 DMOD

RB 4 5 4K

R1 4 6 1.6K

Q3 6 9 8 QMOD

R2 8 0 1K

RC 4 7 130

APPENDIX A   EXAMPLE DATA DECKS                                PAGE 10-3

Q4 7 6 10 QMOD

DVBEDROP 10 3 DMOD

Q5 3 8 0 QMOD

.ENDS NAND

.SUBCKT ONEBIT 1 2 3 4 5 6

* NODES:  INPUT(2), CARRY-IN, OUTPUT, CARRY-OUT, VCC

X1 1 2 7 6 NAND

X2 1 7 8 6 NAND

X3 2 7 9 6 NAND

X4 8 9 10 6 NAND

X5 3 10 11 6 NAND

X6 3 11 12 6 NAND

X7 10 11 13 6 NAND

X8 12 13 4 6 NAND

X9 11 7 5 6 NAND

.ENDS ONEBIT

.SUBCKT TWOBIT 1 2 3 4 5 6 7 8 9

* NODES:  INPUT - BIT0(2) / BIT1(2), OUTPUT - BIT0 / BIT1,

*        CARRY-IN, CARRY-OUT, VCC

X1 1 2 7 5 10 9 ONEBIT

X2 3 4 10 6 8 9 ONEBIT

.ENDS TWOBIT

.SUBCKT FOURBIT 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15

*NODES:  INPUT - BIT0(2) / BIT1(2) / BIT2(2) / BIT3(2),

* OUTPUT - BIT0 / BIT1 / BIT2 / BIT3, CARRY-IN, CARRY-OUT, VCC

X1 1 2 3 4 9 10 13 16 15 TWOBIT

X2 5 6 7 8 11 12 16 14 15 TWOBIT

.ENDS FOURBIT

*** DEFINE NOMINAL CIRCUIT

.MODEL DMOD D

.MODEL QMOD NPN(BF=75 RB=100 CJE=1PF CJC=3PF)

VCC 99 0 DC 5V

VIN1A 1 0 PULSE(0 3 0 10NS 10NS   10NS   50NS)

VIN1B 2 0 PULSE(0 3 0 10NS 10NS   20NS  100NS)

VIN2A 3 0 PULSE(0 3 0 10NS 10NS   40NS  200NS)

VIN2B 4 0 PULSE(0 3 0 10NS 10NS   80NS  400NS)

VIN3A 5 0 PULSE(0 3 0 10NS 10NS  160NS  800NS)

VIN3B 6 0 PULSE(0 3 0 10NS 10NS  320NS 1600NS)

VIN4A 7 0 PULSE(0 3 0 10NS 10NS  640NS 3200NS)

VIN4B 8 0 PULSE(0 3 0 10NS 10NS 1280NS 6400NS)

X1 1 2 3 4 5 6 7 8 9 10 11 12 0 13 99 FOURBIT

RBIT0 9 0 1K

RBIT1 10 0 1K

RBIT2 11 0 1K

RBIT3 12 0 1K

RCOUT 13 0 1K

.PLOT TRAN V(1) V(2) V(3) V(4) V(5) V(6) V(7) V(8)

.PLOT TRAN V(9) V(10) V(11) V(12) V(13)

.PRINT TRAN V(1) V(2) V(3) V(4) V(5) V(6) V(7) V(8)

.PRINT TRAN V(9) V(10) V(11) V(12) V(13)

APPENDIX A   EXAMPLE DATA DECKS                                PAGE 10-4

*** (FOR THOSE WITH MONEY (AND MEMORY) TO BURN)

.TRAN 1NS 6400NS

.OPTIONS ACCT LIST NODE LIMPTS=6401

.END

10.5  Circuit 5

The following deck simulates a transmission-line  inverter.

Two  transmission-line  elements are required since two propaga-

tion modes are excited.  In the case  of  a  coaxial  line,  the

first  line  (T1) models the inner conductor with respect to the

shield, and the second line (T2) models the shield with  respect

to the out- side world.

TRANSMISSION-LINE INVERTER

V1 1 0 PULSE(0 1 0 0.1N)

R1 1 2 50

X1 2 0 0 4 TLINE

R2 4 0 50

.SUBCKT TLINE 1 2 3 4

T1 1 2 3 4 Z0=50 TD=1.5NS

T2 2 0 4 0 Z0=100 TD=1NS

.ENDS TLINE

.TRAN 0.1NS 20NS

.PLOT TRAN V(2) V(4)

.END

CHAPTER 11

APPENDIX B   NONLINEAR DEPENDENT SOURCES

SPICE allows circuits to contain dependent sources  charac-

terized by any of the four equations

i=f(v)          v=f(v)          i=f(i)          v=f(i)

where the functions must be polynomials, and the  arguments  may

be  multidimensional.  The polynomial functions are specified by

a set of coefficients p0, p1, ..., pn.  Both the number  of  di-

mensions  and the number of coefficients are arbitrary.  The me-

aning of the coefficients depends upon the dimension of the  po-

lynomial, as shown in the following examples:

Suppose that the function is one-dimensional  (that  is,  a

function of one argument).  Then the function value fv is deter-

mined by the following expression in fa (the function argument):

fv = p0 + (p1*fa) + (p2*fa**2) + (p3*fa**3) + (p4*fa**4)

+ (p5*fa**5) + ...

Suppose now that the function is two-dimensional, with  ar-

guments  fa and fb.  Then the function value fv is determined by

the following expression:

fv = p0 + (p1*fa) +  (p2*fb)  +  (p3*fa**2)  +  (p4*fa*fb)  +

(p5*fb**2)  +  (p6*fa**3)  +  (p7*fa**2*fb)  +  (p8*fa*fb**2)

+ (p9*fb**3) + ...

Consider now the case  of  a  three-dimensional  polynomial

function with arguments fa, fb, and fc.  Then the function value

fv is determined by the following expression:

fv = p0 + (p1*fa) + (p2*fb) + (p3*fc) + (p4*fa**2) + (p5*fa*fb)

+ (p6*fa*fc)+(p7*fb**2) + (p8*fb*fc) + (p9*fc**2) + (p10*fa**3)

+ (p11*fa**2*fb) + (p12*fa**2*fc) + (p13*fa*fb**2)+(p14*fa*fb*fc)

+ (p15*fa*fc**2) + (p16*fb**3) + (p17*fb**2*fc) + (p18*fb*fc**2)

+ (p19*fc**3) + (p20*fa**4) + ...

Note:  if the polynomial is one-dimensional and exactly one

coefficient is specified, then SPICE assumes it to be p1 (and p0

= 0.0), in order to facilitate the input  of  linear  controlled

sources.

APPENDIX B   NONLINEAR DEPENDENT SOURCES                       PAGE 11-2

For all four of the dependent sources described below,  the

initial  condition  parameter  is described as optional.  If not

specified, SPICE assumes 0 the initial condition  for  dependent

sources  is  an initial 'guess' for the value of the controlling

variable.  The program uses this initial condition to obtain the

dc  operating  point of the circuit.  After convergence has been

obtained, the program continues iterating to  obtain  the  exact

value for the controlling variable.  Hence, to reduce the compu-

tational effort for the dc operating point (or if the polynomial

specifies  a  strong  nonlinearity), a value fairly close to the

actual controlling variable should be specified for the  initial

condition.

11.1  Voltage Controlled Current Sources

General form

_______ ____

GXXXXXXX N+ N- <POLY(ND)> NC1+ NC1- ... P0 <P1 ...> <IC=...>

Examples

________

G1 1 0 5 3 0 0.1M

GR 17 3 17 3 0 1M 1.5M IC=2V

GMLT 23 17 POLY(2) 3 5 1 2 0 1M 17M 3.5U IC=2.5, 1.3

N+ and N- are the positive and negative nodes,  respective-

ly.  Current flow is from the positive node, through the source,

to the negative node.  POLY(ND) only has to be specified if  the

source  is  multi-dimensional  (one-dimensional is the default).

If specified, ND is the number of dimensions, which must be  po-

sitive.  NC1+, NC1-, ...  Are the positive and negative control-

ling nodes, respectively.  One pair of nodes must  be  specified

for  each  dimension.   P0,  P1,  P2, ..., Pn are the polynomial

coefficients.  The (optional) initial condition is  the  initial

guess  at  the  value(s)  of the controlling voltage(s).  If not

specified, 0.0 is assumed.  The polynomial specifies the  source

current as a function of the controlling voltage(s).  The second

example above describes a current source with value

I = 1E-3*V(17,3) + 1.5E-3*V(17,3)**2

note that since the source nodes are the same as the controlling

nodes, this source actually models a nonlinear resistor.

11.2  Voltage Controlled Voltage Sources

General form

_______ ____

EXXXXXXX N+ N- <POLY(ND)> NC1+ NC1- ... P0 <P1 ...> <IC=...>

Examples

________

APPENDIX B   NONLINEAR DEPENDENT SOURCES                       PAGE 11-3

E1 3 4 21 17 10.5 2.1 1.75

EX 17 0 POLY(3) 13 0 15 0 17 0 0 1 1 1 IC=1.5,2.0,17.35

N+ and N- are the positive and negative nodes,  respective-

ly.   POLY(ND)  only has to be specified if the source is multi-

dimensional (one-dimensional is the default).  If specified,  ND

is  the  number  of  dimensions,  which must be positive.  NC1+,

NC1-, ...  are the positive and negative controlling nodes, res-

pectively.   One pair of nodes must be specified for each dimen-

sion.  P0, P1, P2, ..., Pn are the polynomial coefficients.  The

(optional)  initial  condition  is  the  initial  guess  at  the

value(s) of the controlling voltage(s).  If not  specified,  0.0

is  assumed.   The  polynomial specifies the source voltage as a

function of the  controlling  voltage(s).   The  second  example

above describes a voltage source with value

V = V(13,0) + V(15,0) + V(17,0)

(in other words, an ideal voltage summer).

11.3  Current Controlled Current Sources

General form

_______ ____

FXXXXXXX N+ N- <POLY(ND)> VN1 <VN2 ...> P0 <P1 ...> <IC=...>

Examples

________

F1 12 10 VCC 1MA 1.3M

FXFER 13 20 VSENS 0 1

N+ and N- are the positive and negative nodes,  respective-

ly.  Current flow is from the positive node, through the source,

to the negative node.  POLY(ND) only has to be specified if  the

source  is  multi-dimensional  (one-dimensional is the default).

If specified, ND is the number of dimensions, which must be  po-

sitive.  VN1, VN2, ...  are the names of voltage sources through

which the controlling current flows;  one name must be specified

for  each dimension.  The direction of positive controlling cur-

rent flow is from the positive node, through the source, to  the

negative  node  of each voltage source.  P0, P1, P2, ..., Pn are

the polynomial coefficients.  The (optional)  initial  condition

is  the  initial  guess  at  the  value(s)  of  the  controlling

current(s) (in Amps).  If not specified, 0.0  is  assumed.   The

polynomial  specifies  the  source  current as a function of the

controlling current(s).  The first  example  above  describes  a

current source with value

I = 1E-3 + 1.3E-3*I(VCC)

APPENDIX B   NONLINEAR DEPENDENT SOURCES                       PAGE 11-4

11.4  Current Controlled Voltage Sources

General form

_______ ____

HXXXXXXX N+ N- <POLY(ND)> VN1 <VN2 ...> P0 <P1 ...> <IC=...>

Examples

________

HXY 13 20 POLY(2) VIN1 VIN2 0 0 0 0 1 IC=0.5 1.3

HR 4 17 VX 0 0 1

N+ and N- are the positive and negative nodes,  respective-

ly.   POLY(ND)  only has to be specified if the source is multi-

dimensional (one-dimensional is the default).  If specified,  ND

is  the number of dimensions, which must be positive.  VN1, VN2,

... are the names of voltage sources through which the  control-

ling  current flows;  one name must be specified for each dimen-

sion.  The direction of positive  controlling  current  flow  is

from the positive node, through the source, to the negative node

of each voltage source.  P0, P1, P2, ..., Pn are the  polynomial

coefficients.   The  (optional) initial condition is the initial

guess at the value(s) of the controlling current(s)  (in  Amps).

If  not specified, 0.0 is assumed.  The polynomial specifies the

source voltage as a function of the controlling current(s).  The

first example above describes a voltage source with value

V = I(VIN1)*I(VIN2)

CHAPTER 12

APPENDIX C  BIPOLAR MODEL EQUATIONS

(Gmin terms omitted)

Acknowledgment:  This section has been contributed by  Bill

Bidermann at HP labs.

12.1  D C  MODEL

qVB'E'   qVB'C'       qVB'C'          qVB'C'

______   ______       ______          ______

IC=  IS   NF*kT    NR*kT   IS   NR*kT           NC*kT

__                    __

QB( e      - e      )-BR( e      -1)-ISC( e      -1)

qVB'E'         qVB'C'          qVB'E'          qVB'C'

______         ______          ______          ______

IB= IS   NF*kT     IS   NR*kT           NE*kT           NC*kT

__             __

BF( e      -1)+BR( e      -1)+ISE( e      -1)+ISC( e     -1)

NOTE:  The last two terms in the  expression  of  the  base

current  IB represent the components due to recombination in the

BE and BC space charge regions at low injection.

If IRB not specified

RBB' = RBM + RB-RBM

______

QB

If IRB specified

RBB' = 3(RB-RBM) * (TAN(Z)-Z)+RBM

______________

Z*TAN(Z)*TAN(Z)

Where:

-1+(144IB/(pi*pi*IRB)+1)**0.5

_____________________________

Z =   24/(pi*pi)*(IB/IRB)**0.5

Q1

__

QB = 2 (1+(1+4Q2)**0.5)

APPENDIX C  BIPOLAR MODEL EQUATIONS                            PAGE 12-2

______1______

_

Q1 =  1- VB'C' - VB'E'

_____   _____

VAF    VAR

qVB'E'          qVB'C'

______          ______

Q2 = IS   NF*kT      IS   NR*kT

__              __

IKF( e      -1)+IKR( e      -1)

NOTE:  IRB is the current where the base  resistance  falls

halfway  to  its minimum value.  VAF and VAR are forward and re-

verse Early voltages respectively.  IKF and  IKR  determine  the

high  current  beta rolloff with IC.  ISE, ISC, NE and NC deter-

mine the low current beta rolloff with IC.

12.2  A C  MODEL

qVB'E'

______

___d___      IS   NF*kT              VB'E'-MJ

_         __                      ________

CBE = d VB'E'( TFF*QB( e      -1)) + CJE( 1- VJE )

Where:

VB'C'

_____

2     1.44VTF

TFF = TF ( 1 + XTF * (IF/(IF+ITF))  * e        )

qVB'E'

______

NF*kT

IF = IS( e      -1)

CB1 = CBC*(1-XCJC)

CB2 = CBC * XCJC

qVB'C'

______

qIS     kT            VB'C' -MJC

___                   _____ ____

CBC = TR (NR*kT e      )+CJC ( 1- VJC )

VC'S' -MJS

_____ ____

CSS = CJS (1- VJS )

_V_-M

_

NOTE: all junction capacitances of the form  C0*(1-phi) revert

to the form

M*(V-FC*phi)

____________

C0/((1-FC)**M)  *  (1 +  phi(1-FC)  )

when V > FC*phi   ( For CSS assumes FC = 0 )

APPENDIX C  BIPOLAR MODEL EQUATIONS                            PAGE 12-3

12.3  NOISE MODEL

2    4kT

___

IRBB'   = RBB' DELTA f

2   4kT

___

IRC   = RC  DELTA f   Thermal noise

2   4kT

___

IRE   = RE  DELTA f

2                KF*IB**AF

_________

IBN   = 2qIB DELTA f+    f     DELTA f

Note:  The first term is shot noise and the second term  is

flicker noise.

2

ICN   = 2qIC DELTA f

Note: This is shot noise.

12.4  TEMPERATURE EFFECTS

All junctions have dependences identical to the diode model  but

all N factors are considered equal 1.

BF and  BR go as (__T_)XTB

_  _

TNOM

when NF=1.  This is done through appropriate changes in BF ,  BR

and ISE, ISC according to the following equations respectively:

__T_ XTB

_

BF' (or BR') = BF (or BR)*(TNOM)

qEG T-TNOM

___ ______

T  (XTI-XTB)   Nk  T*TNOM

_

ISE' (or ISC') = ISE (or ISC) *(TNOM)          *e

12.5  EXCESS PHASE

This is a delay (linear phase) in the gm  generator  in  AC

analysis.   It is also used in transient analysis using a Bessel

polynomial approximation.  Excess phase, PTF,  is  specified  as

APPENDIX C  BIPOLAR MODEL EQUATIONS                            PAGE 12-4

the number of extra degrees of phase at the frequency

__1__

_

f = 2piTF  Hertz

SPICE INDEX

___________

Page

----

1. TYPES OF ANALYSIS ...............................  1-1

1.1 DC Analysis .................................  1-1

1.2 AC Small Signal Analysis ....................  1-1

1.3 Transient Analysis ..........................  1-2

1.4 Analysis at Different Temperatures ..........  1-3

2. CONVERGENCE .....................................  2-1

3. INPUT FORMAT ....................................  3-1

4. CIRCUIT DESCRIPTION .............................  4-1

5. TITLE, COMMENT, AND END CARDS ...................  5-1

5.1 Title Card ..................................  5-1

5.2 END Card ....................................  5-1

5.3 Comment Card ................................  5-3

6. ELEMENT CARDS ...................................  6-1

6.1 Resistors ...................................  6-1

6.2 Capacitors and Inductors ....................  6-1

6.3 Coupled Mutual Inductors ....................  6-2

6.4 Transmission Lines, Lossless ................  6-2

6.5 Linear Dependent Sources ....................  6-3

6.6 Linear Voltage Controlled Current Sources ...  6-3

6.7 Linear Voltage Controlled Voltage Sources ...  6-4

6.8 Linear Current Controlled Current Sources ...  6-4

6.9 Linear Current Controlled Voltage Sources ...  6-4

6.10 Independent Sources ........................  6-5

6.10.1 Pulse ...............................  6-6

6.10.2 Sinusoidal ..........................  6-6

6.10.3 Exponential .........................  6-7

6.10.4 Piece-Wise Linear ...................  6-7

6.10.5 Single-Frequency FM .................  6-7

7. SEMICONDUCTOR DEVICES ...........................  7-1

7.1 Junction Diodes .............................  7-2

7.2 Bipolar Junction Transistors ................  7-2

7.3 Junction Field Effect Transistors ...........  7-3

7.4 MOSFETs .....................................  7-3

7.5 MODEL Card ..................................  7-4

7.6 Diode Model .................................  7-4

7.7 NPN and PNP BJT Models ......................  7-5

7.8 N and P Channel JFET Models .................  7-7

7.9 N and P Channel MOSFET Models ...............  7-7

8. SUBCIRCUITS .....................................  8-1

8.1 SUBCKT Card .................................  8-1

8.2 ENDS Card ...................................  8-2

8.3 Subcircuit Calls ............................  8-2

SPICE INDEX Cont'd

__________________

Page

----

9. CONTROL CARDS ...................................  9-1

9.1 TEMP Card ...................................  9-1

9.2 WIDTH CARD ..................................  9-1

9.3 OPTIONS Card ................................  9-2

9.4 OP Card .....................................  9-3

9.5 DC Card .....................................  9-4

9.6 NODESET Card ................................  9-4

9.7 IC Card .....................................  9-4

9.8 TF Card .....................................  9-5

9.9 SENS Card ...................................  9-6

9.10 AC Card ....................................  9-6

9.11 DISTO Card .................................  9-6

9.12 NOISE Card .................................  9-7

9.13 TRAN Card ..................................  9-8

9.14 FOUR Card ..................................  9-9

9.15 PRINT Card .................................  9-9

9.16 PLOT Card ..................................  9-10

10. APPENDIX A - EXAMPLE DATA DECKS ................. 10-1

10.1 Circuit 1 .................................. 10-1

10.2 Circuit 2 .................................. 10-1

10.3 Circuit 3 .................................. 10-2

10.4 Circuit 4 .................................. 10-2

10.5 Circuit 5 .................................. 10-4

11. APPENDIX B - NONLINEAR DEPENDENT SOURCES ........ 11-1

11.1 Voltage Controlled Current Sources ......... 11-2

11.2 Voltage Controllde Voltage Sources ......... 11-2

11.3 Current Controlled Current Sources ......... 11-3

11.4 Current Controlled Voltage Sources ......... 11-4

12. APPENDIX C - BIPOLAR MODEL EQUATIONS ............ 12-1

12.1 DC Model ................................... 12-1

12.2 AC Model ................................... 12-2

12.3 NOISE Model ................................ 12-3

12.4 Temperature Effects ........................ 12-3

12.5 Excess Phase ............................... 12-3

#####