VcEd Yamaha 4 OP Voice Editor

By: Charles A. Brand

Copyright (c) 1990

INSTRUCTIONS:

-------------

Intro

The VcEd Voice Editor is written for the Yamaha DS55 4 Operator FM

Synthesis Keyboard/Synthesizer.  The Editor allows you to build and Edit

your own Voices or Tones, and then lets you transmit them to the keyboard or

save them to a file.  Using the Editor is quite easy, and it allows you to

start from scratch or start with a Pre-Set voice by selecting a voice on the

keyboard and then use the Menu Selection in the Editor to Send the voice

from the keyboard to the Amiga and start Editing from there.

You Edit a voice, by using the mouse to click on the parameter value that

you want to change.  You work on one Operator at a time, and you click on

the Operator number to increment to the next Operator.

Start-Up

Upon starting VcEd you will see Initialized Voice data on the screen. At

this point you can start Editing from Scratch.  To change a parameter,

simply Click the pointer over top of the parameter and it will be

highlighted.  At this time you can use the Gadgets, at the Lower Right hand

corner of the screen, to change the parameter value.

The Gadgets

The Gadgets consist of a Slider Gadget, Left and Right Arrow Gadgets, and

a TEST Gadget.  The Gadgets function pretty much like you would expect them

to.

The Slider Gadget is clicked on and then Slid Left and Right, and while

doing so, it will show a value in the box below it.

The Left and Right Arrow Gadgets, when clicked on, will increment or

decrement the value in the box below the Slider Gadget.

The TEST gadget will transmit the voice, as it sits plus the changes you

are making to the voice, to the Keyboard, then sends a 'Note On' command

(Middle C) to let you hear the voice. After the tone has played, VcEd then

Sends the Voice back to the Keyboard the way it was before the changes. After

you set the parameter to the value that you want you have to click on the OK

Gadget to exit from Edit mode.

The TEST Gadget is also active while IN or OUT of Edit mode, however once

you leave Edit mode, the Voice you will hear is the voice as it sounded

before you changed a parameter, UNLESS you have the AUTO UPDATE MODE on, and

in this case, everytime you exit Edit Mode the new voice will automatically

be re-transmitted to the Keyboard. Otherwise, after you leave Edit Mode, you

will have to Send the voice to the Keyboard yourself with the Menu selection

shown below.

The Menus

The Main Menu has two selections, they are 'Init Voice' and 'Quit'. 'Init

Voice' initializes the voice to a standard 4 Operator INIT VOICE as defined

by Yamaha in the DX11 Owners Manual. I assumed that this was a Yamaha

Standard. 'Quit' does exactly that, it returns you to the CLI or Workbench.

The Load Menu allows you to Load a Voice from the Keyboard or from a

file.  You can load a voice from the keyboard, and start editing it from

that point. This is handy if there is a voice, in the Keyboard, that you

would like to make changes to.  You can Load a voice, that you were editing

or Saved earlier, from a disk file.  This is handy if you had a voice you

were Editing and just did not have time to finish, you can come back to it

later.

The Save Menu allows you to Save the voice to a disk file, or Send the

voice to the Keyboard.  You can Save a voice to a disk file to edit later or

just to keep around as your own INIT VOICE. After Editing a Voice to the way

you want it, you can Send it to the Keyboard and place it into the USER Voice

number you want by:

1. Transmit the voice to the keyboard with the Send Voice Menu selection.

2. On the keyboard Press the 'Store' button then the '-1' button to turn

off the memory protect.

3. Press the 'Store' button again and enter the USER MEMORY Voice number

that you wish to send the voice to. (TWO Digits. ex. 09)

4. At this time, the Keyboard will be asking you if it is OK, and showing

the name of the voice that is already at that location.

5. Press the '+1' button if you wish to store the voice, or the '-1'

button to cancel the operation.

6. At this time it would be a good idea to write protect the memory again

by Pressing the 'Store' button again and then the '+1' button.

7. The Voice is now in User Memory permanently until you change it

yourself.

Note that anytime during editing, you can transmit your voice to the

Keyboard without wiping out any of your USER Voices, because the voice goes

into an Edit Buffer and allows you to hear that voice until you put it into

USER MEMORY or select another voice with the '+1' or '-1' buttons or use the

NUMBER buttons to select another voice.

The last Menu is the 'Misc' Menu and is used to toggle the AUTO UPDATE

Mode ON or OFF. The current setting is shown to the Right hand side of the

Voice Name Parameter and is shown as either 'AUTO OFF' or 'AUTO ON'. It is

set to OFF at Start-Up. When this Mode is selected, the Voice you are

Editing will automatically be Sent to the Keyboard whenever you Exit Edit

Mode by Clicking on the OK Gadget. When this Mode is OFF, you must Send the

Voice to the Keyboard yourself with the 'Send Voice' Menu selection.

Finally, each Menu selection has a key Sequence assigned to it also, so

that you can use the keyboard instead of the pull-down Menus. They are :

Main:

Init Voice            -  Right Amiga  I  (Initialize)

Quit                  -  Right Amiga  Q  (Quit)

Load:

Load Voice from File  -  Right Amiga  L  (Load)

* Load Voice from DS55  -  Right Amiga  R  (Recieve)

Save:

* Send Voice to File    -  Right Amiga  S  (Save)

Send Voice to DS55    -  Right Amiga  T  (Transmit)

Misc:

Automatic Update      -  Right Amiga  A  (AUTO ON/OFF)

* Note that the Menu Items marked with an asterisk are not

available in the DEMO Version of VcEd.

(Hey, its only $10.00. I need a hard drive bad!!)

Notes

Some of the parameters, when clicked on, will Toggle, and will not need

the Edit Gadgets. This is because they only have two states or possible

values. These are : AME (0 or 1), Osc. Mode (Ratio or Fixed), Delay (On or

Off), Delay Short/Long (Short or Long), Port Mode (FULL T or Fingered),

Poly/Mono (Poly or Mono) (Gee, I bet you could-a guessed that last one), and

finally LFO Sync (On or Off). If the AUTO Update Mode is on, then toggled

parameters will still automatically update the voice to the Keyboard.

Make sure your Keyboard is on before trying to Recieve a voice from it,

or the program will send the Voice Edit Buffer Dump Request to Nothing and

sit and wait for ever!!!!!  If this happens, turn your keyboard on and

manually transmit a bulk dump to VcEd.  Your screen will appear to be a mass

of crap at this point but it won't be locked up anymore. Now select the INIT

VOICE Menu selection from the Main Menu. Now your set to do some editing

again.  Don't forget you have 4 Operators to program!!!!!

How to Order the Fully Functional VcEd

You can Order the fully functional Version of VcEd from me at :

Charles A. Brand

95 Parana Dr.

Newark, Ohio 43055

Phone : 614-745-5361

The Cost is $10.00 and you get the Source code written in fully commented

'C' language, a file that expalains the program line by line, and  a layout

of the buffer data and explanation of how it is implemented. I promise that

I have worked my butt off on this project and that it is worth every penny

of what I am asking. (Just ask me I'll tell you!)

Any donation of $15.00 or more, will also get a SysEx program and a Voice

Printout program, also with the 'C' Source Code for both.

The SysEx will save standard 32 Voice and 100 Voice Bulk Dumps to disk

and back to the Keyboard (DX and DS55).  This program will also do System

Set-up Dumps as well.

The PrintDump program will print out the Voice parameter data from bulk

dump files. You have the choice of a single voice or all 32 Voices of a

standard 32 voice Yamaha 4 Op bulk dump.

The Source for both of the extra programs is also fully commented.

Feel free to contact me with any questions, comments, or complaints and I

will try my best to change, enhance, or help anybody do the same to this

program. (Please do not call collect!)